# List of Appendices

Contents

3.1 A	Applicable Regulations

3.2 A 	Reporting

3.2 B 	CIF Inspection Data

3.2 C 	Historical Wait Times (3 years) and other Inspection Data

3.2 D	Video Monitoring System Specifications

3.2 E	Undercarriage Video Inspection System Specifications

3.2 F	Web Cam Snapshot View Specifications

3.2 G	Maintenance and Operations of Building and Grounds

3.2 H	Lifts, Brakes, and Air Compressors Specifications

3.2 J	State Audits Requirements

3.3 A	Mechanical Inspection Process

3.4 A 	OBD Hardware Specification

3.4 B	Legacy OBD Flow Process

3.4 C	OBD Exclusion List

3.4 D 	AAMVA Barcode Definitions

3.4 E	Vehicle Inspection Report

3.4 F	Rules Engine Communications and Logic Specifications

3.5 A	Legacy VID (Data and Table Volumes)

3.5 B	Legacy DEXML and XML Schema

3.6 A	BIT/MIT Vehicle Specifications and Diagrams

# Appendix 3.1A: Applicable Regulations

Code of Federal Regulations (CFR) Part 51, Subpart S (Inspection/Maintenance Program Requirements, 51.350 et seq.)

https://ecfr.io/Title-40/se40.2.51_1350

Code of Federal Regulations (CFR) Part 85 (Control of Air Pollution from Mobile Source)

https://ecfr.io/Title-40/cfr85_main

New Jersey Administrative Code, Title 7, Chapter 27, Air Pollution Control

7:27-14, Control and Prohibition of Air Pollution from Diesel-Powered Motor Vehicles https://www.nj.gov/dep/aqm/currentrules/Sub14.pdf

7:27-15 Control and Prohibition of Air Pollution from Gasoline-Fueled Motor Vehicles https://www.nj.gov/dep/aqm/currentrules/Sub15.pdf

7:27B-4 Air Test Method 4: Testing Procedures for Diesel-Powered Motor Vehicles https://www.state.nj.us/dep/aqm/27B4.pdf

7:27B-5 Air Test Method 5: Testing Procedures for Gasoline-Fueled Motor Vehicles https://www.state.nj.us/dep/aqm/27B5.pdf

New Jersey Administrative Code, Title 13, Chapter 20, Enforcement Service

http://www.lexisnexis.com/hottopics/njcode

# Appendix 3.2A: Reporting

Partial Report List

Emission Tests by Facility and Test Type

Heavy Duty Diesel Inspections

Failure Modes by Model Year (Gas)

Failure Modes by Model Year (Diesel)

PIF Inspector by Station

ERF No Match

Manual PIN Entries

Missing Re-exam - buses

Pass Rate Variance

High Pass Rate

High Fail Rate

Inspector Sign-On Statistics (visible to client)

Inspection Sequence Sticker Report

PASS Sticker Report

FAIL Sticker Report

Sticker Summary Report

Inspector Sign-On Statistics

Workstation Activity Report

Total Rejection Breakdown by Facility

Monthly PIF Rejection Breakdown

Diesel Testing by Facility and Test Type

Diesel Testing by Model Year and Test Type Diesel Testing by Inspection Type

Bus Inspections - Off Cycle

Diesel Fail rate/Volume by PIF

OBD Failure Mode MTD Synopsis (Gas)

OBD Failure Mode MTD Synopsis (Diesel)

Mechanical Defect Failure Mode MTD Synopsis

OBD Reset Report

High Volume of Self Repairs

OBD Retest Too Soon

Dirty Opacity

Initial Retest Too Soon

OBD Non-Comms

Inspector Inspections by Hour

ERF Suspended

Network Summary

OBD Failure Mode Synopsis (Gas)

OBD Failure Mode Synopsis (Diesel)

Bad Inspection Records

Continuous Monitors Status Report

Manual VIN Entered

Network Fail Rates by Test Type

OBD Bypass

OBD Fail rate by Protocol

OBD VIN Mismatch Opacity Bypass

PIF Enrollment / Activation Issues

Software and Reference Table Versions

Vehicle Information Changed

Workstation Lockouts

Workstation Transaction Summary

48 Hour Failure Count

Brakes Fail rate/Volume by PIF

Buses Missing a Scheduled Inspection

Buses Missing Re-inspection in 30 days

Bypass Monthly Report

Collector Car Report

Commercial Bus Inspections

Commercial Bus Inspections - Fiscal Year Report

Credentials Fail rate/Volume by PIF

DTCs by Frequency

Exhaust System Fail rate/Volume by PIF

Failed at St. A, Pass at St. B

Initial Pass PIF Mileage Errors

Misc. Safety Fail rate/Volume by PIF

Multiple Passes for same VIN

OBD Bypasses by Model Year

OBD Fail rate/Volume by PIF

OBD PIF Failure

Opacity PIF Failures

Opacity Testing by Facility and Test Type

Opacity Testing by Model Year and Test Type

Mechanical Defects and Emissions Results

Mechanical Defects Fail rate/Volume by PIF

School Bus Inspections

School Bus Inspections - Fiscal Year Report

School Bus Not Performed at BIT

Steering and Suspension Fail rate/Volume by PIF Test Details

Total Handlings PIF

VIN Character Count

Visual Emissions Fail rate/Volume by PIF

PIF End of Day Report

PIF Inspection Detail Report

PIF Sticker Usage Report

Monthly QA Reports:

The reports that need to be sent to the State monthly would be but not limited to:

    - Administrative Audits

    - Inspector Performance Audits

    - Equipment Audits

    - Call Center Stats Report

    - Equipment Service Report

Each report needs to contain the following elements:

    - Administrative Audit:

      - Date of Audit

      - Station Details:

        - License # (Could be multiple)

        - Station Name

        - Trade Name

        - Address

        - Contact Person

        - Phone Number

        - Software Version

        - Class

      - Workstation ID

      - Auditor Details:

        - Auditor Name

        - Auditor ID #

        - Route #

      - Date of Last Audit

      - Sticker Audit Results

      - Inspector Audit Results

      - Invoice Audit Results

      - Administrative Action/Corrective Measures Taken

    - Inspector Performance Audit:

      - Date of Audit

      - Station Details:

        - License # (Could be multiple)

        - Station Name

        - Trade Name

        - Address

        - Contact Person

        - Phone Number

        - Software Version

        - Class

      - Workstation ID

      - Auditor Details:

        - Auditor Name

        - Auditor ID #

        - Route #

      - Date of Last Audit

      - Results - Pass/Fail

      - Administrative Action/Corrective Measures Taken

    - Equipment Audits:

      - Date of Audit

      - Station Details:

        - License # (Could be multiple)

        - Station Name

        - Trade Name

        - Address

        - Contact Person

        - Phone Number

        - Software Version

        - Class

      - Workstation ID

      - Auditor Details:

        - Auditor Name

        - Auditor ID #

        - Route #

      - Date of Last Audit

      - Results:

        - Pass /Fail Initial Try

        - Pass/Fail 2nd Try

      - Administrative Action/Corrective Measures Taken

    - Call Center Report:

      - Call Description/Type

      - Number of Calls – Sum

      - Minutes of Calls - Sum

      - Hours of Calls - Sum

    - Equipment Service Report

      - Date of Call

      - Time of Call

      - Request ID#

      - Name of Call Center Contact

      - Station Details:

        - License # (Could be multiple)

        - Station Name

        - Trade Name

        - Address

        - Contact Person

        - Phone Number

        - Software Version

        - Class

      - Workstation ID

      - Auditor Details:

        - Auditor Name

        - Auditor ID #

        - Route #

      - Call Description

      - Resolution

        - Parts Used/Replaced

        - Warranty Issue

        - Service Performed

      - Call Request Closed Date

      - Call Request Closed Time

      - Call Request Closed by

      - Business Days to Close Request

      - SLA Missed

      - Comments

| MVC REPORT LIST |  |  |  |
| --- | --- | --- | --- |
|  | Report Type | Carry |  |
| Bus Inspections - Off Cycle | M | Y | Bus Inspections - Off Cycle |
| Buses Missing a Scheduled Inspection | M | Y | Buses Missing a Scheduled Inspection |
| Buses Missing Re-inspection in 30 days | M | Y | Buses Missing Re-inspection in 30 days |
| Commercial Bus Inspections | M | Y | Commercial Bus Inspections |
| Commercial Bus Inspections - Fiscal Year | M | Y | Commercial Bus Inspections - Fiscal Year Report |
| ERF Suspended | M | Y | ERF Suspended |
| Missing Re-exam - buses | Q | Y | Missing Re-exam - buses |
| OBD Bypass | T | Y | OBD Bypass |
| PIF End of Day Report | M | Y | PIF End of Day Report |
| PIF Enrollment / Activation Issues | Q | Y | PIF Enrollment / Activation Issues |
| PIF Inspection Detail Report | M | Y | PIF Inspection Detail Report |
| PIF Sticker Usage Report | M | Y | PIF Sticker Usage Report |
| Safety and Emissions Results | Q | Y | Safety and Emissions Results |
| School Bus Inspections | M | Y | School Bus Inspections |
| School Bus Inspections - Fiscal Year Report | M | Y | School Bus Inspections - Fiscal Year Report |
| School Bus Not Performed at BIT | T | Y | School Bus Not Performed at BIT |
| Workstation Activity Report | M | Y | Workstation Activity Report |
| Workstation Transaction Summary | T | Y | Workstation Transaction Summary |

NJMVIS Call Summary Report 		Report Period: December, 2023

| Call Description | Number of Calls | Minutes | Hours |
| --- | --- | --- | --- |
| 5-Year Exemption | 67 | 528.5 | 8.8 |
| Appointment Request | 114 | 1114.0 | 18.6 |
| Caller Hung-Up | 84 | 526.0 | 8.8 |
| Command Center | 18 | 160.2 | 2.7 |
| Complaint - Customer Service | 1 | 0.3 | 0.0 |
| Complaint - Inspection Policy | 2 | 114.3 | 1.9 |
| Complaint - Vehicle Damage | 7 | 82.4 | 1.4 |
| General Wrong Number | 11 | 24.7 | 0.4 |
| Inquiry - Credentials | 93 | 761.5 | 12.7 |
| Inquiry - Directions | 78 | 757.4 | 12.6 |
| Inquiry - Emissions | 44 | 669.5 | 11.2 |
| Inquiry- Facility Wait Time | 183 | 1146.3 | 19.1 |
| Inquiry - Hours of Operation | 78 | 498.9 | 8.3 |
| Inquiry - Inspection Procedures | 337 | 1481.5 | 24.7 |
| Inquiry- Lost & Found | 8 | 82.5 | 1.4 |
| Inquiry - PIF VIIS | 13 | 74.1 | 1.2 |
| Inquiry- PIF/ERF | 5 | 11.4 | 0.2 |
| Inquiry - Redir. To State Svcs. | 24 | 251.2 | 4.2 |
| Internal Call Transfer | 18 | 234.4 | 3.9 |
| Plate Return | 1 | 33.9 | 0.6 |
| Referred to MYS | 221 | 1537.1 | 25.6 |
| Referred to Specialty Site | 12 | 191.3 | 3.2 |
| Station Call | 7 | 60.7 | 1.0 |
| Totals: | 1426 | 10342.1 | 172.4 |
| Appointments | 4225 |  |  |
| Grand Total: | 5651 |  |  |
| Average answer delay: | 00:00:06 |  |  |

Tuesday, January 2, 2024										Page 1 of 1

NJMVIS Call Summary Report 		Report Period: November, 2023

| Call Description | Number of Calls | Minutes | Hours |
| --- | --- | --- | --- |
| 5-Year Exemption | 70 | 407.7 | 6.8 |
| Appointment Request | 102 | 407.6 | 6.8 |
| Caller Hung-Up | 111 | 315.7 | 5.3 |
| Command Center | 26 | 122.6 | 2.0 |
| Complaint - Customer Service | 1 | 0.9 | 0.0 |
| Complaint - Hours of Operation | 1 | 0.3 | 0.0 |
| Complaint - Inspection Policy | 2 | 13.2 | 0.2 |
| Complaint - Lost Credentials | 3 | 10.1 | 0.2 |
| Complaint - Vehicle Damage | 3 | 2.7 | 0.0 |
| General Wrong Number | 11 | 23.1 | 0.4 |
| Inquiry - Credentials | 73 | 391.5 | 6.5 |
| Inquiry - Directions | 75 | 491.7 | 8.2 |
| Inquiry - Emissions | 48 | 491.5 | 8.2 |
| Inquiry - Facility Wait Time | 190 | 951.0 | 15.8 |
| Inquiry - Hours of Operation | 155 | 677.8 | 11.3 |
| Inquiry - Inspection Procedures | 406 | 1831.9 | 30.5 |
| Inquiry - Lost & Found | 12 | 67.7 | 1.1 |
| Inquiry - PIF VIIS | 12 | 66.8 | 1.1 |
| Inquiry - PIF/ERF | 11 | 118.8 | 2.0 |
| Inquiry - Redir. To State Svcs. | 20 | 107.5 | 1.8 |
| Internal Call Transfer | 16 | 209.5 | 3.5 |
| Plate Return | 2 | 0.5 | 0.0 |
| Referred to MVS | 239 | 992.3 | 16.5 |
| Referred to Specialty Site | 17 | 263.9 | 4.4 |
| Station Call | 11 | 10.6 | 0.2 |
| Totals: | 1617 | 7977.0 | 133.0 |
| Appointments: | 4833 |  |  |
| Grand Total: | 6450 |  |  |
| Average answer delay: | 00:00:06 · |  |  |

Friday, December I, 2023										Page 1 of 1

NJMVIS Call Summary Report 		Report Period: October, 2023

| Call Description | Number of Calls | Minutes | Hours |
| --- | --- | --- | --- |
| 5-Year Exemption | 112 | 540.1 | 9.0 |
| Appointment Request | 126 | 1069.4 | 17.8 |
| Caller Hung-Up | 117 | 549.7 | 9.2 |
| Command Center | 28 | 103.6 | 1.7 |
| Complaint - Customer Service | 1 | 10.1 | 0.2 |
| Complaint - Inspection Policy | 3 | 56.0 | 0.9 |
| Complaint - Lost Credentials | 2 | 4.9 | 0.1 |
| Complaint- Vehicle Damage | 7 | 26.7 | 0.4 |
| General Wrong Number | 11 | 15.7 | 0.3 |
| Inquiry - Credentials | 110 | 630.0 | 10.5 |
| Inquiry - Directions | 102 | 599.7 | 10.0 |
| Inquiry - Emissions | 44 | 430.6 | 7.2 |
| Inquiry - Facility Wait Time | 194 | 1089.7 | 18.2 |
| Inquiry - Hours of Operation | 66 | 465.5 | 7.8 |
| Inquiry - Inspection Procedures | 437 | 2090.3 | 34.8 |
| Inquiry - Lost & Found | 12 | 154.4 | 2.6 |
| Inquiry - PIF VIIS | 15 | 156.1 | 2.6 |
| Inquiry - PIF/ERF | 15 | 82.2 | 1.4 |
| Inquiry - Redir. To State Svcs. | 19 | 261.0 | 4.3 |
| Internal Call Transfer | 15 | 71.3 | 1.2 |
| Plate Return | 4 | 2.3 | 0.0 |
| Referred to MVS | 238 | 1045.7 | 17.4 |
| Referred to Specialty Site | 19 | 69.0 | l.2 |
| Station Call | 16 | 9.8 | 0.2 |
| Totals: | 1713 | 9533.7 | 158.9 |

Appointments			5102

Grand Total:			6815

Average answer delay:		00:00:06

Wednesday, November 1, 2023									Page 1 of 1

NJMVIS Call Summary Report 		Report Period: September 2023

| Call Description | Number of Calls | Minutes | Hours |
| --- | --- | --- | --- |
| 5-Year Exemption | 102 | 608.9 | 10.1 |
| Appointment Request | 135 | 488.9 | 8.1 |
| Caller Hung-Up | 126 | 548.8 | 9.1 |
| Command Center | 22 | 128.6 | 2.1 |
| Complaint - Inspection Policy | 2 | 41.5 | 0.7 |
| Complaint - Lost Credentials | 1 | 4.5 | 0.1 |
| Complaint - Vehicle Damage | 4 | 17.6 | 0.3 |
| Compliment | 1 | 7.3 | 0.1 |
| General Wrong Number | 21 | 150.7 | 2.5 |
| Inquiry - Credentials | 105 | 788.8 | 13.1 |
| Inquiry - Directions | 104 | 698.7 | 11.6 |
| Inquiry - Emissions | 50 | 410.4 | 6.8 |
| Inquiry - Facility Wait Time | 217 | 1190.9 | 19.8 |
| Inquiry - Hours of Operation | 98 | 344.7 | 5.7 |
| Inquiry - Inspection Procedures | 454 | 2340.5 | 39.0 |
| Inquiry - Lost & Found | 5 | 26.6 | 0.4 |
| Inquiry - PIF VIIS | 28 | 146.8 | 2.4 |
| Inquiry - PIF/ERF | 4 | 6.7 | 0.1 |
| Inquiry - Redir. To State Svcs. | 14 | 198.9 | 3.3 |
| Internal Call Transfer | 8 | 36.5 | 0.6 |
| Plate Return | 7 | 2.0 | 0.0 |
| Referred to MYS | 253 | 978.0 | 16.3 |
| Referred to Specialty Site | 10 | 7.8 | 0.1 |
| Station Call | 9 | 5.0 | 0.1 |
| Totals: | 1780 | 9179.0 | 153.0 |
| Appointments | 5061 |  |  |
| Grand Total: | 6841 |  |  |
| Average answer delay: | 00:00:06 |  |  |

Monday, October 2, 2023										Page 1 of 1

NJMVIS Call Summary Report 		Report Period: August 2023

| Call Description | Number of Calls | Minutes | Hours |
| --- | --- | --- | --- |
| 5-Year Exemption | 114 | 743.6 | 12.4 |
| Appointment Request | 139 | 542.5 | 9.0 |
| Caller Hung-Up | 132 | 542.1 | 9.0 |
| Command Center | 25 | 36.7 | 0.6 |
| Complaint - Customer Service | 2 | 10.2 | 0.2 |
| Complaint - Inspection Policy | 2 | 22.4 | 0.4 |
| Complaint - Lost Credentials | 1 | 18.0 | 0.3 |
| Complaint - Vehicle Damage | 8 | 27.5 | 0.5 |
| General Wrong Number | 35 | 139.1 | 2.3 |
| Inquiry - Credentials | 152 | 806.5 | 13.4 |
| Inquiry - Directions | 111 | 645.5 | 10.8 |
| Inquiry - Emissions | 51 | 587.4 | 9.8 |
| Inquiry - Facility Wait Time | 269 | 1645.7 | 27.4 |
| Inquiry - Hours of Operation | 91 | 547.1 | 9.1 |
| Inquiry - Inspection Procedures | 559 | 2923.5 | 48.7 |
| Inquiry - Lost & Found | 6 | 10.9 | 0.2 |
| Inquiry - PIF VIIS | 19 | 125.2 | 2.1 |
| Inquiry - PIF/ERF | 20 | 200.4 | 3.3 |
| Inquiry - Redir. To State Svcs. | 22 | 147.9 | 2.5 |
| Internal Call Transfer | 17 | 78.8 | 1.3 |
| Plate Return | 3 | 0.8 | 0.0 |
| Referred to MYS | 283 | 1242.1 | 20.7 |
| Referred to Specialty Site | 8 | 5.9 | 0.1 |
| Station Call | 6 | 2.9 | 0.0 |
| Totals: | 2075 | 11052.8 | 184.2 |
| Appointments: | 6073 |  |  |
| Grand Total: | 8148 |  |  |
| Average answer delay: | 00:00:06 |  |  |

Friday, September 1, 2023										Page 1 of 1

NJMVIS Call Summary Report 		Report Period: July 2023

| Call Description | Number of Calls | Minutes | Hours |
| --- | --- | --- | --- |
| 5-Year Exemption | 86 | 362.8 | 6.0 |
| Appointment Request | 124 | 412.4 | 6.9 |
| Caller Hung-Up | 119 | 244.4 | 4.1 |
| Command Center | 26 | 217.5 | 3.6 |
| Complaint - Customer Service | 1 | 11.3 | 0.2 |
| Complaint - Inspection Policy | 1 | 10.5 | 0.2 |
| Complaint - Lost Credentials | 5 | 13.4 | 0.2 |
| Complaint - Vehicle Damage | 1 | 5.0 | 0.1 |
| General Wrong Number | 22 | 66.3 | 1.1 |
| Inquiry - Credentials | 134 | 324.9 | 5.4 |
| Inquiry - Directions | 114 | 450.0 | 7.5 |
| Inquiry - Emissions | 29 | 181.4 | 3.0 |
| Inquiry - Facility Wait Time | 336 | 1003.3 | 16.7 |
| Inquiry -Hours of Operation | 102 | 306.6 | 5.1 |
| Inquiry - Inspection Procedures | 508 | 1851.4 | 30.9 |
| Inquiry - Lost & Found | 6 | 22.4 | 0.4 |
| Inquiry - PIF VIIS | 18 | 12.5 | 0.2 |
| Inquiry - PIF/ERF | 13 | 36.4 | 0.6 |
| Inquiry - Redir. To State Svcs. | 15 | 44.3 | 0.7 |
| Internal Call Transfer | 18 | 55.8 | 0.9 |
| Plate Return | 7 | 4.4 | 0.1 |
| Referred to MVS | 280 | 675.3 | 11.3 |
| Referred to Specialty Site | 13 | 11.3 | 0.2 |
| Station Call | 36 | 53.5 | 0.9 |
| Totals: | 2014 | 6377.1 | 106.3 |
| Appointments: | 5955 |  |  |
| Grand Total: | 7969 |  |  |
| Average answer delay: | 00:00:07 |  |  |

Tuesday, August 1, 2023									Page 1 of 1

# Appendix 3.2B: CIF Inspection Data

| Station Name | Appointment Required | No. of Lanes | Year Built | Address | Wait Time Gate/Tickets | Exempt From Wait Time | MDDV Smoke Opacity Testing |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Central Inspection Facilities |  |  |  |  |  |  |  |
| Bakers Basin | NO | 5 | 1957 | 3200 Brunswick Pike, Lawrenceville, NJ 08648 | YES | NO | YES |
| Cape May | YES | 1 | 1938 | 6 West Shelby Ave, Cape May Court House, NJ 08210 | NO | YES | YES |
| Cherry Hill | NO | 5 | 1957 | 617 Hampton Rd, Cherry Hill, NJ 08002 | YES | NO | YES |
| Deptford | NO | 4 | 1972 | 725 Tanyard Rd, Deptford, NJ 08075 | YES | NO | YES |
| Eatontown | NO | 4 | 1958 | 109 Route 36, Eatontown, NJ 07724 | YES | NO | YES |
| Flemington | NO | 3 | 1978 | 181 Route 31-202, Ringoes, NJ 08551 | YES | NO | YES |
| Freehold | NO | 6 | 2000 | 801 Okerson Rd, Freehold, NJ 07728 | YES | NO | YES |
| Kilmer | NO | 6 | 1975 | 33 Kilmer Rd, Edison, NJ 08817 | YES | NO | YES |
| Lakewood | NO | 6 | 2000 | 1145 Route 70, Lakewood, NJ 08701 | YES | NO | YES |
| Lodi | NO | 4 | 1959 | 8 Mill Street, Lodi, NJ 07644 | YES | NO | YES |
| Manahawkin | NO | 3 | 1972 | 220 Recovery Rd, Manahawkin, 08050 | YES | NO | YES |
| Mays Landing | NO | 4 | 1991 | 1477 19th Street, Mays Landing, NJ 08330 | YES | NO | YES |
| Millville | NO | 2 | 1938 | 1406 Wheaton Ave, Millville, NJ 08332 | NO | YES | NO |
| Newton | NO | 2 | 1938 | 90 Moran Street, Newton, NJ 07860 | YES | NO | NO |
| Paramus | NO | 5 | 1967 | 20 West Century Rd, Paramus, NJ 07652 | YES | NO | YES |
| Rahway | NO | 6 | 1957 | 1140 Woodbridge Rd & East Hazel Wood Ave, Rahway, NJ 07065 | YES | NO | YES |
| Randolph | NO | 6 | 1987 | 160 Canfield Ave, Randolph, NJ 07869 | YES | NO | YES |
| South Brunswick | NO | 6 | 1990 | 2236 Route 130, Dayton, NJ 08810 | YES | NO | YES |
| Salem | YES | 1 | 1938 | 185 Woodstown Rd, Salem, NJ 08079 | NO | YES | YES |
| Secaucus | NO | 4 | 1963 | 360 County Ave & Secaucus Rd. 07094 | YES | NO | YES |
| Southampton | NO | 4 | 1989 | 1875 Route 38, Southampton, NJ 08088 | YES | NO | YES |
| Washington | YES | 1 | 1938 | Rt. 31 North & Pershing Ave, Washington, NJ 07882 | NO | YES | YES |
| Wayne | NO | 5 | 1958 | 481 Route 46 West, Wayne, NJ 07470 | YES | NO | YES |
| Winslow | NO | 3 | 2000 | 550 Spring Garden Rd, Ancora, NJ 08037 | YES | NO | YES |
| Specialty Inspection Facilities |  |  |  |  |  |  |  |
| Eatontown | YES | 1 | 1958 | 109 Rte. 36W, Eatontown 07724 | NO | N/A | YES |
| Toms River | YES | 1 | 1938 | 4 James St, Toms River, NJ 08753 | NO | N/A | YES |
| Secaucus | YES | 1 | 1963 | 360 County Ave & Secaucus Rd. 07094 | NO | N/A | YES |
| Winslow | YES | 1 | 1991 | 550 Spring Garden Rd, Ancora, NJ 08037 | NO | N/A | YES |

Operations Chart Average Number of Lanes

|  | 2022 |  | 2023 |  | 2024 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Week Day | Saturday | Week Day | Saturday | Week Day | Saturday |
| January | 72 | 74 | 78 | 75 | 80 | 78 |
| February | 80 | 81 | 78 | 73 | 84 | 63 |
| March | 83 | 82 | 80 | 77 | 84 | 84 |
| April | 83 | 81 | 83 | 81 | 85 | 84 |
| May | 84 | 79 | 84 | 79 | 87 | 84 |
| June | 85 | 79 | 85 | 80 | 85 | 86 |
| July | 84 | 81 | 85 | 83 | 87 | 85 |
| August | 84 | 79 | 87 | 83 | 87 | 86 |
| September | 80 | 77 | 87 | 83 | 89 | 85 |
| October | 82 | 78 | 87 | 83 | 88 | 86 |
| November | 79 | 76 | 86 | 83 | 89 | 88 |
| December | 75 | 62 | 84 | 81 | 94 | 85 |
| AVERAGE | 81 | 77 | 84 | 80 | 86 | 83 |

# Appendix 3.2C: Historical Wait Time Violations (3 Years)

Table A - Calculated Number of Monthly Average Wait Time Violations for the Past Three Years

|  | 2022 |  |  |  |  |  |  |  |  |  |  |  | 2023 |  |  |  |  |  |  |  |  |  |  |  | 2024 |  |  |  |  |  |  |  |  |  |  |  | 3-Year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Facility | J | F | M | A | M | J | J | A | S | O | N | D | J | F | M | A | M | J | J | A | S | O | N | D | J | F | M | A | M | J | J | A | S | O | N | D | Total |
| Deptford |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 | 1 |  |  |  |  | 4 |
| Bakers Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Eatontown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  | 1 |
| Flemington |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kilmer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rahway |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| South Brunswick |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cherry Hill |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  |  | 2 |
| Manahawkin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mays Landing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |
| Southampton |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  | 1 |
| Winslow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lodi |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Newark |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| Newton |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Paramus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Randolph |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Secaucus |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wayne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  | 1 |
| Lakewood |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  | 1 |
| Freehold |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 3 | 6 | 0 | 0 | 0 | 0 |  |

Other State Inspections

| Number of Inspections |  |  |  |
| --- | --- | --- | --- |
| Type of Inspection | 2022 | 2023 | 2024 |
| Diesel PIF | 62,823 | 63,046 | 51,217 |
| School Bus | 65,981 | 69,388 | 70,949 |
| Heavy Duty Diesel Roadside | 2,094 | 4,207 | 3,220 |
| Commercial Bus in Terminal | 11,123 | 10,733 | 10,421 |
| Mobile Inspection Team | 13,917 | 15,088 | 11,247 |

Number of PIF Stations by Number of Inspections Performed

| Month | Number of Inspections in 2022 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0-20 | 21-50 | 51-100 | >100 |
|  | Number of PIF Stations |  |  |  |
| January | 496 | 258 | 56 | 7 |
| February | 442 | 299 | 63 | 13 |
| March | 364 | 326 | 105 | 23 |
| April | 380 | 320 | 92 | 17 |
| May | 383 | 303 | 101 | 21 |
| June | 339 | 328 | 118 | 24 |
| July | 370 | 293 | 108 | 18 |
| August | 337 | 305 | 124 | 31 |
| September | 358 | 324 | 92 | 23 |
| October | 384 | 308 | 78 | 21 |
| November | 442 | 269 | 66 | 16 |
| December | 448 | 261 | 70 | 11 |

| Month | Number of Inspections in 2023 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0-20 | 21-50 | 51-100 | >100 |
|  | Number of PIF Stations |  |  |  |
| January | 407 | 278 | 82 | 16 |
| February | 422 | 280 | 71 | 11 |
| March | 360 | 316 | 95 | 23 |
| April | 371 | 322 | 74 | 22 |
| May | 339 | 308 | 106 | 27 |
| June | 321 | 325 | 101 | 33 |
| July | 388 | 282 | 80 | 27 |
| August | 338 | 304 | 108 | 27 |
| September | 391 | 282 | 78 | 19 |
| October | 370 | 286 | 95 | 21 |
| November | 398 | 274 | 77 | 14 |
| December | 425 | 245 | 80 | 11 |

| Month | Number of Inspections in 2024 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0-20 | 21-50 | 51-100 | >100 |
|  | Number of PIF Stations |  |  |  |
| January | 429 | 253 | 73 | 11 |
| February | 374 | 297 | 70 | 15 |
| March | 357 | 294 | 89 | 21 |
| April | 366 | 301 | 83 | 22 |
| May | 336 | 304 | 102 | 22 |
| June | 337 | 298 | 102 | 19 |
| July | 326 | 288 | 100 | 24 |
| August | 326 | 291 | 105 | 26 |
| September | 355 | 289 | 82 | 17 |
| October | 339 | 285 | 91 | 27 |
| November | 408 | 250 | 67 | 13 |
| December | 433 | 226 | 63 | 12 |

Vehicle Population by Model Year (As of Dec 2023)

| Model Year | # of Registered Vehicles | Model Year | # of Registered Vehicles | Model Year | # of Registered Vehicles |
| --- | --- | --- | --- | --- | --- |
| 2025 | 1,351 | 1979 | 2,703 | 1933 | 166 |
| 2024 | 265,930 | 1978 | 2,028 | 1932 | 353 |
| 2023 | 444,340 | 1977 | 1,671 | 1931 | 505 |
| 2022 | 380,964 | 1976 | 1,638 | 1930 | 537 |
| 2021 | 408,088 | 1975 | 1,229 | 1929 | 359 |
| 2020 | 384,303 | 1974 | 1,857 | 1928 | 144 |
| 2019 | 434,073 | 1973 | 2,229 | 1927 | 75 |
| 2018 | 381,673 | 1972 | 2,856 | 1926 | 89 |
| 2017 | 349,728 | 1971 | 2,495 | 1925 | 30 |
| 2016 | 320,612 | 1970 | 3,255 | 1924 | 54 |
| 2015 | 313,714 | 1969 | 3,762 | 1923 | 123 |
| 2014 | 271,522 | 1968 | 3,079 | 1922 | 43 |
| 2013 | 264,191 | 1967 | 3,216 | 1921 | 23 |
| 2012 | 233,501 | 1966 | 3,143 | 1920 | 19 |
| 2011 | 205,876 | 1965 | 2,690 | 1919 | 10 |
| 2010 | 187,661 | 1964 | 1,593 | 1918 | 10 |
| 2009 | 147,283 | 1963 | 1,214 | 1917 | 12 |
| 2008 | 178,278 | 1962 | 885 | 1916 | 11 |
| 2007 | 163,353 | 1961 | 597 | 1915 | 21 |
| 2006 | 139,535 | 1960 | 558 | 1914 | 13 |
| 2005 | 126,316 | 1959 | 626 | 1913 | 23 |
| 2004 | 112,402 | 1958 | 464 | 1912 | 15 |
| 2003 | 92,499 | 1957 | 1,173 | 1911 | 16 |
| 2002 | 70,233 | 1956 | 869 | 1910 | 14 |
| 2001 | 51,544 | 1955 | 1,154 | 1909 | 14 |
| 2000 | 43,754 | 1954 | 449 | 1908 | 1 |
| 1999 | 31,683 | 1953 | 535 | 1907 | 5 |
| 1998 | 23,413 | 1952 | 425 | 1906 | 5 |
| 1997 | 19,663 | 1951 | 499 | 1905 | 5 |
| 1996 | 13,154 | 1950 | 560 | 1904 | 2 |
| 1995 | 13,181 | 1949 | 376 | 1903 | 4 |
| 1994 | 9,884 | 1948 | 424 | 1902 | 2 |
| 1993 | 7,585 | 1947 | 284 | 1901 | 2 |
| 1992 | 5,742 | 1946 | 253 | 1900 | 2 |
| 1991 | 5,618 | 1945 | 44 | 1899 | 2 |
| 1990 | 5,256 | 1944 | 38 |  |  |
| 1989 | 5,188 | 1943 | 29 |  |  |
| 1988 | 4,814 | 1942 | 126 |  |  |
| 1987 | 5,498 | 1941 | 299 |  |  |
| 1986 | 4,364 | 1940 | 388 |  |  |
| 1985 | 3,455 | 1939 | 219 |  |  |
| 1984 | 2,660 | 1938 | 159 |  |  |
| 1983 | 1,639 | 1937 | 293 |  |  |
| 1982 | 1,452 | 1936 | 235 |  |  |
| 1981 | 1,584 | 1935 | 130 |  |  |
| 1980 | 1,623 | 1934 | 258 |  |  |

| Total Number of Vehicles by Model Year |  |
| --- | --- |
| Pre-81 | 57,339 |
| 81-95 | 77,920 |
| 96+up | 6,059,286 |
| Total # of Vehicles | 6,194,545 |
| OBD% | 98 |

# Appendix 3.2D: Video Security System Specifications

SECURITY CAMERAS

The cameras shall be an all-in-one vandal/tamper resistant  8K or better resolution color dome camera with motorized zoom lens designed for indoor/outdoor applications providing multiple H.265 (MPEG4, Part 10) and simultaneous MJPEG streams that can be individually configured. The dome camera shall be designed to provide 30fps with or without audio and shall conform to the ONVIF and PSIA standards. The camera shall have no less than 3840(H)×2160(V) pixels. The camera shall be Power over Ethernet (IEEE802.3af Class1) compliant for indoor/outdoor applications.

The camera shall offer Day/Night functionality with high sensitivity and color infrared capability for use in low light and indoor/outdoor applications and shall be prepackaged with an autofocus or varifocal IR corrected lens to allow zoom and focus adjustment. The camera shall be constructed of cast-aluminum housing with polycarbonate dome bubble and auto tracking inner liner. The dome camera shall be designed to protect against water ,dust to IP67 / NEMA 4 and impact rated IK10 standards or better. The camera shall be designed to be surface mounted to a wall or ceiling and wall mounted with optional accessories. The camera shall provide automatic white balance, automatic exposure, gain control, electronic shutter, and backlight compensation. The camera shall provide user configurable saturation, contrast, brightness, and sharpness controls from the web-interface. The camera shall provide an automated IR filter that will automatically switch from color to monochrome enhancing low lighting with IR illumination. The camera shall require less than 0.007 Lux @ F1.5(0.007 fc) to produce a monochrome image when in night mode and 0.0007 Lux@F1.5

The cameras shall be networked to a recording system (DVR) for recording in real time and event recording. The camera shall support DTS (Direct-To-Storage) for recording of real time and event recording using network attached storage. The camera shall have an event handler allowing the camera to send an image or video clip to on-camera storage, FTP site, email address, and or network attached storage when receiving an internally or externally generated event. Storage of video shall be at a minimum of 365 days before data write over. Video shall be accessible via a web service available to the State at configurable access hours based on weekdays/weekends. The DVR shall be capable of producing video images to a Flash Drive or external hard drive.

NETWORKING REQUIREMENTS

The security system shall incorporate a web server allowing for the viewing of the camera only. The camera shall be Power over Ethernet (IEEE802.3af Class1) compliant or (IEEE802.3af Class 2) compliant. The camera shall have a 100 Mbps or greater Ethernet RJ-45 connection that can deliver data at a maximum rate of 35 Mbps or greater. The camera shall have RTSP (Real Time Streaming Protocol) support for compatibility with media players such as VLC Media Player, Apple QuickTime and Windows Media Player. The camera shall deliver 8.0-megapixel quality video or better at a frame rate of 30fps via TCP/IP over CAT5 or CAT6 cable. The camera shall not require unique or proprietary client software for viewing or controlling the camera. The camera shall be available to State permissioned users

running versions ofMicrosoft Edge™, Firefox™, Chrome™, or Mozilla™. The camera shall provide secure user and administrator password protection.

# Appendix 3.2E: Undercarriage Video Inspection System Specifications

SPECIFICATIONS FOR UNDERCARRIAGE VIDEO INSPECTION SYSTEM

Inspection Camera

The color cameras shall have 8K or better resolution, 0.02 fc (0.2 lux) sensitivity..

The camera position shall have a four-direction adjustment, allowing for adjustment of pan, tilt, and vertical height in the dome and lens rotation (roll), for any angle of view required.

These cameras are not required to be networked or able to record, but may be configured to do so by the NGSystem Contractor {Vendor}.

Undercarriage Housing

The cameras housing shall be mounted in-ground and flush with the ground. The housing shall be a rugged camera enclosure offering weather, weight, and vehicle drive-over protection to video camera/lens combinations. The unit shall be available with a screening material that shall provide protection against UV radiation and corrosion. Housing shall also include equipment to prevent freeze-up in subfreezing temperatures as well as to prevent moisture and condensation from accumulation in hot, humid temperatures. The housing shall be constructed of rust-proof material and the window shall be Lexan polycarbonate. Waterproof seals and gaskets shall provide protection to the camera/lens assembly. The undercarriage housing shall provide adequate back-lighting for the camera system to allow the inspection and identification of vehicle undercarriage components (e.g. catalytic converters).

Monitor

A monitor shall be provided for each undercarriage inspection camera. The monitor shall be a flat panel monitor and of an adequate size that allows Lane Technicians to clearly view as the vehicle being inspected is driven over the undercarriage camera system. flat panel monitor. The resolution must be greater than 1024x768 with a minimum of 24-bit true color. Digital connector shall be available. If required, any housing shall also include equipment to prevent freeze-up in sub-freezing temperatures as well as to prevent moisture and condensation from accumulating in hot, humid temperatures. Mounting of monitor depends upon facility structure.

Current System

The Contractor {Vendor} shall use and maintain the current Undercarriage Video Inspection System that is currently installed at the Centralized Facilities until the Contractor{Vendor} replaces the system as required by the Contract. The current system is a Smith & Wesson Swift UVIS. The system is an all-weather in-ground system. The system contains one camera covering 180 degrees angles and is elucidated with two wide angle long life illuminators low light conditions.

# Appendix 3.2F: Web Cam Snapshot View Specifications

SPECIFICATIONS FOR QUEUE WEB CAM VIEW

Camera specifications described within Appendix 3.2E: Video Security System Specifications shall apply to the Queue Camera Public Viewing System.

# Appendix 3.2G: Maintenance and Operations of Buildings and Grounds

|  |  |  |  |  |  | Appendix F - Facility Buildings, Utilities and Grounds |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Table F-1 State and Contractor Responsibilities |  |  |  |  |  |  |  |  |  |  |
|  | Station Name | Acres | Building Square Footage | Operation | Shared Building | Building related Routine Maintenance Janitorial & Repair | Gas/ Oil | Water | Well Maintenance | Sewer | Septic System Maintenance | Trash Removal | Electric | Phone | Grounds | Internet Service Connectivity |
|  | Bakers Basin | 12.53 | 23128 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Bakers Basin |  |  | Driver testing Field House |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
|  | Bakers Basin |  |  | MVC Agency |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
|  | Bakers Basin |  |  | On site Driver Road Test Course |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
|  | Cape May | 0.46 | 4890 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Cherry Hill | 9.00 | 22905 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Cherry Hill |  |  | Driver Road Test |  | State | State | Contractor |  | Contractor |  | Contractor | State | state | Contractor | State |
|  | Cherry Hill |  |  | Driver Testing Office- Trailer |  | State | State | Contractor |  | Contractor |  | Contractor | State | state | Contractor | State |
|  | Cherry Hill |  |  | Future Driver testing office |  | State | State | Contractor |  | Contractor |  | Contractor | State | state | Contractor | State |
|  | Deptford | 20.00 | 18785 | Inspection Station |  | Contractor | Contractor |  | Contractor | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Eatontown | 13.05 | 20671 | Inspection Station | Yes | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Eatontown |  |  | MVC Agency | Yes | State | State | Contractor |  | Contractor |  | Contractor | state | state | Contractor | State |
|  | Eatontown |  |  | Regional Service Center | Yes | State | State | Contractor |  | Contractor |  | Contractor | state | state | Contractor | State |
|  | Eatontown |  |  | Driver testing Field House |  | State | State | State |  | Contractor |  | Contractor | state | state | Contractor | State |
|  | Eatontown |  |  | On site Driver Road Test Course |  | State | State | State |  | Contractor |  | Contractor | state | state | Contractor | State |
|  | Eatontown |  |  | Specialty Inspection Facility |  | Contractor | State | State |  | Contractor |  | Contractor | State | State | Contractor | Contractor |
|  | Flemington | 4.50 | 11690 | Inspection Station |  | Contractor | Contractor |  | State |  | State | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Flemington |  |  | MVC Agency |  | State | State |  | State |  | State | state | state | state | Contractor | State |
|  | Freehold | 14.80 | 23490 | Inspection Station |  | Contractor | Contractor | Contractor |  |  | State | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Freehold |  |  | MVC Agency |  | State | State | State |  |  | State | state | state | state | Contractor | State |
|  | Kilmer | 5.48 | 24070 | Inspection Station | Yes | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Kilmer |  |  | Driver Testing Office | Yes | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | state | Contractor | State |
|  | Kilmer |  |  | Driver Road Test |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | state | Contractor | State |
|  | Lakewood | 3.93 | 23490 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Lodi | 13.84 | 23130 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Lodi |  |  | Driver testing Field House |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
|  | Lodi |  |  | MVC Agency |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
|  | Lodi |  |  | On site Driver Road Test Course |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
|  | Manahawkin | 5.10 | 7895 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Mays Landing | 11.13 | 17905 | Inspection Station | Yes | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Mays Landing |  |  | Driver Testing Office | Yes | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | state | Contractor | State |
|  | Mays Landing |  |  | Driver Road Test |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | state | Contractor | State |
|  | Millville | 0.65 | 8000 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Newton | 1.46 | 8140 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Paramus | 5.46 | 21230 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Rahway | 16.50 | 23132 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
|  | Rahway |  |  | Driver testing Field House |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
|  | Rahway |  |  | MVC Agency |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
|  | Rahway |  |  | On site Driver Road Test Course |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |

| Station Name | Acres | Building Square Footage | Operation | Shared Building | Building related Routine Maintenance Janitorial & Repair | Gas/ Oil | Water | Well Maintenance | Sewer | Septic System Maintenance | Trash Removal | Electric | Phone | Grounds | Internet Service Connectivity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Randolph |  |  | Inspection Station |  | State | Contractor | Contractor |  |  | Contractor | Contractor | Contractor | Contractor | Contractor | Contractor |
| Randolph |  |  | Driver Road Test |  | State | State | Contractor |  |  | State | Contractor | State | state | Contractor | State |
| Randolph |  |  | MVC Agency |  | State | State | State |  |  | State | state | state | state | Contractor | State |
| Salem | 1.20 | 4480 | Inspection Station | Yes | Contractor | Contractor | Contractor |  |  | Contractor | Contractor | Contractor | Contractor | Contractor | Contractor |
| Salem |  |  | Driver Testing Office | Yes | Contractor | Contractor | Contractor |  |  | Contractor | Contractor | Contractor | state | Contractor | State |
| Secaucus | 6.10 | 17715 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
| Secaucus |  |  | Specialty Inspection Facility |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | State | State | Contractor | Contractor |
| South Brunswick | 10.00 | 21490 | Inspection Station |  | Contractor | Contractor | Contractor |  |  | Contractor | Contractor | Contractor | Contractor | Contractor | Contractor |
| South Brunswick |  |  | MVC Agency |  | State | State | State |  |  | Contractor | state | state | state | Contractor | State |
| Southampton | 9.50 | 17905 | Inspection Station | Yes | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor | Contractor |
| Toms River | 1.20 |  | Specialty Inspection/Training Facility |  | State | State | State |  | State | State | Contractor | State | State | Contractor | Contractor |
| Washington | 0.37 | 4787 | Inspection Station | Yes | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
| Wayne | 15.00 | 29426 | Inspection Station |  | Contractor | Contractor | Contractor |  | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
| Wayne |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wayne |  |  | On site Driver Road Test Course |  | State | State |  |  | Contractor |  | Contractor | state | state | Contractor | State |
| Wayne |  |  | Driver Testing Office |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
| Wayne |  |  | MVC Agency |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
| Wayne |  |  | Driver testing Field House |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
| Wayne |  |  | On site Driver Road Test Course after new field house constructed |  | State | State | State |  | Contractor |  | state | state | state | Contractor | State |
| Winslow | 36.99 | 9456 | Inspection Station |  | Contractor | Contractor |  | Contractor | Contractor |  | Contractor | Contractor | Contractor | Contractor | Contractor |
| Winslow |  | 13198 | Specialty Inspection Facility |  | Contractor | State |  | Contractor | Contractor |  | state | state | state | Contractor | Contractor |

Contractor indicates the contractor is responsible.

State indicates the State is responsible

Future indicates that MVC plans to construct a new building on site to house this operation.

**NOTES The Randolph Service Core is a trailer leased by the current contractor at approximately $300. per month. The new contractor will need to negotiate and pay for the monthly rent

with the owner of the trailer (and fund the rental) or acquire another trailer (and fund the rental or purchase plus all associated hookup costs).

|  |  | Facility Buildings, Utilities and Grounds |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Contractor's Monthly payment to MVC for shared site electric and gas |  |  |  |  |
|  |  | Eatontown | Newark | Wayne | TOTAL MONTHLY PAYMENT |
|  |  | $2,545.26 | $2,121.05 | $2,121.05 | $6,787.36 |
|  | At these shared sites it is not feasible to separate the electric and gas usage. |  |  |  |  |
|  | The state is considering construction which may result in separating some of these sites. |  |  |  |  |
|  | Once construction is completed, the contractor's responsibility for these utilities will be adjusted |  |  |  |  |
|  | to reflect the contractor's actual usage. |  |  |  |  |

| 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  |  | Facility Buildings, Utilities and Grounds |  |  |  |  |  |  |
| 3 |  |  | Expenditures for Facility Renovations, Maintenance and Repairs |  |  |  |  |  |  |
| 4 | Facility Expenditures |  |  |  |  |  |  |  |  |
| 5 | Facility Renovations, Maintenance and Repairs Includes items such as or repairs to: carpentry, plumbing, electrical , heating, ventilation , air conditioning, parking lot lighting, locksmithing, painting, plumbing, roofing and repairs to parking lots, sidewalks ,driveways, curbing, paving, lane floor repairs, and erosion control measures. | Fiscal Year (FY) 2018 | FY 2019 | FY 2020 | FY 2021 | FY 2022 | FY 2023 | FY 2024 | FY 2025 |
| 6 |  | $563,395.65 | $400,977.74 | $414,620.78 | $453,107.88 | $474,185.74 | $237,692.49 | $1,197,192.80 | $1,220,870.46 |
| 7 |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |

# Appendix 3.2H: Lifts, Brakes and Air Compressors Specifications

BRAKE TESTING

The brake tester shall be of the “drive-on and stop” type and must be capable of quickly and accurately measuring and recording the brake effort of a motor vehicle’s actual stopping conditions. A brake dynamometer will not be sufficient. The vehicle weighing mechanism shall be placed in a location such that “backing up” the vehicle is not required during the brake test process.

Side to side brake balance for each axle, front to rear brake balance, vehicle deceleration, vehicle weight, parking brake balance and deceleration, and minimum and maximum vehicle test speed must be tested.

The video monitor and computer shall be mounted so that the information displayed can be read by a person sitting in the driver’s seat of the vehicle being tested. A protective enclosure, which can be locked, shall be provided for the video monitor and computer to prevent vandalism and to minimize exposure to the elements when the brake tester is not in use.

The brake tester shall be equipped with a programmable computer. The contractor shall be responsible for ensuring that the brake testing equipment has the most current software. The contractor shall be responsible for procuring software updates as the manufacturer makes the updates available. The brake test computer shall interface with a computer system for data collection and storage and shall be connected to a printer using standard plugs, sockets and cables. The computer shall be equipped with a numerical keyboard which can be used for data entry purposes.

When a vehicle is driven onto the brake tester and stopped, the following information shall be displayed on a video monitor.

- Total braking effort in pounds.

- Percentage of braking effort developed by the front axle.

- Percentage of braking effort developed by rear axle.

- The difference in percentage of braking effort developed by the left and right wheels on the rear axle.

- Brake efficiency (total braking effort in pounds divided by weight of vehicle in pounds)

- Curves depicting the amplitude and duration of the braking force generated by each wheel. A different color shall be used for each of four wheels.

- A pass-fail indicator which automatically compares the braking effort recorded with the legal standard.

Each motorist whose vehicle fails the brake inspection will be given a printout of the results. The printout must show numerically and graphically the braking force for each wheel. The printout must also graphically demonstrate the side-to-side brake balance of each wheel and overall brake deceleration. This document is independent of the VIR.

There is at least one heavy-duty brake tester at each centralized facility. All new heavy-duty brake tester shall be designed to adequately withstand the stresses involved in a continuous, production line style of motor vehicle testing, and shall have a maximum vehicle drive-over weight capacity of no fewer than 44,000 pounds per axle. (Tandem axles are considered to be one axle). The heavy-duty brake tester shall be able to test vehicles up to 40,000 pounds and shall be equipped with a total of four (4) wheel testing plates. The wheel testing plates shall be

positioned so that one plate is used for each front wheel and each rear wheel of the vehicle being tested. The maximum vehicle wheelbase capacity for the single stop test shall not be less than 318 inches. Other specifications include:

- he top surface of each wheel testing plate shall be equipped with a skid resistant surface.

- The entire surface of each wheel testing plate shall be coated with corrosion resistant finish.

- A metal frame must support the wheel testing plates. The metal frame shall be designed so that it can be easily mounted on a level concrete surface, or, if mounted in concrete, it would appear to be a level testing surface.

- The frame assembly must be designed to accommodate a floor drain under each wheel testing plate location.

- The entire package of metal frame shall be coated with corrosion resistant finish.

- Strain gauge transducers must measure the braking effort of each wheel. The measurement range of the transducers shall be from 0 to 10,000 pounds per wheel. The resolution of the transducers shall be no more than 10 pounds.

- The measurement range of the brake tester shall be from 0 to 40,000 pounds total braking effort. The resolution of the brake tester shall be no more than 10 pounds per wheel. The accuracy of the brake tester shall be ½ per cent of the maximum measuring range.

- A means for readily testing and adjusting the accuracy of each individual wheel testing plate and its associated circuitry.

- Operation without damage, and within specified calibration limits in ambient air temperatures, ranging from 0 to 100 degrees Fahrenheit while exposed to direct sunlight.

- The electronic circuitry shall be on modular, plug in type construction, for ease of servicing.

- The interval of time, from the instant the vehicle being tested is stopped on the wheel testing plates and the information required is displayed on the video monitor, until the brake testing machine is ready for another test, shall not exceed 75 seconds.

## Lift Manual

Rev. 06/25/10

Introduction

A vehicle steering, and suspension system is designed to provide directional control and a smooth but safe ride. It provides stable handling and accurate turning. All vehicles must have a safe and tight steering system to ensure that the vehicle is safe to drive on the roads. Accidents or unsafe vehicle control can occur if the vehicles steering or suspension system is worn out or broken. To ensure this, all vehicles that are over 50,000 miles are required to have their steering systems checked during the vehicle inspection process. The most accurate way of doing this is to raise the vehicle off the ground and physically shake the vehicles front tires and see if there is any looseness evident in the steering and suspension components. Any looseness indicated by more the one Quarter inch of play in the vehicles steering system indicates that there is worn steering or suspension parts that need to be repaired.

Components

The Lift

The lift is an air pressure-controlled plate that can be raised and lowered accurately by the inspector. The lift is a large wide plate that has a rubber coated top. When raised, the lift plate is wide and long enough to ensure that it will contact something metal on the undercarriage of the vehicle. Because of the new lift plate design the vehicle is “sitting on top of a table” ensuring the vehicle will not move or fall off the lift while being raised.

Lift handle control

The lift handle controls the raising and lowering of the lift. To raise the lift the inspector shall pull the handle upward. To lower the lift, push down on the lift handle. Caution must be used when raising the lift as the lift can go up very fast. The inspector shall lift handle up slightly, feeding just a little air into the lift assembly ensuring the lift rises

slowly and smoothly. The lift shall be raised just enough that both tires are off the ground. Once the vehicle is raised enough where the tires leave the ground the inspector will release the lift handle into its neutral center position.

To lower the lift, push down on the lift handle. The handle should be pushed down forcefully and fully to allow maximum air to escape out of the lift assembly ensuring a quicker descent rate. Since the lift has a maximum descent rate, the lift is lowered at a controlled rate, so the inspector does not have to worry about the lift dropping too quickly which ensures that the vehicle is safely lowered. The lift handle should be held down until all the air extinguishes out of the lift and that the lift is completely lowered. The inspector shall always visually confirm that the lift plate is lowered fully and sitting completely on the ground before proceeding.

Since the lift is equipped with internal safety locks, clicking will sometimes be audible when the lift is raised. If the lift was to lose air pressure due to a massive air leak, the locks would engage to prevent the lift from lowering at an uncontrolled rate. Sometimes because of these safety locks, the inspector will find that when they go to lower a vehicle, the air will extinguish out of the lift however the lift plate does not lower. In this case, the inspector shall raise the lift back up, allowing the vehicle to be lifted again slightly, thus allowing the lift safety locks to release automatically. The inspector can then push down on the lift handle again and the vehicle should lower. If the lift still does not lower, retry lifting the vehicle higher than the first attempt and then lowering. If the lift still does no lower, notify the station manager. The station manager shall confirm that the stations air compressor is functional as the lift locks will not release if the systems air pressure is inadequate. If the vehicle cannot be removed from the lift, the station manager should notify their Regional Manager and the Facilities Manager immediately for assistance.

Lifting Vehicles

Lifting Points

There are three main places that you want the lift plate to contact to lift the vehicle from. Frame rails, control arms (A-Frames) or solid axle assembly. When the lift plate is raised, typically it will contact the undercarriage of the vehicle with the back edge of the

lift plate first. Then as the lift is further raised the front part of the plate will make contact with the rest of the undercarriage of the vehicle.

1) Frame Rails

Typically, most vehicles have solid or molded metal frame rails running half the length of the vehicle from the front to the back. This is a great place for the lift plate to contact and lift the vehicle. The inspector shall verify that the vehicle is centered on the lift enough that the lift plate will contact the right and left side frame rails.

The control arms which also go by the name of A –Frames, connect the vehicles wheels to the vehicles sub-frame. Control arms vary in size and width and are usually made of metal but in recent years newer sports cars for weight reduction have been making control arms out of aluminum. Whichever material that the control arms are made of does not matter because the control arms are perfect for lifting the vehicle. The inspector needs to ensure that the vehicle is centered on the lift plate, so the plate contacts each side of the vehicles control arms respectively. The larger the vehicle, it is even more important for the vehicle to be exactly centered on the lift plate.

Solid axle assembles means the vehicle will have a solid front tube or beam running between the front wheels. Most large trucks or 4WD vehicles will be equipped with this type of front axle assemble. On 4WD vehicles, the solid axle will also have a front differential as part of the assembly commonly known to inspectors as a “pumpkin”. This is a perfect place for the lift plate to contact to lift the vehicle from. However, since the pumpkin housing is large and round it will cause the vehicle to usually go up higher on the pumpkin side resulting in the vehicle being lifted crooked and unevenly. To allow the vehicle to be lifted evenly one lifting block should be placed on the opposite side. On these types of vehicles, the license plate centering procedure might not be the best lineup. Always look that the lift plate contacts good lifting points on both sides of the vehicle. On Large 4WD vehicles that means that the lift plate will contact the pumpkin on one side and the placed lifting block on the other side as illustrated above.

Special Lifting Situations

Most vehicles will be able to be lifted from one of the three lifting points. In some cases, however, there might be something hanging lower than the front suspension components of the vehicle. On some 4-cylinder engines the exhaust system usually comes from the front of the engine and then goes under the front of the car. In these cases, the inspector must ensure that the exhaust is not hanging too low and that the lift plate will not contact and lift the car first from the low hanging exhaust system as shown below.

In some cases, the enclosed rubber lifting blocks might need to be used. The inspector would need to slide them between the lift plate and the appropriate lifting point thus allowing enough space to exist between the lifting plate and the hanging exhaust system. After the vehicle is lowered, the inspector will need to immediately remove the blocks so as not to damage anything on the undercarriage of the car when the vehicle is moved forward.

Do Not Lift Vehicles

Some vehicles should not be lifted because there are no appropriate lifting points available. This situation usually occurs on vehicles that have a full cover, usually a plastic covering that completely obscures the whole front suspension undercarriage of the vehicle. Because we do not want to be lifting a car from a plastic cover of which we do not know what is behind that cover, these types of vehicles will not be lifted. Other no lift type vehicles might have other objects in the way that we might not be able to clear or use the lifting blocks to gain clearance around, for example, steering dampeners.

Inspectors that are uncertain whether that vehicle can be safely lifted, they should contact the senior lane technician of that lane for assistance. Whenever an inspector makes a decision not to lift a vehicle which has over 50,000 miles the inspector must notify the station manager immediately.

Vehicles that have air suspension are another concern. If you suspect that the vehicle has air suspension which can be identified by the absence of coil springs and the addition of bellow type bags and air hoses, the air suspension should be turned off. If the inspector does not know where the air suspension off switch is, the inspector shall not lift the vehicle and the manager notified.

Safe Lifting Procedures

 Ensure engine if OFF, transmission is in NEUTRAL and parking brake is OFF before lifting the vehicle.

 Ensure you are wearing the provided safety gloves before touching tires and wheels.

 Ensure vehicle is correctly aligned before lifting the vehicle.

 Use mirrors to check for proper lifting points under vehicle.

 Ensure lift is only raised high enough that the front wheels are no more than six

(6) inches off the ground.

 Ensure you are inside the vehicle with door closed and in full control of the vehicle when operating the vehicle or when starting or stopping the engine.

Vehicles Unable to be Lifted

| Make | Model | Year | Comment |
| --- | --- | --- | --- |
| Audi | A4 | 2005 | Covered–No Lift |
| Audi | A6 | 2005 | Covered–No Lift |
| Audi | A8 | 2004 - 2017 | Covered–No Lift |
| Audi | R8 | 2007 - 2017 | Covered–No Lift |
| Audi | S8 | 2008 | Covered–No Lift |
| BMW | 6 Series | 2006 | Covered–No Lift |
| BMW | 7 Series | 2005 | Covered–No Lift |
| BMW | X3 | 2004 - 2009 | Covered–No Lift |
| BMW | X5 | 2000 - 2017 | Covered–No Lift |
| BMW | X6 | 2008 - 2017 | Covered–No Lift |
| BMW | Z4 | 2003 | Covered–No Lift |
| Ford | Excursion | 2000 - 2005 | Shock Dampner in way - No Lift |
| Ford | F-250 | 1975 - 2017 | Shock Dampner in way - No Lift |
| Ford | F-350 | 1975 - 2017 | Shock Dampner in way - No Lift |
| Ford | F-450 | 1975 – 2017 | Shock Dampner in way - No Lift |
| Ford | Freestar | 2007 | Covered–No Lift |
| Infiniti | G35 | 2003 - 2008 | Covered–No Lift |
| Infiniti | G37 | 2008 - 2009 | Covered–No Lift |
| Infiniti | G37X | 2009 | Covered–No Lift |
| Infiniti | M35 | 2010 - 2017 | Covered–No Lift |
| Infiniti | M35X | 2010 - 2017 | Covered–No Lift |
| Jaguar | XJ Series | 2004 - 2017 | Air Suspension - No Lift |
| Jaguar | XJ8 | 2004 - 2017 | Air Suspension - No Lift |
| Jaguar | XJR | 2004 – 2017 | Air Suspension - No Lift |
| Jeep | Grand Cherokee | 2008 | Covered–No Lift |
| Land Rover | Range Rover | 1993 - 2009 | Air Suspension - No Lift |
| Lexus | LS 400 | 1993-2000 | Air Suspension - No Lift |
| Lexus | RX330 | 2005 | Covered–No Lift |
| Lincoln | MK7 | 1984 - 1992 | Air Suspension - No Lift |
| Lincoln | MK8 | 1993 - 1998 | Air Suspension - No Lift |
| Lincoln | Navigator | 1998 - 2017 | Air Suspension - No Lift |
| Lincoln | Town car | 1990 - 2017 | Air Suspension - No Lift |
| Mercedes-Benz | 380 | 1985 | Oil Pan in way - No Lift |
| Mercedes-Benz | C Class | 2004 - 2007 | Covered–No Lift |
| Mercedes-Benz | CL Class | 2004 - 2007 | Covered–No Lift |
| Mercedes-Benz | CLK Class | 2004 - 2007 | Covered–No Lift |
| Mercedes-Benz | CLS Class | 2006 - 2017 | Air Suspension - No Lift /Some Covered |
| Mercedes-Benz | E Class | 2000 - 2017 | Air Suspension - No Lift /Some Covered |
| Mercedes-Benz | GL Class | 2007 - 2017 | Air Suspension - No Lift /Some Covered |
| Mercedes-Benz | M Class | 2005 - 2017 | Air Suspension - No Lift /Some Covered |
| Mercedes-Benz | S Class | 2000 - 2017 | Air Suspension - No Lift /Some Covered |
| Mercedes-Benz | SL Class | 2004 - 2017 | Air Suspension - No Lift /Some Covered |
| Mercedes-Benz | SLK Class | 2004 -2017 | Covered–No Lift |
| Mercedes-Benz | SLR McLaren | 2005 -2007 | Covered–No Lift |
| Nissan | GT-R | 2009 - 2017 | Covered–No Lift |
| Porsche | 911 | 2000 - 2017 | Covered–No Lift |
| Smart | ForTwo | 2005 - 2013 | Covered–No Lift |
| Subaru | Forrester | 1999 | Covered–No Lift |
| Subaru | Legacy | 2001 - 2002 | Low Exhaust - No Lift |
| Subaru | Outback | 1999 | Low Exhaust - No Lift |
| Volvo | V70xc | 2001 | Covered–No Lift |

No Lift Notification

In the event a manager has determined a vehicle is unable to be lifted or requires special handling, the manager shall email the Manager of Training requesting the vehicle be added to the list. As a minimum the email must include the vehicle year, make and model, pictures, drive train type. The Manager of Training will research that vehicle for possible inclusion on the no lift list update.

Stationary Two-Stage Air Compressor Compresseur D’Air Stationnaire À Deux Étages Compresor de Aire Estacionarrio de 2 Etapas

Lista de Repuestos	CI070080V, CI073080H, CI073080H5, CI073080V, CI073080V5, CI071082VMS, CI073080VMS, EX8404

CI050080HA, CI050080VA, CI051080HA, CI051080VA, CI051080VMSA, CI053080HA,

CI053080H5,  CI053080VA,  CI053080V5,  CI071080VMS, CI071080VPMS,CI070080H,

Replacement Parts List

Liste de Pièces de Rechange

Lista de Repuestos	CI070080V, CI073080H, CI073080H5, CI073080V, CI073080V5, CI071082VMS, CI073080VMS, EX8404

CI050080HA, CI050080VA, CI051080HA, CI051080VA, CI051080VMSA, CI053080HA,

CI053080H5,  CI053080VA,  CI053080V5,  CI071080VMS, CI071080VPMS,CI070080H,

Replacement Parts List

Liste de Pièces de Rechange

| Specifications |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Discharge | Motor | Displacement | Delivery | Max. | Pump | Unit |
| Pressure | HP | CFM | CFM | Pressure | RPM | Weight |
| 175 psi | 5.0 | 21.1 | 17.4 | 175 | 700 | 510 lbs |
| 175 psi | 7.5 | 31.4 | 25.1 | 175 | 1020 | 540 lbs |
| Spécifications |  |  |  |  |  |  |
| Pression dé | Moteur | Déplacement | Débit | Pression | Pompe | Poids du |
| Débit | HP | m3/min | m3/min | Max. | RPM | Modèle |
| 1207 kPa | 5,0 | 0,6 | 0,5 | 1207 kPa | 700 | 232 Kg |
| 1207 kPa | 7,5 | 0,89 | 0,71 | 1207 kPa | 1020 | 245 Kg |
| Specifications |  |  |  |  |  |  |
| Presión de | CP del | Desplazamiento | Suministro | Presión | RPM del | Peso de |
| Salida | Motor | m3/min | m3/min | Máx | Cabezal | la unidad |
| 12,07 bar | 5,0 | 0,6 | 0,5 | 12,07 Bar | 700 | 232 Kg |
| 12,07 bar | 7,5 | 0,89 | 0,71 | 12,07 Bar | 1020 | 245 Kg |

5 HP Data based on 1725 RPM electric motor with a service factor of 1.15. 71/2 HP Data based on 3450 RPM electric motor with a service factor of 1.15.

Données de 5 HP basées sur les moteurs électriques de 1725 RPM avec facteur d’utilisation de 1.15.

Données de 71/2 HP basées sur les moteurs électriques de 3450 RPM avec facteur d’utilisation de 1.15.

Los valores presentados para 5 CP están basados en motores eléctricos de 1725 RPM wcon un factor de servicio de 1.15.

Los valores presentados para 71/2 CP están basados en motores eléctricos de 3450 RPM wcon un factor de servicio de 1.15.

*Data based on B section belts and pulleys.

*Données basées sur la section B courroies et poulies.

*Basado en bandas y poleas del corte B.

NOTE: Flow rate data determined in accordance with United States’ ASME power test code.

REMARQUE: Les renseignements concernant le taux de débit sont déterminés conformément au code d’essai de puissance ASME des É-U.

NOTA: El flujo se calculó según los códigos para pruebas de potencia de la organización ASME

© 2003 Campbell Hausfeld/Scott Fetzer	For parts, product & service information

visit www.chpower.com

IN253604AV 10/03

Replacement Parts List

Liste de Pièces de Rechange

CI050080HA, CI050080VA, CI051080HA, CI051080VA, CI051080VMSA, CI053080HA, CI053080H5, CI053080VA, CI053080V5, CI071080VMS, CI071080VPMS, CI070080H,

Lista de Repuestos	CI070080V, CI073080H, CI073080H5, CI073080V, CI073080V5, CI071082VMS, CI073080VMS, EX8404

17  18	1

19

21

20

4

21

19	22

1	18  20	24

8

24

22

16

15	5

6

12	13

14

9

16 11

14

17	24

4	5

10	35	23

8

5

24

5

23	96

13 15

12

2

31

7	32

34 10 16

11

3

33

26	7

30

29	37

38

36

25	28

28	27

27

www.chpower.com

2

Replacement Parts List	Stationary Two-Stage Air Compressors

For Replacement Parts, Call 1-800-543-6400

Please provide following information:	Address parts correspondence to:

-Model number	The Campbell Group / Attn: Parts Department

-Serial number (if any)	100 Production Drive

Qty

CI071080VPMS CI073080VMS

CI051080VMSA

Description

CI073080H5 CI073080V5

EX8404

CI051080HA*  CI053080HA	CI053080H5 CI071080VMS CI073080H† CI051080VA*  CI053080VA	CI053080V5 CI071082VMS CI073080V†

Ref. No.

Qty

CI071080VPMS CI073080VMS

CI051080VMSA

Description

CI073080H5 CI073080V5

EX8404

CI051080HA*  CI053080HA	CI053080H5 CI071080VMS CI073080H† CI051080VA*  CI053080VA	CI053080V5 CI071082VMS CI073080V†

Ref. No.

-Part description and number as shown in parts list	Harrison, OH 45030 USA

- Air compressor pump	CI050000PB	CI050000PB	CI050000PB	CI070000PB	CI070000PB	CI070000PB	1

Air compressor pump (CI07180VPMS)

(Pressure Lubricated)	--	--	--	CI070000PP	--	--	1

- Horizontal air tank	TF003603AJ	TF003603AJ	TF003603AJ	TF003603AJ	TF003603AJ	TF003603AJ	1

- Vertical air tank	TF006607AJ	TF006607AJ	TF006607AJ	TF006607AJ	TF006607AJ	TF006607AJ	1

4	7/16” - 14 x 11⁄2” Hex head screw	ST070645AV	ST070645AV	ST070645AV	ST070645AV	ST070645AV	ST070645AV 	4

5	7/16” Washer	ST070916AV	ST070916AV	ST070916AV	ST070916AV	ST070916AV	ST070916AV	8

6	7/16” - 14 Hex nut	ST070729AV	ST070729AV	ST070729AV	ST070729AV	ST070729AV	ST070729AV	4

7	3/8” NPT Drain cock	ST127700AV	ST127700AV	ST127700AV	ST127700AV	ST127700AV	ST127700AV	1

8	Discharge tube	TF061500AP	TF061500AP	TF061500AP	TF061500AP	TF061500AP	TF061500AP	1

9	Nut and ferrule	ST072321AV	ST072321AV	ST072321AV	ST072321AV	ST072321AV	ST072321AV	1

10	3/4” check valve	CV003205AV    CV003205AV    CV003205AV    CV003205AV    CV003205AV    CV003205AV	1

11	1/4” Compression fitting	ST011701AV	ST011701AV	ST011701AV	ST011701AV	ST011701AV	ST011701AV	1

12	1/4” NPT, 300 psi pressure gauge	GA016703AV    GA016703AV    GA016703AV    GA016703AV   GA016703AV   GA016703AV	1

13	1/4” - 18 x 21⁄2” Pipe nipple	ST070387AV	ST070387AV	ST070387AV	ST070387AV	ST070387AV	ST070387AV 	1

14	Safety valve	V-215200AV	V-215200AV	V-215200AV	V-215200AV	V-215200AV	V-215200AV	1

15 ♦ Pressure switch, 145/175	CW207561AV  CW207561AV  CW207561AV  CW207561AV   CW207561AV  CW207561AV	1

16	Copper tube: horizontals - 6 ft.

verticals - 2 ft.	R-060600AY	R-060600AY	R-060600AY	R-060600AY	R-060600AY	R-060600AY	1

17	Motor	MC022393AV   MC022394AV   MC301503AV   MC301521AV  MC301500AV  MC301504AV	1

18	5/16” - 18 x 1/2” Set screw	ST026400AV	ST026400AV	ST026400AV	ST026400AV	ST026400AV	ST026400AV	2

19	Belt	BT022000AV	BT022000AV	BT006000AV	BT008500AV	BT008500AV	BT008500AV	1

20	Pulley	PU008003AV   PU008003AV   PU009755AV   PU009750AV   PU009750AV   PU009757AV	1

21	3/8” - 16 x 11⁄4” Screw	ST070638AV	ST070638AV	ST070654AV	ST070638AV	ST070638AV	ST070654AV 	4

22	3/8” Washer	ST070914AV	ST070914AV	ST070914AV	ST070914AV	ST070914AV	ST070914AV	4

23	3/8” - 16 Nut	ST033500AV	ST033500AV	ST033500AV	ST033500AV	ST033500AV	ST033500AV	4

- 3/8” External tooth washer	ST072608AV	ST072608AV	ST072608AV	ST072608AV	ST072608AV	ST072608AV	2

- Belt guard front, horizontal unit	BG217000AV   BG217000AV   BG217000AV   BG217000AV   BG217000AV   BG217000AV	1

Belt guard front, vertical unit	BG217001AV   BG217001AV   BG217001AV   BG217001AV   BG217001AV   BG217001AV	1

26	Belt guard back	BG217100AV   BG217100AV   BG217100AV   BG217100AV   BG217100AV   BG217100AV	1

27	Cap	ST075400AV	ST075400AV	ST075400AV	ST075400AV	ST075400AV	ST075400AV	5

28	Hex flange nut	ST116201AV	ST116201AV	ST116201AV	ST116201AV	ST116201AV	ST116201AV	5

29	Bolt	ST016500AV	ST016500AV	ST016500AV	ST016500AV	ST016500AV	ST016500AV	2

30	Washer	ST011200AV	ST011200AV	ST011200AV	ST011200AV	ST011200AV	ST011200AV	2

31	1/4” - 20 x 3/4” Bolt	ST074415AV	ST074415AV	ST074415AV	ST074415AV	ST074415AV	ST074415AV	1

32	1/4” Washer	ST070910AV	ST070910AV	ST070910AV	ST070910AV	ST070910AV	ST070910AV	1

- Beltguard bracket	TF063400AV	TF063400AV	TF063400AV	TF063400AV	TF063400AV	TF063400AV	1

- 90˚ compression fitting	ST072231AV	ST072231AV	ST072231AV	ST072231AV	ST072231AV	ST072231AV	1

35	1/2” x 1/4” reducer bushing	ST071428AV	ST071428AV	ST071428AV	ST071428AV	ST071428AV	ST071428AV	1

  Isolation pads (pack/3 - verticals)	ZZ000043AJ	ZZ000043AJ	ZZ000043AJ	ZZ000043AJ	ZZ000043AJ	ZZ000043AJ	1

   Isolation pads (pack/4 - horizontals)	ZZ000127AJ	ZZ000127AJ	ZZ000127AJ	—	ZZ000127AJ	ZZ000127AJ	1

	Warning decal	DK630000AV   DK630000AV   DK630000AV   DK630000AV   DK630000AV   DK630000AV	1

UNITS WITH MAGNETIC STARTERS ONLY

| 36 | Magnetic starter bracket |  | BG215300AV | — — BG215300AV |  |  | BG215300AV | — 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37 | Self-threading screw |  | ST016500AV | — — ST016500AV |  |  | ST016500AV | — 2 |
| 38 | .A Magnetic starter assembly |  | ST122001AJ | — — ST122010AJ |  |  | ST122011AJ | — 1 |
|  | 8-32 x 7/16” Hex screw |  | ST074407AV | — — ST074407AV |  |  | ST074407AV | — 4 |
| (♦) Unloader valve for pressure switch available separately, part number CW000200AV (.A) Heater element for magnetic starter |  |  |  |  |  |  |  |  |
| PS005550AV |  |  | 1 | — | — | — | 3 | 3 |
| PS005551AV |  |  | — | 3 | 3 | 1 | — | — |
| () Not shown |  | ()Available separately |  |  |  |  |  |  |
| * Models CI050080H/V are similar but without items 17-24 † Models CI070080H/V are similar but without items 17-24 |  |  |  |  |  |  |  |  |

www.chpower.com

3

Replacement Parts List

Liste de Pièces de Rechange

CI050080HA, CI050080VA, CI051080HA, CI051080VA, CI051080VMSA, CI053080HA, CI053080H5, CI053080VA, CI053080V5, CI071080VMS, CI071080VPMS, CI070080H,

A

CL

C

D

B

E

A

CL

C

D

B

E

C

CL

D

B

E

C

CL

D

B

E

Lista de Repuestos	CI070080V, CI073080H, CI073080H5, CI073080V, CI073080V5, CI071082VMS, CI073080VMS, EX8404

Dim.

Item Article Descripción

Key Réf. Clave

Dim.

Item Article Descripción

Key Réf. Clave

  - Overall width	32.5"

Largeur totale	82,6 cm

Ancho total	82,6

  - Bolt down-width (120˚ apart)	27.0" Largeur de boulonnage (120˚ apart)	68,6 cm Distancia después de apretar el perno (120˚ apart)	68,6

  - Overall height	75.0"

Hauteur totale	190,5 cm

Altura total	190,5

| D | Tank outlet to ground | 40.0" |
| --- | --- | --- |
|  | Sortie du réservoir à la terre Conexión del tanque a tierra | 101,6 cm 101,6 |
| E | Overall depth | 25.5" |
|  | Profondeur totale | 64,8 cm |
|  | Distancia total | 64,8 |

| Key Item Réf. Article Clave Descripción Dim. |  |  |
| --- | --- | --- |
| A B | Overall width Largeur totale Ancho total Bolt down-width Largeur de boulonnage | 64.0" 162,6 cm 162,6 40" 101,6 cm |
|  | Distancia después de apretar el perno | 101,6 |
| C | Overall height | 49" |

Hauteur totale	124,5 cm

Altura total	124,5

  - Tank outlet to ground	14.0" Sortie du réservoir à la terre	35,6 cm

Conexión del tanque a tierra	35,6

  -   Bolt down depth	18.0"

  Profondeur de boulonnage	45,7 cm

 Distancia después de apretar el perno	45,7

  -  Overall depth	20.5"

 Profondeur totale	52,1 cm

  Distancia total	52,1

() Not shown ()  Pas indiquée () No se muestra

www.campbellhausfeld.com

Garage Sales NJIL-12 Preventive Maintenance

Since the Garage Sales NJIL-12 was specifically made for this program, here is the PM Check.

Garage Equipment Sales & Service PM Check List

Pneumatic Operated Front Suspension Lift Annual NJIL-12

    - Remove lift assembly from canister for visual inspection;

    - Check for worn or loose components and repair;

    - Pressure test air bags, safety release cylinders & all fittings and repair as needed;

    - Lubricate & inspect glide blocks and safety latch pivot points;

    - Lean canister & reinstall lift;

    - Check operation of all controls.

Specifications for Lifts with Air Compressors

Each Specialty Inspection Facility is equipped with a pneumatic lift with an air compressor to provide an air supply for the lifts operation. The contractor will be responsible to maintain these lifts along with air compressors used by the current lifts at the Specialty Inspection Facilities (SIF). Currently, there are two different models used at the three SIF. Here is the list of the facilities and type of compressor used.

| SIF | Lift Make | Lift Model | Air Compressor Make | Air Compressor Model |
| --- | --- | --- | --- | --- |
| Westfield | Garage Sales | NJIL-12 | Champion | Advantage VR5- 8 |
| Asbury Park | W-K Industries | SWK-12-18S | Campbell Hausfield | CI073080V |
| Winslow | W-K Industries | SWK-12-18S | Champion | Advantage VR5- 8 |

Information about each type of lift and air compressor are listed separately.

VR5-8 Champion 5 HP 80 Gallon Vertical Advantage Series Air Compressor Fully Packaged

VR5-8 FEATURES AND BENEFITS

- Motor starter mounted & wired -----saves on electrical wiring cost

- Low oil monitor mounted & wired-----stops motor when oil level is low

- Auto tank drain mounted & piped -----removes water from tank automatically

- Isolators pads -----protects unit from out-of-level floors & reduces noise

- Factory oil fill with ChampLub -----long pump life

- Air-cooled aftercooler mounted & piped----- removes 70% of moisture FIVE YEAR WARRANTY ON PUMP

:• , - .. · :.

,    ·	. .  ,., .. ,,,     .,	..

n,-,,, .• '.,"I."-_ '

:."".. ,' !'

•   .	-	-•-- -- ·•-•  '•·r   ..

;--

;	·:  \		.:

:• , - .. · :.

,    ·	. .  ,., .. ,,,     .,	..

n,-,,, .• '.,"I."-_ '

:."".. ,' !'

•   .	-	-•-- -- ·•-•  '•·r   ..

;--

;	·:  \		.:

- 6,000 lbs to 50,000 lbs lift capacity

- 6,000 lbs to 50,000 lbs lift capacity

  - 5 year manufacturer Warranty

  - Can be customized to fit most Hoist makes and/or models OR designed to fit any Pit

,,. •

,,. •

·

W-K Rolling Jacks f o r:

Exclusive Manufactures Established 1990 Patent Pending In USA & CANADA*

'	: .

l     •     . "

- •

. .

- ..,..	1 ,1

-               ?'"	• • •'

.,..      ,.	r .,,;: ,. r H

: .

Exclusive Manufactures Established 1990 Patent Pending In USA & CANADA*

'	: .

l     •     . "

- •

. .

- ..,..	1 ,1

-               ?'"	• • •'

.,..      ,.	r .,,;: ,. r H

: .

Light/ Mid-Size Auto, Heavy Duty, Oil and Lube Retro Fits, Mass Transit, Trucking, Mining and any Hoist/ Pit Applications

Product Line	Easy Installation

Since 1990, W-K Industries "Air-Up" series pneumatic jacks have performed around the clock on numerous above ground 4-post hoists and pits throughout the U.S.A. and Canada lifting every­ thing from passenger vehicles to heavy transit buses to trucks and recreational vehicles. Currently, there are eleven models of the "Air-Up" series pneumatic jacks, providing the user with a wide range of load capacities to fit their specific requirements. New models are currently being developed and will be intro­duced as they are ready for the market. The current models available are in two main lift segments: Light Weight to Mid-Size (6,000lbs - 12,000 lbs) and Heavy Duty (15,000 lbs - 50,000 lbs).

A minimum of 120 psi provides the jacks a lifting capacity of 6,000 to 50,000 lbs. Installation of the "Air-Up" series pneumatic jack is simple;

- Centre the jack on the hoist or pit and lock into place.

- Connect the air supply.

- Lift. The jack is ready to go!

Light Weight to

Mid- Size Lifts

Light Weight to

Mid- Size Lifts

MODEL	           W-K 7	W-K 9	W-K 12

LIFTING

CAPACITY	6,000 lbs	7,000 lbs	9,000 lbs	12,000 lbs

}f - I

..,,._. .,

W-K50

#### L...

- ,,i_.

:_:"I.

###### Industry Applications

W-K "Air-Up" series pneumatic jacks have been successfully used in the following industry applications;

###### Business Value

Since 1990, the proven reliability and performance of the W-K "Air-Up" series pneumatic jacks has greatly reduced

- Automotive Repair

- Emergency Vehicles

- Freight Trucks

- Construction Vehicles

- Oil & Lube retro fits

- Mass Transit

- Recreational Vehicles

- Military Vehicles

- Mining Equipment

- All Service Pit applications

the downtime of fleet vehicles. They have also improved the efficiency of service operations to provide a low maintenance, worry-free, and cost-effective solution for vehicle repair in multiple industries and applications.

  - rolling jacks can be customized to fit your specific business requirement and applications, making it versatile and adaptable to all lift applications as independent products or as an accessory to your product existing product set.

| ADAPTABILITY ACANUS | BRADBURY | GRAND | MOHAWK |
| --- | --- | --- | --- |
| ACU-LIFT TO ALL HOISTS AND PITS | CHALLENGER | HENNESSYIAMMCO | OMER |
| ALM | CUNNINGHAM | HOFFMAN | ROTARY |
| The W-K "Air Up" series rolling jacks can AMERICAN | FMC | HUNTER | SPACESAVER |
| be modified to fit any 4-post hoist or pit. BAYMATE | FORWARD | HYDRA | STEN HOJ |
| W-K products have been installed and BEN PEARSON | GEMINI | LESCO | WHEELTRON/C |
| have been operating successfully since BEND-PAK | GIROLIFT | LINCOLN | WORTHINGTON |
| 1990 on the following hoists: BENWIL | GLOBE | MALCAN | AND MANY MORE. |

Can be fitted to any hoist design

Can ride on the pit ledge

Can be fitted to any existing pit rail system

Can be modified to fit any custom built rail system on pit/hoist

Flat roller design available	Grooved roller design available

NEW PRODUCTS

Available in 20 gallon, 40 gallon, and other sizes customized to your application.

OIL DRAIN PANS

OIL DRAIN PANS

Images shown may differ from actual products, confirmed at time of order.

(A LAIR 1.8 I N CH LIFT STRO KE	K /  STAND-ALONE  uFT

Can be modified to fit in pit, on rail, and/or in-ground applications. Can also be used with all truck racks. Currently available in 12,000 lbs and 25,000 lbs lift capacity. Other capacities available so on. Call to enquire.

"-·-	-	-	-

-, ------ - - - - -

"-·-	-	-	-

-, ------ - - - - -

- - - - P-aten-t Pen-d-i-n-g--i-n  USA &-Cana-da  *- -- - - - - - - - -- -  -  -  -  -  -  -

www.wksalesdist.com sales@wksalesdist.com

Installation, Operation & Maintenance Manual

MODELS

WK6, WK7.5 & WK9

6,000 lb	7,500 lb	9,000 lb

ROLLING JACK

IMPORTANT: READ THIS MANUAL COMPLETELY BEFORE INSTALLING or OPERATING JACK

CAUTION - Before using this product, read and fully understand the operating instructions and all decals on the product. This is necessary to prevent injury to the operator and damage to the product. - Roll jack forward before moving vehicle on or off lift. - Do not attempt to use this jack for anything other than its intended purpose. - If jack begins to tilt as the vehicle is raised, STOP! Lower jack and reposition the vehicle. - Do not use this jack if it is visibly worn, distorted or damaged. - Always wear appropriate eye protection.

WARNING

- Maximum lift capacity is listed on the jack (6000 lbs, 7,500 lbs & 9,000 lbs) Exceeding this could result in severe personal injury or death.

- Lift vehicle at manufacturer’s recommended pick-up points only. Vehicle weight must be evenly distributed on each jack lift pad.

- Keep fingers and hands clear of all pinch points at all times.

WARNING

- Maximum lift capacity is listed on the jack (6000 lbs, 7,500 lbs & 9,000 lbs) Exceeding this could result in severe personal injury or death.

- Lift vehicle at manufacturer’s recommended pick-up points only. Vehicle weight must be evenly distributed on each jack lift pad.

- Keep fingers and hands clear of all pinch points at all times.

Installation

- Install a ¼ NPT male quick disconnect coupling in Air Valve & Handle assembly. Match to shop fittings.

- Remove the M6 nuts and M6 x 150mm bolts from side members.

Note: For flat rollers, the installer should make sure the roller assemblies are tightened before using the jack, after installed in the rail, so that the wheels sit about ¼” below the bottom of the wheel tube. Grooved rollers are ok.

- Install the side members into the jack, Fig 1. Replace the M6 nuts and bolts through the tab back into the side member.

Fig. 1 – Installing Side Member

- Place jack assembly on lift rack rails.

- Once side members are adjusted and jack is centered between the rails, tighten the (4) hex bolts (2 per side) to lock in place, Fig 2.

Fig. 2 – Side Member Bolts

- Jack requires a clean, dry and lubricated air source at 125 lbs.

Start up Instructions

- Once the jack is centered between the runways and air is connected, it may require to be run up and down a few times.

- If the jack has issues going up/down completely, follow the following steps.

  - Raise the jack to the full height, oil the guide pins (do not grease) and lower the jack back down. Continue to raise and lower the jack a few times, oil if necessary.

  - With the jack completely lowered loosen the four bolts button head bolts, re-tighten the bolts in a star pattern. Only tighten the bolts enough to flatten the lock washer.

Runway Spacer Installation on Challenger 4P12 & 4015

- Remove the washer/nut from the two inside anchor bolt locations on each column.

- Install the spacer plate over the anchors and tighten the washer/nut back onto anchors.

- Using a torque wrench, torque each anchor to 80 ft-lbs, Fig 3.

Fig. 3 –Spacer Plate Installation

Runway Spacer Installation on other lift models

- After the jack is installed on the hoist rails, lower the hoist to its lowest position. If the jack touches the ground a spacer will be required. A spacer will be installed at the base of each post and will keep the runway from lowering. Spacers can be ordered in different sizes depending on the requirement. Common sizes used are 1 inch tall and ½ inch tall, Fig. 4.

Operation

Fig. 4 –Spacer Plate Installation

Note: Before lifting a vehicle, operate the jack through a couple of cycles to become familiar with the controls.

- Roll jack forward before moving vehicle on lift rack. Be sure vehicle is centered on rack, apply parking brake and chock wheels.

- Roll jack(s) to the vehicle manufacturer’s recommended pick-up points. Extend lift arms to proper lift points. Select the correct height adapter to reduce the gap between top of jack and pick-up point, secure adapter by sliding them under the lift brackets. Adapters can be arranged or stacked in various combinations to directly engage with the vehicle.

- Raise jack by pressing the lever on the air valve to the right to inflate the jack to a desired height or maximum position of approx. 8 inches. Leave the jack inflated in this position, the spring loaded safety blocks are in position should air pressure decrease.	If maximum height isn’t required rest jack into a lock position with the jack still pressurized.

- When lowering jack press the yellow safety release lever on the right side of the jack all the way down with your right hand and push the air valve lever to the left to release the air pressure with your left hand. Both levers must be held firmly until the jack is in the bottom position. If the jack wasn’t at maximum height, you must inflate and raise the jack sufficiently to free the safety catches before pressing the safety lever down and releasing the air pressure.

Maintenance

Weekly:	Inspect jack and its components for damage or excessive wear. Replace parts as required. See parts list.

Oil guide pins-DO NOT GREASE. The guide pins (visible when jack is in the up position) should be oiled every week or sooner if necessary with gear oil (80-90 grade).

Confirm that the clean/filtered shop air is being bused (water separator is recommended). Check the air pressure being used to verify that the minimum of 125 psi is being used.

Check the button-head bolts to ensure that they are tight. The bolts can be tightened by attaching an Allen key to the bolt on the bottom of the guide pin (under the jack) to hold the guide pin securely while the top bolt is being tightened, Fig 4.

Ensure that the rail is clean and free from debris before rolling the jack into position.

Fig. 4 – Bolt Check

IMPORTANT!

Failure to keep lift free of corrosive agents and solvents will lead to reduced service life, which could result in property damage and/or personal injury. If any problems are encountered, contact your local service representative.

Parts Breakdown

| ITEM # | PART # | QTY/JACK | DESCRIPTION |
| --- | --- | --- | --- |
| 1 | WK6-100 | 1 | RJAB6 AIR BAG |
|  | WK9-100 |  | RJAB7.5 & RJAB9 AIR BAG |
| 2 | QRJ9-02-00 | 1 | TOP WELD |
| 3 | QRJ9-01-00 | 1 | BASE WELD |
| 4 | QRJ9-12-00 | 2 | FOOT PAD, PIN |
| 5 | QRJ9-13-00 | 2 | FOOT PAD, SHORT |
| 6 | QRJ9-14-00 | 2 | FOOT PAD, TALL |
| 7 | QRJ9-09-00 | 2 | ARM WELD |
| 8 | QRJ9-08 | 2 | LOCK PIVOT |
| 9 | QRJ9-03 | 4 | LOCK SHAFT |
| 10 | QRJ9-16-00 | 1 | Y-BAR LOCK RELEASE |
| 11 | QRJ9-17 | 1 | LOCK RELEASE ROD |
| 12 | B40467 | 4 | M6 NUT |
| 13 | QRJ9-06 | 4 | LOCK SHAFT STOP |
| 14 | QRJ9-33 | 4 | M12 X 35MM LG. FLAT HEAD BOLT |
| 15 | QRJ9-35 | 4 | M16 LOCK WASHER |
| 16 | QRJ9-36 | 4 | M16 X 35MM LG. BUTTON HEAD CAP SCREW |

| 17 | QRJ9-34 | 4 |  | M6 X 45MM LG. SOCKET HEAD CAP SCREW |
| --- | --- | --- | --- | --- |
| 18 | QRJ9-07 | 4 |  | LOCK RELEASE SPRING |
| 19 | QRJ9-11 | 4 |  | LOCK |
| 20 | QRJ9-23 | 1 |  | LOCK HANDLE COVER |
| 21 | WK9-805 | 4 |  | M10 X 20MM LG. H.H.C.S. |
| 22 | A1153 | 2 |  | 3/8-16 X ¾” LG. LOCKING FLANGE HEX CAP SCREW |
| 25 | QRJ9-05 | 1 |  | VALVE BRACKET |
| 26 | QRJ9-38 | 1 |  | M6 X 12MM LG. FLAT HEAD BOLT |
| 27 | WK15-802 | 1 |  | ¼ NPT X 45 DEGREE STREET ELBOW |
| 28 | WK15-818 | 1 |  | M6 X 30MM LG. H.H.C.S. |
| 29 | 298-2-6 | 1 |  | ¼ NPT CONTROL VALVE |
| 30 | SD121 | 2 |  | ¼ NPTM X ¼ TUBE ELBOW |
| 31 | WK9-700 | 1 |  | ¼ NPT FLOW RESTRICTOR |
| 32 | 00911 | 18” |  | ¼” DIAMETER POLY TUBING |
| 33 | WK15-816 | 2 |  | 3/8-16 X 4-1/2” LG. FLAT HEAD BOLT |
| 34 | QRJ9-39 | 1 |  | M6 X 80MM LG. FLAT HEAD BOLT |
| 35 | QRJ9-37 | 4 |  | Ø5MM X 50MM LG ROLL PIN |
| 36 | QRJ9-22 | 2 |  | ARM STOP |
| 37 | QRJ9-41 | 4 |  | M6 X 20MM LG SOCKET HEAD CAP SCREW |
| 38 | RJAB-01 | 1 |  | DECAL, LIFT AREA |
| 39 | RJAB-02 | 1 |  | DECAL, PULL OUT LEFT |
| 40 | RJAB-03 | 1 |  | DECAL, PULL OUT RIGHT |
| 41 | RJAB-04 | 1 |  | DECAL, OIL GUIDE BARS |
| 42 | RJAB-05 | 1 |  | DECAL, KEEP HANDS CLEAR |
| 43 | RJAB-06 | 1 |  | DECAL, CAUTION CENTER LOAD |
| 44 | RJAB-08 | 1 |  | DECAL, GREEN ENVIRONMENTALLY CLEAN |
| 45 | RJAB-09 | 1 |  | DECAL, NOTICE RAISE LIFT BEFORE LOWERING |
| 46 | RJAB-10 | 1 |  | DECAL, STOP DO NOT USE HANDLE |
| 47 | RJAB-16 | 1 |  | DECAL, 6,000LB CAPACITY |
|  | RJAB-15 |  |  | DECAL, 7,500LB CAPACITY |
|  | RJAB-17 |  |  | DECAL, 9,000LB CAPACITY |
| 48 | RJAB-20 | 1 |  | DECAL, WK LOGO |

| PART # | QTY/LIFT | DESCRIPTION |
| --- | --- | --- |
| 9-103 | 1 | SAFETY BLOCK ASSEMBLY |
| QRJ9-34 | 4 | M6X45MM LG. SOCKET HEAD BOLT |
| QRJ9-07 | 4 | LOCK SPRING |
| QRJ9-11 | 4 | LOCK PAWL |
| 9-106 | 1 | LOCK RELEASE ROD ASSEMBLY |
| QRJ9-17 | 1 | LOCK RELEASE ROD |
| B31061 | 2 | M6 NYLON LOCKNUT |
| 9-108 | 1 | AIR VALVE ASSEMBLY |
| WK-298200-6-369 | 1 | LIFT CONTROL VALVE, ¼ NPT |
| WK15-802 | 1 | ¼ NPT X 45 DEGREE ST. ELBOW |
| SD121 | 1 | ¼ NPT X 90 DEGREE TUBE ELBOW |
| WK9-700 | 1 | ¼ NPT FLOW RESTRICTOR |
| 9-109 | 1 | AIR TUBE ASSEMBLY |
| 00910 | 24 IN | ¼” AIR LINE |
| 9-118 | 1 | EXTENSION ARM ASSEMBLY |
| QRJ9-22 | 2 | ARM STOP LOOP |
| QRJ9-37 | 4 | 5MM X 50MM ROLL PIN |
| QRJ9-09-00 | 2 | ARM WELD |
| 9-119 | 1 | EXTENSION ARM LOOP (STOP) |
| QRJ9-22 | 2 | ARM STOP LOOP |
| QRJ9-37 | 4 | 5MM X 50MM ROLL PIN |
| 9-123 | 1 | LOCK LEG SET |
| QRJ9-03 | 4 | LOCK LEG |
| QRJ9-06 | 4 | LOCK LEG STOP |
| QRJ9-33 | 4 | M12 X 35MM LG. FLAT HEAD BOLT |
| QRJ9-35 | 4 | M16 LOCK WASHER |
| QRJ9-36 | 4 | M16 X 35MM LG. BUTTON HEAD BOLT |

| D FLOOR | B |
| --- | --- |
| PIT WALL |  |
|  | G A |

TYPE 1

TYPE 2

D	B

TYPE 4

| MODEL 6K 7K 9K 15K 18K 25K (CIRCLE ONE) |  |  |
| --- | --- | --- |
| TYPE (CIRCLE ONE) 1 2 3 4 5 |  |  |
| ITEM | DESCRIPTION | DIMENSION |
| A | Distance between the pit walls |  |
| B | Inside width of the rail |  |
| C | Inside height of the rail |  |
| D | Height of the guard rail |  |
| E | Distance from the floor to the top of the rail |  |
| F | Width of the lip (rail edge) |  |
| G | Thickness of rail |  |

| D FLOOR |  |
| --- | --- |
| PIT WALL | B |

C

E C

A

FLOOR

TYPE 3

PIT WALL

A

TYPE 5

D	B

FLOOR

F E

D	PIT

PLEASE NOTE:

FLOOR

B	C

PIT

WALL	A

WALL	 		C

G

A

- IF THE STYLE OF THE RAIL DOES NOT MATCH ANY OF THE DESIGNS SHOWN THEN PLEASE SUBMIT A CROSS-SECTIONAL DRAWING OF THE RAIL WITH ALL OF THE REQUIRED MEASURMENTS.

- WHEN THERE ARE OTHER ITEMS IN THE PIT THAT MAY INTERFERE WITH THE ROLLING OF THE AIR JACK (i.e. PIT COVERS, OIL DRAIN PANS, LIGHTING IN THE RAILS, OTHER RAILS, etc.) THEN PLEASE SUBMIT A DRAWING THAT INCLUDES THE LOCATION OF THE ITEMS & MEASUREMENTS.

It is the customer’s responsibility to ensure the pit structure is sufficient to carry loads. This pit spec sheet only provides dimensional measurements for jack installation. Customer assumes all liability for structural design and integrity.

| DATE: | MEASURED BY: | DEALER: | SIGNATURE: |
| --- | --- | --- | --- |

"

TOP OF RUNWAY

TYPE 1

C

B

F

F

H

FLOOR

A

TOP OF RUNWAY

D	TYPE 2

E

C

A

| HOIST MANF: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HOIST MODEL: |  |  |  |  |  |  |  |  |
| HOIST CAPC: |  |  |  |  |  |  |  |  |
| HOIST RAIL TYPE (CIRCLE ONE) |  | 1 |  | 2 |  | 3 | 4 | 5 |
| MODEL OF JACK REQUESTED (CIRCLE ONE) |  | 6K | 7K | 9K |  | 15K | 18K | 25K |
| ITEM | DESCRIPTION |  |  |  | DIMENSION |  |  |  |
| A | Distance between the rails |  |  |  |  |  |  |  |
| B | Height of the runway |  |  |  |  |  |  |  |
| C | Distance between bottom of runway and floor |  |  |  |  |  |  |  |
| D | Distance between top of runway and floor |  |  |  |  |  |  |  |
| E | Distance between top of rail and floor |  |  |  |  |  |  |  |
| F | Width of rail |  |  |  |  |  |  |  |
| G | Height of rail |  |  |  |  |  |  |  |
| H | Thickness of rail |  |  |  |  |  |  |  |

F

B	 	G		D E

H

FLOOR

TOP OF RUNWAY

TYPE 3

C

A

B	F

D

H	E

FLOOR

A

A

| TOP OF RUNWAY | G |
| --- | --- |
| TYPE 4 B | D |

E

C

FLOOR

A

TOP OF RUNWAY	TOP OF RUNWAY

TYPE 5

B	F

D

TYPE 6

B	F

G	D

C	H	G	E

FLOOR

E

C

FLOOR

PLEASE NOTE:

IF THE STYLE OF THE RAIL DOES NOT MATCH ANY OF THE ABOVE DESIGNS THEN PLEASE SUBMIT A CROSS-SECTIONAL DRAWING OF THE RAIL WITH ALL OF THE REQUIRED MEASURMENTS.

| DATE: | MEASURED BY: | DEALER: | SIGNATURE: |
| --- | --- | --- | --- |

W-K Industries Limited

Exclusive Manufacturers of Air-Up Series Jacks

Toll: 1-800-353-0550 -Tel: 905-602-6457 -Fax: 905-602-6562

www.w-kindustries.com

# Appendix 3.2J: State Audit Requirements

##### State of New Jersey

Department of Environmental Protection (DEP) Bureau of Mobile Sources (BMS)

Quality Assurance & Auditing Section (QAAS)

Motor Vehicle Inspection & Maintenance Program Emissions Equipment Audit Protocol

Version 2.2 – 10/29/2014

  - Requirements

This protocol sets forth the criteria to be utilized by an auditor representing the Department of Environmental Protection (DEP) for performing an audit of the emissions testing equipment, documenting the results of the audit, outlining the steps to be taken should any of the emissions testing equipment fail any portion of an audit, defining the schedule for audits, and transmitting results to the vehicle inspection database (VID).

  - Schedule

This audit protocol can be implemented on a regular or unscheduled basis at any facility that performs inspections (CIFs, PIFs, PFFs, SIFs, BITs, and MITs) to ensure that the emissions testing equipment meets the appropriate criteria during enhanced emissions inspections. The ultimate objective of the audits is to maximize the overall effectiveness of the program through ensuring accurate and consistent testing at all facilities. All unscheduled audits will be conducted unannounced without prior notice being given to the facility.

In an effort to minimize the impact of audits on the facility and customer wait times, audits will be conducted as follows:

For all facilities, audits will normally be conducted during typical business hours. These hours are 8:00 am to 5:00 pm, Monday through Friday, excluding State holidays. Audits can be conducted outside of these hours with the audit supervisor’s approval in advance as well as the concurrence of the station owner.

Any station that is experiencing high wait times and is operating at capacity will not be audited. The auditor should contact their supervisor and/or the audit supervisor before leaving a station that is experiencing high wait times to inform them of the situation.

  - Required Equipment

The following equipment will be required to implement the audit procedures:

    - Audit laptop

    - 1997 or newer vehicle

    - OBD verification tester capable of simulating all in use vehicle communications protocols

    - SAE-J1978/1939 compliant OBD scan tool

    - Weather station and/or multi-meter with thermocouple attachment

    - Sample vehicle registration barcodes

    - Handheld optical tachometer

    - 3 Neutral Density Filters (NDFs) of known opacity spanning the typically used part of the opacity scale

  - Procedure

Upon entering the station, an auditor will contact and inform the Station Manager that an emissions equipment audit will be performed in accordance with the procedures and schedule outlined herein. At such time that the Station Manager is informed of the audit, no further servicing of any equipment on the workstation that is subject to audit may occur without the expressed approval of the auditor. The Station Manager will not interfere with the commencement of an audit. The auditor will not perform any adjustments or repairs to the station’s equipment prior to, during, or after the audit. An auditor can perform a visual observation of the interior of the workstation to check for tampering.

Audits will be performed upon a workstation in its as-is/in-service condition through the state audit menus located in the workstation software. The auditor will be required to scan his ID badge and enter his PIN or biometrics to enter “state audit mode.” This information must remain confidential to the auditor. Every time an auditor enters “state audit mode,” a special audit record will be written. At the conclusion of an audit the results and detailed data elements will be automatically transmitted to the VID.

Audits will be performed using the audit component of the software installed on the workstation. Audit functions will be performed on the OBD communication link, diesel opacity meter (including ancillary equipment), and any other required inspection equipment to ensure proper operation. The Decentralized Equipment Audit Inspection Report (DE-AIR) form and audit software contains the component criteria to be used during the test procedures. Either the audit procedures described below, the DE-AIR form, or the workstation audit software contains the limits for determining the pass/fail results for each individual audit element.

At the start of an audit the software will automatically populate as much information as possible in the provided boxes under the ‘Information” tab on the audit software. These include:

    - Station Name

    - Start and End Date/Time

    - Workstation Cabinet Serial Number

    - Opacity Meter Serial Number

    - OBD Module Serial Number

    - OBD Module Firmware Version

    - Workstation ID

    - Station Manager or Recipient

    - DEP Representative conducting audit

    - Status of workstation at completion of audit

If the equipment passes an audit, the auditor will:

    - Inform the Station Manager that the workstation can be used for official inspections.

    - Complete and sign the audit summary sheet and DE-AIR form as necessary.

    - Provide a copy to the Station Manager and request the form be signed for concurrence.

If the equipment fails an audit, the auditor will:

    - Set the “State Audit Lockout” in the workstation audit software.

    - Notify the Station Manager that the workstation has been placed out of service.

    - Complete and sign the audit summary sheet and DE-AIR form as necessary.

    - Provide a copy to the Station Manager and request the form be signed for concurrence.

    - Explain the details of the failure to the Station Manager and the re-audit procedure including relevant DEP contact information.

    - Request the Station Manager contact the appropriate repair technician and make arrangements for repair and/or replacement of the equipment that failed the audit.

Any component of the equipment that fails an audit check is to be re-calibrated (if applicable) by the repair technician, station manager or auditor. Following any required service or repair, the auditor will perform a follow-up audit, also referred to as a re-audit, to ensure the equipment passes the audit.

If a repair technician makes corrections while the auditor is still present, the auditor will:

      - Immediately perform a re-audit of the equipment in order to minimize the amount of workstation downtime, provided there is sufficient time left in the auditor’s workday.

If the equipment passes the re-audit, the auditor will:

      - Remove the “State Audit Lockout” from the workstation audit software or the State management console.

      - Inform the Station Manager that the workstation can be used for official inspections.

      - Complete and sign the audit summary sheet and DE-AIR form as necessary.

      - Provide a copy to the Station Manager and request the form be signed for concurrence.

If the repair technician repairs the workstation but the auditor is not available to re-audit, the workstation can be unlocked by the audit supervisor through the State management console if:

- The repair technician has corrected the problem and performs a self-audit of the equipment in question. The self-audit shall be performed in accordance with the methods and tolerances set forth below and must be completed to the satisfaction of the Station Manager and the NJDEP. This information must be recorded on the DE-AIR form and signed by the repair technician and Station Manager. The repair technician must also indicate his company name on the DE-AIR.

- The Station Manager faxes a copy of the self-audit documentation and repair work order to NJDEP Bureau of Mobile Sources at (609) 530-4504. The fax cover sheet must be labeled “Self-Audit - Attention: Audit Supervisor”.

AUDIT FUNCTIONS

Part 1 - OBD System

The inspection workstation must include the hardware and software necessary to access the onboard computer systems on all OBD eligible vehicles, check for proper MIL (Malfunction Indicator Light) command, determine readiness, and recover stored fault codes using the SAE standardized link. In addition to the physical cable check included in this section of these procedures, the system will be subjected to additional audit checks to ensure communications functionality with all in use protocols as well as verify proper readiness monitor status and fault code retrieval from tested vehicles.

The first part of the OBD audit is a visual check of the OBD module, associated wiring, and diagnostic link connectors for broken casing, exposed wiring, bent pins, tampering, etc. Next, an OBD module self-check shall be performed in accordance with manufacturer’s procedure.

The system shall then be subject to one of two types of audits. The primary type is a full system check, using a verification tester to test all in use protocols. The secondary type will be performed in the event that the primary audit tool is not available. This will entail the use of the auditor’s vehicle and an OBD scan tool to verify the results.

Primary OBD Audit - Using Verification Tester

- The first step of the audit is to configure the verification tester using the OBDII verification tester software running on the audit laptop. The verification tester shall be capable of holding different scenarios; therefore one scenario for each in use protocol to be tested should be set up. The auditor should select either “Completed,” “Not Completed,” or “Unsupported” for each of the 11 monitors in the verification tester software.	All three types of settings should be included in each individual scenario. A protocol should then be randomly selected in the upper right of the screen. The status for the MIL (either “ON” or “OFF”) shall be selected and should be varied among scenarios.	From 1 to 6 emissions-related Diagnostic Trouble Codes (DTCs) should also be entered in the Stored DTCs section of the screen. All data from each scenario should be recorded on the auditor’s worksheet.

- Once the desired test settings are configured in the verification tool software the tester should be programmed with these scenarios following the manufacturer’s instructions.

- The auditor will take the verifier to the workstation to be audited and the OBD module should then be plugged into the J1962 diagnostic connector on the

endplate of the verification tool. The workstation should be set in OBD audit mode and the prompts on the screen followed to have the OBD module scan the verification tool. The results of the scan should be displayed on the screen. The status of the MIL and readiness monitors along with any retrieved DTCs should be compared to the values on the auditor’s worksheet.

- The pass/fail result for each item should be marked on the left side of the form. For each item, if the readiness monitor, MIL status, or retrieved DTC matches the value from the auditor’s worksheet, the “pass” box should be marked; otherwise the “fail” box should be marked. This test shall be repeated for each supported protocol using a different scenario for each.

Secondary OBD Audit - Using Auditor’s Vehicle

- For the secondary method, the first step is to scan the audit vehicle with an OBD compliant scan tool and record the results for readiness, MIL status and DTCs on the auditor’s worksheet. Then connect the test system to the vehicle and the results will be displayed on the screen. Again, for each item, if the readiness monitor, MIL status, or retrieved DTC matches the value on the auditor’s worksheet, the “pass” box should be marked; otherwise the “fail” box should be marked.

In the event that an OBD protocol fails to communicate with the workstation while performing either the primary or secondary audit type, the following steps will be taken:

  - The workstation’s OBD module will be reset.

  - The auditor’s verifier or vehicle will be scanned with an OBD compliant scan tool to ensure the verifier or vehicle is functioning normally.

  - The audit will be repeated to determine the final outcome.

Any OBD related failure will result in the audited workstation being locked out from testing using the “State Audit Lockout” until the equipment is repaired and has passed a re-audit.

Part 2 – Diesel Opacity Meter Audit

To perform the audit, the auditor will login to the workstation and select “Opacity Meter” from the state audit menu. For Stations that conduct opacity testing and do not have a workstation the auditor shall complete the data entry using the State management console.

The first part of the audit shall be a visual verification of the opacity meter to ensure all required equipment is available. This shall include, but not be limited to: smoke head,

oil temperature probe, RPM sensors, and other ancillary equipment. Part of this check shall also include examination of all equipment connections and cables for signs of excessive wear or damage. Lastly this check shall include verification that the Station’s neutral density filter (NDF) is not expired and the proper certification documentation is available.

Next, the opacity meter will be subject to an audit of the internal weather station which will be tested for both temperature and barometric pressure accuracy using the auditor’s weather station.

The RPM probes will then be checked for accuracy using the auditor’s hand-held optical tachometer or an OBD scanner.

Next, the oil temperature probes will be checked for accuracy using the auditor’s weather station or a multi-meter with a thermocouple attachment.

Finally, the smoke head will be checked for accuracy using the auditor’s three NDFs.

If an opacity meter fails any component of the audit or if an audit cannot be performed on the opacity meter due to an equipment malfunction, it shall be locked out from testing using the “State Audit Lockout” until the equipment is repaired and has passed a re- audit.

MVC AUDITS

CIF Audits

MVC is responsible for conducting audits at the CIFs to ensure the quality and integrity of the emission programs. MVC conducts the following audits:

    - Covert Audits are unannounced audits to ensure that all licensed inspectors are following all procedures and protocols set by the State.

    - Post-Covert Audits are interviews conducted with the responsible inspector if any infractions are discovered.

    - Remote Observation Audits are unannounced audits to ensure that all licensed inspectors are followed all policies, procedures, and protocols related to the inspection process. These audits are performed by stationing an auditor(s) at a discreet distance from a Central Lane with the best possible view of the inspection process.

    - Performance Audits are performed on an ongoing basis to ensure that every inspector has full knowledge of procedures, policies, during the inspection process.

    - Reconciliation Audits are conducted on a monthly basis to review the monthly billing invoice from the contractor.

    - Sticker Inventory Audits are conducted twice a year at each CIF to ensure that the contractor is following the established protocol. The audit determines whether all stickers assigned to the CIF are accounted for, all journal entries are completed, and all security and procedures for the issuance of all stickers are followed.

    - Warehouse/Distribution Center Audits are done on a periodic or as needed basis at the contractor sticker distribution center to ensure the established protocol (to be developed by the contractor and approved by the State) is followed.

The auditor will need to record each audit on the web portal. The auditor will need to record date of the audit, CIF and auditor’s description. In addition, the auditor will need to record the results of these three (3) questions listed below as yes, no or not applicable.

    - Are the inspection stickers and related items properly secured?

    - Are inspection logs, ledgers and inventories maintained in accordance within State policies and procedures?

    - Are inspector(s) conducting proper vehicle inspections?

Audit Schedule

DEP audits will be conducted according to the following schedule:

    - All facilities with more than two lanes may be audited, unscheduled and unannounced, during normal operating hours. The core operating hours for DEP staff are Monday through Friday from 8 AM until 5:00 PM.

    - For all two-lane facilities, audits will be scheduled by appointment on a normal workday or as otherwise agreed. Only one (1) lane will be closed at a time at two-lane facilities.

    - For all single lane facilities, audits will be scheduled by appointment on a normal workday or as otherwise agreed.

This schedule does not limit the authority of DEP/MVC to conduct an audit at any time.

Within 24 hours’ notice provided by the State, the contractor shall provide access during non- working hours to perform any checks that the State may require.

The contractor shall immediately cease vehicle testing at any lane or station where the equipment or system in the inspection lane or station fails a quality control or quality assurance audit. The contractor shall repair or correct the lane or station, and the State shall verify that the repair or correction has been made, before vehicle testing can resume at that lane or station.

PIF Audits

MVC is responsible for conducting audits at the PIFs to ensure the quality and integrity of the emission programs. MVC conducts the following audits:

    - Covert Audits are unannounced audits to ensure that all licensed inspectors are following all procedures and protocols set by the State.

    - Post-Covert Audits are interviews conducted with the responsible inspector if any infractions are discovered.

    - Remote Observation Audits are unannounced audits to ensure that all licensed inspectors are followed all policies, procedures, and protocols related to the inspection process. These audits are performed by stationing an auditor(s) at a discreet distance from a Central Lane with the best possible view of the inspection process.

    - Performance Audits are performed on an ongoing basis to ensure that every inspector has full knowledge of procedures, policies, during the inspection process.

    - Reconciliation Audits are conducted on a monthly basis to review the monthly billing invoice from the contractor.

    - Record audits are conducted by reviewing or screening at least monthly to assess station performance and identify problems that may indicate potential fraud or incompetence. This includes but not limited to the following:

- Automated record analysis to identify statistical inconsistencies, unusual patterns, and other discrepancies;

- Visits to inspection stations to review records not already covered in the electronic analysis (if any); and

- Comprehensive accounting for all official forms that can be used to demonstrate compliance with the program.

- Equipment audits. During overt site visits, auditors shall conduct quality control evaluations of the required test equipment, including (where applicable):

The auditor will need to record each audit on the web portal. The auditor will need to record date of the audit, PIF and auditor’s description. In addition, the auditor will need to record the results of these four (4) questions listed below as yes, no or not applicable.

    - Are the inspection stickers and related items properly secured?

    - Are inspection logs, ledgers and inventories maintained in accordance within State policies and procedures?

    - Are the current signs, licenses and rate charts posted in full public view?

    - Are inspector(s) conducting proper vehicle inspections?

# Appendix 3.3A: Mechanical Inspection Process

| RSC_CD | RSC_DESC | RSC_FAIL_EFFECT_ON_INSPECTION | RSC_RSCC_CD | RSC_DISPLAY_SEQ |
| --- | --- | --- | --- | --- |
| 2 | Service brake: insufficient braking effort | F | B01 | 1 |
| 5 | Service brake: insufficient pedal reserve | F | B01 | 3 |
| 8 | Service brake: rupture in brake system | 4 | B01 | 5 |
| 19 | Parking brake does not hold vehicle | F | B02 | 1 |
| 85 | Headlights totally inoperative | 4 | Q01 | 1 |
| 107 | Tail lights ‐ both out | 4 | Q01 | 3 |
| 108 | 3rd stoplight is out | A | Q01 | 5 |
| 115 | Stop lights ‐ all lights out | 4 | Q01 | 7 |
| 119 | Stop light(s) out or missing | F | Q01 | 9 |
| 123 | Turn/warning signal out or missing | F | Q01 | 11 |
| 205 | Pedals ‐ pads missing and non-skid surface | F | B01 | 7 |
| 230 | Fuel system leak | 4 | Q04 | 1 |
| 232 | Unusually dangerous conditions | 4 | Q04 | 3 |
| 239 | Shift indicator missing/incorrect | F | Q04 | 5 |
| 292 | Tire has exposed ply or cord | 4 | Q03 | 1 |
| 328 | Excessive looseness in steering box (lash) | 4 | S01 | 1 |
| 329 | Suspension ‐ wheel rock | F | S01 | 3 |
| 331 | Suspension ‐ broken/excess looseness/linkage | F | S01 | 5 |
| 339 | Dangerously loose tie rod, ball joint, pitman arm | 4 | S01 | 7 |
| 353 | Parking brake has insufficient reserve | F | B02 | 3 |
| 363 | Exhaust system leak | F | X01 | 1 |
| 364 | Exhaust system leak venting into vehicle | 4 | X01 | 3 |
| 407 | Brake equalization: Failure | F | B03 | 1 |
| 422 | Steering/Suspension: other failure | F | S01 | 9 |
| 425 | Glass: other failure | F | Q02 | 1 |
| 445 | Turn/Warning Lights: other failure | F | Q01 | 13 |
| 448 | Marker/Clear/Reflector: Failure | F | Q01 | 15 |
| 449 | Wiring/Switches: Failure | F | Q01 | 17 |
| 450 | Exhaust System: other failure | F | X01 | 5 |
| 467 | Motorcycle Failure | F | Q05 | 1 |
| 474 | Glass failure: windshield | F | Q02 | 3 |
| 475 | Glass failure: rear windshield | F | Q02 | 5 |
| 476 | Glass failure: left front window | F | Q02 | 7 |
| 477 | Glass failure: right front window | F | Q02 | 9 |
| 478 | Glass failure: left rear window | F | Q02 | 11 |
| 479 | Glass failure: right rear window | F | Q02 | 13 |
| 480 | Glass failure: tinting | F | Q02 | 15 |
| 481 | Mirror failure: left mirror | F | Q02 | 17 |
| 482 | Mirror failure: right mirror | F | Q02 | 19 |
| 483 | Mirror failure: interior mirror | F | Q02 | 21 |
| 484 | Wheel failure | F | Q03 | 3 |
| 485 | Tire failure | F | Q03 | 5 |
| 487 | Fuel component contacting any moving parts | 4 | Q04 | 7 |
| 488 | Shattered windshield impairing driver's vision | 4 | Q02 | 23 |
| 511 | Service brake: Advisory | A | B01 | 9 |
| 512 | Service brake: other failure | F | B01 | 11 |
| 513 | Service brake: other 48 hour failure | 4 | B01 | 13 |
| 514 | Parking brake: other failure | F | B02 | 5 |
| 515 | Registration: Advisory | A | C02 | 3 |
| 516 | Insurance card: Advisory | A | C03 | 3 |
| 517 | Plates: Advisory | A | C04 | 3 |
| 518 | Collector Vehicle Rejection | F | C05 | 1 |
| 520 | Headlight rejection: left high/low | F | Q01 | 19 |
| 521 | Headlight rejection: right high/low | F | Q01 | 21 |
| 522 | Headlights: Advisory | A | Q01 | 23 |
| 523 | Stop lights: other failure | F | Q01 | 25 |
| 524 | Stop lights: Advisory | A | Q01 | 27 |
| 525 | Tail lights: other failure | F | Q01 | 29 |
| 526 | Horn failure | F | Q04 | 9 |

| 527 | Wipers inoperative/missing/damaged | F | Q04 | 11 |
| --- | --- | --- | --- | --- |
| 531 | Driver license: Advisory | A | C01 | 1 |
| 533 | Odometer: Advisory | A | C06 | 1 |
| 2001 | License | S | C21 | 45 |
| 2002 | Registration | 3 | C21 | 47 |
| 2003 | Insurance | S | C21 | 49 |
| 2004 | Plates | 3 | C21 | 51 |
| 2005 | Title | S | C21 | 53 |
| 2101 | Service Door | S | Q21 | 55 |
| 2102 | Entry Steps | 3 | Q21 | 57 |
| 2103 | Side Doors | S | Q21 | 59 |
| 2104 | Doors/Steps: Head Bump Pad | 3 | Q21 | 61 |
| 2105 | Grab handles | S | Q21 | 63 |
| 2106 | Stirrups | 3 | Q21 | 65 |
| 2107 | Stepwell Light | 3 | Q21 | 67 |
| 2108 | Doors/Steps: Seals | S | Q21 | 69 |
| 2109 | Step Tread | 3 | Q21 | 71 |
| 2120 | Emergency Egress: Lettering ‐ out of service | S | Q22 | 73 |
| 2121 | Emergency Egress: Ignition ‐ Interlock | S | Q22 | 75 |
| 2122 | Emergency Egress: Door buzzer | S | Q22 | 77 |
| 2123 | Emergency Egress: Door handles | S | Q22 | 79 |
| 2124 | Emergency Egress: Head bump pads | 3 | Q22 | 81 |
| 2125 | Backup lights with door open, A, B, C, D | S | Q22 | 83 |
| 2126 | Door slide bar | S | Q22 | 85 |
| 2127 | Emergency Egress: Reflective tape | S | Q22 | 87 |
| 2128 | Emergency Egress: Door hinges | S | Q22 | 89 |
| 2129 | Push out instructions ‐ L | S | Q22 | 91 |
| 2130 | Push out instructions ‐ R | S | Q22 | 93 |
| 2131 | Roof hatch instructions ‐ F | S | Q22 | 95 |
| 2132 | Roof hatch instructions ‐ R | S | Q22 | 97 |
| 2133 | Push out buzzer broken ‐ L | S | Q22 | 99 |
| 2134 | Push out buzzer broken ‐ R | S | Q22 | 101 |
| 2135 | Roof hatch buzzer broken ‐ F | S | Q22 | 103 |
| 2136 | Roof hatch buzzer broken ‐ R | S | Q22 | 105 |
| 2137 | Emergency exit handles | S | Q22 | 107 |
| 2140 | Crash Barriers Mounting | S | Q23 | 109 |
| 2141 | Crash Barriers: Padding | 3 | Q23 | 111 |
| 2142 | Crash Barriers: Covering ‐ out of service | S | Q23 | 113 |
| 2143 | Crash Barriers: Covering ‐ 30 day | 3 | Q23 | 115 |
| 2144 | Crash Barriers: Stanchions | 3 | Q23 | 117 |
| 2150 | Missing Sunshield | 3 | Q24 | 119 |
| 2151 | Non transparent Sunshield | 3 | Q24 | 121 |
| 2152 | Nonadjustable Sunshield | 3 | Q24 | 123 |
| 2153 | Broken Sunshield ‐ out of service | S | Q24 | 125 |
| 2154 | Broken Sunshield ‐ 30 day | 3 | Q24 | 127 |
| 2160 | Fire Extinguisher | S | Q25 | 129 |
| 2161 | Emergency Equip Mounting | S | Q25 | 131 |
| 2162 | Emergency Equip: Charge | S | Q25 | 133 |
| 2163 | Emergency Equip: Inspection Tag | S | Q25 | 135 |
| 2164 | First Aid Kit Mounting | 3 | Q25 | 137 |
| 2165 | Emergency Equip: 2 in Bandage Compress | 3 | Q25 | 139 |
| 2166 | Emergency Equip: 3 in Bandage Compress | 3 | Q25 | 141 |
| 2167 | Emergency Equip: Adhesive Bandage | 3 | Q25 | 143 |
| 2168 | Emergency Equip: Adhesive Tape Rolls | 3 | Q25 | 145 |
| 2169 | Emergency Equip: Non‐sterile Triangular Bandages | 3 | Q25 | 147 |
| 2170 | Emergency Equip: Pair rounded end scissors | 3 | Q25 | 149 |
| 2171 | Emergency Equip: Sterile Eye Patch | 3 | Q25 | 151 |
| 2172 | Emergency Equip: Sterile Gauze Pads 36x36 | 3 | Q25 | 153 |
| 2173 | Emergency Equip: Sterile Gauze Pads 3x3 | 3 | Q25 | 155 |
| 2174 | Emergency Equip: Sterile Gauze Roller Bandage | 3 | Q25 | 157 |

| 2175 | First Aid Kit Size | 3 | Q25 | 159 |
| --- | --- | --- | --- | --- |
| 2176 | Emergency Equip: Portable Warning Device | 3 | Q25 | 161 |
| 2177 | Emergency Equip: Wrecking Bar | 3 | Q25 | 163 |
| 2178 | Emergency Equip: Seat Belt Cutter | S | Q25 | 165 |
| 2190 | Speedometer | 3 | Q26 | 167 |
| 2191 | Oil gauge | 3 | Q26 | 169 |
| 2192 | Voltmeter | 3 | Q26 | 171 |
| 2193 | High Beam Indicator | 3 | Q26 | 173 |
| 2194 | Water temp | 3 | Q26 | 175 |
| 2195 | Air / Vacuum Brake Gauge | S | Q26 | 177 |
| 2196 | Amp meter | 3 | Q26 | 179 |
| 2197 | Odometer | 3 | Q26 | 181 |
| 2198 | Fuel gauge | 3 | Q26 | 183 |
| 2199 | Horn | 3 | Q26 | 185 |
| 2210 | Gear Shift Indicator Mounting | 3 | Q27 | 187 |
| 2211 | Gear Shift Indicator Pattern | 3 | Q27 | 189 |
| 2212 | Gear Shift Indicator Shift detent | 3 | Q27 | 191 |
| 2213 | Gear Shift Indicator Floor boot | S | Q27 | 193 |
| 2220 | Heaters: Shield hose | S | Q28 | 195 |
| 2221 | Defogging fans ‐ L ‐ out of service | S | Q28 | 197 |
| 2222 | Defogging fans ‐ L ‐ 30 day | 3 | Q28 | 199 |
| 2223 | Defogging fans ‐ R ‐ out of service | S | Q28 | 201 |
| 2224 | Defogging fans ‐ R ‐ 30 day | 3 | Q28 | 203 |
| 2225 | Defrosters ‐ L | 3 | Q28 | 205 |
| 2226 | Defrosters ‐ R | 3 | Q28 | 207 |
| 2227 | Heaters ‐ L | 3 | Q28 | 209 |
| 2228 | Heaters ‐ R | 3 | Q28 | 211 |
| 2229 | Rear Heaters ‐ 30 day | 3 | Q28 | 213 |
| 2230 | Heaters: Regulating valve | 3 | Q28 | 215 |
| 2240 | Seat Mounting ‐ out of service | S | Q29 | 217 |
| 2241 | Seat Mounting ‐ 30 day | 3 | Q29 | 219 |
| 2242 | Seat Condition | 3 | Q29 | 221 |
| 2243 | Seat Missing | S | Q29 | 223 |
| 2244 | Seat belts | S | Q29 | 225 |
| 2245 | Seat Belt Locking retractor | S | Q29 | 227 |
| 2246 | Restraining device | S | Q29 | 229 |
| 2247 | Dome light | 3 | Q29 | 231 |
| 2248 | Interior: Cleanliness | 3 | Q29 | 233 |
| 2249 | Interior: Ceiling | 3 | Q29 | 235 |
| 2250 | Interior: Side walls ‐ out of service | S | Q29 | 237 |
| 2251 | Interior: Side walls ‐ 30 day | 3 | Q29 | 239 |
| 2252 | Interior: Storage compartment | 3 | Q29 | 241 |
| 2253 | Air conditioning/mount location ‐ out of service | S | Q29 | 243 |
| 2254 | Air conditioning/mount location ‐ 30 day | 3 | Q29 | 245 |
| 2255 | Floor ‐ out of service | S | Q29 | 247 |
| 2256 | Floor ‐ 30 day | 3 | Q29 | 249 |
| 2257 | Aisle/clearance | 3 | Q29 | 251 |
| 2300 | Identification: Lettering ‐ 30 day | 3 | Q30 | 253 |
| 2301 | Identification: Color | 3 | Q30 | 255 |
| 2302 | Advertising | 3 | Q30 | 257 |
| 2303 | Identification: GVW + 26,001 | 3 | Q30 | 259 |
| 2310 | Body: Conditioning ‐ out of service | S | Q31 | 261 |
| 2311 | Body: Conditioning ‐ 30 day | 3 | Q31 | 263 |
| 2312 | Bumper ‐ out of service | S | Q31 | 265 |
| 2313 | Bumper ‐ 30 day | 3 | Q31 | 267 |
| 2314 | Body: Rub Rails | 3 | Q31 | 269 |
| 2315 | Fenders | 3 | Q31 | 271 |
| 2316 | Crossing Arm ‐ out of service | S | Q31 | 273 |
| 2317 | Crossing Arm ‐ 30 day | 3 | Q31 | 275 |
| 2318 | Body: Hoods | S | Q31 | 277 |

| 2330 | Warning Equipment: Red/Amber | S | Q32 | 279 |
| --- | --- | --- | --- | --- |
| 2331 | Warning Equipment: Switching | S | Q32 | 281 |
| 2332 | Warning Equipment: Indicator Light | S | Q32 | 283 |
| 2333 | Warning Equipment: Stop Arm | S | Q32 | 285 |
| 2334 | Warning Equipment: Brake Pilot Light | 3 | Q32 | 287 |
| 2335 | Warning Equipment: Backup Alarm | S | Q32 | 289 |
| 2336 | Warning Equipment: Roof Strobe light | S | Q32 | 291 |
| 2340 | Lights: Parking | 3 | Q33 | 293 |
| 2341 | Directional signal ‐ out of service | S | Q33 | 295 |
| 2342 | Directional signal ‐ 30 day | 3 | Q33 | 297 |
| 2343 | Lights: Hazard | 3 | Q33 | 299 |
| 2344 | Lights: Marker | 3 | Q33 | 301 |
| 2345 | Tail Lights ‐ out of service | S | Q33 | 303 |
| 2346 | Tail Lights‐ 30 day | 3 | Q33 | 305 |
| 2347 | Lights: Clearance | 3 | Q33 | 307 |
| 2348 | Headlights L out | 3 | Q33 | 309 |
| 2349 | Headlights R out | 3 | Q33 | 311 |
| 2350 | Headlights L Aim | 3 | Q33 | 313 |
| 2351 | Headlights R Aim | 3 | Q33 | 315 |
| 2352 | Stop Lights ‐ out of service | S | Q33 | 317 |
| 2353 | Stop Lights ‐ 30 day | 3 | Q33 | 319 |
| 2354 | Lights: License Plate | 3 | Q33 | 321 |
| 2355 | High beam L | S | Q33 | 323 |
| 2356 | High beam R | S | Q33 | 325 |
| 2357 | Lights: Wiring/switching | S | Q33 | 327 |
| 2358 | Exterior Indicator Lights | 3 | Q33 | 329 |
| 2359 | Exterior Backup Lights | S | Q33 | 331 |
| 2360 | Lights: Other ‐ out of service | S | Q33 | 333 |
| 2361 | Lights: Other ‐ 30 day | 3 | Q33 | 335 |
| 2362 | ID Lights | 3 | Q33 | 337 |
| 2363 | Exterior Reflectors | 3 | Q33 | 339 |
| 2380 | Glazing: Windshield | S | Q34 | 341 |
| 2381 | Glazing: Left Side | S | Q34 | 343 |
| 2382 | Glazing: Right Side | S | Q34 | 345 |
| 2383 | Rear Glazing ‐ out of service | S | Q34 | 347 |
| 2384 | Visual obstruction | 3 | Q34 | 349 |
| 2385 | Glazing: Gradient tint | 3 | Q34 | 351 |
| 2386 | Rear View Mirror | S | Q31 | 353 |
| 2387 | Cross Over Mirror | S | Q31 | 355 |
| 2388 | Interior Mirror | S | Q29 | 232 |
| 2389 | Rear View Convex Mirror | S | Q31 | 359 |
| 2400 | Wiper inoperable | S | Q35 | 361 |
| 2401 | Washers inoperable | 3 | Q35 | 363 |
| 2402 | Wiper Blades ‐ out of service | S | Q35 | 365 |
| 2403 | Wiper Blades ‐ 30 day | 3 | Q35 | 367 |
| 2404 | Wipers: No fluid | 3 | Q35 | 369 |
| 2405 | Wipers: Sweep | S | Q35 | 371 |
| 2420 | Under body: Undercoating | 3 | Q36 | 373 |
| 2421 | Under body: Shackles | S | Q36 | 375 |
| 2422 | Under body: Springs | S | Q36 | 377 |
| 2423 | Under body: Floor Supports | S | Q36 | 379 |
| 2424 | Under body: Drive Shaft Guards | S | Q36 | 381 |
| 2425 | Under body: Cross Members | S | Q36 | 383 |
| 2426 | Under body: Shocks | S | Q36 | 385 |
| 2427 | Under body: Fluid Leaks ‐ out of service | S | Q36 | 387 |
| 2428 | Under body: Fluid Leaks ‐ 30 day | 3 | Q36 | 389 |
| 2429 | Under body: Body Clips | S | Q36 | 391 |
| 2430 | Under body: Body Bolts | S | Q36 | 393 |
| 2431 | Under body: Grease Seal | S | Q36 | 395 |
| 2432 | Under body: Pinion Seal | 3 | Q36 | 397 |

| 2433 | Under body: Wheel cylinders | S | Q36 | 399 |
| --- | --- | --- | --- | --- |
| 2434 | Under body: Air Tanks | S | Q36 | 401 |
| 2435 | Under body: Carrier Bearings | 3 | Q36 | 403 |
| 2436 | Transmission Leak ‐ out of service | S | Q36 | 405 |
| 2437 | Transmission Leak ‐ 30 day | 3 | Q36 | 407 |
| 2438 | Fuel Tank Mounting | S | Q36 | 409 |
| 2439 | Fuel Tank Cage | S | Q36 | 411 |
| 2440 | Fuel Tank Fill Cap | S | Q36 | 413 |
| 2441 | Fuel Tank Heat Shield | S | Q36 | 415 |
| 2442 | Fuel Tank Lines | S | Q36 | 417 |
| 2460 | Under hood: Firewall | S | Q37 | 419 |
| 2461 | Drive Belts ‐ out of service | S | Q37 | 421 |
| 2462 | Drive Belts ‐ 30 day | 3 | Q37 | 423 |
| 2463 | Underhood: Wire/Hoses | S | Q37 | 425 |
| 2464 | Battery ‐ out of service | S | Q37 | 427 |
| 2465 | Battery ‐ 30 day | 3 | Q37 | 429 |
| 2466 | Under hood: Fuel Leaks | S | Q37 | 431 |
| 2467 | Under hood: Coolant Lines | S | Q37 | 433 |
| 2468 | Under hood: Air Cleaner | 3 | Q37 | 435 |
| 2469 | Under hood: Heater Shutoff Valve | 3 | Q37 | 437 |
| 2470 | Under hood: Antifreeze Leak ‐ out of service | S | Q37 | 439 |
| 2471 | Under hood: Antifreeze Leak ‐ 30 day | 3 | Q37 | 441 |
| 2472 | Under hood: Oil Leak ‐ out of service | S | Q37 | 443 |
| 2473 | Under hood: Oil Leak ‐ 30 day | 3 | Q37 | 445 |
| 2474 | Power steering ‐ out of service | S | Q37 | 447 |
| 2475 | Power steering ‐ 30 day | 3 | Q37 | 449 |
| 2476 | Under hood: Brake Fluid | S | Q37 | 451 |
| 2477 | Under hood: Compressor Leak | S | Q37 | 453 |
| 2478 | Under hood: Fluid Leaks ‐ 30 day | 3 | Q37 | 455 |
| 2490 | Handicap: Ramp/power lift | 3 | Q38 | 457 |
| 2491 | Handicap: Door fastening device tether | 3 | Q38 | 459 |
| 2492 | Handicap: Lift door ‐ out of service | S | Q38 | 461 |
| 2494 | Handicap: Interlock | S | Q38 | 465 |
| 2495 | Handicap: Buzzer | S | Q38 | 467 |
| 2496 | Handicap: Identification | 3 | Q38 | 469 |
| 2497 | Handicap: Light | 3 | Q38 | 471 |
| 2498 | Handicap: Hand rail | 3 | Q38 | 473 |
| 2499 | Handicap: Manual pump handle | 3 | Q38 | 475 |
| 2500 | Handicap: Two way radio | 3 | Q38 | 477 |
| 2501 | Handicap: Fire Extinguisher System | 3 | Q38 | 479 |
| 2510 | Chaffing Hoses | S | Q39 | 481 |
| 2511 | Coolant Leak ‐ out of service | S | Q39 | 483 |
| 2512 | Coolant Leak ‐ 30 day | 3 | Q39 | 485 |
| 2513 | Firewall | S | Q39 | 487 |
| 2514 | Frayed Belts ‐ out of service | S | Q39 | 489 |
| 2515 | Frayed Belts ‐ 30 day | 3 | Q39 | 491 |
| 2516 | Frayed Wires | S | Q39 | 493 |
| 2517 | Fuel Leaks | S | Q39 | 495 |
| 2518 | Heater Shutoff | 3 | Q39 | 497 |
| 2519 | Loose Battery | S | Q39 | 499 |
| 2520 | Oil Leak ‐ out of service | S | Q39 | 501 |
| 2521 | Oil Leak ‐ 30 day | 3 | Q39 | 503 |
| 2522 | Other ‐ out of service | S | Q39 | 505 |
| 2600 | Chains available on bus (when required) | 3 | S21 | 515 |
| 2601 | Steering & Suspension: Looseness | S | S21 | 517 |
| 2602 | Wheel Rock ‐ L | S | S21 | 519 |
| 2603 | Wheel Rock ‐ R | S | S21 | 521 |
| 2604 | Wheel Lash | S | S21 | 523 |
| 2630 | Tires/Wheels: Condition | S | Q40 | 527 |
| 2631 | Rims ‐ out of service | S | Q40 | 529 |

| 2632 | Rims ‐ 30 day | 3 | Q40 | 531 |
| --- | --- | --- | --- | --- |
| 2633 | Lugs ‐ out of service | S | Q40 | 533 |
| 2634 | Lugs ‐ 30 day | 3 | Q40 | 535 |
| 2635 | Studded Snow tires on drive wheels after April 1st | 3 | Q40 | 537 |
| 2650 | Parking Brake: Reserve | S | B21 | 539 |
| 2651 | Parking Brake Inoperative | S | B21 | 541 |
| 2652 | Parking Brake: Indicator Light | S | B21 | 543 |
| 2653 | Parking Brake: Other ‐ out of service | S | B21 | 545 |
| 2660 | Insufficient Pedal Reserve | S | B22 | 547 |
| 2661 | Brakes: Warning Device | S | B22 | 549 |
| 2662 | Brakes: Test @ 20mph stop distance | S | B22 | 551 |
| 2663 | Calipers/Rotors | S | B22 | 553 |
| 2664 | Brake Leaks | S | B22 | 555 |
| 2665 | Brakes: Hose/Tubing | S | B22 | 557 |
| 2680 | Air Chamber ‐ Front | S | B23 | 561 |
| 2681 | Air Chamber: LS 16, 20, 24 ‐ 2" Max | S | B23 | 563 |
| 2682 | Air Chamber: LS 30, 24W ‐ 2.5" Max | S | B23 | 565 |
| 2683 | Air Chamber: LS T‐16, 20, 24 ‐ 1 3/4" Max | S | B23 | 567 |
| 2684 | Air Chamber: LS T‐30 ‐ 2" Max | S | B23 | 569 |
| 2685 | Air Chamber: LS T‐9, 9 12 ‐ 1 3/8" Max | S | B23 | 571 |
| 2686 | Air Chamber ‐ Rear | S | B23 | 573 |
| 2687 | Air Chamber: RS 16, 20, 24 ‐ 2" Max | S | B23 | 575 |
| 2688 | Air Chamber: RS 30, 24W ‐ 2.5" Max | S | B23 | 577 |
| 2689 | Air Chamber: RS T‐30 ‐ 2" Max | S | B23 | 579 |
| 2690 | Air Chamber: RS T‐16, 20, 24 ‐ 1 3/4" Max | S | B23 | 581 |
| 2691 | Air Chamber: RS T‐9, 9 12 ‐ 1 3/8" Max | S | B23 | 583 |
| 2700 | Exhaust System Mounting | S | X21 | 585 |
| 2701 | Exhaust System Leaks | S | X21 | 587 |
| 2702 | Exhaust System Noise | 3 | X21 | 589 |
| 2703 | Exhaust System Patched | S | X21 | 591 |
| 2704 | Exhaust System Termination point | S | X21 | 593 |
| 3001 | License | S | C31 | 83 |
| 3002 | Registration | S | C31 | 89 |
| 3003 | Insurance | S | C31 | 85 |
| 3004 | Plates | S | C31 | 91 |
| 3005 | Title | S | C31 | 87 |
| 3101 | Service Door | S | Q41 | 93 |
| 3102 | Entry Steps | S | Q41 | 101 |
| 3103 | Side Doors | S | Q41 | 95 |
| 3104 | Doors/Steps: Head Bump Pad | S | Q41 | 103 |
| 3105 | Doors/Steps: Grab handles | S | Q41 | 97 |
| 3106 | Doors/Steps: Stirrups | S | Q41 | 105 |
| 3107 | Stepwell Light | S | Q41 | 107 |
| 3108 | Doors/Steps: Seals | S | Q41 | 99 |
| 3109 | Step Tread | S | Q41 | 109 |
| 3110 | Kneeler Operation | S | Q41 | 111 |
| 3111 | Kneeler Signal | S | Q41 | 113 |
| 3112 | Kneeler Interlock | S | Q41 | 115 |
| 3113 | Rear Door Operation | S | Q41 | 117 |
| 3114 | Rear Door Signal | S | Q41 | 119 |
| 3115 | Rear Door Sensitive Edge | S | Q41 | 121 |
| 3116 | Rear Door Interlock | S | Q41 | 123 |
| 3120 | Emergency Egress: Lettering | S | Q42 | 125 |
| 3121 | Emergency Egress: Ignition ‐ Interlock | S | Q42 | 127 |
| 3122 | Emergency Egress: Door buzzer | S | Q42 | 129 |
| 3123 | Emergency Egress: Door handles | S | Q42 | 131 |
| 3124 | Emergency Egress: Head bump pads | S | Q42 | 163 |
| 3125 | Backup lights with door open, A, B, C, D | S | Q42 | 133 |
| 3126 | Door slide bar | S | Q42 | 135 |
| 3127 | Emergency Egress: Reflective tape | S | Q42 | 137 |

| 3128 | Emergency Egress: Door hinges | S | Q42 | 139 |
| --- | --- | --- | --- | --- |
| 3129 | Push out instructions ‐ L | S | Q42 | 141 |
| 3130 | Push out instructions ‐ R | S | Q42 | 143 |
| 3131 | Roof hatch instructions ‐ F | S | Q42 | 145 |
| 3132 | Roof hatch instructions ‐ R | S | Q42 | 147 |
| 3133 | Push out buzzer broken ‐ L | S | Q42 | 149 |
| 3134 | Push out buzzer broken ‐ R | S | Q42 | 151 |
| 3135 | Roof hatch buzzer broken ‐ F | S | Q42 | 153 |
| 3136 | Roof hatch buzzer broken ‐ R | S | Q42 | 155 |
| 3137 | Emergency exit handles | S | Q42 | 157 |
| 3138 | Emergency Egress: Hatch | S | Q42 | 159 |
| 3139 | Emergency Egress: Door | S | Q42 | 161 |
| 3140 | Emergency Egress: Signs | S | Q42 | 165 |
| 3141 | Crash Barriers ‐ Mounting | S | Q43 | 173 |
| 3142 | Emergency Egress: Easily Opened | S | Q42 | 167 |
| 3143 | Crash Barriers: Padding | S | Q43 | 175 |
| 3144 | Emergency Egress: Signal | S | Q42 | 169 |
| 3145 | Crash Barriers: Covering | S | Q43 | 177 |
| 3146 | Push out windows | S | Q42 | 171 |
| 3147 | Crash Barriers: Stanchions | S | Q43 | 179 |
| 3150 | Missing Sunshield | S | Q44 | 183 |
| 3151 | Non transparent Sunshield | S | Q44 | 185 |
| 3152 | Nonadjustable Sunshield | S | Q44 | 187 |
| 3153 | Broken Sunshield | S | Q44 | 181 |
| 3160 | Fire Extinguisher | S | Q45 | 189 |
| 3161 | Emergency Equip: Mounting | S | Q45 | 191 |
| 3162 | Emergency Equip: Charge | S | Q45 | 193 |
| 3163 | Emergency Equip: Inspection Tag | S | Q45 | 195 |
| 3164 | First Aid Kit Mounting | S | Q45 | 199 |
| 3165 | Emergency Equip: 2 in Bandage Compress | S | Q45 | 201 |
| 3166 | Emergency Equip: 3 in Bandage Compress | S | Q45 | 203 |
| 3167 | Emergency Equip: Adhesive Bandage | S | Q45 | 205 |
| 3168 | Emergency Equip: Adhesive Tape Rolls | S | Q45 | 207 |
| 3169 | Emergency Equip: Non‐sterile Triangular Bandages | S | Q45 | 209 |
| 3170 | Emergency Equip: Pair rounded end scissors | S | Q45 | 211 |
| 3171 | Emergency Equip: Sterile Eye Patch | S | Q45 | 213 |
| 3172 | Emergency Equip: Sterile Gauze Pads 36x36 | S | Q45 | 215 |
| 3173 | Emergency Equip: Sterile Gauze Pads 3x3 | S | Q45 | 217 |
| 3174 | Emergency Equip: Sterile Gauze Roller Bandage | S | Q45 | 219 |
| 3175 | Emergency Equip: First Aid Kit Size | S | Q45 | 221 |
| 3176 | Emergency Equip: Portable Warning Device | S | Q45 | 223 |
| 3177 | Emergency Equip: Wrecking Bar | S | Q45 | 225 |
| 3178 | Emergency Equip: Seat Belt Cutter | S | Q45 | 197 |
| 3179 | Emergency Warning Equipment | S | Q45 | 227 |
| 3190 | Speedometer | S | Q46 | 231 |
| 3191 | Oil gauge | S | Q46 | 233 |
| 3192 | Voltmeter | S | Q46 | 235 |
| 3193 | High Beam Indicator | S | Q46 | 237 |
| 3194 | Water temp | S | Q46 | 239 |
| 3195 | Air / Vacuum Brake Gauge | S | Q46 | 229 |
| 3196 | Amp meter | S | Q46 | 241 |
| 3197 | Odometer | S | Q46 | 243 |
| 3198 | Fuel gauge | S | Q46 | 245 |
| 3199 | Horn | S | Q46 | 247 |
| 3200 | Instruments: Brake indicator | S | Q46 | 249 |
| 3201 | Instruments: Stop lamp indicator | S | Q46 | 251 |
| 3202 | Instruments: Turn signal | S | Q46 | 253 |
| 3203 | Low air signal Audible | S | Q46 | 255 |
| 3204 | Low air signal Visual | S | Q46 | 257 |
| 3205 | Low air signal Signal pressure | S | Q46 | 259 |

| 3206 | Low air signal (activates ‐ characters) | S | Q46 | 261 |
| --- | --- | --- | --- | --- |
| 3210 | Gear Shift Indicator ‐ Mounting | S | Q47 | 265 |
| 3211 | Gear Shift Indicator Pattern | S | Q47 | 267 |
| 3212 | Gear Shift Indicator Shift detent | S | Q47 | 269 |
| 3213 | Gear Shift Indicator Floor boot | S | Q47 | 263 |
| 3220 | Heaters: Shield hose | S | Q48 | 271 |
| 3221 | Defogging fans ‐ L | S | Q48 | 273 |
| 3223 | Defogging fans ‐ R | S | Q48 | 275 |
| 3225 | Defrosters ‐ L | S | Q48 | 277 |
| 3226 | Defrosters ‐ R | S | Q48 | 279 |
| 3227 | Heaters ‐ L | S | Q48 | 281 |
| 3228 | Heaters ‐ R | S | Q48 | 283 |
| 3229 | Rear Heater | S | Q48 | 285 |
| 3230 | Heaters: Regulating valve | S | Q48 | 287 |
| 3231 | Heater | S | Q48 | 289 |
| 3232 | AC | S | Q48 | 291 |
| 3233 | Defroster | S | Q48 | 293 |
| 3240 | Seat Mounting | S | Q49 | 295 |
| 3242 | Seat Condition | S | Q49 | 311 |
| 3243 | Seat Missing | S | Q49 | 297 |
| 3244 | Seat belts | S | Q49 | 299 |
| 3245 | Seat Belt Locking retractor | S | Q49 | 301 |
| 3246 | Interior: Restraining device | S | Q49 | 303 |
| 3247 | Dome light | S | Q49 | 313 |
| 3248 | Interior: Cleanliness | S | Q49 | 315 |
| 3249 | Interior: Ceiling | S | Q49 | 317 |
| 3250 | Interior: Side walls | S | Q49 | 305 |
| 3252 | Interior: Storage compartment | S | Q49 | 319 |
| 3253 | Air conditioning/mount location | S | Q49 | 307 |
| 3255 | Interior: Floor | S | Q49 | 309 |
| 3257 | Aisle/clearance | S | Q49 | 321 |
| 3258 | Modesty Panels | S | Q49 | 323 |
| 3259 | Interior: Handles | S | Q49 | 325 |
| 3260 | Interior Lighting | S | Q49 | 327 |
| 3261 | Interior: Passenger Signal | S | Q49 | 329 |
| 3262 | Driver's Seatbelt | S | Q49 | 331 |
| 3263 | Driver's Seat | S | Q49 | 333 |
| 3264 | Driver's Partition | S | Q49 | 335 |
| 3265 | Driver's Partition Tint | S | Q49 | 337 |
| 3266 | Driver's Partition Curtain | S | Q49 | 339 |
| 3267 | Driver's Partition Integrity | S | Q49 | 341 |
| 3268 | Visor/Sunscreen | S | Q49 | 343 |
| 3269 | Interior: Signage | S | Q49 | 345 |
| 3300 | Identification: Lettering | S | Q50 | 347 |
| 3301 | Identification: Color | S | Q50 | 349 |
| 3302 | Advertising | S | Q50 | 351 |
| 3303 | Identification: GVW + 26,001 | S | Q50 | 353 |
| 3304 | Exterior Bus Unit Numbers | S | Q50 | 355 |
| 3305 | Identification: Lettering | S | Q50 | 357 |
| 3306 | MVC Number | S | Q50 | 359 |
| 3307 | USDOT Number | S | Q50 | 361 |
| 3308 | MVC Charter/Route Numbers | S | Q50 | 363 |
| 3310 | Exterior‐Body: Conditioning | S | Q51 | 365 |
| 3312 | Bumper | S | Q51 | 367 |
| 3314 | Rub Rails | S | Q51 | 373 |
| 3315 | Fenders | S | Q51 | 375 |
| 3316 | Crossing Arm | S | Q51 | 369 |
| 3318 | Hoods | S | Q51 | 371 |
| 3319 | Body Damage | S | Q51 | 377 |
| 3320 | Body Integrity | S | Q51 | 379 |

| 3330 | Warning Equipment: Red/Amber | S | Q52 | 381 |
| --- | --- | --- | --- | --- |
| 3331 | Warning Equipment: Switching | S | Q52 | 383 |
| 3332 | Warning Equipment: Indicator Light | S | Q52 | 385 |
| 3333 | Warning Equipment: Stop Arm | S | Q52 | 387 |
| 3334 | Warning Equipment: Brake Pilot Light | S | Q52 | 393 |
| 3335 | Warning Equipment: Backup Alarm | S | Q52 | 389 |
| 3336 | Warning Equipment: Roof Strobe light | S | Q52 | 391 |
| 3340 | Lights: Parking | S | Q53 | 415 |
| 3341 | Directional signal | S | Q53 | 395 |
| 3343 | Lights: Hazard | S | Q53 | 417 |
| 3344 | Lights: Marker | S | Q53 | 419 |
| 3345 | Tail Lights | S | Q53 | 397 |
| 3347 | Lights: Clearance | S | Q53 | 421 |
| 3348 | Headlights L out | S | Q53 | 399 |
| 3349 | Headlights R out | S | Q53 | 401 |
| 3350 | Headlights L Aim | S | Q53 | 423 |
| 3351 | Headlights R Aim | S | Q53 | 425 |
| 3352 | Stop Lights | S | Q53 | 403 |
| 3354 | Lights: License Plate | S | Q53 | 427 |
| 3355 | High beam L | S | Q53 | 405 |
| 3356 | High beam R | S | Q53 | 407 |
| 3357 | Exterior‐Lights: Wiring/switching | S | Q53 | 409 |
| 3358 | Exterior Indicator Lights | S | Q53 | 429 |
| 3359 | Exterior Backup Lights | S | Q53 | 411 |
| 3360 | Exterior‐Lights: Other | S | Q53 | 413 |
| 3362 | Exterior‐Lights: ID Lights | S | Q53 | 431 |
| 3363 | Exterior Reflectors | S | Q53 | 433 |
| 3364 | ICC Cluster Lamps | S | Q53 | 435 |
| 3365 | License Plate Lamp | S | Q53 | 437 |
| 3380 | Glazing: Windshield | S | Q54 | 439 |
| 3381 | Glazing: Left Side | S | Q54 | 441 |
| 3382 | Glazing: Right Side | S | Q54 | 443 |
| 3383 | Rear Glazing | S | Q54 | 445 |
| 3384 | Visual obstruction | S | Q54 | 457 |
| 3385 | Glazing: Gradient tint | S | Q54 | 459 |
| 3386 | Rear View Mirror | S | Q51 | 447 |
| 3387 | Cross Over Mirror | S | Q51 | 449 |
| 3388 | Interior Mirror | S | Q49 | 314 |
| 3389 | Rear View Convex Mirror | S | Q51 | 453 |
| 3390 | Glazing: Openings | S | Q54 | 455 |
| 3400 | Wiper inoperable | S | Q55 | 461 |
| 3401 | Washers inoperable | S | Q55 | 467 |
| 3402 | Wiper Blades | S | Q55 | 463 |
| 3404 | Wipers: No fluid | S | Q55 | 469 |
| 3405 | Wipers: Sweep | S | Q55 | 465 |
| 3420 | Under body: Undercoating | S | Q56 | 513 |
| 3421 | Under body: Shackles | S | Q56 | 471 |
| 3422 | Under body: Springs | S | Q56 | 473 |
| 3423 | Under body: Floor Supports | S | Q56 | 475 |
| 3424 | Under body: Drive Shaft Guards | S | Q56 | 477 |
| 3425 | Under body: Cross Members | S | Q56 | 479 |
| 3426 | Under body: Shocks | S | Q56 | 481 |
| 3427 | Under body: Fluid Leaks | S | Q56 | 483 |
| 3429 | Under body: Body Clips | S | Q56 | 485 |
| 3430 | Under body: Body Bolts | S | Q56 | 487 |
| 3431 | Under body: Grease Seal | S | Q56 | 489 |
| 3432 | Under body: Pinion Seal | S | Q56 | 515 |
| 3433 | Under body: Wheel cylinders | S | Q56 | 491 |
| 3434 | Under body: Air Tanks | S | Q56 | 493 |
| 3435 | Under body: Carrier Bearings | S | Q56 | 517 |

| 3436 | Transmission Leak | S | Q56 | 495 |
| --- | --- | --- | --- | --- |
| 3438 | Fuel Tank Mounting | S | Q56 | 497 |
| 3439 | Fuel Tank Cage | S | Q56 | 499 |
| 3440 | Fuel Tank Fill Cap | S | Q56 | 501 |
| 3441 | Fuel Tank Heat Shield | S | Q56 | 503 |
| 3442 | Fuel Tank Lines | S | Q56 | 505 |
| 3443 | Frame Cracks | S | Q56 | 507 |
| 3444 | Frame Integrity | S | Q56 | 509 |
| 3445 | Frame Mounts | S | Q56 | 511 |
| 3460 | Firewall | S | Q57 | 519 |
| 3461 | Drive Belts | S | Q57 | 521 |
| 3463 | Under hood: Wire/Hoses | S | Q57 | 523 |
| 3464 | Battery | S | Q57 | 525 |
| 3466 | Under hood: Fuel Leaks | S | Q57 | 527 |
| 3467 | Under hood: Coolant Lines | S | Q57 | 529 |
| 3468 | Under hood: Air Cleaner | S | Q57 | 541 |
| 3469 | Under hood: Heater Shutoff Valve | S | Q57 | 543 |
| 3470 | Under hood: Antifreeze Leak | S | Q57 | 531 |
| 3472 | Under hood: Oil Leak | S | Q57 | 533 |
| 3474 | Under hood: Power steering | S | Q57 | 535 |
| 3476 | Under hood: Brake Fluid | S | Q57 | 537 |
| 3477 | Under hood: Compressor Leak | S | Q57 | 539 |
| 3478 | Under hood: Fluid Leaks | S | Q57 | 545 |
| 3479 | Under hood: Battery Hold Down | S | Q57 | 547 |
| 3480 | Under hood: Battery Maintenance | S | Q57 | 549 |
| 3490 | Handicap: Ramp/power lift | S | Q58 | 557 |
| 3491 | Handicap: Door fastening device tether | S | Q58 | 559 |
| 3492 | Handicap: Lift door | S | Q58 | 551 |
| 3494 | Handicap: Interlock | S | Q58 | 553 |
| 3495 | Handicap: Buzzer | S | Q58 | 555 |
| 3496 | Handicap: Identification | S | Q58 | 561 |
| 3497 | Handicap: Light | S | Q58 | 563 |
| 3498 | Handicap: Hand rail | S | Q58 | 565 |
| 3499 | Handicap: Manual pump handle | S | Q58 | 567 |
| 3500 | Handicap: Two way radio | S | Q58 | 569 |
| 3501 | Handicap: Fire Extinguisher System | S | Q58 | 571 |
| 3502 | Wheel Chair Lift Signal | S | Q58 | 573 |
| 3503 | Wheel Chair Lift Interlock | S | Q58 | 575 |
| 3504 | Wheel Chair Lift Operation | S | Q58 | 577 |
| 3510 | Chaffing Hoses | S | Q59 | 579 |
| 3511 | Coolant Leak | S | Q59 | 581 |
| 3513 | Firewall | S | Q59 | 583 |
| 3514 | Frayed Belts | S | Q59 | 585 |
| 3516 | Frayed Wires | S | Q59 | 587 |
| 3517 | Fuel Leaks | S | Q59 | 589 |
| 3518 | Heater Shutoff | S | Q59 | 597 |
| 3519 | Loose Battery | S | Q59 | 591 |
| 3520 | Oil Leak | S | Q59 | 593 |
| 3522 | Other | S | Q59 | 595 |
| 3600 | Chains available on bus (when required) | S | S31 | 625 |
| 3601 | Looseness | S | S31 | 607 |
| 3602 | Air Suspension Operation | S | S31 | 627 |
| 3603 | Wheel Rock ‐ L | S | S31 | 609 |
| 3604 | Air Suspension Air Leaks | S | S31 | 629 |
| 3605 | Wheel Rock ‐ R | S | S31 | 611 |
| 3606 | Air Suspension Bellows | S | S31 | 631 |
| 3607 | Wheel Lash | S | S31 | 613 |
| 3608 | Air Suspension Leveling Valves | S | S31 | 633 |
| 3609 | Steering Wheel Play | S | S31 | 615 |
| 3610 | Air Suspension Hose / Tube Adequacy | S | S31 | 635 |

| 3611 | Steering Shaft | S | S31 | 617 |
| --- | --- | --- | --- | --- |
| 3612 | Springs | S | S31 | 637 |
| 3613 | Tie Rod End | S | S31 | 619 |
| 3614 | Shock Absorbers | S | S31 | 639 |
| 3615 | Draglink | S | S31 | 621 |
| 3616 | Radius Arms | S | S31 | 641 |
| 3617 | Power Steering Components | S | S31 | 623 |
| 3618 | Sway Bars | S | S31 | 643 |
| 3630 | Tires/Wheels: Condition | S | Q60 | 647 |
| 3631 | Rims | S | Q60 | 649 |
| 3633 | Lugs | S | Q60 | 651 |
| 3635 | Studded Snow tires on drive wheels after April 1st | S | Q60 | 655 |
| 3636 | Tires: Studs | S | Q60 | 653 |
| 3650 | Parking Brake: Reserve | S | B31 | 657 |
| 3651 | Parking Brake Inoperative | S | B31 | 659 |
| 3652 | Parking Brake: Indicator Light | S | B31 | 661 |
| 3653 | Parking Brake: Other | S | B31 | 663 |
| 3660 | Insufficient Brake Pedal Reserve | S | B32 | 665 |
| 3661 | Brakes: Warning Device | S | B32 | 667 |
| 3662 | Brakes: Test @ 20mph stop distance | S | B32 | 669 |
| 3663 | Calipers/Rotors | S | B32 | 671 |
| 3664 | Brakes: Leaks | S | B32 | 673 |
| 3665 | Brakes: Hose/Tubing | S | B32 | 675 |
| 3666 | Brake Lining Cracks | S | B32 | 677 |
| 3667 | Brake Lining Thickness | S | B32 | 679 |
| 3668 | Brake Lining Lining Integrity | S | B32 | 681 |
| 3669 | Brake Drum/Rotor Cracks | S | B32 | 683 |
| 3670 | Brake Drum/Rotor Thickness | S | B32 | 685 |
| 3671 | Brakes: Wheel Cylinder | S | B32 | 687 |
| 3672 | Brake Supply Air Leaks | S | B32 | 689 |
| 3673 | Brake Supply Fluid Leaks | S | B32 | 691 |
| 3674 | Brake Booster | S | B32 | 693 |
| 3675 | Brake Hose/Line Chaffing | S | B32 | 695 |
| 3676 | Brake Hose / Tube Integrity | S | B32 | 697 |
| 3680 | Air Chamber ‐ Front | S | B33 | 701 |
| 3681 | Air Chamber: LS 16, 20, 24 ‐ 2" Max | S | B33 | 703 |
| 3682 | Air Chamber: LS 30, 24W ‐ 2.5" Max | S | B33 | 705 |
| 3683 | Air Chamber: LS T‐16, 20, 24 ‐ 1 3/4" Max | S | B33 | 707 |
| 3684 | Air Chamber: LS T‐30 ‐ 2" Max | S | B33 | 709 |
| 3685 | Air Chamber: LS T‐9, 9 12 ‐ 1 3/8" Max | S | B33 | 711 |
| 3686 | Air Chamber ‐ Rear | S | B33 | 713 |
| 3687 | Air Chamber: RS 16, 20, 24 ‐ 2" Max | S | B33 | 715 |
| 3688 | Air Chamber: RS 30, 24W ‐ 2.5" Max | S | B33 | 717 |
| 3689 | Air Chamber: RS T‐30 ‐ 2" Max | S | B33 | 719 |
| 3690 | Air Chamber: RS T‐16, 20, 24 ‐ 1 3/4" Max | S | B33 | 721 |
| 3691 | Air Chamber: RS T‐9, 9 12 ‐ 1 3/8" Max | S | B33 | 723 |
| 3692 | Air Chamber Types mismatched on axle | S | B33 | 725 |
| 3693 | Air Chamber L/ Front (Axle 1) | S | B33 | 727 |
| 3694 | Air Chamber R/ Front (Axle 1) | S | B33 | 729 |
| 3695 | Air Chamber L/ Drive (Axle 2) | S | B33 | 731 |
| 3696 | Air Chamber R/ Drive (Axle 2) | S | B33 | 733 |
| 3697 | Air Chamber L/ Tag (Axle 3) | S | B33 | 735 |
| 3698 | Air Chamber R/ Tag (Axle 3) | S | B33 | 737 |
| 3699 | Air Chamber Securement | S | B33 | 739 |
| 3700 | Exhaust System Mounting | S | X31 | 741 |
| 3701 | Exhaust System Leaks | S | X31 | 743 |
| 3702 | Exhaust System Noise | S | X31 | 749 |
| 3703 | Exhaust System Patched | S | X31 | 745 |
| 3704 | Exhaust System Termination point | S | X31 | 747 |
| 4001 | License | F | C41 | 1 |

| 4002 | Registration | F | C41 | 3 |
| --- | --- | --- | --- | --- |
| 4003 | Insurance | F | C41 | 5 |
| 4004 | Plates | F | C41 | 7 |
| 4005 | Title | F | C41 | 9 |
| 5001 | Spec: License | S | C21 | 145 |
| 5002 | Spec: Registration | S | C21 | 147 |
| 5003 | Spec: Insurance | S | C21 | 149 |
| 5004 | Spec: Plates | S | C21 | 151 |
| 5005 | Spec: Title | S | C21 | 153 |
| 5101 | Spec: Service Door | S | Q21 | 155 |
| 5102 | Spec: Entry Steps | S | Q21 | 157 |
| 5103 | Spec: Side Doors | S | Q21 | 159 |
| 5104 | Spec: Doors/Steps: Head Bump Pad | S | Q21 | 161 |
| 5105 | Spec: Grab handles | S | Q21 | 163 |
| 5106 | Spec: Stirrups | S | Q21 | 165 |
| 5107 | Spec: Stepwell Light | S | Q21 | 167 |
| 5108 | Spec: Doors/Steps: Seals | S | Q21 | 169 |
| 5109 | Spec: Step Tread | S | Q21 | 171 |
| 5120 | Spec: Emergency Egress: Lettering ‐ out of service | S | Q22 | 173 |
| 5121 | Spec: Emergency Egress: Ignition ‐ Interlock | S | Q22 | 175 |
| 5122 | Spec: Emergency Egress: Door buzzer | S | Q22 | 177 |
| 5123 | Spec: Emergency Egress: Door handles | S | Q22 | 179 |
| 5124 | Spec: Emergency Egress: Head bump pads | S | Q22 | 181 |
| 5125 | Spec: Backup lights with door open, A, B, C, D | S | Q22 | 183 |
| 5126 | Spec: Door slide bar | S | Q22 | 185 |
| 5127 | Spec: Emergency Egress: Reflective tape | S | Q22 | 187 |
| 5128 | Spec: Emergency Egress: Door hinges | S | Q22 | 189 |
| 5129 | Spec: Push out instructions ‐ L | S | Q22 | 191 |
| 5130 | Spec: Push out instructions ‐ R | S | Q22 | 193 |
| 5131 | Spec: Roof hatch instructions ‐ F | S | Q22 | 195 |
| 5132 | Spec: Roof hatch instructions ‐ R | S | Q22 | 197 |
| 5133 | Spec: Push out buzzer broken ‐ L | S | Q22 | 199 |
| 5134 | Spec: Push out buzzer broken ‐ R | S | Q22 | 201 |
| 5135 | Spec: Roof hatch buzzer broken ‐ F | S | Q22 | 203 |
| 5136 | Spec: Roof hatch buzzer broken ‐ R | S | Q22 | 205 |
| 5137 | Spec: Emergency exit handles | S | Q22 | 207 |
| 5140 | Spec: Crash Barriers Mounting | S | Q23 | 209 |
| 5141 | Spec: Crash Barriers: Padding | S | Q23 | 211 |
| 5142 | Spec: Crash Barriers: Covering ‐ out of service | S | Q23 | 213 |
| 5144 | Spec: Crash Barriers: Stanchions | S | Q23 | 217 |
| 5150 | Spec: Missing Sunshield | S | Q24 | 219 |
| 5151 | Spec: Non transparent Sunshield | S | Q24 | 221 |
| 5152 | Spec: Nonadjustable Sunshield | S | Q24 | 223 |
| 5153 | Spec: Broken Sunshield ‐ out of service | S | Q24 | 225 |
| 5160 | Spec: Fire Extinguisher | S | Q25 | 229 |
| 5161 | Spec: Emergency Equip Mounting | S | Q25 | 231 |
| 5162 | Spec: Emergency Equip: Charge | S | Q25 | 233 |
| 5163 | Spec: Emergency Equip: Inspection Tag | S | Q25 | 235 |
| 5164 | Spec: First Aid Kit Mounting | S | Q25 | 237 |
| 5165 | Spec: Emergency Equip: 2 in Bandage Compress | S | Q25 | 239 |
| 5166 | Spec: Emergency Equip: 3 in Bandage Compress | S | Q25 | 241 |
| 5167 | Spec: Emergency Equip: Adhesive Bandage | S | Q25 | 243 |
| 5168 | Spec: Emergency Equip: Adhesive Tape Rolls | S | Q25 | 245 |
| 5169 | Spec: Emerg Equip: Non‐sterile Triangular Bandages | S | Q25 | 247 |
| 5170 | Spec: Emergency Equip: Pair rounded end scissors | S | Q25 | 249 |
| 5171 | Spec: Emergency Equip: Sterile Eye Patch | S | Q25 | 251 |
| 5172 | Spec: Emergency Equip: Sterile Gauze Pads 36x36 | S | Q25 | 253 |
| 5173 | Spec: Emergency Equip: Sterile Gauze Pads 3x3 | S | Q25 | 255 |
| 5174 | Spec: Emerg Equip: Sterile Gauze Roller Bandage | S | Q25 | 257 |
| 5175 | Spec: First Aid Kit Size | S | Q25 | 259 |

| 5176 | Spec: Emergency Equip: Portable Warning Device | S | Q25 | 261 |
| --- | --- | --- | --- | --- |
| 5177 | Spec: Emergency Equip: Wrecking Bar | S | Q25 | 263 |
| 5178 | Spec: Emergency Equip: Seat Belt Cutter | S | Q25 | 265 |
| 5190 | Spec: Speedometer | S | Q26 | 267 |
| 5191 | Spec: Oil gauge | S | Q26 | 269 |
| 5192 | Spec: Voltmeter | S | Q26 | 271 |
| 5193 | Spec: High Beam Indicator | S | Q26 | 273 |
| 5194 | Spec: Water temp | S | Q26 | 275 |
| 5195 | Spec: Air / Vacuum Brake Gauge | S | Q26 | 277 |
| 5196 | Spec: Amp meter | S | Q26 | 279 |
| 5197 | Spec: Odometer | S | Q26 | 281 |
| 5198 | Spec: Fuel gauge | S | Q26 | 283 |
| 5199 | Spec: Horn | S | Q26 | 285 |
| 5210 | Spec: Gear Shift Indicator Mounting | S | Q27 | 287 |
| 5211 | Spec: Gear Shift Indicator Pattern | S | Q27 | 289 |
| 5212 | Spec: Gear Shift Indicator Shift detent | S | Q27 | 291 |
| 5213 | Spec: Gear Shift Indicator Floor boot | S | Q27 | 293 |
| 5220 | Spec: Heaters: Shield hose | S | Q28 | 295 |
| 5221 | Spec: Defogging fans ‐ L ‐ out of service | S | Q28 | 297 |
| 5223 | Spec: Defogging fans ‐ R ‐ out of service | S | Q28 | 301 |
| 5225 | Spec: Defrosters ‐ L | S | Q28 | 305 |
| 5226 | Spec: Defrosters ‐ R | S | Q28 | 307 |
| 5227 | Spec: Heaters ‐ L | S | Q28 | 309 |
| 5228 | Spec: Heaters ‐ R | S | Q28 | 311 |
| 5230 | Spec: Heaters: Regulating valve | S | Q28 | 315 |
| 5240 | Spec: Seat Mounting ‐ out of service | S | Q29 | 317 |
| 5242 | Spec: Seat Condition | S | Q29 | 321 |
| 5243 | Spec: Seat Missing | S | Q29 | 323 |
| 5244 | Spec: Seat belts | S | Q29 | 325 |
| 5245 | Spec: Seat Belt Locking retractor | S | Q29 | 327 |
| 5246 | Spec: Restraining device | S | Q29 | 329 |
| 5247 | Spec: Dome light | S | Q29 | 331 |
| 5248 | Spec: Interior: Cleanliness | S | Q29 | 333 |
| 5249 | Spec: Interior: Ceiling | S | Q29 | 335 |
| 5250 | Spec: Interior: Side walls ‐ out of service | S | Q29 | 337 |
| 5252 | Spec: Interior: Storage compartment | S | Q29 | 341 |
| 5253 | Spec: AC/mount location ‐ out of service | S | Q29 | 343 |
| 5255 | Spec: Floor ‐ out of service | S | Q29 | 347 |
| 5257 | Spec: Aisle/clearance | S | Q29 | 351 |
| 5301 | Spec: Identification: Color | S | Q30 | 355 |
| 5302 | Spec: Advertising | S | Q30 | 357 |
| 5303 | Spec: Identification: GVW + 26,001 | S | Q30 | 359 |
| 5310 | Spec: Body: Conditioning ‐ out of service | S | Q31 | 361 |
| 5312 | Spec: Bumper ‐ out of service | S | Q31 | 365 |
| 5314 | Spec: Body: Rub Rails | S | Q31 | 369 |
| 5315 | Spec: Fenders | S | Q31 | 371 |
| 5316 | Spec: Crossing Arm ‐ out of service | S | Q31 | 373 |
| 5318 | Spec: Body: Hoods | S | Q31 | 377 |
| 5330 | Spec: Warning Equipment: Red/Amber | S | Q32 | 379 |
| 5331 | Spec: Warning Equipment: Switching | S | Q32 | 381 |
| 5332 | Spec: Warning Equipment: Indicator Light | S | Q32 | 383 |
| 5333 | Spec: Warning Equipment: Stop Arm | S | Q32 | 385 |
| 5334 | Spec: Warning Equipment: Brake Pilot Light | S | Q32 | 387 |
| 5335 | Spec: Warning Equipment: Backup Alarm | S | Q32 | 389 |
| 5336 | Spec: Warning Equipment: Roof Strobe light | S | Q32 | 391 |
| 5340 | Spec: Lights: Parking | S | Q33 | 393 |
| 5341 | Spec: Directional signal ‐ out of service | S | Q33 | 395 |
| 5343 | Spec: Lights: Hazard | S | Q33 | 399 |
| 5344 | Spec: Lights: Marker | S | Q33 | 401 |
| 5345 | Spec: Tail Lights ‐ out of service | S | Q33 | 403 |

| 5347 | Spec: Lights: Clearance | S | Q33 | 407 |
| --- | --- | --- | --- | --- |
| 5348 | Spec: Headlights L out | S | Q33 | 409 |
| 5349 | Spec: Headlights R out | S | Q33 | 411 |
| 5350 | Spec: Headlights L Aim | S | Q33 | 413 |
| 5351 | Spec: Headlights R Aim | S | Q33 | 415 |
| 5352 | Spec: Stop Lights ‐ out of service | S | Q33 | 417 |
| 5354 | Spec: Lights: License Plate | S | Q33 | 421 |
| 5355 | Spec: High beam L | S | Q33 | 423 |
| 5356 | Spec: High beam R | S | Q33 | 425 |
| 5357 | Spec: Lights: Wiring/switching | S | Q33 | 427 |
| 5358 | Spec: Exterior Indicator Lights | S | Q33 | 429 |
| 5359 | Spec: Exterior Backup Lights | S | Q33 | 431 |
| 5360 | Spec: Lights: Other ‐ out of service | S | Q33 | 433 |
| 5362 | Spec: ID Lights | S | Q33 | 437 |
| 5363 | Spec: Exterior Reflectors | S | Q33 | 439 |
| 5380 | Spec: Glazing: Windshield | S | Q34 | 441 |
| 5381 | Spec: Glazing: Left Side | S | Q34 | 443 |
| 5382 | Spec: Glazing: Right Side | S | Q34 | 445 |
| 5383 | Spec: Rear Glazing ‐ out of service | S | Q34 | 447 |
| 5384 | Spec: Visual obstruction | S | Q34 | 449 |
| 5385 | Spec: Glazing: Gradient tint | S | Q34 | 451 |
| 5386 | Spec: Rear View Mirror | S | Q31 | 453 |
| 5387 | Spec: Cross Over Mirror | S | Q31 | 455 |
| 5388 | Spec: Interior Mirror | S | Q29 | 332 |
| 5389 | Spec: Rear View Convex Mirror | S | Q31 | 459 |
| 5400 | Spec: Wiper inoperable | S | Q35 | 461 |
| 5401 | Spec: Washers inoperable | S | Q35 | 463 |
| 5402 | Spec: Wiper Blades ‐ out of service | S | Q35 | 465 |
| 5404 | Spec: Wipers: No fluid | S | Q35 | 469 |
| 5405 | Spec: Wipers: Sweep | S | Q35 | 471 |
| 5420 | Spec: Under body: Undercoating | S | Q36 | 473 |
| 5421 | Spec: Under body: Shackles | S | Q36 | 475 |
| 5422 | Spec: Under body: Springs | S | Q36 | 477 |
| 5423 | Spec: Under body: Floor Supports | S | Q36 | 479 |
| 5424 | Spec: Under body: Drive Shaft Guards | S | Q36 | 481 |
| 5425 | Spec: Under body: Cross Members | S | Q36 | 483 |
| 5426 | Spec: Under body: Shocks | S | Q36 | 485 |
| 5427 | Spec: Under body: Fluid Leaks ‐ out of service | S | Q36 | 487 |
| 5429 | Spec: Under body: Body Clips | S | Q36 | 491 |
| 5430 | Spec: Under body: Body Bolts | S | Q36 | 493 |
| 5431 | Spec: Under body: Grease Seal | S | Q36 | 495 |
| 5432 | Spec: Under body: Pinion Seal | S | Q36 | 497 |
| 5433 | Spec: Under body: Wheel cylinders | S | Q36 | 499 |
| 5434 | Spec: Under body: Air Tanks | S | Q36 | 501 |
| 5435 | Spec: Under body: Carrier Bearings | S | Q36 | 503 |
| 5436 | Spec: Transmission Leak ‐ out of service | S | Q36 | 505 |
| 5438 | Spec: Fuel Tank Mounting | S | Q36 | 509 |
| 5439 | Spec: Fuel Tank Cage | S | Q36 | 511 |
| 5440 | Spec: Fuel Tank Fill Cap | S | Q36 | 513 |
| 5441 | Spec: Fuel Tank Heat Shield | S | Q36 | 515 |
| 5442 | Spec: Fuel Tank Lines | S | Q36 | 517 |
| 5460 | Spec: Under hood: Firewall | S | Q37 | 519 |
| 5461 | Spec: Drive Belts ‐ out of service | S | Q37 | 521 |
| 5463 | Spec: Under hood: Wire/Hoses | S | Q37 | 525 |
| 5464 | Spec: Battery ‐ out of service | S | Q37 | 527 |
| 5466 | Spec: Under hood: Fuel Leaks | S | Q37 | 531 |
| 5467 | Spec: Under hood: Coolant Lines | S | Q37 | 533 |
| 5468 | Spec: Under hood: Air Cleaner | S | Q37 | 535 |
| 5469 | Spec: Under hood: Heater Shutoff Valve | S | Q37 | 537 |
| 5470 | Spec: Under hood: Antifreeze Leak ‐ out of service | S | Q37 | 539 |

| 5472 | Spec: Under hood: Oil Leak ‐ out of service | S | Q37 | 543 |
| --- | --- | --- | --- | --- |
| 5474 | Spec: Power steering ‐ out of service | S | Q37 | 547 |
| 5476 | Spec: Under hood: Brake Fluid | S | Q37 | 551 |
| 5477 | Spec: Under hood: Compressor Leak | S | Q37 | 553 |
| 5490 | Spec: Handicap: Ramp/power lift | S | Q38 | 557 |
| 5491 | Spec: Handicap: Door fastening device tether | S | Q38 | 559 |
| 5492 | Spec: Handicap: Lift door ‐ out of service | S | Q38 | 561 |
| 5494 | Spec: Handicap: Interlock | S | Q38 | 565 |
| 5495 | Spec: Handicap: Buzzer | S | Q38 | 567 |
| 5496 | Spec: Handicap: Identification | S | Q38 | 569 |
| 5497 | Spec: Handicap: Light | S | Q38 | 571 |
| 5498 | Spec: Handicap: Hand rail | S | Q38 | 573 |
| 5499 | Spec: Handicap: Manual pump handle | S | Q38 | 575 |
| 5500 | Spec: Handicap: Two way radio | S | Q38 | 577 |
| 5501 | Spec: Handicap: Fire Extinguisher System | S | Q38 | 579 |
| 5510 | Spec: Chaffing Hoses | S | Q39 | 581 |
| 5511 | Spec: Coolant Leak ‐ out of service | S | Q39 | 583 |
| 5513 | Spec: Firewall | S | Q39 | 587 |
| 5514 | Spec: Frayed Belts ‐ out of service | S | Q39 | 589 |
| 5516 | Spec: Frayed Wires | S | Q39 | 593 |
| 5517 | Spec: Fuel Leaks | S | Q39 | 595 |
| 5518 | Spec: Heater Shutoff | S | Q39 | 597 |
| 5519 | Spec: Loose Battery | S | Q39 | 599 |
| 5520 | Spec: Oil Leak ‐ out of service | S | Q39 | 601 |
| 5522 | Spec: Other ‐ out of service | S | Q39 | 605 |
| 5600 | Spec: Chains available on bus (when required) | S | S21 | 615 |
| 5601 | Spec: Steering & Suspension: Looseness | S | S21 | 617 |
| 5602 | Spec: Wheel Rock ‐ L | S | S21 | 619 |
| 5603 | Spec: Wheel Rock ‐ R | S | S21 | 621 |
| 5604 | Spec: Wheel Lash | S | S21 | 623 |
| 5630 | Spec: Tires/Wheels: Condition | S | Q40 | 627 |
| 5631 | Spec: Rims ‐ out of service | S | Q40 | 629 |
| 5633 | Spec: Lugs ‐ out of service | S | Q40 | 633 |
| 5635 | Spec: Stud Snow tires on drive wheels after Apr 1 | S | Q40 | 637 |
| 5650 | Spec: Parking Brake: Reserve | S | B21 | 639 |
| 5651 | Spec: Parking Brake Inoperative | S | B21 | 641 |
| 5652 | Spec: Parking Brake: Indicator Light | S | B21 | 643 |
| 5653 | Spec: Parking Brake: Other ‐ out of service | S | B21 | 645 |
| 5660 | Spec: Insufficient Pedal Reserve | S | B22 | 647 |
| 5661 | Spec: Brakes: Warning Device | S | B22 | 649 |
| 5662 | Spec: Brakes: Test @ 20mph stop distance | S | B22 | 651 |
| 5663 | Spec: Calipers/Rotors | S | B22 | 653 |
| 5664 | Spec: Brake Leaks | S | B22 | 655 |
| 5665 | Spec: Brakes: Hose/Tubing | S | B22 | 657 |
| 5680 | Spec: Air Chamber ‐ Front | S | B23 | 661 |
| 5681 | Spec: Air Chamber: LS 16, 20, 24 ‐ 2" Max | S | B23 | 663 |
| 5682 | Spec: Air Chamber: LS 30, 24W ‐ 2.5" Max | S | B23 | 665 |
| 5683 | Spec: Air Chamber: LS T‐16, 20, 24 ‐ 1 3/4" Max | S | B23 | 667 |
| 5684 | Spec: Air Chamber: LS T‐30 ‐ 2" Max | S | B23 | 669 |
| 5685 | Spec: Air Chamber: LS T‐9, 9 12 ‐ 1 3/8" Max | S | B23 | 671 |
| 5686 | Spec: Air Chamber ‐ Rear | S | B23 | 673 |
| 5687 | Spec: Air Chamber: RS 16, 20, 24 ‐ 2" Max | S | B23 | 675 |
| 5688 | Spec: Air Chamber: RS 30, 24W ‐ 2.5" Max | S | B23 | 677 |
| 5689 | Spec: Air Chamber: RS T‐30 ‐ 2" Max | S | B23 | 679 |
| 5690 | Spec: Air Chamber: RS T‐16, 20, 24 ‐ 1 3/4" Max | S | B23 | 681 |
| 5691 | Spec: Air Chamber: RS T‐9, 9 12 ‐ 1 3/8" Max | S | B23 | 683 |
| 5700 | Spec: Exhaust System Mounting | S | X21 | 685 |
| 5701 | Spec: Exhaust System Leaks | S | X21 | 687 |
| 5702 | Spec: Exhaust System Noise | S | X21 | 689 |
| 5703 | Spec: Exhaust System Patched | S | X21 | 691 |

| 5704 | Spec: Exhaust System Termination point | S | X21 | 693 |
| --- | --- | --- | --- | --- |
| 8000 | Inspection terminated | F | Q04 | 13 |
| 8001 | Inspection terminated ‐ no emission test | F | Q04 | 15 |
| 8002 | Brakes: inspection not performed | F | B01 | 15 |
| 8004 | Safety Equipment: inspection not performed | F | Q01 | 31 |
| 8006 | Exhaust System: inspection not performed | F | X01 | 7 |
| 8012 | Safety Failure: Emission test not performed | F | Q04 | 17 |
| 8019 | Steering & Suspension ‐ inspection not performed | F | S01 | 11 |
| 8030 | Inspection terminated | S | Q59 | 599 |
| 8031 | Inspection terminated ‐ no emission test | S | Q59 | 601 |
| 8032 | Safety Equipment: inspection not performed | S | Q59 | 603 |
| 8033 | Safety Failure: Emission test not performed | S | Q59 | 605 |
| 8034 | Steering & Suspension ‐ inspection not performed | S | S31 | 645 |
| 8035 | Brakes: inspection not performed | S | B32 | 699 |
| 8036 | Exhaust System: inspection not performed | S | X31 | 751 |
| 8040 | Inspection terminated | S | Q39 | 507 |
| 8041 | Inspection terminated ‐ no emission test | S | Q39 | 509 |
| 8042 | Safety Equipment: inspection not performed | S | Q39 | 511 |
| 8043 | Safety Failure: Emission test not performed | 3 | Q39 | 513 |
| 8044 | Steering & Suspension ‐ inspection not performed | S | S21 | 525 |
| 8045 | Brakes: inspection not performed | S | B22 | 559 |
| 8046 | Exhaust System: inspection not performed | S | X21 | 595 |
| 8101 | Emission test not done due to vehicle condition | A | Q04 | 18 |
| 9001 | Brakes: automated test not performed | F | B03 | 3 |
| 9002 | Brakes: automated test failed | F | B03 | 5 |
| 9005 | Brakes: automated test advisory | A | B03 | 7 |

| Code | Definition |
| --- | --- |
| 3 | A rejection but the bus is not put out of service if repairs are made within thirty days. After thirty days if not repair the bus is put out of service and can't transport passengers. |
| 4 | 48 hour rejection. This is a serious rejection which means that the vehicle must be repaired within 48 hours. |
| A | Advisory. Customer is advised about a specific condition, but it is not required to be recertified. |
| F | Failure. |
| S | Out of service. This means that a bus cannot carry passengers until repairs are made. |

The Bus inspection process shall include, but not be limited to, the following data elements: CREDENTIALS

- License

- Registration

- Insurance

- Plates IDENTIFICATION

- Lettering

- Color

- Advertising 4. GVW+26,001

SCHOOL BUS WARNING EQUIPMENT

- Red/Amber

- Switching

- Indicator

- Stop Arm

- Brake Pilot Light

- Backup Alarm BUS EXTERIOR

- Conditioning

- Bumper

- Rub Rails

- Fenders

- Crossing Arm

- Hoods MIRRORS

- Rear View

- Cross Over

- Interior

- Rear View Convex UNDERBODY

- Undercoating

- Shackles

- Springs

- FL Supports

- Guards

- Members

- Shocks

- Leaks

- Clips

- Bolts

- Seal

- Cylinders

- Air Tanks

- Bearings

- Leak

FUEL LINES/TANKS

- Mounting

- Cage

- Fill Cap

- Heat Shield

- Lines UNDERHOOD

- Firewall

- Drive Belts

- Wire/Hoses

- Battery

- Fuel Leaks

- Coolant ‐ Lines

- Air‐ Cleaner

- Heater Shut Off‐ Valve

- Antifreeze ‐ Leak

- Oil‐ Leak

- Power ‐ Steering

- Brake ‐ Fluid

- Compress ‐ Leak

- Other TIRES

- Condition

- Rims

- Lugs

- Snow tires on drive wheels

- Or chains available on bus (when required) DOOR/STEPS

- Service door

- Entry steps

- Side doors

- Heat bump pad

- Grab handles

- Entry steps

- Stirrups

- Stepwell light

- Seals

- Step tread

EMERGENCY EXITS DOOR/WINDOWS ROOF HATCHES

- Lettering

- Ignition ‐ Interlock

- Door buzzer

- Door handles

- Heat bump ‐ pads

- Backup lights with door open – B, C & D

- Door side bar

- Reflective tape

- Door hinges

- Push out instructions ‐ L, R

- Roof hatch instructions ‐ F, R

- Push out buzzer/broken ‐ L, R

- Roof hatch buzzer/broken ‐ F, R CRASH BARRIERS

- Mounting

- Padding

- Covering

- Stanchions SUNSHIELD

- Missing

- Non transparent

- Non adjustable

- Broken

- Fire extinguisher ‐ mounting

- First aid kit ‐ mounting

- Charge

- Missing equipment

- Size

- Portable warming ‐ device

- Wrecking ‐ bar INSTRUMENTS

- Speedometer

- Oil gauge

- Voltmeter

- High beam indicator

- Water temp

- Air vacuum

- Ammeter

- Odometer

- Fuel gauge

WINDSHIELD WIPER/WASHERS HORN

- Wiper inoperable

- Washers inoperable

- Blades

- No fluid

- Sweep

- Horn

GEAR SHIFT INDICATOR

- Mounting

- Pattern

- Shift detent

- Floor boot HEATERS

- Shield hose

- Defogging fans ‐ L, R

- Defrosters ‐ L, R

- Heaters ‐ L, R

- Rear

- Mounting

- Condition

- Missing

- Seat belts

- Cutter

- Driver refractor

- Restraining device BUS INTERIORS

- Dome light

- Cleanliness

- Ceiling

- Sidewalls

- Storage comp

- Air conditioning/mount location

- Floor

- Aisle/clearance STEERING & SUSPENSION

- Looseness

- Wheel rock ‐ L, R

- Wheel lash SERVICE BRAKE

- Reserve warning

- Device

- Test @20mph Stop distance

- Calipers/Rotors

- Leak

- Hose/Tubing

- Air chamber (Front), (Rear) PARKING BRAKE

- Reserve

- Inoperative

- Other LIGHTS

- Parking

- Directional signal headlights

- Hazard

- Marker

- Tail

- Clearance

- L Aim

- R Aim

- Stop

- Plate

- High beam

- Wiring/switching

- Indicator

- Backup

- Other

- ID

- Reflectors GLAZING

- W/S

- L/S

- R/S

- Rear

- Visual obstruction

- Gradient tint EXHAUST SYSTEMS

- Mounting

- Leaks

- Noise

- Patched

- Termination point HANDICAPPED RQ ONLY

- Rump/power lift

- Door fastening device tether

- Lift door

- Interlock

- Buzzer

- Identification

- Light

- Fluid leaks

- Hand rail

- Manual pump handle

- Two way radio MISCELLANEOUS

# Appendix 3.4A: OBD Hardware Specification

ETNJ3 OBD Module Hardware Specifications - v1.4

- Applicable Vehicles

  - Light Duty Gasoline - Model year 1996 and newer with a GVWR less than or equal to 8,500 pounds.

  - Light Duty Diesel - Model year 1997 and newer with a GVWR less than or equal to 8,500 pounds.

  - Medium Duty Gasoline & Diesel - Model year 2008 and newer with a GVWR between 8,501 and 14,000 pounds.

  - Heavy Duty Gasoline & Diesel - Model year 2014 and newer with a GVWR greater than or equal to 14,001 pounds

  - Other fuel types such as Compressed Natural Gas (CNG), Liquefied Natural Gas (LNG), and hybrids follow the GVWR and model year cutoffs of their gasoline counterparts for the purposes of determining OBD applicability.

- Power

  - The OBD module shall be primarily self-powered by a rechargeable battery; however use of a 12 volt source through a pin on the DLC from the vehicle being tested is permitted as a secondary power source option.

  - The battery used to provide the primary source of power for the OBD module shall initially provide a continuous run time of 8 hours without being supplied 12 volt power from a vehicle and add no more than 16 ounces to the weight of the device.

- OBD Testing Logic

  - Individual OBD module commands shall be sent locally by the NGworkstation software by issuing SAE J2534 commands to the greatest extent possible.

  - The OBD module shall not receive any commands from the NGVID to perform a test; however portions of the test may be controlled by the rules engine. Which portions of the test and what vocabulary is needed will be determined during the software requirements and design phase of the contract.

  - The OBD test flowchart located in Appendix 3.4A of this RFP is given as an example to show the level of complexity involved in the current OBD test. The OBD test for the NGsystem will be modified using this as a starting point. These modifications will be made jointly between the State and the NGcontractor during the requirements and design phase of the contract in order to leverage the knowledge and experience of all parties involved to ensure the best possible OBD test for the State.

  - The final OBD test design shall be subject to State approval.

- Communications With NGWorkstation

  - The OBD module shall communicate with the NGworkstation wirelessly.

  - The OBD module shall have a minimum range of 75 feet with a clear line of sight from the transmitter to the receiver.

  - The NGcontractor may provide as an upgrade the option to purchase a wireless range extender to increase the minimum range specified in 4.2.

  - The OBD module shall provide a method of securing the communications link to the NGworkstation. Data can either be encrypted or an alternative security protocol can be used subject to approval by the State.

  - The OBD module shall be FCC Part 15 Class B approved.

  - There shall be no loss of communications between the OBD module and the NGworkstation while being subjected to electromagnetic or radio frequency interference typically found in an automobile service garage from running engines, air compressors, welders, cell phones, etc.

  - There shall be no loss of communications between the OBD module and the NGworkstation while being subjected to radio transmissions from citizen’s band, emergency band, and other radio types of transmissions.

- SAE J1962 Cable and Diagnostic Link Connectors (DLC)

  - The minimum cable length shall be one meter and the maximum cable length shall be five meters. The contractor may offer cables in a variety of lengths with the shortest being included in the standard equipment package with an option to purchase additional length cables.

  - The cable should be of heavy gauge with strain reliefs at the cable ends and a SAE J1962 male DLC connector.

  - An adapter shall be provided as standard equipment to allow for testing of vehicles that use the SAE J1939-13 Deutsch 9 pin round connector. This adapter will plug into the SAE J1962 cable at the male DLC end.

- Indicator Lights

  - The OBD module shall have an indicator light to inform the operator the device has sufficient power to perform an OBD test.

  - The OBD module shall have an indicator light to inform the operator the device has established active communications with the vehicle.

  - The communications light specified in 6.2 shall blink or flash to inform the operator as messages are being sent and received between the OBD module and the vehicle.

- OBD Module Self-Check

  - The OBD module shall be able to perform a self-check on the network layer of the device that includes the ability to check both the SAE J1962 and SAE J1939-13 connectors depending on the vehicle being tested.

  - The self-check will be used to verify that the OBD module hardware and cable is fully functional in the event of a vehicle communications failure during an OBD inspection. The self-check may also be required to be performed at regular intervals, i.e. weekly.

  - The results and corresponding data from the self-check shall be made available to the NGworkstation for transmission to the NGVID.

  - The self-check mechanism shall be designed in such a way that it cannot be used to simulate a real OBD vehicle during an inspection.

  - The self-check mechanism is not required to be integrated into the OBD module itself, it may be integrated into the NGworkstation as an alternative.

- OBD Module Identification

  - The OBD module shall have a unique electronic serial number that shall be made available to the NGworkstation for transmission to the NGVID.

- OBD Module Firmware

  - The OBD module shall have a firmware version that shall be made available to the NGworkstation for transmission to the NGVID.

  - The OBD module shall have a method that allows the NGworkstation to update the firmware installed on the device.

  - If a firmware update is required for any reason, the contractor shall update all OBD modules in use within two weeks of receiving State approval of the new firmware version.

- Log Files

  - The NGworkstation shall log all requests to and responses from the OBD module. The messages in the log shall be labeled in such a fashion that makes it easy to distinguish between requests and responses.

  - The log shall contain a timestamp of when the log was created as well as identifying inspection and vehicle information. For example, the test record number could be used to identify the inspection and the license plate number could be used to identify the vehicle.

  - The log shall either be transmitted and stored on the NGVID or stored locally on the NGworkstation. If the logs are to be stored locally on the NGworkstation the State shall

be given read only remote access to retrieve the log files without having to request the files from the contractor.

- OBD Module Durability

  - The OBD module shall be ruggedized to protect itself from accidental drops.

  - The OBD module’s durability shall be tested by performing a series of drop tests from a height of three feet.

  - The OBD module drop tests shall consist of a series of 45 drops including an equal number of free fall, corner, and edge drops.

  - The OBD module shall be considered durable if after all drop tests have been completed the OBD module is still functioning for all in-use OBD communications protocols with its casing still intact.

- OBD Communication Rate

  - The OBD communications rate shall be calculated independently for the three vehicle categories listed in 12.4, 12.5, and 12.6. The communications rate calculation shall exclude vehicles that the State has determined are unable to communicate with the OBD module (e.g. grey market vehicles).

  - For the purposes of this calculation the definition of successful communication with a vehicle shall be when all of the data required to make a pass/fail determination was collected by the OBD module (i.e. MIL command status and readiness monitor status).

  - The OBD communications rate shall be calculated on a per vehicle basis using a three month rolling window. This means that vehicles that fail for communications during a month will be tracked for the following two months, and if the vehicle successfully communicates during a re-inspection it is considered as having passed communications for the purposes of this calculation.

  - Category 1: OBD eligible vehicles with a GVWR <= 8,500 pounds shall have a communications rate of greater than or equal to 99.9%

  - Category 2: OBD eligible vehicles with a GVWR between 8,501 and 14,000 pounds shall have a communications rate of greater than or equal to 99.0%.

  - Category 3: OBD eligible vehicles with a GVWR greater than or equal to 14,001 pounds shall have a communications rate of greater than or equal to 95%.

  - The OBD module shall be able to communicate with vehicles that by design are currently known to be non-complaint with relevant SAE specifications.

  - The State will have the ability to exclude certain year/make/model combinations from the OBD testing requirement to handle unforeseen circumstances beyond its control, but no combinations will be initially excluded.

  - The NGcontractor may submit a request to the State to have a vehicle excluded from this calculation or the OBD testing requirement. This request shall provide a justification as to why a specific vehicle or year/make/model combination should be excluded along with supporting data that shows the vehicle is non-complaint with SAE specifications and no solution can be provided.

  - The State may at any time adjust the communication rate goals individually up or down for each vehicle category listed in 12.4, 12.5, and 12.6. Any adjustment made by the State will be given to the NGcontractor in writing at least three months prior to the effective date. The State will provide a written justification of the rate adjustment and all supporting data to the NGcontractor upon request.

  - Service Level Agreements associated with communications rate goals are contained in the SLA section in the main body of the RFP.8

- Alternative Hardware Configuration for Centralized Testing

  - The NGcontractor may propose an alternative hardware configuration to be used only at the Central Inspection Facilities.

  - All hardware must be of a common core architecture, and the only requirements of this specification that are allowed to be modified for this alternative are:

    - Power - the OBD module may use an alternative primary power source other than the battery required by Section 2. However use of a 12 volt source through a pin on the DLC from the vehicle being tested is not permitted as a primary power source option.

    - Communications with NGWorkstation - the OBD module may use an alternative method to communicate with the NGworkstations other than the wireless specification required by Section 4.1 & 4.2. Alternatives can include but are not limited to a direct connection such a serial, USB, or Ethernet; or wireless communications with a reduced range.

    - Any alternative proposed shall be documented by the NGcontractor and submitted to the State for approval.

- Specification References

  - Any conflicts between requirements that are contained in this specification and those contained in the references below, this specification overrides the conflicting requirement.

  - If the contractor has a reason to believe a conflicting requirement listed in this specification will adversely affect the performance of the OBD module, an explanation should be submitted to the State along with supporting documentation, evidence, and data. After examination of the documentation and supporting data certain requirements contained herein may be removed by the State.

  - References included in this section may not be the only specifications the OBD module must meet in order to communicate and retrieve data from all of the vehicles required in Section 1.

  - SAE J1962 - Diagnostic Connector

  - SAE J1978 - OBDII Scan Tool - Equivalent to ISO/DIS 15031-4

  - SAE J1979 - E/E Diagnostic Test Modes

  - SAE J2534 - Recommended Practice for Pass Through Vehicle Reprogramming

  - SAE J1939/13 - Off Board Diagnostic Connector

  - SAE J1939/72 - Application Layer Diagnostics

# Appendix 3.4B: Legacy OBD Flow Process

Testing/Process Requirements

- In the case of a vehicle with multiple OBD modules responding to a request, the data from each module should be combined using "Inclusive Or" to record the complete response (except for RPM data).

- If the MIL fails to illuminate KOEO there must be a retry loop of a configurable length (Default 1).

- If the workstation OBD Module does not acknowledge a reset request there must be a retry loop of a configurable length (Default 2).

  - The retry loop will prompt the inspector “No Workstation OBD Module Found - Check the Connection to the OBD Module, and try again.” The software will then wait for a configurable amount of time (Default 0 seconds) and then resend the reset command.

  - If the module does not acknowledge the reset command by the end of the retry loop the system will prompt the inspector: “Communications with the Workstation OBD Module cannot be established - system will be locked out.” The software will set the ‘OBDII Interface Not Responding’ lockout, and abandon/abort the inspection (for PIFs) or resume the inspection in another lane (for CIFs).

- If communications with the vehicle is unsuccessful there must be a retry loop of a configurable length (Default 3).

  - The retry loop will prompt the inspector “Ignition Off, Reseat the Connector, KOER, and try again.” Then go back to reset the OBD Module and try to establish communications again.

  - If the vehicle does not establish communications by the end of the retry loop the vehicle will fail for communications.

- After sending a request to the vehicle for RPM, if the module loses vehicle communications or if the reported RPM value is 0, there must be a retry loop of configurable length (Default 3).

  - The retry loop will prompt the inspector “No RPM detected – Ensure vehicle is KOER, and try again”

  - If there is no RPM by the end of the loop record Null in the OBD RPM field if the vehicle lost communications after the RPM request was sent, or record 0 in the OBD RPM field if the vehicle module is reporting it as such.

- If continuous monitor support is required and any of the three continuous monitors are ‘Not Ready’ or ‘Not Supported’ there must be a retry loop of configurable length (Default 5).

- If the MIL is commanded on but no codes were collected during the DTC request command there must be a retry loop of configurable length (Default 5).

- If the OBD Module responds with a communications timeout message the software must re-establish communications with the vehicle and continue the process - skipping the command that caused the timeout message - provided that the response data is not used in any pass/fail determination. For example, if after sending a request for Mode $09 PID $06 (CVN) the module responds with a timeout, the software must re-establish communications with the vehicle, then Mode $09 PID $06 is skipped and Mode $01 PID $1C is sent (Extended Data). When a command is skipped a code should be recorded in the ‘Emission Clarification’ data element. The structure and content should be identical between CIF & PIF to simplify data processing and will be designed by Parsons and SGS.  If the command that caused the timeout message is used in a pass/fail decision the command cannot be skipped. If communications cannot be re-established by the end of the Init Retry limit (Default 3), and readiness and DTC information has been recorded the process can proceed to the end test routine. If communications cannot be re-established by the end of the Init Retry limit (Default 3), and readiness and DTC information has NOT been recorded the process must proceed to the next step (see Error Handling page of the flow diagram).  If communications cannot be re-established by the end of the Init Retry limit (Default 3), and the current process is readiness or DTC request then the vehicle will fail for communications.

- The structure and storage of the OBD Log file will be determined by SGS and Parsons during design. However, the log file must include date/time stamp, identifying vehicle data, a serial data log (‘D5 data dump’ see Vetronix Interface Control Document – Appendix C), and a record delimiter. The log file structure must be identical between CIFs and PIFs; however, the storage method may be different. For example, based on the volume of tests performed it may necessary to log one days’ worth of testing into one file for a CIF, and one months’ worth of data for a PIF.

Data Element Requirements

- OBDBulbCheck

  - Type: String

  - Values: (P)ass/(F)ail

- OBDVisualMIL

  - Type: String

  - Values (P)ass/(F)ail

- OBDCommunications

  - Type: String

  - Values: (P)ass/(F)ail/e(X)cluded

- OBDConnector

  - Type: String

  - Values: (P)ass/(D)amaged/(M)issing/(O)bstructed

- OBDProtocol

  - Type: Integer

  - Range: 0-9

  - Reference table must be compiled and placed on VIIS for conversion purposes

- OBDRPM

  - Type: Integer

  - Range : 0-9999

- ModuleID1, ModuleID2, ModuleID3

  - Type: 2 digit Hex

  - Must be recorded in ascending order (i.e. if 4 modules respond to Mode

$01 PID $00 with a response order of $F3,$C2,$10,$2E the id’s should be recorded as: ModuleID1 = 10 , ModuleID2 = 2E , ModuleID3 = C2

- PID00, PID20, PID40

  - Type: 8 digit Hex with spaces after each 2 digits (aA bB cC dD)

  - If responses are received from multiple modules ‘inclusive or’ must be used to determine the final PID (i.e. Mode $01 PID $00 and Module $10 =

$B3 $3C $B8 $11 and Module $18 = $C0 $50 $80 $00 then PID00 = FE 7C B8 11 )

- PID Count

  - Type : Integer

  - Range : 0 – 999

  - Sum of all individual PIDs supported ( i.e. PID00 = FE 7C B8 11 and PID20 = 95 81 80 FC then PIDCount = 31 )

- Readiness Monitors (Misfire, FuelSystem, CompComponents, Catalyst, HeatedCatalyst, Evap, SecondaryAir, ACSystem, O2Sensor, O2SensorHeator, EGR)

  - Type: String

  - Values: (R)eady, (X)Not Ready, (U)nsupported

  - If responses are received from multiple modules ‘inclusive or’ must be used to determine the readiness state of the vehicle.

- OBDReadiness

  - Type: String

  - Values: (P)ass, (F)ail, e(X)cluded

- OBDTotalUnsetMonitors

  - Type: Integer

  - Range: 0-11

- MILCommand

  - Type: String

  - Values: (P)ass, (F)ail

  - If the MIL is commanded on by any module responding to Mode $01 PID

$01 MILCommand should be set to (F)ail.

- DTC1,DTC2,…,DTC10

  - Type: 5 character String

  - Catalyst Codes take order precedence. If any module responds with a catalyst code P0420~P0439 these codes must be listed first. The remaining codes may be listed in any order.

- DTCCount

c. Sum of DTCs reported by all modules.

- OBDVIN

  - Type: 17 character String

  - Leading/trailing pad characters ($00) must be removed

  - Once the pad characters are removed, if the length of data exceeds 17 characters only the first 17 characters should be recorded

- OBDCAL_ID

  - Type: 16 character String

  - Leading/trailing pad characters ($00) must be removed

- OBDCVN

  - Type: ?? character String (Parsons & SGS will determine an appropriate length)

  - Leading/trailing pad characters ($00) must be removed

- OBDType

  - Type: 2 digit Hex

  - Reference table must be compiled and placed on VIIS for conversion purposes

- OBDMILOnDistance

  - Type: Integer

b. Range: 0 – 65,535 km (0 – 40722 miles)

c. Must be converted to miles

- OBDWarmups

  - Type: Integer

  - Range: 0 – 255

- OBDCodeClearDistance

  - Type: Integer

b. Range: 0 – 65,535 km (0 – 40722 miles)

c. Must be converted to miles

- OBDMILOnTime

- OBDCodeClearTime

  - Type: Integer

  - Range: 0 – 65,535 minutes

- OBDOverall

  - Type: String

  - Values: (P)ass, (F)ail

- CATDTC

  - Type: Variant

  - Values: (Y)es, (N)o, (C)leared

  - If (Y)es the value must persist, no matter how long it is until the next inspection.

  - When the value is set to (C)leared it must be recorded in the inspection record as (C)leared.

  - When the value is set to (Y)es it must be recorded in the inspection record as (Y)es.

  - During the Begin Vehicle Test Transaction (BVT) the VID must return the CATDTC value from the last inspection.

  - During the begin test rules the workstation must set CATDTC to (N)o if the last inspection had a CATDTC value of (C)leared.

# Appendix 3.4C: OBD Exclusion List

| Model Year | Make | Model | VIN Mask | Communications Exclusion | RPM Exclusion | Readiness Exclusion | Continuous Monitor Exclusion | CVN Exclusion | Catalyst Retest Exclusion | OBD Bypass Allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1996 | CHRYSLER | CIRRUS | * | N | N | Y | N | N | N | N |
| 1996 | CHRYSLER | CONCORDE | * | N | N | Y | N | N | N | N |
| 1996 | CHRYSLER | LHS | * | N | N | Y | N | N | N | N |
| 1996 | CHRYSLER | NEW YORKER | * | N | N | Y | N | N | N | N |
| 1996 | CHRYSLER | SEBRING | * | N | N | Y | N | N | N | N |
| 1996 | CHRYSLER | TOWN & COUNTRY | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | AVENGER | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | CARAVAN | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | DAKOTA | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | INTREPID | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | NEON | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | RAM PICKUP | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | RAM VAN | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | RAM WAGON | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | STEALTH | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | STRATUS | * | N | N | Y | N | N | N | N |
| 1996 | DODGE | VIPER | * | N | N | Y | N | N | N | N |
| 1996 | EAGLE | SUMMIT | * | N | N | Y | N | N | N | N |
| 1996 | EAGLE | TALON | * | N | N | Y | N | N | N | N |
| 1996 | EAGLE | VISION | * | N | N | Y | N | N | N | N |
| 1996 | FORD | CLUB WAGON | * | N | N | N | Y | N | N | N |
| 1996 | FORD | ECONOLINE | * | N | N | N | Y | N | N | N |
| 1996 | FORD | F150 | * | N | N | N | Y | N | N | N |
| 1996 | INFINITI | G20 | * | N | N | Y | N | N | N | N |
| 1996 | INFINITI | I30 | * | N | N | Y | N | N | N | N |
| 1996 | INFINITI | J30 | * | N | N | Y | N | N | N | N |
| 1996 | INFINITI | Q45 | * | N | N | Y | N | N | N | N |
| 1996 | JEEP | CHEROKEE | * | N | N | Y | N | N | N | N |
| 1996 | JEEP | GRAND CHEROKEE | * | N | N | Y | N | N | N | N |
| 1996 | MAZDA | MPV | * | N | N | Y | Y | N | N | N |
| 1996 | MITSUBISHI | 3000GT | * | N | N | Y | N | N | N | N |
| 1996 | MITSUBISHI | DIAMANTE | * | N | N | Y | N | N | N | N |
| 1996 | MITSUBISHI | ECLIPSE | * | N | N | Y | N | N | N | N |
| 1996 | MITSUBISHI | GALANT | * | N | N | Y | N | N | N | N |
| 1996 | MITSUBISHI | MIGHTY MAX | * | N | N | Y | N | N | N | N |
| 1996 | MITSUBISHI | MIRAGE | * | N | N | Y | N | N | N | N |
| 1996 | MITSUBISHI | MONTERO | * | N | N | Y | N | N | N | N |
| 1996 | NISSAN | 200SX | * | N | N | Y | N | N | N | N |
| 1996 | NISSAN | 240SX | * | N | N | Y | N | N | N | N |
| 1996 | NISSAN | 300ZX | * | N | N | Y | N | N | N | N |
| 1996 | NISSAN | ALTIMA | * | N | N | Y | N | N | N | N |
| 1996 | NISSAN | MAXIMA | * | N | N | Y | N | N | N | N |
| 1996 | NISSAN | PATHFINDER | * | N | N | Y | N | N | N | N |
| 1996 | NISSAN | PICKUP | * | N | N | Y | N | N | N | N |

| Model Year | Make | Model | VIN Mask | Communications Exclusion | RPM Exclusion | Readiness Exclusion | Continuous Monitor Exclusion | CVN Exclusion | Catalyst Retest Exclusion | OBD Bypass Allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1996 | NISSAN | QUEST | * | N | N | Y | N | N | N | N |
| 1996 | NISSAN | SENTRA | * | N | N | Y | N | N | N | N |
| 1996 | PLYMOUTH | BREEZE | * | N | N | Y | N | N | N | N |
| 1996 | PLYMOUTH | NEON | * | N | N | Y | N | N | N | N |
| 1996 | PLYMOUTH | VOYAGER | * | N | N | Y | N | N | N | N |
| 1996 | SAAB | 900 | * | N | N | Y | N | N | N | N |
| 1996 | SAAB | 9000 | * | N | N | Y | N | N | N | N |
| 1996 | SUBARU | IMPREZA | * | N | N | Y | N | N | N | N |
| 1996 | SUBARU | LEGACY | * | N | N | Y | N | N | N | N |
| 1996 | SUBARU | SVX | * | N | N | Y | N | N | N | N |
| 1996 | VOLVO | 850 SERIES | * | N | N | Y | N | N | N | N |
| 1996 | VOLVO | 960 SERIES | * | N | N | Y | N | N | N | N |
| 1997 | CADILLAC | DEVILLE | * | N | N | N | Y | N | N | N |
| 1997 | CADILLAC | ELDORADO | * | N | N | N | Y | N | N | N |
| 1997 | CADILLAC | SEVILLE | * | N | N | N | Y | N | N | N |
| 1997 | EAGLE | TALON | * | N | N | Y | N | N | N | N |
| 1997 | FORD | TAURUS | ???????2????????? | N | N | N | Y | N | N | N |
| 1997 | MAZDA | MPV | * | N | N | Y | Y | N | N | N |
| 1997 | MITSUBISHI | 3000GT | * | N | N | Y | N | N | N | N |
| 1997 | MITSUBISHI | DIAMANTE | * | N | N | Y | N | N | N | N |
| 1997 | MITSUBISHI | ECLIPSE | * | N | N | Y | N | N | N | N |
| 1997 | MITSUBISHI | GALANT | * | N | N | Y | N | N | N | N |
| 1997 | MITSUBISHI | MIRAGE | * | N | N | Y | N | N | N | N |
| 1997 | MITSUBISHI | MONTERO | * | N | N | Y | N | N | N | N |
| 1997 | MITSUBISHI | MONTERO SPORT | * | N | N | Y | N | N | N | N |
| 1997 | NISSAN | 200SX | * | N | N | Y | N | N | N | N |
| 1997 | OLDSMOBILE | AURORA | * | N | N | N | Y | N | N | N |
| 1997 | SAAB | 900 | * | N | N | Y | N | N | N | N |
| 1997 | SAAB | 9000 | * | N | N | Y | N | N | N | N |
| 1997 | TOYOTA | PASEO | * | N | N | Y | N | N | N | N |
| 1997 | TOYOTA | TERCEL | * | N | N | Y | N | N | N | N |
| 1997 | VOLVO | 850 SERIES | * | N | N | Y | N | N | N | N |
| 1997 | VOLVO | 960 SERIES | * | N | N | Y | N | N | N | N |
| 1998 | EAGLE | TALON | * | N | N | Y | N | N | N | N |
| 1998 | FORD | TAURUS | ???????2????????? | N | N | N | Y | N | N | N |
| 1998 | MAZDA | MPV | * | N | N | N | Y | N | N | N |
| 1998 | MITSUBISHI | 3000GT | * | N | N | Y | N | N | N | N |
| 1998 | MITSUBISHI | DIAMANTE | * | N | N | Y | N | N | N | N |
| 1998 | MITSUBISHI | ECLIPSE | * | N | N | Y | N | N | N | N |
| 1998 | MITSUBISHI | GALANT | * | N | N | Y | N | N | N | N |
| 1998 | MITSUBISHI | MIRAGE | * | N | N | Y | N | N | N | N |
| 1998 | MITSUBISHI | MONTERO | * | N | N | Y | N | N | N | N |
| 1998 | MITSUBISHI | MONTERO SPORT | * | N | N | Y | N | N | N | N |
| 1998 | SAAB | 900 | * | N | N | Y | N | N | N | N |

| Model Year | Make | Model | VIN Mask | Communications Exclusion | RPM Exclusion | Readiness Exclusion | Continuous Monitor Exclusion | CVN Exclusion | Catalyst Retest Exclusion | OBD Bypass Allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1998 | SAAB | 9000 | * | N | N | Y | N | N | N | N |
| 1998 | VOLVO | C70 | * | N | N | Y | N | N | N | N |
| 1998 | VOLVO | S70 | * | N | N | Y | N | N | N | N |
| 1998 | VOLVO | S90 | * | N | N | Y | N | N | N | N |
| 1998 | VOLVO | V70 | * | N | N | Y | N | N | N | N |
| 1998 | VOLVO | V90 | * | N | N | Y | N | N | N | N |
| 1999 | BUICK | CENTURY | * | N | N | N | Y | N | N | N |
| 1999 | BUICK | LESABRE | * | N | N | N | Y | N | N | N |
| 1999 | BUICK | PARK AVENUE | * | N | N | N | Y | N | N | N |
| 1999 | BUICK | REGAL | * | N | N | N | Y | N | N | N |
| 1999 | BUICK | RIVIERA | * | N | N | N | Y | N | N | N |
| 1999 | CHEVROLET | CAMARO | * | N | N | N | Y | N | N | N |
| 1999 | CHEVROLET | LUMINA | * | N | N | N | Y | N | N | N |
| 1999 | CHEVROLET | MALIBU | * | N | N | N | Y | N | N | N |
| 1999 | CHEVROLET | MONTE CARLO | * | N | N | N | Y | N | N | N |
| 1999 | CHEVROLET | VENTURE | * | N | N | N | Y | N | N | N |
| 1999 | FORD | TAURUS | ???????2????????? | N | N | N | Y | N | N | N |
| 1999 | OLDSMOBILE | ALERO | * | N | N | N | Y | N | N | N |
| 1999 | OLDSMOBILE | CUTLASS | * | N | N | N | Y | N | N | N |
| 1999 | OLDSMOBILE | EIGHTY EIGHT | * | N | N | N | Y | N | N | N |
| 1999 | OLDSMOBILE | INTRIGUE | * | N | N | N | Y | N | N | N |
| 1999 | OLDSMOBILE | SILHOUETTE | * | N | N | N | Y | N | N | N |
| 1999 | PONTIAC | BONNEVILLE | * | N | N | N | Y | N | N | N |
| 1999 | PONTIAC | FIREBIRD | * | N | N | N | Y | N | N | N |
| 1999 | PONTIAC | GRAND AM | * | N | N | N | Y | N | N | N |
| 1999 | PONTIAC | GRAND PRIX | * | N | N | N | Y | N | N | N |
| 1999 | PONTIAC | MONTANA | * | N | N | N | Y | N | N | N |
| 1999 | SAAB | 9-5 | * | N | N | N | Y | N | N | N |
| 2000 | BUICK | CENTURY | * | N | N | N | Y | N | N | N |
| 2000 | BUICK | LESABRE | * | N | N | N | Y | N | N | N |
| 2000 | BUICK | PARK AVENUE | * | N | N | N | Y | N | N | N |
| 2000 | BUICK | REGAL | * | N | N | N | Y | N | N | N |
| 2000 | CHEVROLET | CAMARO | * | N | N | N | Y | N | N | N |
| 2000 | CHEVROLET | IMPALA | * | N | N | N | Y | N | N | N |
| 2000 | CHEVROLET | LUMINA | * | N | N | N | Y | N | N | N |
| 2000 | CHEVROLET | MALIBU | * | N | N | N | Y | N | N | N |
| 2000 | CHEVROLET | MONTE CARLO | * | N | N | N | Y | N | N | N |
| 2000 | CHEVROLET | VENTURE | * | N | N | N | Y | N | N | N |
| 2000 | JAGUAR | XJ8 | * | N | N | N | Y | N | N | N |
| 2000 | JAGUAR | XK8 | * | N | N | N | Y | N | N | N |
| 2000 | JAGUAR | XKR | * | N | N | N | Y | N | N | N |
| 2000 | OLDSMOBILE | ALERO | 1G3N??2E?YC?????? | N | N | N | Y | N | N | N |
| 2000 | OLDSMOBILE | INTRIGUE | * | N | N | N | Y | N | N | N |
| 2000 | OLDSMOBILE | SILHOUETTE | * | N | N | N | Y | N | N | N |

| Model Year | Make | Model | VIN Mask | Communications Exclusion | RPM Exclusion | Readiness Exclusion | Continuous Monitor Exclusion | CVN Exclusion | Catalyst Retest Exclusion | OBD Bypass Allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2000 | PONTIAC | BONNEVILLE | 1G2HZ541?Y4?????? | N | N | N | Y | N | N | N |
| 2000 | PONTIAC | FIREBIRD | 2G2FS?2K?Y2?????? | N | N | N | Y | N | N | N |
| 2000 | PONTIAC | GRAND AM | 1G2N??2E?Y??????? | N | N | N | Y | N | N | N |
| 2000 | PONTIAC | GRAND PRIX | * | N | N | N | Y | N | N | N |
| 2000 | PONTIAC | MONTANA | * | N | N | N | Y | N | N | N |
| 2000 | VOLVO | S40 | * | N | N | N | Y | N | N | N |
| 2000 | VOLVO | V40 | * | N | N | N | Y | N | N | N |
| 2001 | JAGUAR | XJ8 | * | N | N | N | Y | N | N | N |
| 2001 | JAGUAR | XK8 | * | N | N | N | Y | N | N | N |
| 2001 | OLDSMOBILE | AURORA | * | N | N | N | Y | N | N | N |
| 2002 | JAGUAR | X-TYPE | * | N | N | N | Y | N | N | N |
| 2002 | JAGUAR | XJ8 | * | N | N | N | Y | N | N | N |
| 2003 | JAGUAR | S-TYPE | * | N | N | N | Y | N | N | N |
| 2003 | JAGUAR | X-TYPE | * | N | N | N | Y | N | N | N |
| 2003 | JAGUAR | XJ8 | * | N | N | N | Y | N | N | N |
| 2003 | PORSCHE | BOXSTER | * | N | N | N | Y | N | N | N |
| 2003 | VOLVO | C70 | * | N | N | N | Y | N | N | N |
| 2004 | JAGUAR | S-TYPE | * | N | N | N | Y | N | N | N |
| 2004 | JAGUAR | X-TYPE | * | N | N | N | Y | N | N | N |
| 2004 | JAGUAR | XJ SERIES | * | N | N | N | Y | N | N | N |
| 2004 | JAGUAR | XJ8 | * | N | N | N | Y | N | N | N |
| 2004 | JAGUAR | XJR | * | N | N | N | Y | N | N | N |
| 2004 | VOLVO | C70 | * | N | N | N | Y | N | N | N |
| 2005 | JAGUAR | S-TYPE | * | N | N | N | Y | N | N | N |
| 2005 | JAGUAR | X-TYPE | * | N | N | N | Y | N | N | N |
| 2005 | JAGUAR | XJ SERIES | * | N | N | N | Y | N | N | N |
| 2005 | JAGUAR | XJ8 | * | N | N | N | Y | N | N | N |
| 2005 | JAGUAR | XJR | * | N | N | N | Y | N | N | N |
| 2005 | JAGUAR | XKR | * | N | N | N | Y | N | N | N |
| 2006 | JAGUAR | S-TYPE | * | N | N | N | Y | N | N | N |
| 2006 | JAGUAR | X-TYPE | * | N | N | N | Y | N | N | N |
| 2006 | JAGUAR | XJ8 | * | N | N | N | Y | N | N | N |
| 2006 | JAGUAR | XK8 | * | N | N | N | Y | N | N | N |
| 2013 | RAM | 1500 | * | N | N | N | Y | N | N | N |

# Appendix 3.4D: AAMVA Barcode Definitions

AAMVA Barcodes

The AAMVA barcode format begins with an AAMVA header. Each field in the barcode shall be preceded by a 3-position Field Identifier (FID). This shall allow any process to interpret the data in a barcode without knowing its precise structure. Different applications can then process only the data that is relevant. The impact of layout changes would be limited to those applications that have direct need of the data being modified.

To allow for variation in the barcode without the necessity of software changes, software shall NOT treat the barcode as a fixed format string. Instead, it shall treat the barcode as a collection of data elements that are identified individually. The software shall "unstring" the data and extract the relevant data elements. This will allow the State to change the barcode content for other purposes without impacting the other processes.

The AAMVA Header has the following format:

@_ _ _AAMVA63603602nn

@ = First character in AAMVA compliant barcode

_ = character used as the Data Element Separator (line feed) ASCII hex "0A"

_ = character used as the Record Separator (record separator) ASCII hex "1C"

NOTE: the legacy barcode document specified hex "1E", however, the Record Separator character is not used in the barcode at all

_ = character used as the Segment Terminator (carriage return) ASCII hex "0D" AAMVA= indicates that the barcode meets AAMVA Best Practices

636036= indicates the jurisdiction (New Jersey) that printed the barcode 02 = AAMVA Best Practices version 02

nn = number of AAMVA Subfiles in the barcode

The Header is followed by a Subfile Designator, one for each Subfile in the AAMVA portion of the barcode:

xxiiiijjjj

xx = Subfile Type

iiii = offset from the beginning of the barcode to the start of the Subfile (i.e., how many characters appear in the string before the subfile starts; the subfile starts in position iiii+1) jjjj = length of Subfile, including terminator characters

The AAMVA Subfiles follow the Subfile Designators. The last Subfile is followed by the Segment Terminator.

Legacy Registration Barcodes

This appendix describes the registration barcodes currently in use. These barcodes will be used by the inspection system for most of the duration of this contract.

Example 1

Registration Document Barcode β = blank

| Field ID | Field Name | Format | Size | Value |
| --- | --- | --- | --- | --- |
| RAM | Registration Plate Number | A | 10 - fixed | GFFD592βββ |
| VAD | VIN | A | 17 - fixed | ABPL265FW11429301 |
| EAD | inspection Expiration Date | YYYYMMDD | 8 - fixed | 20161130 |
| ZJA | NJ1 Indicator | A | 1 - fixed | N |
| ZJB | VIN Extension | A | 2 - fixed | ββ |
| ZJC | Document Type | A | 2 - fixed | RG |

NOTE: Field RAM (Registration Plate Number) is a 10-position fixed-length field, left-justified, space- padded. Only A - Z, 0 - 9, and a dash ("-") are allowable characters. Use the leftmost non-blank characters to determine plate number (up to 10 characters).

Using the above data, the barcode data would be the following data as a continuous string:

@ 	AAMVA6360360205RG00590016VH00750023IN00980014ZJ01120019

RGRAMGFFD592βββ_VHVADABPL265FW11429301_INEAD19991130_ZJZJAN_ZJBββ_ZJCRG_

This data can be broken down as: AAMVA Header with 05 subfiles

@ 	AAMVA6360360205

Subfile Designators

Subfile type (RG), offset (0059), length (0016) RG00590016

Subfile type (VH), offset (0075), length (0023) VH00750023

Subfile type (IN), offset (0098), length (0014) IN00980014

Subfile type (ZJ), offset (0112), length (0019) ZJ01120019

Subfile Data

Subfile ID: RG Field ID's and Data

Registration Plate Number (with segment terminator) RAMGFFD592βββ_

Subfile ID: VH Field ID's and Data

VIN (with segment terminator) VADABPL265FW11429301_

Subfile ID: IN

Field ID's and Data

Inspection Expiration Date (with segment terminator) EAD19991130_

Subfile ID: ZJ

Field ID's and Data

NJ1 Indicator (with data element separator) ZJAN_

VIN Extension (with data element separator) ZJBββ_

Document Type (with segment terminator) ZJCRG_

Future Registration Barcodes

MVC is preparing for future upgrades to the NJ registration system which will include an expanded barcode. It will also conform to the AAMVA standard. The final layout will be provided when it is available.

VIR Fail Barcodes

A PDF417 Barcode shall display on failing VIRs. The VIR Fail Barcode shall not be an AAMVA- standard barcode. Instead it shall be a fixed length and have a version control in the format of “VIRxx”. The barcode shall be printed using 10 x 30 mil elements and error correction level 5. The width of the barcode shall be less than 3 inches, which can be accomplished by limiting the width to 13 columns.

The VIR shall print both the legacy barcode and a new barcode based on new system requirements to be determined during the design phase. This new barcode shall be similar to the legacy barcode but may contain different fields.

The list below shows the fields in the legacy VIR barcode and their lengths.

| Inspection Section | XMLType | Fields Name | Size |
| --- | --- | --- | --- |
| Versioning | N/A ? | BarCodeVersion | 5 |
| Compliance | InspectionParmsType | OverallInspectionResult | 1 |
| Compliance | InspectionParmsType | EmComplianceDate | 10 |
| Compliance | InspectionParmsType | EmExpirationDate | 10 |
| Compliance | InspectionParmsType | InspComplianceDate | 10 |
| Compliance | InspectionParmsType | InspTypeCode | 1 |
| Compliance | InspectionParmsType | InspCycleTypeCode | 1 |
| Compliance | InspectionParmsType | InspDate | 10 |
| Compliance | InspectionParmsType | InspExpirationDate | 10 |
| Compliance | InspectionParmsType | InspModeCode | 1 |
| Compliance | InspectionParmsType | NewStickerDate | 10 |
| Documents | CredentialsDetailsType | overallCredentialsResult | 1 |
| Emissions | DieselEmDetailsType | OpacityReading | 2 |
| Emissions | DieselEmDetailsType | OpacityStd | 5 |
| Emissions | DieselEmDetailsType | OpacityResult | 1 |
| Emissions | EmissionDetailsType | MiscEmRejectionComment | 50 |
| Emissions | EmissionDetailsType | MiscEmRejectionResult | 1 |
| Emissions | EmissionDetailsType | OverallEmResult | 1 |
| Emissions | EmissionDetailsType | LiquidLeakResult | 1 |
| Emissions | EmissionDetailsType | VisibleSmokeResult | 1 |
| Emissions | GasCapDetailsType | GasCapResult | 1 |
| Emissions | IdleEmDetailsType | IdleOverallResult | 1 |
| Emissions | IdleEmDetailsType | IdleTestFinal HC | 5 |
| Emissions | IdleEmDetailsType | IdleStd HC | 1 |
| Emissions | IdleEmDetailsType | IdleTestFinal CO | 5 |
| Emissions | IdleEmDetailsType | IdleStd CO | 1 |
| Emissions | IdleEmDetailsType | IdleTestFinal CO2 | 5 |
| Emissions | IdleEmDetailsType | IdleStd CO2 | 1 |

| Emissions | IdleEmDetailsType | IdleTestFinal O2 | 5 |
| --- | --- | --- | --- |
| Emissions | IdleEmDetailsType | IdleStd O2 | 1 |
| Emissions | IndicatorLightDetailsType | IndicatorLightResult | 1 |
| Emissions | IndicatorLightItemDetailsType | IndicatorLightItemCode | 1 |
| Emissions | IndicatorLightItemDetailsType | IndicatorLightItemCode | 1 |
| Emissions | IndicatorLightItemDetailsType | IndicatorLightItemCode | 1 |
| Emissions | OBDEmDetailsType | CatDTCCode | 1 |
| Emissions | OBDEmDetailsType | DLCResultCode | 1 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCCode | 5 |
| Emissions | OBDEmDetailsType | DTCStoredCount | 2 |
| Emissions | OBDEmDetailsType | MilCommandOnResultCode | 1 |
| Emissions | OBDEmDetailsType | OBDCommResultCode | 1 |
| Emissions | OBDEmDetailsType | OBDResult | 1 |
| Emissions | OBDEmDetailsType | ReadinessResult | 1 |
| Emissions | OBDEmDetailsType | UnsetMonitorCount | 2 |
| Emissions | PrimaryEmDetailsType | PrimaryEmResult | 1 |
| Emissions | ReadinessMonitorDetailsType | AirConditioningResult | 1 |
| Emissions | ReadinessMonitorDetailsType | CatalystResult | 1 |
| Emissions | ReadinessMonitorDetailsType | CompComponentResult | 1 |
| Emissions | ReadinessMonitorDetailsType | EgrSystemResult | 1 |
| Emissions | ReadinessMonitorDetailsType | EvaporativeSystemResult | 1 |
| Emissions | ReadinessMonitorDetailsType | FuelSystemResult | 1 |
| Emissions | ReadinessMonitorDetailsType | HeatedCatalystResult | 1 |
| Emissions | ReadinessMonitorDetailsType | MisfireResult | 1 |
| Emissions | ReadinessMonitorDetailsType | OxygenSensorHeaterResult | 1 |
| Emissions | ReadinessMonitorDetailsType | OxygenSensorResult | 1 |
| Emissions | ReadinessMonitorDetailsType | SecondaryAirSystemResult | 1 |
| Emissions | SafetyDetailsType | ExhaustSystemResult | 1 |
| Emissions | TamperDetailsType | TamperItemCode | 1 |
| Emissions | TamperDetailsType | TamperItemResult | 1 |
| Emissions | TamperDetailsType | TamperItemResult | 1 |
| Emissions | TamperDetailsType | TamperItemResult | 1 |
| Emissions | TSIEmDetailsType | TSIOverallResult | 1 |
| Emissions | TSI_IdleEmDetailsType | IdleTestFinal HC | 5 |
| Emissions | TSI_IdleEmDetailsType | IdleStd HC | 1 |

| Emissions | TSI_IdleEmDetailsType | IdleTestFinal CO | 5 |
| --- | --- | --- | --- |
| Emissions | TSI_IdleEmDetailsType | IdleStd CO | 1 |
| Emissions | TSI_IdleEmDetailsType | IdleTestFinal CO2 | 5 |
| Emissions | TSI_IdleEmDetailsType | IdleStd CO2 | 1 |

| New Jersey VIIS－Reporting System | Version: 0.2 |
| --- | --- |
| Specification Document | Date: June 19, 2009 |

| Emissions | TSI_IdleEmDetailsType | IdleTestFinal O2 | 5 |
| --- | --- | --- | --- |
| Emissions | TSI_IdleEmDetailsType | IdleStd O2 | 1 |
| Emissions | TSI_IdleEmDetailsType | TSI_HighIdleTestFinal HC | 5 |
| Emissions | TSI_IdleEmDetailsType | TSI_HighIdleStd HC | 1 |
| Emissions | TSI_IdleEmDetailsType | TSI_HighIdleTestFinal CO | 5 |
| Emissions | TSI_IdleEmDetailsType | TSI_HighIdleStd CO | 1 |
| Emissions | TSI_IdleEmDetailsType | TSI_HighIdleTestFinal CO2 | 5 |
| Emissions | TSI_IdleEmDetailsType | TSI_HighIdleStd CO2 | 1 |
| Emissions | TSI_IdleEmDetailsType | TSI_HighIdleTestFinal O2 | 5 |
| Emissions | TSI_IdleEmDetailsType | TSI_HighIdleStd O2 | 1 |
| Emissions Safety | VehiclePhysicalParmsType Variant ‐ SafetyResultCodeType OR SBUResultCodeType OR CBUResultCodeType | DualExhaust overallSafetyResult | 1 1 |
| Safety | InspectionParmsType | SafetyComplianceDate | 10 |
| Safety | InspectionParmsType | SafetyExpirationDate | 10 |
| Safety | InspectionParmsType | safetyTestType | 3 |
| Safety | SafetyDetailsType | BrakeResult | 1 |
| Safety | SafetyDetailsType | MiscSafetyRejectionComment | 50 |
| Safety | SafetyDetailsType | MiscSafetyRejectionResult | 1 |
| Safety | SafetyDetailsType | SafetyConditions | 80 |
| Safety | SafetyDetailsType | SafetyEquipmentResult | 1 |
| Safety | SafetyDetailsType | SteeringSuspensionResult | 1 |
| VehicleInfo | CylindersCodeType | NumOfCylinders | 2 |
| VehicleInfo | CylindersCodeType | OtherEngineCode | 1 |
| VehicleInfo | InspectionParmsType | ChangeICDCode | 1 |
| VehicleInfo | InspectionParmsType | OdometerReading | 6 |
| VehicleInfo | RegDataType | PlateNumber | 10 |
| VehicleInfo | VehicleDataType | VehicleID | 11 |
| VehicleInfo | VehiclePhysicalParmsType | AspirationCode | 1 |
| VehicleInfo | VehiclePhysicalParmsType | Displacement | 4 |
| VehicleInfo | VehiclePhysicalParmsType | DisplUnitCode | 3 |
| VehicleInfo | VehiclePhysicalParmsType | EngineHP | 3 |
| VehicleInfo | VehiclePhysicalParmsType | EngineMake | 20 |
| VehicleInfo | VehiclePhysicalParmsType | EngineYear | 4 |
| VehicleInfo | VehiclePhysicalParmsType | EPAMobile6Code | 6 |
| VehicleInfo | VehiclePhysicalParmsType | FuelCode | 4 |
| VehicleInfo | VehiclePhysicalParmsType | StackSize | 3 |
| VehicleInfo | VehiclePhysicalParmsType | TransmissionCode | 1 |
| VehicleInfo | VehiclePhysicalParmsType | VehicleGVWR | 6 |
| VehicleInfo | VehiclePhysicalParmsType | VehicleMake | 10 |

| New Jersey VIIS－Reporting System | Version: 0.2 |
| --- | --- |
| Specification Document | Date: June 19, 2009 |

| VehicleInfo | VehiclePhysicalParmsType | VehicleModel | 20 |
| --- | --- | --- | --- |
| VehicleInfo | VehiclePhysicalParmsType | VehicleYear | 4 |
| VehicleInfo | VinDataType | VinObd | 17 |
| VehicleInfo | VinDataType | VinVehicle | 17 |
| VehicleInfo | VinDataType | VINDocument | 19 |
| Total Size |  | Actual Barcode Length | 614 |

# Appendix 3.4E: Vehicle Inspection Report

VIR General Information

This is a general specification for the legacy VIR document. The new system VIR may be different which will be determined during the design phase.

The Vehicle Inspection Reports will be printed via the workstation and the web based reporting application. Here are some general VIR rules and notes:

All VIRs:

- The Facility Info section will show the physical address for where an inspection was conducted for MIT and BIT stations, otherwise show the Business Address.

- The Date and Time in the Control Info section relate to the Start Date and Time of the inspection.

- The Facility Info, Vehicle Info, and Control Info sections are the same across ALL VIRs.

- The Overall Result section is the same format for all VIRs, except OPACITY. For the Opacity VIR, the pre-defined text changes a little.

- If an Advisory Safety Condition exists, the Advisory Safety Condition section will appear on the VIR and the Overall Result will say ‘PASS-Advisory’ for a passing inspection and ‘FAIL-Advisory’ for a failing inspection – this is a calculation and not a result value stored in the database.

- In the Secondary Emissions Tests Performed section: MISC Emissions will show a PASS or a SEE EXPLANATION result. If the result says see explanation, then the field in the database was a FAIL and the comment associated with that fail will appear in the Explanation section of the VIR.

- If an inspector is logged in to the workstation via a fingerprint (biometrically), then their signature is not required on the VIR and their name will automatically print on the signature line.

- Barcodes will only print on Failing VIRs.

- The VIR will show the detail sections of the report for the items that Overall FAIL, some passing items do not require extensive data (see samples). For example, Safety Conditions are only printed for the items that FAIL the safety inspection; else that section will not appear on the report. So if the Overall Emissions Result = Fail, then the failing emissions section will appear, else the passing emissions section will appear. Each detail section is dependent on the result in the Overall Results section of the report.

- Credential Failures will display the specific failure items in the Safety sections (Failed and Advisory) – as they were safety items and the same two categories apply.

- The Emissions readings on the VIR will show the FINAL results.

- There is no footer to display on the VIR.

Passing VIRs:

- Passing VIRs will fit on one page, even if this cuts off part of the explanation section (which is pre-sized for passing VIRs).

- The printed text in the header section is consistent for all Passing VIRs (see samples).

- The Explanation section of a passing VIR may be cut off in the print, if it runs over a page.

- Passing VIRs are NOT mandatory to print, it is upon request.

Failing VIRs:

- Do not have a page limit for printing.

- The printed text in the header section is consistent for all Failing VIRs (see samples).

- The Explanation section will dynamically expand in order to display all information listed on the report.

NOTE: under the failing OBD VIR sample below there is a list of specific items to display in the explanation section depending on the OBD results.

- Emissions Failures will print an Emissions Repair Data form at the end of the VIR. This form cannot be split between pages.

NOTE: gas cap and liquid leak secondary emissions tests failures do not require this form to be printed if they are the only failed items.

- Only the Safety Sections (OOS, 48-Hour, Failed, Advisory – this is the order they will be listed in on the VIR) that have a Failing Item of inspection will display on the VIR. The sections (if they display) have pre-defined text (as shown on the VIR samples below) and then will display the items specific value as listed in description field in the INSPECTION_CONDITIONS table. The table will have codes with the items. We will print the description for the Fails (F) in the Failed Safety Conditions section and 48 hour codes (4) in the 48-Hour Safety Conditions section and the Out of Service (O) in the Out Of Service Safety Conditions section and then the Advisory (A) ones will be listed in the Advisory Safety Conditions section.

- Only up to 3 of the 4 Safety sections can appear on a single Failing VIR.

- The Calculated Date in the Failed Safety Conditions pre-defined text (if section is printed), is calculated as: IF ((48-Hour Fail) and (InspDate < InspComplianceDate): THEN take the InspComplianceDate and subtract 2 days, AND display the end of the following month (examples shown below), ELSE (there is no 48-Hour Fail) display Compliance Date.

Examples:

  - 48 hr expiration date is 5/1/2009, regular fail expiration date is 5/31/2009 ( 5/1

- 2days = 4/29, end of following month = 5/31)

  - 48 hr expiration date is 5/15/2009, regular fail expiration date is 6/30/2009 ( 5/15 - 2days = 5/13, end of following month = 6/30)

  - 48 hr expiration date is 12/31/2008, regular fail expiration date is 1/31/2009 ( 12/31 - 2days = 12/29, end of following month = 1/31)

- Failing VIRs still only show the details of the items that Failed. The passing summary sections will print for the passed categories.

- The pre-defined Warranty statement will print on all EMISSIONS (excluding OPACITY) failing VIRs, when the vehicle tested was model year 2001 or newer (above). No warranty statement will print on the Opacity VIRs. Statement: “Vehicles that fail the Emissions Inspection may be eligible for warranty coverage for the required repairs. Vehicle manufacturers are required by federal law to provide federally certified vehicles with Emissions Warranties for at least eight (8) years or eighty thousand (80,000) miles and on some California certified vehicles up to fifteen (15) years or one hundred fifty thousand (150,000) miles. Warranty coverage may vary depending upon vehicle make and model year. For further information, please refer to the Emissions Warranty section of the vehicle owner's manual.”

- The Emission Repair Data form will only print on failing EMISSIONS VIRs (excluding gas cap and liquid leak failures). There will be a text line at the bottom of the form stating; ‘Please present to Certified Emissions Repair Facility for completion.’

- For the OBD Readiness Result, will need to translate X's to Fs (fails) and Rs to Ps (passes) for Readiness. E is excluded.

- A Failing MISC Safety Item will be shown under the appropriate Safety Heading (48-Hour, Failed, etc.) on the report with the entire comment stored with it displayed.

- A Barcode will display on failing VIRs. The barcode location and format is described in Appendix 3.4A.

    - Pass OBD VIR

Vehicle Inspection Report <(Duplicate)>

Please Review This Important Information THIS IS AN OFFICIAL RECORD WHICH MUST BE PRESENTED IF THE VEHICLE IS TO BE REINSPECTED. IF LOST, A DUPLICATE RECORD MAY BE OBTAINED FROM ANY INSPECTION FACILITY OR <WEBSITE>. Your vehicle has PASSED both its SAFETY TEST and its EMISSIONS TEST. The results are summarized in this report. Questions? Visit NJINSPECTIONS.COM or call the Motorist Hotline at 1-888-NJMOTOR.

FACILITY INFO VEHICLE INFO CONTROL INFO South Brunswick VIN: Certificate: 2236 Route 130 Plate: Software Version: Dayton, NJ 08810 Veh. Type: Date: Year: Time: (888)656-6867 Make: Inspection Type: Facility ID: CIF000123 Model: Sticker#: Workstation ID: CL009200 Odometer Test Fee: Fuel Type GVW R:

FINAL RESULT: PASS Overall Safety Results: PASS Overall Emissions Result: PASS Credentials: PASS This test was performed in conformance with section 207(b) of the Federal Clean Air Act and the Inspection Expiration Date is <date>.

Primary Emission Test Performed: OBD Secondary Emissions Test(s) Performed: TEST RESULT: Gas Cap N/A Tampering PASS Visible Smoke PASS Liquid Leak PASS Indicator Light PASS Misc. Emissions See Explanation

EXPLANATION:

Inspector: ERIC K. GOEBEL INL000703

    - Pass Snap Opacity VIR

Vehicle Inspection Report <(Duplicate)>

Please Review This Important Information THIS IS AN OFFICIAL RECORD WHICH MUST BE PRESENTED IF THE VEHICLE IS TO BE REINSPECTED. IF LOST, A DUPLICATE RECORD MAY BE OBTAINED FROM ANY INSPECTION FACILITY OR <WEBSITE>. Your vehicle has PASSED both its SAFETY TEST and its EMISSIONS TEST. The results are summarized in this report. Questions? Visit NJINSPECTIONS.COM or call the Motorist Hotline at 1-888-NJMOTOR.

FACILITY INFO VEHICLE INFO CONTROL INFO South Brunswick VIN: Certificate: 2236 Route 130 Plate: Software Version: Dayton, NJ 08810 Veh. Type: Date: Year: Time: (888)656-6867 Make: Inspection Type: Facility ID: CIF000123 Model: Sticker#: Workstation ID: CL009200 Odometer Test Fee: Fuel Type GVW R:

FINAL RESULT: PASS Overall Safety Results: PASS Overall Emissions Result: PASS Credentials: PASS The Inspection Expiration Date is <date>.

Primary Emission Test Performed: Snap Opacity Secondary Emissions Test(s) Performed: Opacity READING STANDARD RESULT TEST RESULT: FINAL <Ave Value> <value> PASS Gas Cap N/A Tampering PASS Visible Smoke PASS Liquid Leak PASS Indicator Light PASS Misc. Emissions See Explanation

EXPLANATION:

Inspector: ERIC K. GOEBEL INL000703

    - Pass Advisory Condition VIR

Vehicle Inspection Report <(Duplicate)>

Please Review This Important Information THIS IS AN OFFICIAL RECORD WHICH MUST BE PRESENTED IF THE VEHICLE IS TO BE REINSPECTED. IF LOST, A DUPLICATE RECORD MAY BE OBTAINED FROM ANY INSPECTION FACILITY OR <WEBSITE>. Your vehicle has PASSED both its SAFETY TEST and its EMISSIONS TEST. The results are summarized in this report. Questions? Visit NJINSPECTIONS.COM or call the Motorist Hotline at 1-888-NJMOTOR.

FACILITY INFO VEHICLE INFO CONTROL INFO South Brunswick VIN: Certificate: 2236 Route 130 Plate: Software Version: Dayton, NJ 08810 Veh. Type: Date: Year: Time: (888)656-6867 Make: Inspection Type: Facility ID: CIF000123 Model: Sticker#: Workstation ID: CL009200 Odometer Test Fee: Fuel Type GVW R:

FINAL RESULT: PASS-Advisory Overall Safety Results: PASS Overall Emissions Result: PASS Credentials: PASS This test was performed in conformance with section 207(b) of the Federal Clean Air Act and the Inspection Expiration Date is <date>.

Primary Emission Test Performed: IDLE Secondary Emissions Test(s) Performed: Mode: IDLE TEST RESULT: GAS READING STANDARD RESULT Gas Cap N/A HC <V1> Tampering PASS CO% <V2> Visible Smoke PASS CO2% <V3> Liquid Leak PASS O2 <V4> Indicator Light PASS Misc. Emissions See Explanation

Advisory Safety Conditions: The advisory condition(s) listed are not required to be repaired to pass inspection, the vehicle owner must correct them within 60 days from the date of this inspection or the vehicle may be cited for failure to make repairs pursuant to NJSA 39:8-9. This vehicle does not have to be re-inspected for the advisory items. The vehicle must be maintained throughout the inspection cycle. Tires <Advisory code in VID have the code "A" in VID--Table = INSP_CONDITIONS>

EXPLANATION:

Inspector: ERIC K. GOEBEL INL000703

    - Fail OBD VIR

Vehicle Inspection Report <(Duplicate)>

Please Review This Important Information THIS IS AN OFFICIAL RECORD WHICH MUST BE PRESENTED IF THE VEHICLE IS TO BE REINSPECTED. IF LOST, A DUPLICATE RECORD MAY BE OBTAINED FROM ANY INSPECTION FACILITY OR <W EBSITE>. This Vehicle must pass inspections by <compliance date> at any facility, or it may be subjected to a fine(s) and/or registration suspension. RETAIN THIS DOCUMENT FOR USE ON REINSPECTION. Questions? Visit NJINSPECTIONS.COM or call the Motorist Hotline at 1-888-NJMOTOR.

FACILITY INFO VEHICLE INFO CONTROL INFO South Brunswick VIN: Certificate: 2236 Route 130 Plate: Software Version: Dayton, NJ 08810 Veh. Type: Date: Year: Time: (888)656-6867 Make: Inspection Type: Facility ID: CIF000123 Model: Sticker#: Workstation ID: CL009200 Odometer Test Fee: Fuel Type GVW R:

FINAL RESULT: FAIL Overall Safety Results: FAIL Overall Emissions Result: FAIL Credentials: FAIL This test was performed in conformance with section 207(b) of the Federal Clean Air Act and the Inspection Expiration Date is <date>.

Primary Emission Test Performed: OBD OBD SYSTEM - DIAGNOSTIC RESULTS Overall OBD System Result: Bulb Check: PASS OBD Connector: MIL Command Status: Check Engine Light On: PASS OBD Communications: Overall Readiness Result: OBD SYSTEM - READINESS STATUS Engine Misfire Catalytic Converters A/C Refrigerant Fuel System Heated Catalytic Converters Oxygen Sensors Comprehensive Component Evaporative System Oxygen Sensors - Heaters Secondary Air Injection EGR Systems OBD SYSTEM - DIAGNOSTIC TROUBLE CODES PRESENT: Total # of codes Present: Individual Codes Present: Secondary Emissions Test(s) Performed: TEST RESULT: Gas Cap N/A Tampering PASS Visible Smoke PASS Liquid Leak PASS Indicator Light PASS Misc. Emissions See Explanation

Out Of Service Safety Conditions: This vehicle has the following serious Out of Service safety condition(s) that must be repaired and inspected prior to the vehicle returning to service. <Safety code in VID have the code "O" in VID--Table = INSP_CONDITIONS>> 48 Hour Safety Conditions: This vehicle has the following serious 48-Hour safety condition(s) that must be repaired and inspected by <Compliance Date> <Safety code in VID have the code "4" in VID--Table = INSP_CONDITIONS>> Failed Safety Conditions: This vehicle has failed for the following safety conditions that must pass inspection by <Calculated date>. <Safety code in VID have the code "F" in VID--Table = INSP_CONDITIONS>> Advisory Safety Conditions: The advisory condition(s) listed are not required to be repaired to pass inspection, the vehicle owner must correct them within 60 days from the date of this inspection or the vehicle may be cited for failure to make repairs pursuant to NJSA 39:8-9. This vehicle does not have to be re-inspected for the advisory items. The vehicle must be maintained throughout the inspection cycle. Tires <Advisory code in VID have the code "A" in VID--Table = INSP_CONDITIONS>

EXPLANATION:

Vehicles that fail the Emissions Inspection may be eligible for warranty coverage for the required repairs.  Vehicle manufacturers are required by federal law to provide federally certified vehicles with Emissions Warranties for at least eight (8) years or eighty thousand (80,000) miles and on some California certified vehicles up to fifteen (15) years or one hundred fifty thousand (150,000) miles. Warranty coverage may vary depending upon vehicle make and model year. For further information, please refer to the Emissions Warranty section of the vehicle owner's manual.

Inspector:

ERIC K. GOEBEL	INL000703	<BARCODE>

Date of Repair: Repair Facility ID:

Emissions Repair Data

Repair Technician ID:

Repair Invoice Number: Confirmation #:

*Please present to NJ Certified Emissions Repair Facility for completion

Facility Stamp Here

A failing OBD VIR will display specific statements in the Explanation section of the report for failing OBD items. These statements are based on a specific item that has failed the inspection. The chart below lists the item that failed the inspection in the left column, with the corresponding statement that will appear in the Explanation section in the right column. These are pre-defined text statements that will display exactly as they appear in the chart below:

Optional statements for OBD inspection results (based on item of failure):

|  | OBD Message No. 1 - DLC connector failure | This vehicle has failed the emissions inspection due to a damaged, missing, obstructed or modified On-Board Diagnostic (OBD) system connector. To continue with the inspection process, the connector should be repaired and the vehicle reinspected. |  |
| --- | --- | --- | --- |
|  | OBD Message No. 2 - MIL bulb failure | This vehicle has failed the emissions inspection due to an inoperable On-Board Diagnostic Malfunction Indicator Light (OBD MIL). To continue with the inspection process, the MIL bulb should be repaired and the vehicle reinspected. |  |

| OBD Message No. 3 - KOER failure or MIL commanded on | Based on information obtained from the On-Board Diagnostic (OBD) system, this vehicle has failed due to a problem with the pollution control system. To continue with the inspection process, the pollution control system should be repaired and the vehicle reinspected. Fault Codes stored on the OBD system are listed above to assist in repairing the vehicle. |
| --- | --- |
| OBD Message No. 4 - Communications failure | This vehicle has failed the emissions inspection due to a communication problem with the On-Board Diagnostic (OBD) system. To continue with the inspection process, the OBD system should be repaired and the vehicle reinspected. |
| OBD Message No. 5 - Readiness failure | Based on information obtained from the On-Board Diagnostic (OBD) system, the system is not ready to determine the status of the pollution control system on this vehicle. To continue with the inspection process, the vehicle’s OBD system must be ready for testing upon reinspection. See the vehicle owner’s manual for information on "OBD/Readiness Driving Procedures" or contact your vehicle service provider. |
| OBD Message No. 6 - CATDTC = Y AND OBDOverall = F | The On-Board Diagnostic (OBD) system detected a catalyst fault. To continue with the inspection process, the catalyst should be repaired, required monitors including the catalyst monitor set, and the vehicle reinspected. |

    - Fail Snap Opacity VIR

Vehicle Inspection Report <(Duplicate)>

Please Review This Important Information THIS IS AN OFFICIAL RECORD WHICH MUST BE PRESENTED IF THE VEHICLE IS TO BE REINSPECTED. IF LOST, A DUPLICATE RECORD MAY BE OBTAINED FROM ANY INSPECTION FACILITY OR <W EBSITE>. This Vehicle must pass inspections by <compliance date> at any facility, or it may be subjected to a fine(s) and/or registration suspension. RETAIN THIS DOCUMENT FOR USE ON REINSPECTION. Questions? Visit NJINSPECTIONS.COM or call the Motorist Hotline at 1-888-NJMOTOR.

FACILITY INFO VEHICLE INFO CONTROL INFO South Brunswick VIN: Certificate: 2236 Route 130 Plate: Software Version: Dayton, NJ 08810 Veh. Type: Date: Year: Time: (888)656-6867 Make: Inspection Type: Facility ID: CIF000123 Model: Sticker#: Workstation ID: CL009200 Odometer Test Fee: Fuel Type GVW R:

FINAL RESULT: FAIL Overall Safety Results: FAIL Overall Emissions Result: FAIL Credentials: FAIL The Inspection Expiration Date is <date>.

Primary Emission Test Performed: Snap Opacity Secondary Emissions Test(s) Performed: Opacity READING STANDARD RESULT TEST RESULT: FINAL <Ave Value> <value> PASS Gas Cap N/A Tampering PASS Visible Smoke PASS Liquid Leak PASS Indicator Light PASS Misc. Emissions See Explanation

Out Of Service Safety Conditions: This vehicle has the following serious Out of Service safety condition(s) that must be repaired and inspected prior to the vehicle returning to service. <Safety code in VID have the code "O" in VID--Table = INSP_CONDITIONS>> 48 Hour Safety Conditions: This vehicle has the following serious 48-Hour safety condition(s) that must be repaired and inspected by <Compliance Date> <Safety code in VID have the code "4" in VID--Table = INSP_CONDITIONS>> Failed Safety Conditions: This vehicle has failed for the following safety conditions that must pass inspection by <Calculated date>. <Safety code in VID have the code "F" in VID--Table = INSP_CONDITIONS>> Advisory Safety Conditions: The advisory condition(s) listed are not required to be repaired to pass inspection, the vehicle owner must correct them within 60 days from the date of this inspection or the vehicle may be cited for failure to make repairs pursuant to NJSA 39:8-9. This vehicle does not have to be re-inspected for the advisory items. The vehicle must be maintained throughout the inspection cycle. Tires <Advisory code in VID have the code "A" in VID--Table = INSP_CONDITIONS>

EXPLANATION:

| Inspector: ERIC K. GOEBEL INL000703 <BARCODE> |  |
| --- | --- |
| Emissions Repair Data |  |
| Date of Repair: Repair Facility ID: Repair Technician ID: Repair Invoice Number: | Facility Stamp Here |
| Confirmation #: |  |

*Please present to NJ Certified Emissions Repair Facility for completion

    - Fail Advisory Condition VIR

Vehicle Inspection Report <(Duplicate)>

Please Review This Important Information THIS IS AN OFFICIAL RECORD WHICH MUST BE PRESENTED IF THE VEHICLE IS TO BE REINSPECTED. IF LOST, A DUPLICATE RECORD MAY BE OBTAINED FROM ANY INSPECTION FACILITY OR <W EBSITE>. This Vehicle must pass inspections by <compliance date> at any facility, or it may be subjected to a fine(s) and/or registration suspension. RETAIN THIS DOCUMENT FOR USE ON REINSPECTION. Questions? Visit NJINSPECTIONS.COM or call the Motorist Hotline at 1-888-NJMOTOR.

FACILITY INFO VEHICLE INFO CONTROL INFO South Brunswick VIN: Certificate: 2236 Route 130 Plate: Software Version: Dayton, NJ 08810 Veh. Type: Date: Year: Time: (888)656-6867 Make: Inspection Type: Facility ID: CIF000123 Model: Sticker#: Workstation ID: CL009200 Odometer Test Fee: Fuel Type GVW R:

FINAL RESULT: FAIL-Advisory Overall Safety Results: FAIL Overall Emissions Result: FAIL Credentials: FAIL This test was performed in conformance with section 207(b) of the Federal Clean Air Act and the Inspection Expiration Date is <date>.

Primary Emission Test Performed: IDLE Secondary Emissions Test(s) Performed: Mode: IDLE TEST RESULT: GAS READING STANDARD RESULT Gas Cap N/A HC <V1> Tampering PASS CO% <V2> Visible Smoke PASS CO2% <V3> Liquid Leak PASS O2 <V4> Indicator Light PASS Misc. Emissions See Explanation

Out Of Service Safety Conditions: This vehicle has the following serious Out of Service safety condition(s) that must be repaired and inspected prior to the vehicle returning to service. <Safety code in VID have the code "O" in VID--Table = INSP_CONDITIONS>> 48 Hour Safety Conditions: This vehicle has the following serious 48-Hour safety condition(s) that must be repaired and inspected by <Compliance Date> <Safety code in VID have the code "4" in VID--Table = INSP_CONDITIONS>> Failed Safety Conditions: This vehicle has failed for the following safety conditions that must pass inspection by <Calculated date>. <Safety code in VID have the code "F" in VID--Table = INSP_CONDITIONS>> Advisory Safety Conditions: The advisory condition(s) listed are not required to be repaired to pass inspection, the vehicle owner must correct them within 60 days from the date of this inspection or the vehicle may be cited for failure to make repairs pursuant to NJSA 39:8-9. This vehicle does not have to be re-inspected for the advisory items. The vehicle must be maintained throughout the inspection cycle. Tires <Advisory code in VID have the code "A" in VID--Table = INSP_CONDITIONS>

EXPLANATION:

| Vehicles that fail the Emissions Inspection may be eligible for warranty coverage for the required repairs. Vehicle manufacturers are required by federal law to provide federally certified vehicles with Emissions Warranties for at least eight (8) years or eighty thousand (80,000) miles and on some California certified vehicles up to fifteen (15) years or one hundred fifty thousand (150,000) miles. Warranty coverage may vary depending upon vehicle make and model year. For further information, please refer to the Emissions Warranty section of the vehicle owner's manual. |  |
| --- | --- |
| Inspector: ERIC K. GOEBEL INL000703 <BARCODE> |  |
| Emissions Repair Data Date of Repair: |  |
| Repair Facility ID: Repair Technician ID: Repair Invoice Number: Confirmation #: | Facility Stamp Here |

*Please present to NJ Certified Emissions Repair Facility for completion

# Appendix 3.4F: Rules Engine Communications and Logic Specifications

Rules Engine Logic

| General Notes |  |
| --- | --- |
| 1 | Each ruleset must set the rulesVersion number in the initialize rule sheet. The rulesVersion should always be the same for all three rulesets even if there is no change in a particular ruleset. |
| 2 | The aliases defined in the Scope section must be the same in all rule sheets in a ruleset. These aliases must always be used to identify attributes in the rule sheets. All entities are children of the Inspection entity. |
| 3 | The inspExtentCode is used to terminate a ruleset. Almost all rule sheets check this attribute in the Preconditions/Filters section. If the inspExtentCode is set to 'T' for 'Terminate', the rule sheets will not execute, essentially terminating the rules. |
| 4 | Every rule sheet must have a postInfo entry in row N.1 of the Rules (non-conditional) section. The corresponding Rule Statement will be the name of the rule sheet preceded by a dash. For example, the CycleInitialize rule sheet will have a Rule Statement with ID N.1 and text of '-CycleInitialize'. |
| 5 | Every Rules Conditions/Actions column must have a CurrInsp.post row in the Actions section. This forces a track of which rules have fired. The corresponding Rules Statement will start with the name of the rule sheet followed by a colon. For example, each column in CycleInitialize will have a Rule Statement that begins with 'CycleInitialize:'. |
| 6 | CurrInsp.post commands can have values of Info, Warning, or Violation. Normal rules will use Info. If an error is encountered the Warning and Violation values will be used. If the data is valid but incorrect, e.g., a vehicle configuration cannot be tested at the current facility, the Warning value will be used and the Inspector can be given the option to correct the data and try again. If an invalid combination arises that indicates a software problem, the Violation value will be used and the inspection will be terminated. |
| 7 | Warnings and Violations must produce an Err entity. Warnings use this format: Err+=Error.new[errorCode='XXnnn RulesheetName', errorLevel='W', errorDetails='XXnnn Error - error description text. Re-enter data or terminate the inspection'] Violations use tis format: Err+=Error.new[errorCode='XXnnn RulesheetName', errorLevel='V', errorDetails='XXnnn Fatal Error - error description text. Inspection will terminate.'] |
| 8 | The errorLevel can be used by the software to determine the action to be taken. The errorDetails can be displayed on the screen to provide information to the inspector and troubleshooter. |
| 9 | In addition, when a Warning or Violation is set the testExtentCode must be set to 'T'. |
| 10 | VehicleCategory is used to determine the cycle length (6 mos, 12 mos, 24 mos) and whether a vehicle gets a safety inspection or not. |

| 11 | There are two "cycles" that affect the inspection process. One cycle is relative to the current date/month. A vehicle can be inspected "on-cycle" up to two months prior to its sticker expiration date. For example, in March vehicles with a May sticker can be inspected on-cycle, but a June vehicle would be off-cycle. The other cycle is relative to the vehicle's sticker date. A vehicle's sticker is "active" for the length of its cycle (6, 12, or 24 months), with accommodation made for the "two month early" inspection window. So if a 24-month vehicle is presented with a sticker that expired 18 months ago, its new sticker date will be calculated using that date plus 24 months. |
| --- | --- |

Begin Test Cycle Ruleset

CycleInitialize

Initialize values for BeginTestCycle ruleset.

Add a TempHold entity.

Set rules version number.

Default null inspExtentCode to F.

Default null omni2Indicator to N.

Default null inspDate to today.

Default null commercialIndicator to N.

Default null softwareVersion to zero-length string.

Default null transportIndicator to N.

Default null mvcRegCode to 0.

Default null vehicleGVWR to 0.

Terminate if testRecordNumber is null.

CycleValidation_v5

Terminate for invalid pifClassCode.

Terminate for invalid inspTypeCode.

Terminate for invalid facilityType.

Terminate for invalid fuelCode.

Terminate for invalid vehicleGVWR<=0.

SetFuelType

Set fuelType based on fuelCode.

SetVehicleSize

Set vehicleSize based on vehicleGVWR.

SetProgramType

Set CurrInsp.inspProgramType based on CurrInsp.inspTypeCode.

SetNullMvcRegCode_IM

Run only for non-bus, mvcRegCode=0.

Set default mvcRegCode based on commercialIndicator, transportIndicator, omni2Indicator.

SetNullMvcRegCode_Bus

Run only for bus, mvcRegCode=0.

Set default mvcRegCode based on inspProgramType.

SetVehicleCategory_Bus_v5

Run only if the inspProgramType is for a bus.

Compare mvcRegCode and inspProgramType for validity.

Set CurrVehicle.vehicleCategory to 'cbus' or 'sbus'.

Invalid combination will allow re-entry or termination.

SetVehicleCategory_MC_v5

Run only if the inspProgramType is for a motorcycle.

| Set CurrVehicle.vehicleCategory to 'norm'. |  |  |
| --- | --- | --- |
| SetVehicleCategory_IM_v5a |  |  |
| Run only if the inspProgramType is 'IM'. |  |  |
| Terminate if mvcRegCode is not valid for IM inspection. |  |  |
| CheckForDC |  |  |
| Run only if inspProgramType='IM'. |  |  |
| For HD Diesel vehicles, change inspTypeCode and inspProgramType to 'DC'. |  |  |
| SetVehicleCategory_IM_v5c |  |  |
| Run only if the inspProgramType is 'IM'. |  |  |
| Compare mvcRegCode, omin2Indicator, transportIndicator, and commercialIndicator for validity. |  |  |
| Set CurrVehicle.vehicleCategory to 'comm', 'norm', 'trans', or 'omni'. |  |  |
| Invalid combination will allow re-entry or termination. |  |  |
| CheckPgmTypeAndFacility |  |  |
| Compare inspProgramType, facilityType, and pifClassCode for validity. |  |  |
| Invalid combination will allow re-entry or termination. |  |  |
| CheckPgmTypeAndVehicle_Std |  |  |
| Run for non-bus. |  |  |
| Compare inspProgramType, vehicleSize, fuelType, and omni2Indicator for validity. |  |  |
| Some invalid combinations will cause termination. |  |  |
| Some invalid combination will allow re-entry or termination. |  |  |
| SetCycleLength |  |  |
| Set inspExpirationDateCycle length based on vehicleCategory and inspProgramType. |  |  |
| SetCurrMonth |  |  |
| Set reference dates. |  |  |
| Also set aaChgICD to 'E' or 'Y' for this ruleset based on PrevInsp.chgICDCode. |  |  |
| Non-conditional: | [NOTE: inspDate is used instead of today to ease testing.] |  |
| Temp.aaCurrMonth=CurrInsp.inspDate.month |  |  |
| Temp.aaCycleBeg=CurrInsp.inspDate.addMonths (2-CurrInsp.inspExpirationDateCycle ).eom |  |  |
| Temp.aaCycleEnd=CurrInsp.inspDate.addMonths (2).endOfMonth |  |  |
| Temp.aaCycleFutEnd=CurrInsp.inspDate.addMonths(2+CurrInsp.inspExpirationDateCycle).eom |  |  |
| Temp.aaCycleInvalid=CurrInsp.inspDate.addMonths(61).endOfMonth |  |  |
| SetOldStkr_Null |  |  |
| Run only if oldStickerDate is null. |  |  |
| Set oldStickerDate to invalid future date (1/1/2050). This will force a recalculation. |  |  |
| Set Temp.aaOldStickerReset='y' so it can be changed back later. |  |  |
| SetPrevIED_Null |  |  |
| Run only if Prev inspExpirationDate is null. |  |  |
| Set Prev IED to invalid future date (1/1/2050). This will force a recalculation. |  |  |
| Set Temp.aaPrevInspExpirationDateReset='y' so it can be changed back later. |  |  |
| SetPrevInspDate_Null |  |  |
| Run only if Prev inspDate is null. |  |  |
| Set Prev inspDate to invalid past date (1/1/1996). This will force a recalculation. |  |  |
| Set Temp.aaPrevInspDateReset='y' so it can be changed back later. |  |  |
| SetCycleStartDate_1 |  |  |
| The first try to set the cycleStartDate. |  |  |
| There are five date ranges: |  |  |
| <= Temp.aaCycleBeg |  | Invalid past date |
| >Temp.aaCycleBeg and <=Temp.aaCycleEnd |  | Current cycle |
| >Temp.aaCycleEnd and <=Temp.aaCycleFutEnd |  | Next cycle |
| >Temp.aaCycleFutEnd and <=Temp.aaCycleInvalid |  | Dealer cycle |

| >Temp.aaCycleInvalid | Invalid future date |
| --- | --- |
| The Prev IED and the oldStickerDate will be compared to these ranges. |  |
| The "ref date" used is based on the Prev IED or the oldStickerDate. |  |
| If the cycleStartDate is set, aaString will be set to 'done'. |  |
| If the cycleStartDate cannot be set: |  |
| aaString will be set to 'ied', 'stkr', or 'today'. |  |
| aaOffset will be set to zero or 'ref date'.month - aaCurrMonth (-11 to +11). |  |
| SetCycleStartDate_2 |  |
| Run only for non-bus and if Temp.aaString<>'done'. |  |
| Non-conditional: |  |
| Three dates are set using aaOffset: |  |
| Temp.aaDateCurr=CurrInsp.inspDate.addMonths (Temp.aaOffset ).endOfMonth |  |
| Temp.aaDatePrev=Temp.aaDateCurr.addMonths ( -12 ).endOfMonth |  |
| Temp.aaDateNext=Temp.aaDateCurr.addMonths ( 12 ).endOfMonth |  |
| These are the same month for current year, last year, and next year. |  |
| The cycleStartDate is set to one of these dates based on aaOffset. |  |
| SetCycleStartDate_3 |  |
| Run only for bus and if Temp.aaString<>'done'. |  |
| Non-conditional: |  |
| Five dates are set using aaOffset: |  |
| aaDateOffsetP12 | aaOffset+12 |
| aaDateOffsetP6 | aaOffset+6 |
| aaDateOffset | aaOffset |
| aaDateOffsetM6 | aaOffset -6 |
| aaDateOffsetM12 | aaOffset-12 |
| The cycleStartDate is set to one of these dates based on aaOffset. |  |
| SetInspModeCode |  |
| Non-conditional: |  |
| Reset aaCycleBeg to cycleStartDate -3 months, last day of month. |  |
| Set the ModeCode (off-cycle, on-cycle) based on: |  |
| aaCycleBeg |  |
| PrevInsp.inspDat e |  |
| PrevInsp.overallResult |  |
| PrevInsp.inspModeCode |  |
| SetInspCycleTypeCode |  |
| Non-conditional: |  |
| Set aaCycleBeg to cycleStartDate -3 months, last day of month.[redundant] |  |
| Set the ModeCode (off-cycle, on-cycle) based on: |  |
| aaCycleBeg |  |
| PrevInsp.inspDat e |  |
| PrevInsp.overallResult |  |
| PrevInsp.inspModeCode |  |
| SetInspExtentCode |  |
| Set inspExtentCode based on inspCycleTypeCode. |  |
| SetInspExtentCodeBusExpire |  |
| Run only if inspCycleTypeCode is retest. |  |
| Run only if the inspProgramType is for a bus. |  |
| If current insp is more than 30 days after the prev insp, set inspExtentCode to full. |  |

ResetSoftwareVersion_Null

Run REGARDLESS of inspExtentCode value.

Run only if software version is "".

Non-conditional:

Resset software version to null.

ResetOldStkr_Null

Run REGARDLESS of inspExtentCode value.

Run only if oldStickerDate was null.

Non-conditional:

Reset oldStickerDate to null.

ResetPrevIED_Null

Run REGARDLESS of inspExtentCode value.

Run only if Prev inspExpirationDate was null.

Non-conditional:

Reset Prev inspExpirationDate to null.

ResetPrevInspDate_Null

Run REGARDLESS of inspExtentCode value.

Run only if Prev inspDate was null.

Non-conditional:

Reset Prev inspDate to null.

| Begin Test Cycle Data |  |
| --- | --- |
| CurrInsp.cycleStartDate |  |
| 1 | This date is marks the beginning of a vehicle's current cycle. It is actually the last day of the month that begins the cycle. |
| 2 | It is set in the BeginTestCycle ruleset. |
| 3 | For an on-cycle, on-time vehicle, it is the "sticker date". |
| 4 | For an off-cycle, future vehicle, it is the "sticker date" minus one cycle, so it marks the nominal month that the vehicle passed inspection. |
|  | For example, a vehicle with a 10/2010 sticker presented in 2009 would nominally have passed inspection in 2008, so the cycleStartDate will be set to 10/31/2008. |
| 5 | For an on-cycle, late vehicle (within 21 months), it is still the "sticker date". |
| 6 | For a vehicle with a "sticker date" more than 21 months old, the month will be retained but the year will be adjusted to a date as close to "on-time" as possible. |
|  | For example, a vehicle with a 4/2005 sticker presented in 6/2010 will have the cycleStartDate set to 4/2010. |
|  | A vehicle with a "sticker date" more than five years in the future is in error and will follow the same rule. |
| 7 | Vehicle with "sticker date" between 2 and 5 years in the future (with a new car dealer sticker) will retain the same month but will be adjusted to the current cycle. They are not due for inspection and will lose their four-year sticker. |
| CurrInsp.inspExpirationDateCycle |  |
| 1 | This attribute is the length of a vehicle's cycle in months. |
| 2 | It is set in the BeginTestCycle ruleset. |
| 3 | Currently, it is 6 months for buses, 12 months for commercial vehicles and omnibus2s, and 24 months for everything else. |

| Temp.aaChgICD |  |
| --- | --- |
| 1 | This attribute is used to indicate whether to use the oldStickerDate or the Prev inspExpirationDate. |
| 2 | It is set to 'E' for Prev IED or "Y" for oldStickerDate. |
| 3 | Whichever date is used, it is referred to in this document as 'sticker date'. |
| Temp.aaCurrMonth |  |
| 1 | This attribute holds the current month. |
| Temp.aaCycleBeg |  |
| 1 | This attribute is used in both BeginTest and EndTest rules. |
| 2 | In the BeginTest rules it is used to mark the latest date relative to today that the 'sticker date' will be rejected. |
| 3 | Any 'sticker date' that is this date or earlier will be recalculated. |
| 4 | In this case it is today - cycle length + 2 months, last day of the month. |
| 5 | It is also used to decide if an inspection is on- or off-cycle, initial or retest. |
| 6 | In this case it is cycleStartDate - 3 months, last day of the month. |
| 7 | It has a different use/value in the EndTest rules. |
| Temp.aaCycleEnd |  |
| 1 | This attribute is the end of the current inspection cycle relative to today. |
| 2 | It is today + 2 months, last day of the month. |
| Temp.aaCycleFutEnd |  |
| 1 | This attribute is the end of the next inspection cycle relative to today. |
| 2 | It is today + cycle length + 2 months, last day of the month. |
| Temp.aaCycleInvalid |  |
| 1 | This attribute is the last date for a valid dealer sticker relative to today. |
| 2 | It is today + 61 months, last day of the month. |
| Temp.aaDateCurr |  |
| 1 | This attribute is used to store a date based on the 'sticker date', the current year, and aaOffset. |
| Temp.aaDateNext |  |
| 1 | This attribute is used to store a date for next year based on aaDateCurr. |
| Temp.aaDateOffsetP12 |  |
| 1 | This attribute is used to store a date for +12 months based on aaOffset. |
| Temp.aaDateOffsetP6 |  |
| 1 | This attribute is used to store a date for +6 months based on aaOffset. |
| Temp.aaDateOffset |  |
| 1 | This attribute is used to store a date based on aaOffset. |
| Temp.aaDateOffsetM6 |  |
| 1 | This attribute is used to store a date for -6 months based on aaOffset. |

| Temp.aaDateOffsetM12 |  |
| --- | --- |
| 1 | This attribute is used to store a date for -12 months based on aaOffset. |
| Temp.aaDatePrev |  |
| 1 | This attribute is used to store a date for last year based on aaDateCurr. |
| Temp.aaOffset |  |
| 1 | This attribute is used to store a monthly offset for calculations. |
| 2 | It is set and used in both the BeginTest and the EndTest rulesets. |
| 3 | In BeginTestCycle, it is the offset to be used to determine the year for a recalculated sticker date. |
| 4 | It is the difference between the current month and the 'sticker date' month. |
| 5 | Its range is -11 to +11. |
| 6 | It has a different use/value in the EndTest rules. |
| Temp.aaOldStickerReset |  |
| 1 | This attribute indicates whether to set Curr oldStickerDate back to null. |
| Temp.aaPrevInspDateReset |  |
| 1 | This attribute indicates whether to set Prev inspDate back to null. |
| Temp.aaPrevInspExpirationDateReset |  |
| 1 | This attribute indicates whether to set Prev inspExpirationDate back to null. |
| Temp.aaString |  |
| 1 | A temporary string field. |

Begin Test Cycle Notes

CycleInitialize

Add a TempHold entity to Inspection to hold temporary attributes during processing. In the development environment, if the output xml file from BeginTestCycle is to be used as input into BeginTestSelection, this entity must be deleted from the xml file first.

Null values are set to default values. Corticon has some issues with null values, particularly for numeric fields. This process avoids those problems.

CycleValidation_v5

Certain attributes must have valid non-null values. If they do not, this rulesheet will terminate the ruleset.

SetFuelType

Sets fuelType based on fuelCode.

SetVehicleSize

Sets vehicleSize based on vehicleGVWR.

SetProgramType

Sets CurrInsp.inspProgramType based on CurrInsp.inspTypeCode.

SetNullMvcRegCode_IM

| For IM inspections, sets a default mvcRegCode if one was not entered. It is based on the three indicators: commercial, transport, omni2. |  |
| --- | --- |
| SetNullMvcRegCode_Bus |  |
| For bus inspections, sets a default mvcRegCode if one was not entered. It is based on the inspProgramType, CB or SB. |  |
| SetVehicleCategory_Bus_v5 |  |
| For bus inspections, sets the vehicleCategory. It is based on the inspProgramType, CB or SB. | VehicleCategory is used to determine the cycle length (6 mos, 12 mos, 24 mos) and whether a vehicle gets a safety inspection or not. |
| SetVehicleCategory_MC_v5 |  |
| For motorcycles, the vehicleCategory is set to 'norm'. | Motorcycles are no longer inspected. This was kept as a placeholder. |
| SetVehicleCategory_IM_v5a |  |
| Terminates the inspection if the mvcRegCode is not valid for an IM inspection. |  |
| CheckForDC |  |
| Changes the inspProgramType to DC for heavy duty Diesel trucks. Also sets vehicleCategory to 'comm' to force one-year sticker. |  |
| SetVehicleCategory_IM_v5c |  |
| For IM inspections, sets the vehicleCategory based on mvcRegCode and the three indicators. Invalid combinations cause the rules to terminate. |  |
| CheckPgmTypeAndFacility |  |
| Check if inspProgramType is valid at the current facility based on facility type and pifClass. Invalid combinations cause the rules to terminate. |  |
| CheckPgmTypeAndVehicle_Std |  |
| For non-bus vehicles, checks if vehicle configuration is valid for inspProgramType. Invalid combinations cause the rules to terminate. |  |
| SetCycleLength |  |
| Sets the cycle length (6, 12, 24 mos) based on the vehicleCategory and the inspProgramType. | The combination of DC/norm does not occur but is included for completeness. |
| SetCurrMonth |  |
| Sets certain dates required to calculate cycle dates and expiration dates. Also sets temp value aaChgICD which is actually used to decide whether to use the old sticker date or old inspExpirationDate for setting cycle/expiration dates. |  |
| SetOldStkr_Null |  |
| SetPrevIED_Null |  |
| SetPrevInspDate_Null |  |
| These three rulesheets set certain previous dates to default values when they are null. They also set switches so the dates can be changed back to null at the end. |  |
| SetCycleStartDate_1 |  |

This is the first attempt to set the cycleStartDate. It is based on the oldStickerDate if aaChgICD is 'Y' or the previous IED if it is 'E'. If the cycleStartDate is set successfully then aaString is set to 'done' If not, aaString is set to 'today' (use today as the ref date), 'stkr' (use oldStickerDate as the ref date), or 'ied' (use prevIED as the ref date). In addition, an offset is set based on the month of the ref date and the current month.

SetCycleStartDate_2

For non-bus vehicles where the cycle date was not set by SetCycleStartDate_1. This rulesheet runs when the dates are out of range, for example, more than 22 months old or more than 2 months in the future. The cycleStartDate month and year are set based on the offset value from SetCycleStartDate_1.

SetCycleStartDate_3

For bus vehicles where the cycle date was not set by SetCycleStartDate_1. This rulesheet runs when the dates are out of range, for example, more than 4 months old or more than 2 months in the future. The cycleStartDate month and year are set based on the offset value from SetCycleStartDate_1.

SetInspModeCode

Sets the mode for the inspection, i.e., cycle test ('C') or off-cycle ('O') based on previous inspection data.

SetInspCycleTypeCode

Sets the cycle type for the inspection, i.e., initial ('I') or retest ('R') based on previous inspection data.

SetInspExtentCode

Set inspExtentCode based on CurrInsp.inspCycleTypeCode.

SetInspExtentCodeBusExpire

For buses, resets a partial inspection to a full inspection if the previous inspection is more than 30 days old.

ResetSoftwareVersion_Null

ResetOldStkr_Null

ResetPrevIED_Null

ResetPrevInspDate_Null

Reset attributes back to null if they were changed. These will run even if the ruleset has been terminated.

Begin Test Selection Ruleset

TestSelectionInit1

Initialize values for BeginTestSelection ruleset.

Add a TempHold entity.

Set rules version number.

Set CurrInsp.primaryEmTestAllowed = T.

Set CurrInsp.vehicleConditionCheckRequired=F.

Set CurrInsp.gasCapRequired=F

Set CurrInsp.indicatorLightRequired=F

Set CurrInsp.liquidLeakRequired=F

Set CurrInsp.miscEmRequired=T

Set CurrInsp.tamperCheckRequired=F

Set CurrInsp.visibleSmokeRequired=F

Default null inspExtentCode to F.

Default null inspDate to today.

Default null PrevInsp.primaryEmTestCode='N'

Default null PrevInsp.primaryEmResult='N'

Default null PrevInsp.overallSafetyResult='N'

Default null PrevInsp.overallCredentialsResult='N'

Default null CurrInsp.emInspRequested='N'

Default null CurrVehicle.retrofitPresent='N'

Default null CurrInsp.fullInspRequested='N'

Default null CurrInsp.exceptionParms.rPMExclusion='N'

Terminate if testRecordNumber is null.

TestSelectionInit2

Run only if PrevInsp.primaryEmResult<>'N'.

Set PrevInsp.primaryEmResult to 'N' for certain values of:

PrevInsp.primaryEmTestCode

PrevInsp.primaryEmResult

TestSelectionInit3

Set Temp.aaPrevSafetyFail based on PrevInsp.overallSafetyResult.

TestSelectionValidate

Check the following for valid values:

CurrInsp.inspProgramType

CurrVehicle.vehicleSize

CurrVehicle.fuelType

CurrVehicle.vehicleYear=null

CurrVehicle.omni2Indicator

CurrFacility.facilityProgramCode

CurrFacility.pifClassCode

CurrWkstn.gasbenchTestAllowed

CurrWkstn.gasCapTestAllowed

CurrWkstn.oBDTestAllowed

CurrInsp.inspCycleTypeCode

CurrWkstn.smokemeterTestAllowed

CurrInsp.inspTypeCode

CurrVehicle.commercialIndicator

CurrVehicle.transportIndicator

CurrInsp.inspExpirationDateCycle=null

Any invalid value will cause termination.

ResetCatDTCCode

Set CurrInsp catDTCCode based on PrevInsp catDTCCode.

Prev is C or other, Curr = N

Prev is N, Curr = N

Prev is Y, Curr = Y

PrimaryTestSelection_IM

Run only if the inspProgramType is 'IM'.

Based on vehicleSize, fuelType, vehicleYear, and omni2Indicator set:

CurrInsp.primaryEmTestCode

CurrInsp.primaryEmTestRequired

CurrInsp.primaryEmResult = N when Required = F

CurrInsp.safetyTestType

Invalid combination will allow re-entry or termination.

PrimaryTestSelection_SB

Run only if the inspProgramType is 'SB'.

Based on vehicleSize, fuelType, and vehicleYear set:

CurrInsp.primaryEmTestCode

CurrInsp.primaryEmTestRequired

CurrInsp.primaryEmResult = N when Required = F

CurrInsp.safetyTestType

PrimaryTestSelection_CB

Run only if the inspProgramType is 'CB'.

Based on vehicleSize, fuelType, and vehicleYear set:

CurrInsp.primaryEmTestCode

CurrInsp.primaryEmTestRequired

CurrInsp.primaryEmResult = N when Required = F

CurrInsp.safetyTestType

PrimaryTestSelection_DC

Run only if the inspProgramType is 'DC'.

Based on vehicleSize and fuelType set:

CurrInsp.primaryEmTestCode

CurrInsp.primaryEmTestRequired

CurrInsp.primaryEmResult = N when Required = F

CurrInsp.safetyTestType

Invalid combination will allow re-entry or termination.

PrimaryTestSelection_MC

Run only if the inspProgramType is 'MC'.

Non-conditional:

CurrInsp.safetyTestType='MC'

CurrInsp.primaryEmTestCode='N'

CurrInsp.primaryEmResult='N'

CurrInsp.primaryEmTestRequired=F

CurrInsp.gasCapRequired=F

CurrInsp.indicatorLightRequired=F

CurrInsp.liquidLeakRequired=F

CurrInsp.visibleSmokeRequired=F

CurrInsp.tamperCheckRequired=F

CurrInsp.miscEmRequired=F

OBDExceptions

Run only if the primaryEmTestCode = 'O'.

Set the primaryEmTestCode based on obdBypass and fuelType.

Set primaryEmTestRequired to F when primaryEmTestCode is N.

Set CurrInsp.primaryEmResult = N when primaryEmTestRequired = F.

OBDExceptions_Readiness

Run only if the primaryEmTestCode = 'O'.

Run only if inspTypeCode = "CBX" or "SBX"

Set exemptFromReadiness to "Y"

TSIExceptions

Run only if the primaryEmTestCode = 'T'.

Set the primaryEmTestCode based on tsiBypass.

OpacityExceptions

Run only if the primaryEmTestCode = 'D'.

Set the primaryEmTestCode based on opacityBypass.

Set primaryEmTestRequired to F when primaryEmTestCode is N.

Set CurrInsp.primaryEmResult = N when primaryEmTestRequired = F.

SetRpmExclusion

Run only if CurrInsp.exceptionParms.rPMExclusion<>'Y'.

Set CurrInsp.exceptionParms.rPMExclusion = 'Y' based on CurrVehicle.fuelCode.

FacilityCheck

Run only if the primaryEmTestCode <> 'N'.

Set primaryEmTestAllowed based on:

CurrFacility.facilityProgramCode

CurrFacility.pifClassCode

CurrInsp.primaryEmTestCode

Invalid combination will allow re-entry or termination.

GasCapTest

Run only if the inspProgramType<>'MC'.

Set gasCapRequired based on fuelCode and vehicleYear.

Set Result = N when Required = F

IndicatorLightTest_Std

Run only for non-bus.

NOTE: indicatorLightRequired was already defaulted to false.

Set indicatorLightRequired based on:

CurrVehicle.fuelType

CurrVehicle.vehicleSize

CurrVehicle.vehicleYear

CurrInsp.primaryEmTestCode

Set Result = N when Required = F

Terminate for invalid combination.

IndicatorLightTest_Bus

Run only for vehicleSize LD, MD1, HD.

Run only if the inspProgramType<>'MC'.

NOTE: indicatorLightRequired was already defaulted to false.

Set indicatorLightRequired based on:

CurrVehicle.fuelType

CurrVehicle.vehicleSize

CurrVehicle.vehicleYear

CurrInsp.primaryEmTestCode

Set Result = N when Required = F

Terminate for invalid combination.

LiquidLeakCheck

Run only if the inspProgramType<>'MC'.

Set liquidLeakRequired based on fuelType.

Set Result = N when Required = F

TamperCheck

Run only for vehicleSize LD, MD1, HD.

Run only if the inspProgramType<>'MC'.

NOTE: tamperCheckRequired was already defaulted to false.

Set tamperCheckRequired based on fuelType, vehicleYear, and retrofitPresent

Set Result = N when Required = F

VisibleSmokeTest

Run only if the inspProgramType<>'MC'.

Set visibleSmokeRequired based on fuelType.

Set Result = N when Required = F

Elec_NoMiscEm

Run only for vehicleSize LD, MD1, HD.

Run only for non-motorcycle.

NOTE: miscEmRequired was already defaulted to true.

Set miscEmRequired based on fuelCode.

BusInitCheck_PrevEm

Run only if:

CurrInsp.inspProgramType is for a bus

CurrInsp.inspCycleTypeCode='I'

PrevInsp.emExpirationDate<>null

CurrInsp.emInspRequested='N'

emInspRequested=''S' forces emission test, ='N' makes it optional

Set to 'I' by workstation if requested by Inspector (required)

Set CurrInsp.emInspRequested:

to 'S' if bus spec inspection

if PrevInsp.emExpirationDate > today + 2 months (end of month)

(this means previous emission test is still valid)

if Opacity test and Jun, Jul, Aug, set to 'S'

otherwise, set to 'N'

if PrevInsp.emExpirationDate<= today + 2 months (end of month):

(this means previous emission test is expired)

if Opacity test and Dec, Jan, or Feb, set to 'N' (optional)

otherwise, set to 'S' (required)

BusInitCheck_NoPrevEm

Run only if:

CurrInsp.inspProgramType is for a bus

CurrInsp.inspCycleTypeCode='I'

PrevInsp.emExpirationDate=null

CurrInsp.emInspRequested='N'

emInspRequested=''S' forces emission test, ='N' makes it optional

Set to 'I' by workstation if requested by Inspector (required)

Set CurrInsp.emInspRequested:

to 'S' if bus spec inspection

if Opacity test and Jun, Jul, Aug, set to 'S' (required)

otherwise, set to 'N' (optional)

BusInitCheck_Final

Run only if:

CurrInsp.inspProgramType is for a bus

CurrInsp.inspCycleTypeCode='I'

CurrInsp.emInspRequested='N'

Set 'required' switches to false:

CurrInsp.primaryEmTestRequired

CurrInsp.gasCapRequired

CurrInsp.indicatorLightRequired

CurrInsp.liquidLeakRequired

CurrInsp.miscEmRequired

CurrInsp.tamperCheckRequired

CurrInsp.visibleSmokeRequired

NOTE: BusInit rules will not run if Inspector requests the emission tests (emInspRequested='I')

CheckSafetyInspReqd

Set CurrInsp.safetyInspRequired based on:

CurrInsp.safetyTestType

CurrInsp.inspExpirationDateCycle

Temp.aaPrevSafetyFail

NoSafetyInspReqd

Run only if CurrInsp.safetyInspRequired=F

Non-conditional - set safety results to 'N':

CurrSafety.brakeResult

CurrSafety.exhaustSystemResult

CurrSafety.miscSafetyRejectionResult

CurrInsp.overallSafetyResult

CurrSafety.safetyEquipmentResult

CurrSafety.steeringSuspensionResult

ReInspCheckPrimaryEm

Run only if:

CurrInsp.inspExtentCode='P' (partial inspection)

CurrInsp.inspCycleTypeCode = 'R' (retest)

CurrInsp.primaryEmTestRequired = T

CurrInsp.primaryEmTestCode <> 'N'

CurrInsp.emInspRequested='N'

CurrInsp.fullInspRequested='N'

These PrevInsresults must be 'not failed':

PrevInsp.primaryEmResult

PrevInsp.indicatorLightResult

PrevInsp.tamperResult

PrevInsp.visibleSmokeResult

PrevInsp.miscEmRejectionResult

If the conditions are met, set:

CurrInsp.primaryEmTestRequired to false.

CurrInsp.primaryEmResult to 'Z' (previously passed).

ReInspCheckPrimaryEm_Bus

Run only if:

CurrInsp.inspProgramType = 'CB' or CurrInsp.inspProgramType = 'SB'

CurrInsp.inspCycleTypeCode='R' (retest)

CurrInsp.primaryEmTestRequired=T

PrevInsp.primaryEmResult='N' (no prev primaryEm result)

CurrInsp.emInspRequested='N'

CurrInsp.fullInspRequested='N'

For non-spec inspections:

Set CurrInsp.primaryEmTestRequired to false

CurrInsp.primaryEmResult to 'N'

ReInspSetBypasses

Run only if:

CurrInsp.inspExtentCode='P' (partial inspection)

CurrInsp.primaryEmTestCode <> 'N'

CurrInsp.inspCycleTypeCode='R' (retest)

CurrInsp.primaryEmTestRequired = F

CurrInsp.primaryEmResult='Z' (previously passed)

Set the following based on PrevInsp values of the same fields:

CurrInsp.primaryEmTestCode

CurrInsp.obdBypass

CurrInsp.opacityBypass

CurrInsp.tsiBypass

ReInspGasCap

Run only if:

CurrInsp.inspExtentCode='P'

CurrInsp.inspCycleTypeCode = 'R'

CurrInsp.gasCapRequired=T

CurrInsp.fullInspRequested='N'

If PrevInsp.gasCapResult passed, set:

CurrInsp.gasCapRequired to false.

CurrInsp.gasCapResult to 'Z'.

ReInspCheckIndicatorLight

Run only if:

CurrInsp.inspExtentCode='P'

CurrInsp.inspCycleTypeCode = 'R'

CurrInsp.indicatorLightRequired=T

CurrInsp.fullInspRequested='N'

If PrevInsp.indicatorLightResult passed,

AND CurrInsp.primaryEmTest is not required, set:

CurrInsp.indicatorLightRequired to false.

CurrInsp.indicatorLightResult to 'Z'.

ReInspCheckLiquidLeak

Run only if:

CurrInsp.inspExtentCode='P'

CurrInsp.inspCycleTypeCode = 'R'

CurrInsp.liquidleakRequired=T

CurrInsp.fullInspRequested='N'

If PrevInsp.liquidLeakResult passed,

CurrInsp.liquidLeakRequired to false.

CurrInsp.liquidLeakResult to 'Z'.

ReInspTamperCheck

Run only if:

CurrInsp.inspExtentCode='P'

CurrInsp.inspCycleTypeCode = 'R'

CurrInsp.tamperCheckRequired=T

CurrInsp.fullInspRequested='N'

If PrevInsp.tamperResult passed,

CurrInsp.tamperCheckRequired to false.

CurrInsp.tamperResult to 'Z'.

ReInspVisibleSmoke

Run only if:

CurrInsp.inspExtentCode='P'

CurrInsp.inspCycleTypeCode = 'R'

CurrInsp.visibleSmokeRequired= T

CurrInsp.fullInspRequested='N'

If PrevInsp.visibleSmokeResult passed,

AND primaryEmTest is not required, set:

CurrInsp.visibleSmokeRequired to false.

CurrInsp.visibleSmokeResult to 'Z'.

ReInspMiscEm

Run only if:

CurrInsp.inspExtentCode='P'

CurrInsp.inspCycleTypeCode = 'R'

CurrInsp.miscEmRequired=T

CurrInsp.fullInspRequested='N'

CurrInsp.miscEmRequired is ALWAYS set to true.

If PrevInsp.miscEmRejectionResult passed,

AND primaryEmTest is not required, set:

CurrInsp.miscEmRejectionResult to 'Z'.

ReInspCheckSecondaryEm_Bus

Run only if:

CurrInsp.inspProgramType = 'CB' or CurrInsp.inspProgramType = 'SB'

CurrInsp.inspCycleTypeCode='R'

CurrInsp.primaryEmTestRequired=F

CurrInsp.primaryEmResult='N'

CurrInsp.emInspRequested='N'

CurrInsp.fullInspRequested='N'

Non-conditional - set secondary Em tests required to false:

CurrInsp.gasCapRequired

CurrInsp.indicatorLightRequired

CurrInsp.liquidLeakRequired

CurrInsp.miscEmRequired

CurrInsp.tamperCheckRequired

CurrInsp.visibleSmokeRequired

WkstnCheck_PrimaryEm

Run only if primaryEmTestCode <> 'N'.

Run only if primaryEmTestRequired is true.

Check if primary Em test can be performed based on workstation configuration.

Terminate if primary Em test is not allowed by workstation.

WkstnCheck_GasCap

Run only if gasCapRequired is true.

Terminate if gas cap test is not allowed by workstation.

SetVehConditionCheckRequired

Run only if at least one of the following is true:

CurrInsp.gasCapRequired

CurrInsp.indicatorLightRequired

CurrInsp.liquidLeakRequired

CurrInsp.miscEmRequired

CurrInsp.primaryEmTestRequired

CurrInsp.tamperCheckRequired

CurrInsp.visibleSmokeRequired

Set CurrInsp.vehicleConditionCheckRequired=T

| Begin Test Selection Data |  |
| --- | --- |
| Temp.aaCurrMonth |  |
| 1 | This attribute holds the current month. |
| Temp.aaDateCurr |  |
| 1 | InspDate plus 2 months, end of month |
| Temp.aaPrevSafetyFail |  |
| 1 | This switch shows whether the previous inspection failed for safety. |

Begin Test Selection Notes

TestSelectionInit1

Add a TempHold entity to Inspection to hold temporary attributes during processing.

Set certain default values. Null values are set to default values. Corticon has some issues with null values, particularly for numeric fields. This process avoids those problems.

TestSelectionInit2

Sets previous primaryEmResult to 'N' for certain previous primary test data values.

TestSelectionInit3

Sets a switch that determines whether to use the previous safety results.

TestSelectionValidate

Certain attributes must have valid non-null values. If they do not, this rulesheet will terminate the ruleset.

ResetCatDTCCode

Sets the current catDTCCode based on the previous value. This changes a 'C' (Clear) from the last inspection to 'N' (No Cat DTC present).

PrimaryTestSelection_IM

For IM inspections, sets the primaryEmTestCode and the safetyTestType based on vehicle configuration and transport indicators. Invalid combinations will terminate the ruleset.

PrimaryTestSelection_SB

For school bus inspections, sets the primaryEmTestCode and the safetyTestType based on vehicle configuration.

| PrimaryTestSelection_CB |  |
| --- | --- |
| For commercial bus inspections, sets the primaryEmTestCode and the safetyTestType based on vehicle configuration. |  |
| PrimaryTestSelection_DC |  |
| For heavy duty Diesel truck inspections, sets the primaryEmTestCode and the safetyTestType based on vehicle configuration. Invalid combinations will terminate the ruleset. |  |
| PrimaryTestSelection_MC |  |
| For motorcycle inspections, sets primaryEmTestCode to 'N' and sets all "required" flags to False. | Motorcycles are no longer inspected. This was kept as a placeholder. |
| OBDExceptions |  |
| For OBD tests, resets primaryEmTestCode if OBDBypass is True. Gas vehicles change to a TSI test, Diesel vehicles change to no test. |  |
| OBDExceptions_Readiness |  |
| For Bus specification inspections that are OBD tests, sets exemptFromReadiness to "Y" so buses do not have to be ready. |  |
| TSIExceptions |  |
| For TSI tests, resets primaryEmTestCode if TSIBypass is True. Changes test to an Idle test. |  |
| OpacityExceptions |  |
| For Diesel opacity tests, resets primaryEmTestCode if opacityBypass is True. Changes opacity test to no test. |  |
| SetRpmExclusion |  |
| Forces rPMExclusion to 'Y' for electric and hybrid electric vehicles since RPM is not required. |  |
| FacilityCheck |  |
| Checks if selected primary emission test can be performed at the current facility. Invalid combinations will terminate the ruleset. |  |
| GasCapTest |  |
| Checks whether gas cap test is required based on fuelCode and vehicleYear. |  |

IndicatorLightTest_Std

For non-bus vehicles, check whether indicator light test is required based on vehicle configuration and primaryEmTestCode. Invalid combinations will terminate the ruleset.

IndicatorLightTest_Bus

For buses, check whether indicator light test is required based on vehicle configuration and primaryEmTestCode. Invalid combinations will terminate the ruleset.

LiquidLeakCheck

Checks whether liquid leak check is required based on fuelCode.

TamperCheck

Checks whether tamper check is required based on fuelCode, vehicleYear, and retrofit present.

VisibleSmokeTest

Checks whether visible smoke test is required based on fuelCode.

Elec_NoMiscEm

For LD, MD1, HD, electric vehicles, miscellaneous emission test is not required.

BusInitCheck_PrevEm

For initial bus inspections with a previous emExpirationDate, and the Inspector has not requested an emissions test, check if emission test is required based on the current date and the previous emExpirationDate. Initial spec inspections always require an emissions test.

BusInitCheck_NoPrevEm

For initial bus inspections with no previous emExpirationDate, and the Inspector has not requested an emissions test, check if emission test is required based the inspTypeCode. Initial spec inspections always require an emissions test.

BusInitCheck_Final

For initial bus inspections where he Inspector has not requested an emissions test, if neither of the previous two rulesheets set emInspRequested to 'S', set the emission "required" switches to False.

CheckSafetyInspReqd

Check if a safety inspection is required based on the safetyTestType, the cycle length (6, 12, 24) and whether the previous inspection failed for safety.

NoSafetyInspReqd

If no safety inspection is required, set all safety results to 'N'.

ReInspCheckPrimaryEm

On a retest, check if primary emissions test can be marked 'Z' (passed previously). Run only if a primary emission test is required and the Inspector has not requested either an emissions test or a full inspection. If all previous inspection results are "Pass", mark the current primaryEmResult as 'Z' and required as False.

ReInspCheckPrimaryEm_Bus

For a bus retest, if the previous inspection did not require an emissions test and the Inspector has not requested either an emissions test or a full inspection, the emissions test is not required.

ReInspSetBypasses

For a retest where the primary emission test is marked 'Z', set the current bypass values based on the previous values.

ReInspGasCap

For a retest where a full inspection has not been requested, check whether gas cap can be marked as previously passed.

ReInspCheckIndicatorLight

For a retest where a full inspection has not been requested, check whether indicator light can be marked as previously passed. If current primaryEmTest is required, it must be retested.

ReInspCheckLiquidLeak

For a retest where a full inspection has not been requested, check whether liquid leak can be marked as previously passed.

ReInspTamperCheck

For a retest where a full inspection has not been requested, check whether tamper check can be marked as previously passed.

ReInspVisibleSmoke

For a retest where a full inspection has not been requested, check whether the visible smoke test can be marked as previously passed. If current

| primaryEmTest is required, it must be retested. |  |
| --- | --- |
| ReInspMiscEm |  |
| For a retest where a full inspection has not been requested, check whether the miscellaneous emissions test can be marked as previously passed. The miscellaneous emissions test is always set to true. |  |
| ReInspCheckSecondaryEm_Bus |  |
| For a bus retest, if the primary emissions test is not required and the Inspector has not requested either an emissions test or a full inspection, secondary emissions test are not required either. |  |
| WkstnCheck_PrimaryEm |  |
| Check if workstation can perform the primary test required. Invalid combinations will terminate the ruleset. | Current software allows smokemeter tests since the results are manually entered. |
| WkstnCheck_GasCap |  |
| Check if workstation can perform the gas cap test if it is required. Invalid combinations will terminate the ruleset. |  |
| SetVehConditionCheckRequired |  |
| If any emissions test is required, set vehicleConditionCheckRequired to True. |  |

End Test Ruleset

EndTestInitialize

Initialize values for EndTest ruleset.

Add a TempHold entity.

Initialize Temp.aaUsePrevInsp to true.

Set Temp.aaUsePrevInsp to true.

Set Temp.aaString to empty string.

Set CurrWkstn.rulesVersion

Default 'other' inspExtentCode to F.

Default null inspDate to today.

Terminate if testRecordNumber is null.

Terminate if CurrInsp.inspExpirationDateCycle is null.

EndTestValidation

Check the following for valid values:

CurrInsp.cycleStartDate=null

CurrInsp.inspExpirationDateCycle

CurrSafety.brakeResult

CurrSafety.exhaustSystemResult

CurrSafety.miscSafetyRejectionResult

CurrSafety.safetyEquipmentResult

CurrSafety.steeringSuspensionResult

CurrInsp.gasCapResult

CurrInsp.liquidLeakResult

CurrInsp.miscEmRejectionResult

CurrInsp.primaryEmResult

CurrInsp.overallCredentialsResult

CurrInsp.inspModeCode

CurrInsp.tamperResult

CurrInsp.visibleSmokeResult

CurrInsp.indicatorLightResult

CurrInsp.vehicleConditionOK

CurrInsp.safetyInspRequired

Any invalid value will cause termination.

SetNewExpDate_Offset

Non-conditional:

Set newExpirationDate = cycleStartDate + expirationDateCycle (eom).

Set Temp.aaOffset to the number of days for compliance based on inspProgramType.

Use 60 to represent "the end of the following month".

If the inspProgramType is unknown, use 60.

FailRefDates

Non-conditional:

Temp.aaAddToday2Days=today+2 days (for 48-hour fails.)

Set Temp.aaAddOffsetToday.

Set Temp.aaAddOffsetCycle (max compliance date).

CheckPrevInspDate_1

Run only if PrevInsp.inspDate=null (no prev insp available).

Set Temp.aaUsePrevInsp to false.

Set Temp.aaString='nullPrevEmDate'.

CheckPrevInspDate_2

Run only if Temp.aaUsePrevInsp is still true.

If Prev inspDate is before earliest allowed date (aaCycleBeg), set aaUsePrevInsp to false.

SetNullPrevInspCD

Run only if Temp.aaUsePrevInsp is still true.

Run only if PrevInsp.inspComplianceDate=null.

Set Prev inspComplianceDate to CurrInsp.inspDate.

Set Temp.aaPrevInspCDReset='y'.

SetNullPrevSafetyCD

Run only if Temp.aaUsePrevInsp is still true.

Run only if PrevInsp.safetyComplianceDate=null.

Non-conditional - Set Temp.aaPrevSafetyCDReset='y'.

Set PrevInsp.safetyComplianceDate based on PrevInsp.overallSafetyResult.

SetNullPrevEmCD

Run only if Temp.aaUsePrevInsp is still true.

Run only if PrevInsp.emComplianceDate=null.

Non-conditional:

Set Temp.aaPrevInspCDReset='y'.

Set Temp.aaString='nullPrevEmDate'.

Set null Prev emComplianceDate based on:

Prev overallEmResult.

Prev overallSafetyResult.

Temp.aaOffset

NoSafetyInspReqd

Run only if CurrInsp.safetyInspRequired is false.

Set safety results to 'N':

CurrSafety.brakeResult

CurrSafety.exhaustSystemResult

CurrSafety.miscSafetyRejectionResult

CurrSafety.safetyEquipmentResult

CurrSafety.steeringSuspensionResult

SetSafetyCollection

Create a SafetyResult entity for each valid safety result.

Terminate if no valid safety results.

SafetyResultPriority

Set a priority for each safety result.

OverallSafetyResult

Set CurrInsp.overallSafetyResult.

ForceBusMiscEmToN

Run only if:

CurrInsp.inspProgramType='CB' or CurrInsp.inspProgramType='SB'

CurrInsp.miscEmRejectionResult='P' or CurrInsp.miscEmRejectionResult='R'

If all other emResults are 'N', set miscEmRejectionResult to 'N'.

SetEmCollection

Create a EmResult entity for each valid Em result.

Terminate if no valid Em results.

EmResultPriority_Std

Run only for non-bus

Set a priority for each Em result.

EmResultPriority_Bus

Run only for bus

Set a priority for each Em result.

OverallEmResult

Set CurrInsp.overallEmResult.

OverallResult

Set CurrInsp.overallResult based on:

CurrInsp.overallCredentialsResult

CurrInsp.overallSafetyResult

CurrInsp.overallEmResult

SafetyPass

Run only if overallSafetyResult is a pass.

Non-conditional:

Set the following to aaNewExpirationDate.

CurrInsp.safetyComplianceDate

CurrInsp.safetyExpirationDate

SafetyFail_SetED

Run only if overallSafetyResult is not a pass.

Set CurrInsp.safetyExpirationDate based on Curr inspModeCode (cycle/off-cycle).

StdSafetyFail_NoPrev

Run only if overallSafetyresult is not a pass.

Run only if the programType is not for a bus.

Run only if Temp.aaUsePrevInsp=F (prev insp is not to be used).

Set safetyComplianceDate based on:

CurrInsp.inspModeCode

CurrInsp.overallSafetyResult

Temp.aaOffset

Results of '3' and 'S' are treated like 'F' and '4' respectively.

StdSafetyFail_UsePrev

Run only if overallSafetyResult is not a pass.

Run only if the programType is not for a bus.

Run only if Temp.aaUsePrevInsp=T (prev insp is to be used).

Set safetyComplianceDate based on:

CurrInsp.inspModeCode

CurrInsp.overallSafetyResult

PrevInsp.overallSafetyResult

Temp.aaOffset

Results of '3' and 'S' are treated like 'F' and '4' respectively.

BusSafetyFail_NoPrev

Run only if overallSafetyresult is not a pass.

Run only if the programType is for a bus.

Run only if Temp.aaUsePrevInsp=F (prev insp is not to be used).

Set safetyComplianceDate based on overallSafetyResult.

Results of 'F' and '4' are treated like '3' and 'S' respectively.

BusSafetyFail_UsePrev

Run only if overallSafetyResult is not a pass.

Run only if the programType is for a bus.

Run only if Temp.aaUsePrevInsp=T (prev insp is to be used).

Set safetyComplianceDate based on:

CurrInsp.overallSafetyResult

PrevInsp.overallSafetyResult

Results of 'F' and '4' are treated like '3' and 'S' respectively.

SafetyFail_CheckMaxCD

Run only if overallSafetyResult is not a pass.

Run only if CurrInsp.safetyComplianceDate > max compliance date (aaAddOffsetCycle).

Run only if cycle test (inspModeCode='C').

Set CurrInsp.safetyComplianceDate to max compliance date (aaAddOffsetCycle).

StdEmPass

Run only for non-bus.

Run only if overallEmResult is a pass.

Non-conditional:

Set the following to aaNewExpirationDate.

CurrInsp.emComplianceDate

CurrInsp.emExpirationDate

BusEmPass

Run only if overallEmResult is a pass.

Run only if the programType is for a bus.

Non-conditional:

Set the following to aaNewExpirationDate + 6 months.

CurrInsp.emComplianceDate

CurrInsp.emExpirationDate

BusEmNotDone

Run only if the programType is for a bus.

Run only if CurrInsp.overallEmResult='N'.

Set the following based on whether Temp.aaString='nullPrevEmDate' or not.

CurrInsp.emComplianceDate

CurrInsp.emExpirationDate

EmFail_SetED

Run only if overallEmResult is not a pass.

Set CurrInsp.emExpirationDate based on Curr inspModeCode (cycle/off-cycle).

EmFail_NoPrev

Run only if overallEmResult is not a pass.

Run only if Temp.aaUsePrevInsp=F (prev insp is not to be used).

Set emComplianceDate based on overallEmResult.

EmFail_UsePrev

Run only if overallEmResult is not a pass.

Run only if Temp.aaUsePrevInsp=T (prev insp is to be used).

Set emComplianceDate based on:

CurrInsp.inspModeCode

CurrInsp.overallEmResult

PrevInsp.overallEmResult

Temp.aaOffset

EmFail_CheckMaxCD

Run only if overallEmResult is not a pass.

Run only if CurrInsp.emComplianceDate > max compliance date (aaAddOffsetCycle).

Run only if cycle test (inspModeCode='C').

Set CurrInsp.emComplianceDate to max compliance date (aaAddOffsetCycle).

CredentialsPass

Run only if CurrInsp.overallCredentialsResult='A' or CurrInsp.overallCredentialsResult='P'

Non-conditional - set the following to aaNewExpirationDate:

Temp.aaCredCD

Temp.aaCredED

CredentialsFail_SetED

Run only if CurrInsp.overallCredentialsResult<>'A' and CurrInsp.overallCredentialsResult<>'P'

Set aaCredED based on inspModeCode (cycle/off-cycle)

StdCredentialsFail

Run only for non-bus.

Run only if CurrInsp.overallCredentialsResult<>'A' and CurrInsp.overallCredentialsResult<>'P'

Set aaCredCD based on inspModeCode and aaOffset.

BusCredentialsFail

Run only for bus.

Run only if CurrInsp.overallCredentialsResult<>'A' and CurrInsp.overallCredentialsResult<>'P'

Set aaCredCD based on CurrInsp.overallCredentialsResult.

CredentialsFail_CheckMaxCD

Run only if CurrInsp.overallCredentialsResult<>'A' and CurrInsp.overallCredentialsResult<>'P'

Run only if Temp.aaCredCD > max compliance date (aaAddOffsetCycle).

Run only if cycle test (inspModeCode='C').

Set aaCredCD to max compliance date (aaAddOffsetCycle).

CurrInspDates

Non-conditional:

Set Curr inspComplianceDate to earliest of emComplianceDate, safetyComplianceDate, aaCredCD.

Set Curr inspExpirationDate to earliest of emExpirationDate, safetyExpirationDate, and aaCredED.

PassStickerDate

Run only if overallResult is pass.

Non-conditional:

Set newStickerDate to CurrInsp.inspExpirationDate.

ChangeICDCode

Set changeICDCode based on facilityType = MIT/BIT, CurrInsp.overallResult and PrevInsp.changeICDCode.

SetTempCatDTCCodes

Non-conditional:

Set aaCurrCatDTCCode=null

Set aaCurrCatTamperResult='P'

Set aaPrevCatDTCCode to 'N' or 'Y' based on PrevInsp.catDTCCode

SetCatTamperFail

Run only if CurrTamper.tamperItemCode='C'

If catalytic converter tamper failed

AND catRetestExclusion is not 'Y':

Set Temp.aaCurrCatTamperResult to 'F'

Set Temp.aaCurrCatDTCCode to 'Y'

SetCatDTCCode_DTCs

Run only if OBD, aaCurrCatDTCCode not set, and Cat DTC exists:

CurrInsp.primaryEmTestCode='O'

Temp.aaCurrCatDTCCode=null

At least one DTC between P0420 and P0439.

Set aaCurrCatDTCCode based on CurrInsp.exceptionParms.cATRetestExclusion

SetCatDTCCode_PrimaryEm

Run only if aaCurrCatDTCCode not set.

Set aaCurrCatDTCCode based on:

aaPrevCatDTCCode

CurrInsp.primaryEmResult

SetCatDTCCode_Final

Non-conditional - Set CurrInsp.catDTCCode to aaCurrCatDTCCode.

EmFail_DC

Run only if CurrInsp.inspProgramType='DC' and CurrInsp.primaryEmResult='X'.

Terminate the inspection (set inspExtentCode to 'T') if all the following are PASS:

CurrInsp.indicatorLightResult

CurrInsp.visibleSmokeResult

CurrInsp.liquidLeakResult

CurrInsp.tamperResult

CurrInsp.miscEmRejectionResult

ResetPrevICD_Null

Run only if Temp.aaPrevInspCDReset='y'.

Non-conditional:

Set PrevInsp.inspComplianceDate=null

Set Temp.aaPrevInspCDReset='n'

ResetPrevEmCD_Null

Run only if Temp.aaPrevEmCDReset='y'.

Non-conditional:

Set PrevInsp.emComplianceDate=null

Set Temp.aaPrevEmCDReset='n'

ResetPrevSafetyCD_Null

Run only if Temp.aaPrevSafetyCDReset='y'

Non-conditional:

Set PrevInsp.safetyComplianceDate=null

Set Temp.aaPrevSafetyCDReset='n'

SetCatDTCCode_NonObd (This is disabled)

Run only if non-OBD and aaCurrCatDTCCode not set:

CurrInsp.primaryEmTestCode<>'O'

Temp.aaCurrCatDTCCode=null

Set aaCurrCatDTCCode based on:

Temp.aaPrevCatDTCCode

CurrInsp.primaryEmResult

CurrInsp.exceptionParms.cATRetestExclusion

SetCatDTCCode_NotReady (This is disabled)

Run only if OBD and aaCurrCatDTCCode not set:

CurrInsp.primaryEmTestCode='O'

Temp.aaCurrCatDTCCode=null

Set aaCurrCatDTCCode based on:

Temp.aaPrevCatDTCCode

CurrReady.catalystResult

CurrInsp.primaryEmResult

CurrInsp.exceptionParms.cATRetestExclusion

| End Test Data |  |
| --- | --- |
| CurrInsp.cycleStartDate |  |
| 1 | This date is marks the beginning of a vehicle's current cycle. It is actually the last day of the month that begins the cycle. |
| 2 | It is set in the BeginTestCycle ruleset. |
| 3 | For an on-cycle, on-time vehicle, it is the "sticker date". |
| 4 | For an off-cycle, future vehicle, it is the "sticker date" minus one cycle, so it marks the nominal month that the vehicle passed inspection. |
|  | For example, a vehicle with a 10/2010 sticker presented in 2009 would nominally have passed inspection in 2008, so the cycleStartDate will be set to 10/31/2008. |
| 5 | For an on-cycle, late vehicle (within 21 months), it is still the "sticker date". |
| 6 | For a vehicle with a "sticker date" more than 21 months old, the month will be retained but the year will be adjusted to a date as close to "on-time" as possible. |
|  | For example, a vehicle with a 4/2005 sticker presented in 6/2010 will have the cycleStartDate set to 4/2010. |
|  | A vehicle with a "sticker date" more than four years in the future is in error and will follow the same rule. |
| 7 | A vehicle with a "sticker date" between two and four years in the future, i.e., with a new car dealer sticker, will retain the same month but will be adjusted to the current cycle. These vehicles are not due for inspection, and will lose their four- year cycle. |
| CurrInsp.inspExpirationDateCycle |  |
| 1 | This attribute is the length of a vehicle's cycle in months. |
| 2 | It is set in the BeginTestCycle ruleset. |
| 3 | Currently, it can be 6 months for buses, 12 months for commercial vehicles and omnibus2s, and 24 months for everything else. |
| Temp.aaAddOffsetCycle |  |
| 1 | This date is the cycleStartDate plus the offset OR the end of the month following the cycleStartDate. |
| 2 | It is set and used in the EndTest ruleset. |
| 3 | It represents the maximum compliance date that can be set when a vehicle fails. |
| Temp.aaAddOffsetToday |  |
| 1 | This date is today plus the offset OR the end of next month. |
| 2 | It is set and used in the EndTest ruleset. |
| Temp.aaAddToday2Days |  |
| 1 | This date is used for 48-hour fails. |
| 2 | It is set and used in the EndTest ruleset. |

| Temp.aaCredCD |  |
| --- | --- |
| 1 | This attribute holds the credentials compliance date. |
| Temp.aaCredED |  |
| 1 | This attribute holds the credentials expiration date. |
| Temp.aaCurrCatDTCCode |  |
| 1 | This attribute holds the current CatDTCCode. |
| Temp.aaCurrCatTamperResult |  |
| 1 | This attribute shows the currCatTamper pass/fail. |
| Temp.aaCurrMonth |  |
| 1 | This attribute is used in the End Test to hold the month of the new expiration date. |
| Temp.aaCycleBeg |  |
| 1 | This attribute is used in both BeginTest and EndTest rules. |
| 2 | In the EndTest rules it is used to mark the earliest date a vehicle can be inspected relative to its cycleStartDate. |
| 3 | Any prev insp that occurs earlier than this date will be ignored. |
| 4 | It is the cycleStartDate - 2 months, first day of the month. |
| 5 | It has a different use/value in the BeginTest rules. |
| Temp.aaDateNext |  |
| 1 | This attribute is used in the End Test to temporarily hold the new expiration date. |
| Temp.aaNewExpirationDate |  |
| 1 | This date marks the new inspection expiration date when a vehicle passes (safety, emissions, and overall). |
| 2 | It is set and used in the EndTest ruleset. |
| 3 | It is the cycleStartDate plus the inspExpirationDateCycle. |
| Temp.aaMin |  |
| 1 | This attribute is used to store the minimum priority for results collections. |
| Temp.aaOffset |  |
| 1 | This attribute is used to store a monthly offset for calculations. |
| 2 | It is set and used in both the BeginTest and the EndTest rulesets. |
| 3 | In EndTest, it is the offset to be used to determine compliance dates for failed inspections. |
| 4 | It can be 30, 45, or 60; 60 is used to represent "end of next month". |
| 5 | It has a different use/value in the BeginTest rules. |

| Temp.aaPrevCatDTCCode |  |
| --- | --- |
| 1 | This attribute is calculated from the previous CatDTCCode. |
| Temp.aaPrevEmCDReset |  |
| 1 | This attribute indicates whether to set Prev emComplianceDate back to null. |
| Temp.aaPrevInspCDReset |  |
| 1 | This attribute indicates whether to set Prev inspComplianceDate back to null. |
| Temp.aaPrevSafetyCDReset |  |
|  | This attribute indicates whether to set Prev safetyComplianceDate back to null. |
| Temp.aaUsePrevInsp |  |
| 1 | This attribute indicates whether to use any prev insp data. |
| 2 | Prev insps with inspDate earlier than Temp.aaCycleBeg will not be used. |

End Test Notes

EndTestInitialize

Add a TempHold entity to Inspection to hold temporary attributes during processing.

Set certain default values. Null values are set to default values. Corticon has some issues with null values, particularly for numeric fields. This process avoids those problems.

InspExpirationDateCycle must be valid or ruleset will terminate.

EndTestValidation

Certain attributes must have valid non-null values. If they do not, this rulesheet will terminate the ruleset.

SetNewExpDate_Offset

Sets aaNewExpirationDate based on inspExpirationDateCycle, and sets an offset value based on the inspProgramType. This offset is used to calculate compliance dates for failures.

FailRefDates

Sets reference dates for failures based on offset.

CheckPrevInspDate_1

If there is no previous inspection date, set aaUsePrevInsp to False.

CheckPrevInspDate_2

If there is a previous inspection date, but it is before the beginning of the cycle, i.e., prior to aaCycleBeg, set aaUsePrevInsp to False.

SetNullPrevInspCD

If the previous inspection is to be used, but the previous inspection compliance date is null, set it to the current insp date.

SetNullPrevSafetyCD

If the previous inspection is to be used, but the previous safety compliance date is null, set it based on the previous safety result.

SetNullPrevEmCD

If the previous inspection is to be used, but the previous emission compliance date is null, set it based on the previous safety result, previous emission result, and the compliance offset.

NoSafetyInspReqd

If no safety inspection is required, set all safety results to 'N'.

SetSafetyCollection

Create a collection of safety test results from detail safety results.

SafetyResultPriority

Assign a priority number to each safety result in the collection. The lower the number, the higher the priority.

OverallSafetyResult

Select the lowest priority number in the safety collection and assign a test result value.

ForceBusMiscEmToN

For bus, if the miscEmResult is a Pass, but all other emission results are N (not required), set miscEmResult to N.

SetEmCollection

Create a collection of emission test results from detail emission results.

EmResultPriority_Std

For non-bus, assign a priority number to each emission result in the collection. The lower the number, the higher the priority.

EmResultPriority_Std

For bus, assign a priority number to each emission result in the collection. The lower the number, the higher the priority.

OverallEmResult

Select the lowest priority number in the emission collection and assign a test result value.

OverallResult

Set the overall result based on the Credentials result passed in and the calculated overallEmResult and overallSafetyResult.

SafetyPass

If safety passed set both the safety compliance date and safety expiration date to the new expiration date.

SafetyFail_SetED

If safety failed set the safety expiration date based on on-cycle/off-cycle mode code.

StdSafetyFail_NoPrev

For non-bus with no previous data and safety failed, set safety compliance date based on the mode, the safety result and the compliance offset.

StdSafetyFail_UsePrev

For non-bus with previous data and safety failed, set safety compliance date based on the mode, the previous safety result, the current safety result and the compliance offset.

BusSafetyFail_NoPrev

For bus with no previous data and safety failed, set safety compliance date based on the safety result.

BusSafetyFail_UsePrev

For bus with previous data and safety failed, set safety compliance date based on the previous safety result and the current safety result.

SafetyFail_CheckMaxCD

For on-cycle inspections aaAddOffsetCycle is the latest possible compliance date. For failed safety with a calculated safety compliance date after this date, change the compliance date to this date.

StdEmPass

For non-bus with emissions passed set both the emission compliance date and emission expiration date to the new expiration date.

BusEmPass

For bus with emissions passed set both the emission compliance date and emission expiration date to the new expiration date plus six months, since emissions inspection is only required annually.

BusEmNotDone

For bus with no emission test, set the emission compliance date and the emission expiration date to match the later of the current safety expiration date or previous emission expiration date.

EmFail_SetED

For failed emission test, set the emission expiration date based on the on- cycle/off-cycle mode code.

EmFail_NoPrev

For failed emission test with no previous data, set the emission compliance date based on the overall emission result.

EmFail_UsePrev

For failed emission test with previous data, set the emission compliance date based on the on-cycle/off-cycle mode code, the current emission result, the previous emission result, and the compliance offset.

EmFail_CheckMaxCD

For on-cycle inspections aaAddOffsetCycle is the latest possible compliance date. For failed emissions with a calculated emission compliance date after this date, change the compliance date to this date.

CredentialsPass

For credentials passed set both the temp credentials compliance date and expiration date to the new expiration date.

CredentialsFail_SetED

For credentials failed set the temp credentials expiration date based on on-cycle/off-cycle mode code.

StdCredentialsFail

For non-bus with credentials failed, set temp credentials compliance date based on the mode and the compliance offset.

BusCredentialsFail

For bus v failed, set temp credentials compliance date based on the credentials result.

CredentialsFail_CheckMaxCD

| For on-cycle inspections aaAddOffsetCycle is the latest possible compliance date. For failed credentials with a temp credentials compliance date after this date, change the compliance date to this date. |  |
| --- | --- |
| CurrInspDates |  |
| Set the current inspection compliance date and inspection expiration date based on earliest dates for safety, emissions, and credentials. |  |
| PassStickerDate |  |
| For passed inspections, set the newStickerDate to the inspExpirationDate. |  |
| ChangeICDCode |  |
| The chgICDCode is used by the next inspection. If it is failed, this switch is used to decide whether to change the compliance date or leave it as-is. Set the chgICDCode based on whether it is a MIT/BIT or not, the current overall result and the previous chgICDCode. |  |
| SetTempCatDTCCodes |  |
| There are two temp fields for catDTCCode, one for current and one for previous, and another temp code for current catTamperResult. Set the current cat code to null and the current cat tamper result to Pass. Set the temp previous cat code based on the previous catDTCCode. | The next sequence of rulesheets sets the current catDTCCode which affects the pass/fail decision for the next inspection. Once aaCurrCatDTCCode is set to a value the subsequent rulesheets are skipped. |
| SetCatTamperFail |  |
| If the current test has a Cat tamper fail and is not excluded from CatRetest, aaCurrCatDTCCode is set to 'Y'. |  |
| SetCatDTCCode_DTCs |  |
| If the current test is an OBD test with Cat DTCs, aaCurrCatDTCCode is set to 'Y'. |  |
| SetCatDTCCode_PrimaryEm |  |
| Set aaCurrCatDTCCode based on aaPrevCatDTCCode and the current primary emission result. If aaPrevCatDTCCode is 'N', set current to 'N'. Otherwise aaPrevCatDTCCode is 'Y'. If the current primary emission test passed, set aaCurrCatDTCCode to 'C' (Clear); otherwise set it to 'Y'. |  |
| SetCatDTCCode_Final |  |
| Set CurrInsp.catDTCCode to aaCurrCatDTCCode. |  |
| EmFail_DC |  |
| For Diesel Compliance test (DC) where the opacity test was not performed, terminate the inspection if all other emission tests are passed. Otherwise, no action is taken and the inspection will fail for emissions. |  |
| ResetPrevICD_Null |  |
| ResetPrevEmCD_Null |  |
| ResetPrevSafetyCD_Null |  |
| These three steps reset previous inspection attributes back to null. |  |

Rules Engine Input-Output Definition Rules Engine Vocabulary

| Entity_v7 | Attribute_v7 |
| --- | --- |
| CollectorVehicleDetails | collectorVehicleStatus |
| CollectorVehicleDetails | collectorVehicleVoucherDate |
| CollectorVehicleDetails | odometerReading |
| CollectorVehicleDetails | prevInspDate |
| CollectorVehicleDetails | prevOdometerReading |
| CollectorVehicleDetails | qualified |
| CredentialsConditions | credentialsCondition |
| DieselEmDetails | opacityCutpoint |
| DieselEmDetails | opacityReading |
| DieselEmDetails | opacityTestType |
| DieselEmDetails | riseTimeLimit |
| DTCCodes | dTCCode |
| EmResult | description |
| EmResult | priority |
| EmResult | resultCode |
| Error | errorCode |
| Error | errorDetails |
| Error | errorLevel |
| ExceptionParms | bypassOBDAllowed |
| ExceptionParms | cATRetestExclusion |
| ExceptionParms | continuousMonitorExclusion |
| ExceptionParms | cVNExclusion |
| ExceptionParms | exemptFromLift |
| ExceptionParms | exemptFromOBD |
| ExceptionParms | exemptFromOpacity |
| ExceptionParms | exemptFromReadiness |
| ExceptionParms | exemptFromTSI |
| ExceptionParms | requiresRollTest |
| ExceptionParms | rPMExclusion |
| Facility | facilityID |
| Facility | facilityOperationCode |
| Facility | facilityProgramCode |
| Facility | facilityType |
| Facility | fleetIndicator |
| Facility | licenseStatusCode |
| Facility | mobileIndicator |
| Facility | pifClassCode |
| IdleEmDetails | idleTestResult1 |
| IdleEmDetails | idleTestResult2 |
| IndicatorLightItemDetails | indicatorLightItemCode |
| IndicatorLightItemDetails | lightCheckResult |
| IndicatorLightItemDetails | lightOnResult |
| Inspection | catDTCCode |
| Inspection | changeICDCode |

| Entity_v7 | Attribute_v7 |
| --- | --- |
| Inspection | cycleStartDate |
| Inspection | emComplianceDate |
| Inspection | emExpirationDate |
| Inspection | emInspRequested |
| Inspection | fullInspRequested |
| Inspection | gasCapRequired |
| Inspection | gasCapResult |
| Inspection | idleBypass |
| Inspection | indicatorLightRequired |
| Inspection | indicatorLightResult |
| Inspection | inspComplianceDate |
| Inspection | inspCycleTypeCode |
| Inspection | inspDate |
| Inspection | inspExpirationDate |
| Inspection | inspExpirationDateCycle |
| Inspection | inspExtentCode |
| Inspection | inspModeCode |
| Inspection | inspProgramType |
| Inspection | inspTypeCode |
| Inspection | liftRequired |
| Inspection | liquidLeakRequired |
| Inspection | liquidLeakResult |
| Inspection | miscEmRejectionResult |
| Inspection | miscEmRequired |
| Inspection | newStickerDate |
| Inspection | obdBypass |
| Inspection | odometerReading |
| Inspection | offlineInspCode |
| Inspection | oldStickerDate |
| Inspection | opacityBypass |
| Inspection | overallCredentialsResult |
| Inspection | overallEmResult |
| Inspection | overallResult |
| Inspection | overallSafetyResult |
| Inspection | primaryEmResult |
| Inspection | primaryEmTestAllowed |
| Inspection | primaryEmTestCode |
| Inspection | primaryEmTestRequired |
| Inspection | safetyComplianceDate |
| Inspection | safetyExpirationDate |
| Inspection | safetyInspRequired |
| Inspection | safetyTestType |
| Inspection | tamperCheckRequired |
| Inspection | tamperResult |
| Inspection | testRecordNumber |
| Inspection | tsiBypass |
| Inspection | vehicleConditionCheckRequired |

| Entity_v7 | Attribute_v7 |
| --- | --- |
| Inspection | vehicleConditionOK |
| Inspection | visibleSmokeRequired |
| Inspection | visibleSmokeResult |
| ObdEmDetails | dLCResultCode |
| ObdEmDetails | dTCStoredCount |
| ObdEmDetails | milCommandOnResultCode |
| ObdEmDetails | obdCommResultCode |
| ObdEmDetails | readinessResult |
| ObdEmDetails | unsetMonitorCount |
| PrevInspection | catDTCCode |
| PrevInspection | changeICDCode |
| PrevInspection | emComplianceDate |
| PrevInspection | emExpirationDate |
| PrevInspection | gasCapResult |
| PrevInspection | idleBypass |
| PrevInspection | indicatorLightResult |
| PrevInspection | inspComplianceDate |
| PrevInspection | inspCycleTypeCode |
| PrevInspection | inspDate |
| PrevInspection | inspExpirationDate |
| PrevInspection | inspExtentCode |
| PrevInspection | inspModeCode |
| PrevInspection | inspProgramType |
| PrevInspection | inspTypeCode |
| PrevInspection | liquidLeakResult |
| PrevInspection | miscEmRejectionResult |
| PrevInspection | newStickerDate |
| PrevInspection | obdBypass |
| PrevInspection | odometerReading |
| PrevInspection | offlineInspCode |
| PrevInspection | opacityBypass |
| PrevInspection | overallCredentialsResult |
| PrevInspection | overallEmResult |
| PrevInspection | overallResult |
| PrevInspection | overallSafetyResult |
| PrevInspection | primaryEmResult |
| PrevInspection | primaryEmTestCode |
| PrevInspection | safetyComplianceDate |
| PrevInspection | safetyExpirationDate |
| PrevInspection | tamperResult |
| PrevInspection | tsiBypass |
| PrevInspection | visibleSmokeResult |
| ReadinessMonitorDetails | airConditioningResult |
| ReadinessMonitorDetails | catalystResult |
| ReadinessMonitorDetails | compComponentResult |
| ReadinessMonitorDetails | egrSystemResult |
| ReadinessMonitorDetails | evapSystemResult |

| Entity_v7 | Attribute_v7 |
| --- | --- |
| ReadinessMonitorDetails | fuelSystemResult |
| ReadinessMonitorDetails | heatedCatalystResult |
| ReadinessMonitorDetails | misfireResult |
| ReadinessMonitorDetails | oxygenSensorHeaterResult |
| ReadinessMonitorDetails | oxygenSensorResult |
| ReadinessMonitorDetails | secondaryAirSystemResult |
| SafetyConditions | safetyCondition |
| SafetyDetails | brakeResult |
| SafetyDetails | exhaustSystemResult |
| SafetyDetails | miscSafetyRejectionResult |
| SafetyDetails | safetyEquipmentResult |
| SafetyDetails | steeringSuspensionResult |
| SafetyResult | description |
| SafetyResult | priority |
| SafetyResult | resultCode |
| TamperItemDetails | tamperItemCode |
| TamperItemDetails | tamperItemResult |
| TempHold | aaAddOffsetCycle |
| TempHold | aaAddOffsetToday |
| TempHold | aaAddToday2Days |
| TempHold | aaBoolean |
| TempHold | aaChgICD |
| TempHold | aaCredCD |
| TempHold | aaCredED |
| TempHold | aaCurrCatDTCCode |
| TempHold | aaCurrCatTamperResult |
| TempHold | aaCurrMonth |
| TempHold | aaCycleBeg |
| TempHold | aaCycleEnd |
| TempHold | aaCycleFutEnd |
| TempHold | aaCycleInvalid |
| TempHold | aaDateCurr |
| TempHold | aaDateNext |
| TempHold | aaDateOffset |
| TempHold | aaDateOffsetM12 |
| TempHold | aaDateOffsetM6 |
| TempHold | aaDateOffsetP12 |
| TempHold | aaDateOffsetP6 |
| TempHold | aaDatePrev |
| TempHold | aaMax |
| TempHold | aaMin |
| TempHold | aaNewExpirationDate |
| TempHold | aaOffset |
| TempHold | aaOldStickerReset |
| TempHold | aaPrevCatDTCCode |
| TempHold | aaPrevEmCDReset |
| TempHold | aaPrevInspCDReset |

| Entity_v7 | Attribute_v7 |
| --- | --- |
| TempHold | aaPrevInspDateReset |
| TempHold | aaPrevInspExpirationDateReset |
| TempHold | aaPrevSafetyCDReset |
| TempHold | aaPrevSafetyFail |
| TempHold | aaString |
| TempHold | aaUsePrevInsp |
| TSIEmDetails | highTestResult1 |
| TSIEmDetails | highTestResult2 |
| TSIEmDetails | idleTestResult1 |
| TSIEmDetails | idleTestResult2 |
| Vehicle | commercialIndicator |
| Vehicle | engineHP |
| Vehicle | engineMake |
| Vehicle | engineSpeed |
| Vehicle | engineYear |
| Vehicle | fuelCode |
| Vehicle | fuelType |
| Vehicle | mvcRegCode |
| Vehicle | mvcVehicleTypeCode |
| Vehicle | nj1Indicator |
| Vehicle | numberOfPassengers |
| Vehicle | numOfCylinders |
| Vehicle | omni2Indicator |
| Vehicle | otherEngineCode |
| Vehicle | plateJurisdiction |
| Vehicle | plateNumber |
| Vehicle | retrofitPresent |
| Vehicle | transmissionCode |
| Vehicle | transportIndicator |
| Vehicle | vehicleCategory |
| Vehicle | vehicleGVWR |
| Vehicle | vehicleMake |
| Vehicle | vehicleModel |
| Vehicle | vehicleSize |
| Vehicle | vehicleYear |
| Vehicle | vinDocument |
| Vehicle | vinObd |
| Vehicle | vinVehicle |
| Workstation | configVersion |
| Workstation | gasbenchSoftwareVersion |
| Workstation | gasbenchTestAllowed |
| Workstation | gasCapTestAllowed |
| Workstation | oBDTestAllowed |
| Workstation | rulesVersion |
| Workstation | smokemeterTestAllowed |
| Workstation | softwareVersion |

## App_Rules Inputs

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection |  |  |  |  | Required | Required | Required |  |
|  |  |  |  | catDTCCode | Required | nullable | nullable | C, N, Y |
|  |  |  |  | changeICDCode | nullable | nullable | nullable | E, N, M, Y |
|  |  |  |  | cycleStartDate | Required | nullable | Required | Date |
|  |  |  |  | emComplianceDate | nullable | nullable | nullable | Date |
|  |  |  |  | emExpirationDate | nullable | nullable | nullable | Date |
|  |  |  |  | emInspRequested | Required | nullable | nullable | I, N, S |
|  |  |  |  | fullInspRequested | nullable | nullable | nullable | N, Y |
|  |  |  |  | gasCapRequired | Required | nullable | nullable | F, T |
|  |  |  |  | gasCapResult | Required | not available | not available | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | indicatorLightRequired | Required | nullable | nullable | F, T |
|  |  |  |  | indicatorLightResult | Required | not available | not available | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | inspComplianceDate | nullable | nullable | nullable | Date |
|  |  |  |  | inspCycleTypeCode | Required | nullable | Required | I, R |
|  |  |  |  | inspDate | Required | Required | Required | Date |
|  |  |  |  | inspExpirationDate | nullable | nullable | nullable | Date |
|  |  |  |  | inspExpirationDateCycle | Required | nullable | Required | 6, 12, 24 |
|  |  |  |  | inspExtentCode | Required | nullable | Required | F, P, T |
|  |  |  |  | inspModeCode | Required | nullable | Required | C, O |
|  |  |  |  | inspProgramType | Required | nullable | Required | CB, DC, IM, MC, SB |
|  |  |  |  | inspTypeCode | Required | Required | Required | CBA, CBC, CBD, CBG, CBR, CBS, CBX, DC, IMC, MC, SBA, SBC, SBS, SBX |
|  |  |  |  | liftRequired | nullable | nullable | nullable | F, T |
|  |  |  |  | liquidLeakRequired | Required | nullable | nullable | F, T |
|  |  |  |  | liquidLeakResult | Required | not available | not available | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | miscEmRejectionResult | Required | not available | not available | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | miscEmRequired | Required | nullable | nullable | F, T |
|  |  |  |  | newStickerDate | not available | not available | not available | Date |
|  |  |  |  | obdBypass | Required | nullable | nullable | F, T |
|  |  |  |  | odometerReading | nullable | nullable | nullable | Integer |
|  |  |  |  | offlineInspCode | nullable | nullable | nullable | N, Y |
|  |  |  |  | oldStickerDate | nullable | nullable | nullable | Date |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | opacityBypass | Required | nullable | nullable | F, T |
|  |  |  |  | overallCredentialsResult | Required | not available | not available | 3 ,A ,F ,P ,S |
|  |  |  |  | overallEmResult | nullable | not available | not available | 3 ,A ,F ,P ,S |
|  |  |  |  | overallResult | nullable | not available | not available | 3 ,A ,F ,P ,S |
|  |  |  |  | overallSafetyResult | nullable | not available | not available | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | primaryEmResult | Required | not available | not available | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | primaryEmTestAllowed | Required | nullable | nullable | F, T |
|  |  |  |  | primaryEmTestCode | Required | nullable | nullable | D, I, N, O, T |
|  |  |  |  | primaryEmTestRequired | Required | nullable | nullable | F, T |
|  |  |  |  | safetyComplianceDate | nullable | nullable | nullable | Date |
|  |  |  |  | safetyExpirationDate | nullable | nullable | nullable | Date |
|  |  |  |  | safetyInspRequired | Required | nullable | nullable | F, T |
|  |  |  |  | safetyTestType | Required | nullable | nullable | CBU, Diesel, IM, MC, SBU |
|  |  |  |  | tamperCheckRequired | Required | nullable | nullable | F, T |
|  |  |  |  | tamperResult | Required | not available | not available | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | testRecordNumber | Required | Required | Required | Integer |
|  |  |  |  | tsiBypass | Required | nullable | nullable | F, T |
|  |  |  |  | vehicleConditionCheckRequired | nullable | nullable | nullable | F, T |
|  |  |  |  | vehicleConditionOK | Required | nullable | nullable | N, Y |
|  |  |  |  | visibleSmokeRequired | Required | nullable | nullable | F, T |
|  |  |  |  | visibleSmokeResult | Required | not available | not available | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  | collectorVehicleDetails |  |  |  | NOT USED | NOT USED | NOT USED | NOT USED |
|  | credentialsConditions |  |  |  | optional | not available | not available | one‐to‐many |
|  |  |  |  | credentialsCondition | Required | not available | not available | Integer |
|  | dieselEmDetails |  |  |  | optional | not available | optional | one‐to‐one |
|  |  |  |  | opacityCutpoint | Required | not available | nullable | Integer |
|  |  |  |  | opacityReading | Required | not available | not available | Decimal |
|  |  |  |  | opacityTestType | Required | not available | not available | P, R, S |
|  |  |  |  | riseTimeLimit | nullable | not available | nullable | Decimal |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | emResult |  |  |  | Rules use only | Rules use only | Rules use only | one‐to‐many |
|  | error |  |  |  | output only | output only | output only | one‐to‐many |
|  | exceptionParms |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | bypassOBDAllowed | nullable | nullable | nullable | N, Y |
|  |  |  |  | cATRetestExclusion | nullable | nullable | nullable | N, Y |
|  |  |  |  | continuousMonitorExclusion | nullable | nullable | nullable | N, Y |
|  |  |  |  | cVNExclusion | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromLift | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromOBD | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromOpacity | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromReadiness | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromTSI | nullable | nullable | nullable | N, Y |
|  |  |  |  | requiresRollTest | nullable | nullable | nullable | N, Y |
|  |  |  |  | rPMExclusion | nullable | nullable | nullable | N, Y |
|  | facility |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | facilityID | nullable | nullable | nullable | string |
|  |  |  |  | facilityOperationCode | nullable | nullable | nullable | C, S |
|  |  |  |  | facilityProgramCode | Required | nullable | Required | C, D |
|  |  |  |  | facilityType | Required | Required | Required | CIF, PIF, PFF, SIF, MIT, BIT, ERF, DEC, DEF |
|  |  |  |  | fleetIndicator | nullable | nullable | nullable | N, Y |
|  |  |  |  | licenseStatusCode | nullable | nullable | nullable | A, C, P, R, S |
|  |  |  |  | mobileIndicator | nullable | nullable | nullable | N, Y |
|  |  |  |  | pifClassCode | Required | Required | Required | 1, 2, 3, 4, 5 |
|  | idleEmDetails |  |  |  | optional | not available | not available | one‐to‐one |
|  |  |  |  | idleTestResult1 | Required | not available | not available | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult2 | Required | not available | not available | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  | indicatorLightItemDetails |  |  |  | optional | not available | not available | one‐to‐many |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | indicatorLightItemCode | Required | not available | not available | D, M, N |
|  |  |  |  | lightCheckResult | Required | not available | not available | F, P |
|  |  |  |  | lightOnResult | Required | not available | not available | F, P |
|  | obdEmDetails |  |  |  | optional | not available | not available | one‐to‐one |
|  |  |  |  | dLCResultCode | Required | not available | not available | D, M O, P |
|  |  |  |  | dTCStoredCount | nullable | not available | not available | Integer |
|  |  |  |  | milCommandOnResultCode | Required | not available | not available | F, P |
|  |  |  |  | obdCommResultCode | Required | not available | not available | F, P |
|  |  |  |  | readinessResult | nullable | not available | not available | E, R, X |
|  |  |  |  | unsetMonitorCount | nullable | not available | not available | Integer |
|  |  | dTCCodes |  |  | optional | not available | not available | one‐to‐many |
|  |  |  |  | dTCCode | nullable | not available | not available | String |
|  |  | ReadinessMonitorDetails |  |  | optional | not available | not available | one‐to‐one |
|  |  |  |  | airConditioningResult | Required | not available | not available | R, U, X |
|  |  |  |  | catalystResult | Required | not available | not available | R, U, X |
|  |  |  |  | compComponentResult | Required | not available | not available | R, U, X |
|  |  |  |  | egrSystemResult | Required | not available | not available | R, U, X |
|  |  |  |  | evapSystemResult | Required | not available | not available | R, U, X |
|  |  |  |  | fuelSystemResult | Required | not available | not available | R, U, X |
|  |  |  |  | heatedCatalystResult | Required | not available | not available | R, U, X |
|  |  |  |  | misfireResult | Required | not available | not available | R, U, X |
|  |  |  |  | oxygenSensorHeaterResult | Required | not available | not available | R, U, X |
|  |  |  |  | oxygenSensorResult | Required | not available | not available | R, U, X |
|  |  |  |  | secondaryAirSystemResult | Required | not available | not available | R, U, X |
|  | prevInspection |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | catDTCCode | nullable | nullable | nullable | C, N, Y |
|  |  |  |  | changeICDCode | nullable | nullable | nullable | E, N, M, Y |
|  |  |  |  | emComplianceDate | nullable | nullable | nullable | Date |
|  |  |  |  | emExpirationDate | nullable | nullable | nullable | Date |
|  |  |  |  | gasCapResult | nullable | nullable | nullable | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | indicatorLightResult | nullable | nullable | nullable | S ,3 ,F ,N ,P ,R ,X ,Z |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | inspComplianceDate | nullable | nullable | nullable | Date |
|  |  |  |  | inspCycleTypeCode | nullable | nullable | nullable | I, R |
|  |  |  |  | inspDate | nullable | nullable | nullable | Date |
|  |  |  |  | inspExpirationDate | nullable | nullable | nullable | Date |
|  |  |  |  | inspExtentCode | nullable | nullable | nullable | F, P, T |
|  |  |  |  | inspModeCode | nullable | nullable | nullable | C, O |
|  |  |  |  | inspProgramType | nullable | nullable | nullable | CB, DC, IM, MC, SB |
|  |  |  |  | inspTypeCode | nullable | nullable | nullable | CBA, CBC, CBD, CBG, CBR, CBS, CBX, DC, IMC, MC, SBA, SBC, SBS, SBX |
|  |  |  |  | liquidLeakResult | nullable | nullable | nullable | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | miscEmRejectionResult | nullable | nullable | nullable | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | newStickerDate | nullable | nullable | nullable | Date |
|  |  |  |  | obdBypass | nullable | nullable | nullable | F, T |
|  |  |  |  | odometerReading | nullable | nullable | nullable | Integer |
|  |  |  |  | offlineInspCode | nullable | nullable | nullable | N, Y |
|  |  |  |  | oldStickerDate | nullable | nullable | nullable | Date |
|  |  |  |  | opacityBypass | nullable | nullable | nullable | F, T |
|  |  |  |  | overallCredentialsResult | nullable | nullable | nullable | 3 ,A ,F ,P ,S |
|  |  |  |  | overallEmResult | nullable | nullable | nullable | 3 ,A ,F ,P ,S |
|  |  |  |  | overallResult | nullable | nullable | nullable | 3 ,A ,F ,P ,S |
|  |  |  |  | overallSafetyResult | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | primaryEmResult | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | primaryEmTestCode | nullable | nullable | nullable | D, I, N, O, T |
|  |  |  |  | safetyComplianceDate | nullable | nullable | nullable | Date |
|  |  |  |  | safetyExpirationDate | nullable | nullable | nullable | Date |
|  |  |  |  | tamperResult | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | tsiBypass | nullable | nullable | nullable | F, T |
|  |  |  |  | visibleSmokeResult | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  | collectorVehicleDetails |  |  | NOT USED | NOT USED | NOT USED | NOT USED |
|  |  | credentialsConditions |  |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | credentialsCondition | nullable | nullable | nullable | Integer |
|  |  | dieselEmDetails |  |  | optional | optional | optional | one‐to‐one |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | opacityCutpoint | nullable | nullable | nullable | Integer |
|  |  |  |  | opacityReading | nullable | nullable | nullable | Decimal |
|  |  |  |  | opacityTestType | nullable | nullable | nullable | P, R, S |
|  |  |  |  | riseTimeLimit | nullable | nullable | nullable | Decimal |
|  |  | exceptionParms |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | bypassOBDAllowed | nullable | nullable | nullable | N, Y |
|  |  |  |  | cATRetestExclusion | nullable | nullable | nullable | N, Y |
|  |  |  |  | continuousMonitorExclusion | nullable | nullable | nullable | N, Y |
|  |  |  |  | cVNExclusion | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromLift | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromOBD | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromOpacity | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromReadiness | nullable | nullable | nullable | N, Y |
|  |  |  |  | exemptFromTSI | nullable | nullable | nullable | N, Y |
|  |  |  |  | requiresRollTest | nullable | nullable | nullable | N, Y |
|  |  |  |  | rPMExclusion | nullable | nullable | nullable | N, Y |
|  |  | facility |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | facilityID | nullable | nullable | nullable | string |
|  |  |  |  | facilityOperationCode | nullable | nullable | nullable | C, S |
|  |  |  |  | facilityProgramCode | nullable | nullable | nullable | C, D |
|  |  |  |  | facilityType | nullable | nullable | nullable | CIF, PIF, PFF, SIF, MIT, BIT, ERF, DEC, DEF |
|  |  |  |  | fleetIndicator | nullable | nullable | nullable | N, Y |
|  |  |  |  | licenseStatusCode | nullable | nullable | nullable | A, C, P, R, S |
|  |  |  |  | mobileIndicator | nullable | nullable | nullable | N, Y |
|  |  |  |  | pifClassCode | nullable | nullable | nullable | 1, 2, 3, 4, 5 |
|  |  | idleEmDetails |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | idleTestResult1 | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult2 | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  | indicatorLightItemDetails |  |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | indicatorLightItemCode | nullable | nullable | nullable | D, M, N |
|  |  |  |  | lightCheckResult | nullable | nullable | nullable | F, P |
|  |  |  |  | lightOnResult | nullable | nullable | nullable | F, P |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | obdEmDetails |  |  | nullable | nullable | nullable | one‐to‐one |
|  |  |  |  | dLCResultCode | nullable | nullable | nullable | D, M O, P |
|  |  |  |  | dTCStoredCount | nullable | nullable | nullable | Integer |
|  |  |  |  | milCommandOnResultCode | nullable | nullable | nullable | F, P |
|  |  |  |  | obdCommResultCode | nullable | nullable | nullable | F, P |
|  |  |  |  | readinessResult | nullable | nullable | nullable | E, R, X |
|  |  |  |  | unsetMonitorCount | nullable | nullable | nullable | Integer |
|  |  |  | dTCCodes |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | dTCCode | nullable | nullable | nullable | String |
|  |  |  | ReadinessMonitorDetails |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | airConditioningResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | catalystResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | compComponentResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | egrSystemResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | evapSystemResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | fuelSystemResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | heatedCatalystResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | misfireResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | oxygenSensorHeaterResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | oxygenSensorResult | nullable | nullable | nullable | R, U, X |
|  |  |  |  | secondaryAirSystemResult | nullable | nullable | nullable | R, U, X |
|  |  | safetyDetails |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | brakeResult | nullable | nullable | nullable | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | exhaustSystemResult | nullable | nullable | nullable | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | miscSafetyRejectionResult | nullable | nullable | nullable | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | safetyEquipmentResult | nullable | nullable | nullable | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | steeringSuspensionResult | nullable | nullable | nullable | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  | safetyConditions |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | safetyCondition | nullable | nullable | nullable | Integer |
|  |  | tamperItemDetails |  |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | tamperItemCode | nullable | nullable | nullable | C, D, N, R |
|  |  |  |  | tamperItemResult | nullable | nullable | nullable | 3, F, N, P, S, X, Z |
|  |  | tSIEmDetails |  |  | optional | optional | optional | one‐to‐one |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | highTestResult1 | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | highTestResult2 | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult1 | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult2 | nullable | nullable | nullable | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  | vehicle |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | commercialIndicator | nullable | nullable | nullable | N, Y |
|  |  |  |  | cylindersCode | nullable | nullable | nullable | 1 to 16, E, R |
|  |  |  |  | engineHP | nullable | nullable | nullable | Decimal |
|  |  |  |  | engineMake | nullable | nullable | nullable | String |
|  |  |  |  | engineSpeed | nullable | nullable | nullable | L, M, H |
|  |  |  |  | engineYear | nullable | nullable | nullable | Integer |
|  |  |  |  | fuelCode | nullable | nullable | nullable | CNGA, DIES, ELEC, ELED, ELEG, GASO, GCNG, GHYD, GLNG, GLPG, GMET, GPRO, HYDR, METH, PROP |
|  |  |  |  | mvcRegCode | nullable | nullable | nullable | Integer |
|  |  |  |  | mvcVehicleTypeCode | nullable | nullable | nullable | Integer |
|  |  |  |  | nj1Indicator | nullable | nullable | nullable | N, Y |
|  |  |  |  | numberOfPassengers | nullable | nullable | nullable | Integer |
|  |  |  |  | omni2Indicator | nullable | nullable | nullable | N, Y |
|  |  |  |  | plateJurisdiction | nullable | nullable | nullable | String |
|  |  |  |  | plateNumber | nullable | nullable | nullable | String |
|  |  |  |  | regIssueDate | nullable | nullable | nullable | Date |
|  |  |  |  | retrofitPresent | nullable | nullable | nullable | N, Y |
|  |  |  |  | transmissionCode | nullable | nullable | nullable | A, M |
|  |  |  |  | transportIndicator | nullable | nullable | nullable | N, T, Y |
|  |  |  |  | vehicleGVWR | nullable | nullable | nullable | Integer |
|  |  |  |  | vehicleMake | nullable | nullable | nullable | String |
|  |  |  |  | vehicleModel | nullable | nullable | nullable | String |
|  |  |  |  | vehicleSize | nullable | nullable | nullable | LD, MD1, MD2, MD3, HD |
|  |  |  |  | vehicleYear | nullable | nullable | nullable | Integer |
|  |  |  |  | vinDocument | nullable | nullable | nullable | String |
|  |  |  |  | vinObd | nullable | nullable | nullable | String |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | vinVehicle | nullable | nullable | nullable | String |
|  |  | workstation |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | configVersion | nullable | nullable | nullable | String |
|  |  |  |  | gasbenchSoftwareVersion | nullable | nullable | nullable | String |
|  |  |  |  | rulesVersion | nullable | nullable | nullable | String |
|  |  |  |  | softwareVersion | nullable | nullable | nullable | String |
|  | safetyDetails |  |  |  | Required | not available | not available | one‐to‐one |
|  |  |  |  | brakeResult | Required | not available | not available | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | exhaustSystemResult | Required | not available | not available | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | miscSafetyRejectionResult | Required | not available | not available | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | safetyEquipmentResult | Required | not available | not available | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | steeringSuspensionResult | Required | not available | not available | 3, 4, F, N, P, R, S, X, Z, A |
|  |  | safetyConditions |  |  | optional | not available | not available | one‐to‐many |
|  |  |  |  | safetyCondition | Required | not available | not available | Integer |
|  | safetyResult |  |  |  | Rules use only | Rules use only | Rules use only | one‐to‐many |
|  | tamperItemDetails |  |  |  | optional | not available | not available | one‐to‐many |
|  |  |  |  | tamperItemCode | Required | not available | not available | C, D, N, R |
|  |  |  |  | tamperItemResult | Required | not available | not available | 3, F, N, P, S, X, Z |
|  | tempHold |  |  |  | Rules use only | Rules use only | Rules use only | one‐to‐one |
|  | tSIEmDetails |  |  |  | optional | not available | not available | one‐to‐one |
|  |  |  |  | highTestResult1 | Required | not available | not available | 3, F, N, P, S, X, Z |
|  |  |  |  | highTestResult2 | Required | not available | not available | 3, F, N, P, S, X, Z |
|  |  |  |  | idleTestResult1 | Required | not available | not available | 3, F, N, P, S, X, Z |
|  |  |  |  | idleTestResult2 | Required | not available | not available | 3, F, N, P, S, X, Z |
|  | vehicle |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | commercialIndicator | Required | nullable | Required | N, Y |
|  |  |  |  | cylindersCode | nullable | nullable | nullable | 1 to 16, E, R |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | engineHP | nullable | nullable | nullable | Decimal |
|  |  |  |  | engineMake | nullable | nullable | nullable | String |
|  |  |  |  | engineSpeed | nullable | nullable | nullable | L, M, H |
|  |  |  |  | engineYear | nullable | nullable | nullable | Integer |
|  |  |  |  | fuelCode | Required | Required | Required | CNGA, DIES, ELEC, ELED, ELEG, GASO, GCNG, GHYD, GLNG, GLPG, GMET, GPRO, HYDR, METH, PROP |
|  |  |  |  | fuelType | Required | nullable | Required | D, G, N, O |
|  |  |  |  | mvcRegCode | Required | nullable | Required | Integer |
|  |  |  |  | mvcVehicleTypeCode | nullable | nullable | nullable | Integer |
|  |  |  |  | nj1Indicator | nullable | nullable | nullable | N, Y |
|  |  |  |  | numberOfPassengers | nullable | nullable | nullable | Integer |
|  |  |  |  | omni2Indicator | Required | nullable | Required | N, Y |
|  |  |  |  | plateJurisdiction | nullable | nullable | nullable | String |
|  |  |  |  | plateNumber | Required | Required | Required | String |
|  |  |  |  | regIssueDate | nullable | nullable | nullable | Date |
|  |  |  |  | retrofitPresent | Required | nullable | nullable | N, Y |
|  |  |  |  | transmissionCode | nullable | nullable | nullable | A, M |
|  |  |  |  | transportIndicator | Required | nullable | Required | N, T, Y |
|  |  |  |  | vehicleCategory | Required | nullable | Required | cbus, comm, norm, omni, sbus, trans |
|  |  |  |  | vehicleGVWR | Required | Required | Required | Integer |
|  |  |  |  | vehicleMake | nullable | nullable | nullable | String |
|  |  |  |  | vehicleModel | nullable | nullable | nullable | String |
|  |  |  |  | vehicleSize | Required | nullable | Required | LD, MD1, MD2, MD3, HD |
|  |  |  |  | vehicleYear | Required | nullable | Required | Integer |
|  |  |  |  | vinDocument | Required | Required | Required | String |
|  |  |  |  | vinObd | nullable | nullable | nullable | String |
|  |  |  |  | vinVehicle | Required | Required | Required | String |
|  | workstation |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | configVersion | nullable | nullable | nullable | String |

| Entity/Attribute |  |  |  |  | EndTest | BeginTestCycle | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | gasbenchSoftwareVersion | nullable | nullable | nullable | String |
|  |  |  |  | gasbenchTestAllowed | Required | nullable | Required | F, T |
|  |  |  |  | gasCapTestAllowed | Required | nullable | Required | F, T |
|  |  |  |  | oBDTestAllowed | Required | nullable | Required | F, T |
|  |  |  |  | rulesVersion | nullable | nullable | nullable | String |
|  |  |  |  | smokemeterTestAllowed | Required | nullable | Required | F, T |
|  |  |  |  | softwareVersion | nullable | nullable | nullable | String |

CurrInsp.tempHold	Temp (internal to ruleset only)

CurrInsp.error	Err

CurrInsp.obdEmDetails	CurrObd

PrevInsp.obdEmDetails	PrevObd

CurrInsp.safetyDetails	CurrSafety

PrevInsp.safetyDetails	PrevSafety CurrInsp.credentialsConditions	CurrCredConditions PrevInsp.credentialsConditions	PrevCredConditions CurrInsp.exceptionParms	CurrExceptions PrevInspection.exceptionParms	PrevExceptions

CurrInsp.error	Err

CurrInsp.obdEmDetails	CurrObd

PrevInsp.obdEmDetails	PrevObd

CurrInsp.safetyDetails	CurrSafety

PrevInsp.safetyDetails	PrevSafety CurrInsp.credentialsConditions	CurrCredConditions PrevInsp.credentialsConditions	PrevCredConditions CurrInsp.exceptionParms	CurrExceptions PrevInspection.exceptionParms	PrevExceptions

CurrInsp.safetyResult	CurrSafetyResult

CurrInsp.emResult	CurrEmResult

CurrInsp.error	Err

CurrInsp.safetyDetails	CurrSafety

PrevInsp.safetyDetails	PrevSafety

CurrInsp.obdEmDetails	CurrObd

PrevInsp.obdEmDetails	PrevObd

## App_Rules Outputs

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection |  |  |  |  | Required | Required | Required |  |
|  |  |  |  | catDTCCode | unchanged | Set | Set | C, N, Y |
|  |  |  |  | changeICDCode | unchanged | Set | unchanged | E, N, M, Y |
|  |  |  |  | cycleStartDate | Set | static | static | Date |
|  |  |  |  | emComplianceDate | unchanged | Set | unchanged | Date |
|  |  |  |  | emExpirationDate | unchanged | Set | unchanged | Date |
|  |  |  |  | emInspRequested | unchanged | unchanged | Set | I, N, S |
|  |  |  |  | fullInspRequested | unchanged | unchanged | Set | N, Y |
|  |  |  |  | gasCapRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | gasCapResult | unchanged | unchanged | optional | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | indicatorLightRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | indicatorLightResult | unchanged | unchanged | optional | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | inspComplianceDate | unchanged | Set | unchanged | Date |
|  |  |  |  | inspCycleTypeCode | Set | static | static | I, R |
|  |  |  |  | inspDate | optional | optional | optional | Date |
|  |  |  |  | inspExpirationDate | unchanged | Set | unchanged | Date |
|  |  |  |  | inspExpirationDateCycle | Set | static | static | 6, 12, 24 |
|  |  |  |  | inspExtentCode | Set | optional | optional | F, P, T |
|  |  |  |  | inspModeCode | Set | static | static | C, O |
|  |  |  |  | inspProgramType | Set | static | static | CB, DC, IM, MC, SB |
|  |  |  |  | inspTypeCode | optional | static | static | CBA, CBC, CBD, CBG, CBR, CBS, CBX, DC, IMC, MC, SBA, SBC, SBS, SBX |
|  |  |  |  | liftRequired | unchanged | unchanged | unchanged | F, T |
|  |  |  |  | liquidLeakRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | liquidLeakResult | unchanged | unchanged | optional | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | miscEmRejectionResult | unchanged | unchanged | optional | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | miscEmRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | newStickerDate | unchanged | Set | unchanged | Date |
|  |  |  |  | obdBypass | unchanged | unchanged | optional | F, T |
|  |  |  |  | odometerReading | unchanged | unchanged | unchanged | Integer |
|  |  |  |  | offlineInspCode | unchanged | unchanged | unchanged | N, Y |
|  |  |  |  | oldStickerDate | unchanged | unchanged | unchanged | Date |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | opacityBypass | unchanged | unchanged | optional | F, T |
|  |  |  |  | overallCredentialsResult | unchanged | unchanged | unchanged | 3 ,A ,F ,P ,S |
|  |  |  |  | overallEmResult | unchanged | Set | unchanged | 3 ,A ,F ,P ,S |
|  |  |  |  | overallResult | unchanged | Set | unchanged | 3 ,A ,F ,P ,S |
|  |  |  |  | overallSafetyResult | unchanged | Set | optional | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | primaryEmResult | unchanged | unchanged | optional | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | primaryEmTestAllowed | unchanged | static | Set | F, T |
|  |  |  |  | primaryEmTestCode | unchanged | static | Set | D, I, N, O, T |
|  |  |  |  | primaryEmTestRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | safetyComplianceDate | unchanged | Set | unchanged | Date |
|  |  |  |  | safetyExpirationDate | unchanged | Set | unchanged | Date |
|  |  |  |  | safetyInspRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | safetyTestType | unchanged | static | Set | CBU, Diesel, IM, MC, SBU |
|  |  |  |  | tamperCheckRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | tamperResult | unchanged | unchanged | optional | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | testRecordNumber | static | static | static | Integer |
|  |  |  |  | tsiBypass | unchanged | unchanged | optional | F, T |
|  |  |  |  | vehicleConditionCheckRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | vehicleConditionOK | unchanged | unchanged | unchanged | N, Y |
|  |  |  |  | visibleSmokeRequired | unchanged | unchanged | Set | F, T |
|  |  |  |  | visibleSmokeResult | unchanged | unchanged | optional | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  | collectorVehicleDetails |  |  |  | NOT USED | NOT USED | NOT USED | NOT USED |
|  | credentialsConditions |  |  |  | not available | optional | not available | one‐to‐many |
|  |  |  |  | credentialsCondition | static | static | static | Integer |
|  | dieselEmDetails |  |  |  | not available | optional | not available | one‐to‐one |
|  |  |  |  | opacityCutpoint | unchanged | unchanged | unchanged | Integer |
|  |  |  |  | opacityReading | static | static | static | Decimal |
|  |  |  |  | opacityTestType | static | static | static | P, R, S |
|  |  |  |  | riseTimeLimit | unchanged | unchanged | unchanged | Decimal |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | emResult |  |  |  | Rules use only | Rules use only | Rules use only | one‐to‐many |
|  | error |  |  |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | errorCode | optional | optional | optional | String |
|  |  |  |  | errorDetails | optional | optional | optional | String |
|  |  |  |  | errorLevel | optional | optional | optional | I, V, W |
|  | exceptionParms |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | bypassOBDAllowed | unchanged | static | unchanged | N, Y |
|  |  |  |  | cATRetestExclusion | unchanged | static | unchanged | N, Y |
|  |  |  |  | continuousMonitorExclusion | unchanged | static | unchanged | N, Y |
|  |  |  |  | cVNExclusion | unchanged | static | unchanged | N, Y |
|  |  |  |  | exemptFromLift | unchanged | static | unchanged | N, Y |
|  |  |  |  | exemptFromOBD | unchanged | static | unchanged | N, Y |
|  |  |  |  | exemptFromOpacity | unchanged | static | unchanged | N, Y |
|  |  |  |  | exemptFromReadiness | unchanged | static | optional | N, Y |
|  |  |  |  | exemptFromTSI | unchanged | static | unchanged | N, Y |
|  |  |  |  | requiresRollTest | unchanged | static | unchanged | N, Y |
|  |  |  |  | rPMExclusion | unchanged | static | optional | N, Y |
|  | facility |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | facilityID | static | static | static | string |
|  |  |  |  | facilityOperationCode | static | static | static | C, S |
|  |  |  |  | facilityProgramCode | static | static | static | C, D |
|  |  |  |  | facilityType | static | static | static | CIF, PIF, PFF, SIF, MIT, BIT, ERF, DEC, DEF |
|  |  |  |  | fleetIndicator | static | static | static | N, Y |
|  |  |  |  | licenseStatusCode | static | static | static | A, C, P, R, S |
|  |  |  |  | mobileIndicator | static | static | static | N, Y |
|  |  |  |  | pifClassCode | static | static | static | 1, 2, 3, 4, 5 |
|  | idleEmDetails |  |  |  | not available | optional | not available | one‐to‐one |
|  |  |  |  | idleTestResult1 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | idleTestResult2 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  | indicatorLightItemDetails |  |  |  | not available | optional | not available | one‐to‐many |
|  |  |  |  | indicatorLightItemCode | static | static | static | D, M, N |
|  |  |  |  | lightCheckResult | static | static | static | F, P |
|  |  |  |  | lightOnResult | static | static | static | F, P |
|  | obdEmDetails |  |  |  | not available | optional | not available | one‐to‐one |
|  |  |  |  | dLCResultCode | static | static | static | D, M O, P |
|  |  |  |  | dTCStoredCount | static | static | static | Integer |
|  |  |  |  | milCommandOnResultCode | static | static | static | F, P |
|  |  |  |  | obdCommResultCode | static | static | static | F, P |
|  |  |  |  | readinessResult | static | unchanged | static | E, R, X |
|  |  |  |  | unsetMonitorCount | static | static | static | Integer |
|  |  | dTCCodes |  |  | not available | optional | not available | one‐to‐many |
|  |  |  |  | dTCCode | unchanged | unchanged | unchanged | String |
|  |  | ReadinessMonitorDetails |  |  | not available | optional | not available | one‐to‐one |
|  |  |  |  | airConditioningResult | static | static | static | R, U, X |
|  |  |  |  | catalystResult | static | static | static | R, U, X |
|  |  |  |  | compComponentResult | static | static | static | R, U, X |
|  |  |  |  | egrSystemResult | static | static | static | R, U, X |
|  |  |  |  | evapSystemResult | static | static | static | R, U, X |
|  |  |  |  | fuelSystemResult | static | static | static | R, U, X |
|  |  |  |  | heatedCatalystResult | static | static | static | R, U, X |
|  |  |  |  | misfireResult | static | static | static | R, U, X |
|  |  |  |  | oxygenSensorHeaterResult | static | static | static | R, U, X |
|  |  |  |  | oxygenSensorResult | static | static | static | R, U, X |
|  |  |  |  | secondaryAirSystemResult | static | static | static | R, U, X |
|  | prevInspection |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | catDTCCode | static | static | static | C, N, Y |
|  |  |  |  | changeICDCode | static | static | static | E, N, M, Y |
|  |  |  |  | emComplianceDate | static | static | static | Date |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | emExpirationDate | static | static | static | Date |
|  |  |  |  | gasCapResult | static | static | static | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | indicatorLightResult | static | static | static | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | inspComplianceDate | static | static | static | Date |
|  |  |  |  | inspCycleTypeCode | static | static | static | I, R |
|  |  |  |  | inspDate | static | static | static | Date |
|  |  |  |  | inspExpirationDate | static | static | static | Date |
|  |  |  |  | inspExtentCode | static | static | static | F, P, T |
|  |  |  |  | inspModeCode | static | static | static | C, O |
|  |  |  |  | inspProgramType | static | static | static | CB, DC, IM, MC, SB |
|  |  |  |  | inspTypeCode | static | static | static | CBA, CBC, CBD, CBG, CBR, CBS, CBX, DC, IMC, MC, SBA, SBC, SBS, SBX |
|  |  |  |  | liquidLeakResult | static | static | static | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | miscEmRejectionResult | static | static | static | S ,3 ,F ,N ,P ,R ,X ,Z |
|  |  |  |  | newStickerDate | static | static | static | Date |
|  |  |  |  | obdBypass | static | static | static | F, T |
|  |  |  |  | odometerReading | static | static | static | Integer |
|  |  |  |  | offlineInspCode | static | static | static | N, Y |
|  |  |  |  | oldStickerDate | static | static | static | Date |
|  |  |  |  | opacityBypass | static | static | static | F, T |
|  |  |  |  | overallCredentialsResult | static | static | static | 3 ,A ,F ,P ,S |
|  |  |  |  | overallEmResult | static | static | static | 3 ,A ,F ,P ,S |
|  |  |  |  | overallResult | static | static | static | 3 ,A ,F ,P ,S |
|  |  |  |  | overallSafetyResult | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | primaryEmResult | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | primaryEmTestCode | static | static | static | D, I, N, O, T |
|  |  |  |  | safetyComplianceDate | static | static | static | Date |
|  |  |  |  | safetyExpirationDate | static | static | static | Date |
|  |  |  |  | tamperResult | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | tsiBypass | static | static | static | F, T |
|  |  |  |  | visibleSmokeResult | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  | collectorVehicleDetails |  |  | NOT USED | NOT USED | NOT USED | NOT USED |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | credentialsConditions |  |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | credentialsCondition | static | static | static | Integer |
|  |  | dieselEmDetails |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | opacityCutpoint | static | static | static | Integer |
|  |  |  |  | opacityReading | static | static | static | Decimal |
|  |  |  |  | opacityTestType | static | static | static | P, R, S |
|  |  |  |  | riseTimeLimit | static | static | static | Decimal |
|  |  | exceptionParms |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | bypassOBDAllowed | static | static | static | N, Y |
|  |  |  |  | cATRetestExclusion | static | static | static | N, Y |
|  |  |  |  | continuousMonitorExclusion | static | static | static | N, Y |
|  |  |  |  | cVNExclusion | static | static | static | N, Y |
|  |  |  |  | exemptFromLift | static | static | static | N, Y |
|  |  |  |  | exemptFromOBD | static | static | static | N, Y |
|  |  |  |  | exemptFromOpacity | static | static | static | N, Y |
|  |  |  |  | exemptFromReadiness | static | static | static | N, Y |
|  |  |  |  | exemptFromTSI | static | static | static | N, Y |
|  |  |  |  | requiresRollTest | static | static | static | N, Y |
|  |  |  |  | rPMExclusion | static | static | static | N, Y |
|  |  | facility |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | facilityID | static | static | static | string |
|  |  |  |  | facilityOperationCode | static | static | static | C, S |
|  |  |  |  | facilityProgramCode | static | static | static | C, D |
|  |  |  |  | facilityType | static | static | static | CIF, PIF, PFF, SIF, MIT, BIT, ERF, DEC, DEF |
|  |  |  |  | fleetIndicator | static | static | static | N, Y |
|  |  |  |  | licenseStatusCode | static | static | static | A, C, P, R, S |
|  |  |  |  | mobileIndicator | static | static | static | N, Y |
|  |  |  |  | pifClassCode | static | static | static | 1, 2, 3, 4, 5 |
|  |  | idleEmDetails |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | idleTestResult1 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult2 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  | indicatorLightItemDetails |  |  | optional | optional | optional | one‐to‐many |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | indicatorLightItemCode | static | static | static | D, M, N |
|  |  |  |  | lightCheckResult | static | static | static | F, P |
|  |  |  |  | lightOnResult | static | static | static | F, P |
|  |  | obdEmDetails |  |  | nullable | nullable | nullable | one‐to‐one |
|  |  |  |  | dLCResultCode | static | static | static | D, M O, P |
|  |  |  |  | dTCStoredCount | static | static | static | Integer |
|  |  |  |  | milCommandOnResultCode | static | static | static | F, P |
|  |  |  |  | obdCommResultCode | static | static | static | F, P |
|  |  |  |  | readinessResult | static | static | static | E, R, X |
|  |  |  |  | unsetMonitorCount | static | static | static | Integer |
|  |  |  | dTCCodes |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | dTCCode | static | static | static | String |
|  |  |  | ReadinessMonitorDetails |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | airConditioningResult | static | static | static | R, U, X |
|  |  |  |  | catalystResult | static | static | static | R, U, X |
|  |  |  |  | compComponentResult | static | static | static | R, U, X |
|  |  |  |  | egrSystemResult | static | static | static | R, U, X |
|  |  |  |  | evapSystemResult | static | static | static | R, U, X |
|  |  |  |  | fuelSystemResult | static | static | static | R, U, X |
|  |  |  |  | heatedCatalystResult | static | static | static | R, U, X |
|  |  |  |  | misfireResult | static | static | static | R, U, X |
|  |  |  |  | oxygenSensorHeaterResult | static | static | static | R, U, X |
|  |  |  |  | oxygenSensorResult | static | static | static | R, U, X |
|  |  |  |  | secondaryAirSystemResult | static | static | static | R, U, X |
|  |  | safetyDetails |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | brakeResult | static | static | static | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | exhaustSystemResult | static | static | static | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | miscSafetyRejectionResult | static | static | static | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | safetyEquipmentResult | static | static | static | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | steeringSuspensionResult | static | static | static | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  | safetyConditions |  | optional | optional | optional | one‐to‐many |
|  |  |  |  | safetyCondition | static | static | static | Integer |
|  |  | tamperItemDetails |  |  | optional | optional | optional | one‐to‐many |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | tamperItemCode | static | static | static | C, D, N, R |
|  |  |  |  | tamperItemResult | static | static | static | 3, F, N, P, S, X, Z |
|  |  | tSIEmDetails |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | highTestResult1 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | highTestResult2 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult1 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult2 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  | vehicle |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | commercialIndicator | static | static | static | N, Y |
|  |  |  |  | cylindersCode | static | static | static | 1 to 16, E, R |
|  |  |  |  | engineHP | static | static | static | Decimal |
|  |  |  |  | engineMake | static | static | static | String |
|  |  |  |  | engineSpeed | static | static | static | L, M, H |
|  |  |  |  | engineYear | static | static | static | Integer |
|  |  |  |  | fuelCode | static | static | static | CNGA, DIES, ELEC, ELED, ELEG, GASO, GCNG, GHYD, GLNG, GLPG, GMET, GPRO, HYDR, METH, PROP |
|  |  |  |  | mvcRegCode | static | static | static | Integer |
|  |  |  |  | mvcVehicleTypeCode | static | static | static | Integer |
|  |  |  |  | nj1Indicator | static | static | static | N, Y |
|  |  |  |  | numberOfPassengers | static | static | static | Integer |
|  |  |  |  | omni2Indicator | static | static | static | N, Y |
|  |  |  |  | plateJurisdiction | static | static | static | String |
|  |  |  |  | plateNumber | static | static | static | String |
|  |  |  |  | regIssueDate | static | static | static | Date |
|  |  |  |  | retrofitPresent | static | static | static | N, Y |
|  |  |  |  | transmissionCode | static | static | static | A, M |
|  |  |  |  | transportIndicator | static | static | static | N, T, Y |
|  |  |  |  | vehicleGVWR | static | static | static | Integer |
|  |  |  |  | vehicleMake | static | static | static | String |
|  |  |  |  | vehicleModel | static | static | static | String |
|  |  |  |  | vehicleYear | static | static | static | Integer |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | vinDocument | static | static | static | String |
|  |  |  |  | vinObd | static | static | static | String |
|  |  |  |  | vinVehicle | static | static | static | String |
|  |  | workstation |  |  | optional | optional | optional | one‐to‐one |
|  |  |  |  | configVersion | static | static | static | String |
|  |  |  |  | gasbenchSoftwareVersion | static | static | static | String |
|  |  |  |  | rulesVersion | static | static | static | String |
|  |  |  |  | softwareVersion | static | static | static | String |
|  | safetyDetails |  |  |  | not available | Required | Required | one‐to‐one |
|  |  |  |  | brakeResult | unchanged | optional | optional | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | exhaustSystemResult | unchanged | optional | optional | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | miscSafetyRejectionResult | unchanged | optional | optional | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | safetyEquipmentResult | unchanged | optional | optional | 3, 4, F, N, P, R, S, X, Z, A |
|  |  |  |  | steeringSuspensionResult | unchanged | optional | optional | 3, 4, F, N, P, R, S, X, Z, A |
|  |  | safetyConditions |  |  | not available | optional | not available | one‐to‐many |
|  |  |  |  | safetyCondition | static | static | static | Integer |
|  | safetyResult |  |  |  | Rules use only | Rules use only | Rules use only | one‐to‐many |
|  | tamperItemDetails |  |  |  | not available | optional | not available | one‐to‐many |
|  |  |  |  | tamperItemCode | static | static | static | C, D, N, R |
|  |  |  |  | tamperItemResult | static | static | static | 3, F, N, P, S, X, Z |
|  | tempHold |  |  |  | Rules use only | Rules use only | Rules use only | one‐to‐one |
|  | tSIEmDetails |  |  |  | not available | optional | not available | one‐to‐one |
|  |  |  |  | highTestResult1 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | highTestResult2 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult1 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  |  |  |  | idleTestResult2 | static | static | static | 3 ,4 ,F ,N ,P ,S ,X ,Z |
|  | vehicle |  |  |  | Required | Required | Required | one‐to‐one |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | commercialIndicator | optional | static | static | N, Y |
|  |  |  |  | cylindersCode | static | static | static | 1 to 16, E, R |
|  |  |  |  | engineHP | static | static | static | Decimal |
|  |  |  |  | engineMake | static | static | static | String |
|  |  |  |  | engineSpeed | static | static | static | L, M, H |
|  |  |  |  | engineYear | static | static | static | Integer |
|  |  |  |  | fuelCode | static | static | static | CNGA, DIES, ELEC, ELED, ELEG, GASO, GCNG, GHYD, GLNG, GLPG, GMET, GPRO, HYDR, METH, PROP |
|  |  |  |  | fuelType | Set | static | static | D, G, N, O |
|  |  |  |  | mvcRegCode | optional | static | static | Integer |
|  |  |  |  | mvcVehicleTypeCode | static | static | static | Integer |
|  |  |  |  | nj1Indicator | static | static | static | N, Y |
|  |  |  |  | numberOfPassengers | static | static | static | Integer |
|  |  |  |  | omni2Indicator | optional | static | static | N, Y |
|  |  |  |  | plateJurisdiction | static | static | static | String |
|  |  |  |  | plateNumber | static | static | static | String |
|  |  |  |  | regIssueDate | static | static | static | Date |
|  |  |  |  | retrofitPresent | optional | static | optional | N, Y |
|  |  |  |  | transmissionCode | static | static | static | A, M |
|  |  |  |  | transportIndicator | optional | static | static | N, T, Y |
|  |  |  |  | vehicleCategory | Set | static | static | cbus, comm, norm, omni, sbus, trans |
|  |  |  |  | vehicleGVWR | optional | static | static | Integer |
|  |  |  |  | vehicleMake | static | static | static | String |
|  |  |  |  | vehicleModel | static | static | static | String |
|  |  |  |  | vehicleSize | Set | static | static | LD, MD1, MD2, MD3, HD |
|  |  |  |  | vehicleYear | static | static | static | Integer |
|  |  |  |  | vinDocument | static | static | static | String |
|  |  |  |  | vinObd | static | static | static | String |
|  |  |  |  | vinVehicle | static | static | static | String |

| Entity/Attribute |  |  |  |  | BeginTestCycle | EndTest | BeginTestSelection | Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | workstation |  |  |  | Required | Required | Required | one‐to‐one |
|  |  |  |  | configVersion | static | static | static | String |
|  |  |  |  | gasbenchSoftwareVersion | static | static | static | String |
|  |  |  |  | gasbenchTestAllowed | static | static | static | F, T |
|  |  |  |  | gasCapTestAllowed | static | static | static | F, T |
|  |  |  |  | oBDTestAllowed | static | static | static | F, T |
|  |  |  |  | rulesVersion | Set | Set | Set | String |
|  |  |  |  | smokemeterTestAllowed | static | static | static | F, T |
|  |  |  |  | softwareVersion | static | static | static | String |

CurrInsp.tempHold	Temp (internal to ruleset only)

CurrInsp.error	Err

CurrInsp.obdEmDetails	CurrObd

PrevInsp.obdEmDetails	PrevObd

CurrInsp.safetyDetails	CurrSafety

PrevInsp.safetyDetails	PrevSafety CurrInsp.credentialsConditions	CurrCredConditions PrevInsp.credentialsConditions	PrevCredConditions CurrInsp.exceptionParms	CurrExceptions PrevInspection.exceptionParms	PrevExceptions

CurrInsp.error	Err

CurrInsp.obdEmDetails	CurrObd

PrevInsp.obdEmDetails	PrevObd

CurrInsp.safetyDetails	CurrSafety

PrevInsp.safetyDetails	PrevSafety CurrInsp.credentialsConditions	CurrCredConditions PrevInsp.credentialsConditions	PrevCredConditions CurrInsp.exceptionParms	CurrExceptions PrevInspection.exceptionParms	PrevExceptions

CurrInsp.safetyResult	CurrSafetyResult

CurrInsp.emResult	CurrEmResult

CurrInsp.error	Err

CurrInsp.safetyDetails	CurrSafety

PrevInsp.safetyDetails	PrevSafety

CurrInsp.obdEmDetails	CurrObd

PrevInsp.obdEmDetails	PrevObd

# Appendix 3.5A: Legacy VID (Data and Table Volumes)

Legacy Data Tables and Volumes

The legacy VID is approximately 150 gigabytes as of August 2025.

| Table Type | Description | Approximate Volume (May 2024) |
| --- | --- | --- |
| Inspection Data | Inspection data is currently stored on two major tables and several ancillary detail tables. | 63.7 .4 million inspection records |
| Entity License Data | License data for various facilities, inspectors, and other entities are stored on a set of tables that are generally updated from the MVC Business License System (BLS). | 20,000 licensed entities |
| Vehicle Data | Certain vehicle registration data is currently on the legacy VID. | 16.1 million vehicle records |
| Calibration Data | Workstation calibration records are stored on several tables. | 8.8 million calibration records |
| Audit Data | Audit data is not stored on the legacy VID but must be stored on the NGVID. |  |
| Covert Data | Separate tables are used to store and manage covert vehicles. The amount of covert data is minimal. |  |
| Repair Data | Repair data is entered from the inspection workstations and from ERFs using a web- based interface. | 2 million repair records |
| Reference Data | Reference tables store information needed for efficient operation of the NGVID, e.g., the Vehicle Lookup Table (VLT). | Various sizes |
| User | The NGVID shall include tables that contain data that uniquely identify users who have access to the database and their roles. |  |
| Transaction Log | Workstation transaction logs must be kept for statistical and troubleshooting purposes. |  |
| Issues Log | The NGVID shall track and log any errors or system issues to aid in the resolution of any NGVID specific issues that may arise. |  |
| Other Logs | Activity log: The Contractor must maintain a database change log to support recovery, rollback, and other database management and maintenance activities. OBD Communication Log: The Contractor shall maintain a log of OBD communication activity between the vehicle and the inspection system. User Log: The Contractor must maintain a log of user activity. |  |

The two main tables that currently store the actual inspection results are the NJAS table and the InspectionResults table. The NJAS stores most of data elements. All data elements will be stored for historical purposes, but some will be deprecated in the NGVID.

NJAS Inspection Table

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 1 | NJA_RES_SYS_NO | Unique ID (from INSPECTION_RESULTS) | NUMBER(11,0) | No |
| 2 | NJA_TEST_REC_NO | WorkstationTest Record Number | NUMBER(6,0) | Yes |
| 3 | NJA_LIC_ST_ID | Facility ID (Station ID) | VARCHAR2(9 BYTE) | Yes |
| 4 | NJA_ETS_ID | Workstation ID | VARCHAR2(8 BYTE) | Yes |
| 6 | NJA_TECH_ID | Inspector License Number (Pos 2) | VARCHAR2(9 BYTE) | Yes |
| 7 | NJA_TECH_SRC | Inspector License Number Input Source (Pos 2) | VARCHAR2(1 BYTE) | Yes |
| 8 | NJA_VIN | VIN from document | VARCHAR2(19 BYTE) | Yes |
| 9 | NJA_VIN_SRC | VIN Input Source (B = Scanned Registration, M = Manual Entry, V = Scanned Vehicle, or S = Scanned Windshield Sticker) | VARCHAR2(1 BYTE) | Yes |
| 10 | NJA_LIC_NO | License Plate Number | VARCHAR2(10 BYTE) | Yes |
| 11 | NJA_LIC_JUR | License Plate Jurisdiction | VARCHAR2(2 BYTE) | Yes |
| 12 | NJA_LIC_SRC | License Plate Source | VARCHAR2(1 BYTE) | Yes |
| 13 | NJA_MODEL_YR | Vehicle Model Year | NUMBER(4,0) | Yes |
| 14 | NJA_MAKE | Vehicle Make | VARCHAR2(17 BYTE) | Yes |
| 15 | NJA_MODEL | Vehicle Model | VARCHAR2(23 BYTE) | Yes |
| 19 | NJA_GVWR | Gross Vehicle Weight Rating (GVWR) | NUMBER(5,0) | Yes |
| 22 | NJA_NO_OF_CYL | Number of Cylinders (0 to 16, E, R) | VARCHAR2(2 BYTE) | Yes |
| 23 | NJA_ENG_SIZE | Engine Size in Liters | NUMBER(3,1) | Yes |
| 24 | NJA_TRANS_TYPE | Transmission Type (A/M) | VARCHAR2(1 BYTE) | Yes |
| 25 | NJA_DUAL_EXH | Dual Exhaust | VARCHAR2(1 BYTE) | Yes |
| 26 | NJA_FUEL_CD | Fuel Code (from table) | VARCHAR2(4 BYTE) | Yes |
| 27 | NJA_VID_SYS_NO | VID Vehicle Identification Key | VARCHAR2(11 BYTE) | Yes |
| 32 | NJA_CURR_ODO_RDNG | Current Odometer Reading | NUMBER(7,0) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 33 | NJA_PREV_ODO_RDNG | Previous Test Odometer Reading | NUMBER(7,0) | Yes |
| 35 | NJA_TEST_TYPE | Test Type (Initial/Retest/Motorcycle) | VARCHAR2(1 BYTE) | Yes |
| 36 | NJA_TEST_START_DT | Test Date & Test Start Time | DATE | Yes |
| 37 | NJA_TEST_END_TIME | Test Date & Test End Time | DATE | Yes |
| 93 | NJA_VIS_SMOKE_RES | Visible Smoke Test Result | VARCHAR2(1 BYTE) | Yes |
| 97 | NJA_OVERALL_EMISS_RES | Overall Emission Result | VARCHAR2(1 BYTE) | Yes |
| 98 | NJA_CREDENT_RES | Credentials Result | VARCHAR2(1 BYTE) | Yes |
| 99 | NJA_ST_SUSP_RES | Steering and Suspension Result | VARCHAR2(1 BYTE) | Yes |
| 100 | NJA_SAF_EQUIP_RES | Safety Equipment Result | VARCHAR2(1 BYTE) | Yes |
| 102 | NJA_BRAKE_RES | Brakes Result | VARCHAR2(1 BYTE) | Yes |
| 103 | NJA_EX_SYS_RES | Exhaust System Result | VARCHAR2(1 BYTE) | Yes |
| 105 | NJA_MISC_COMNT | Miscellaneous Comment | VARCHAR2(200 BYTE) | Yes |
| 106 | NJA_MISC_COMNT_RES | Miscellaneous Comment Result | VARCHAR2(1 BYTE) | Yes |
| 107 | NJA_OVERALL_SAF_RES | Overall Safety Result | VARCHAR2(1 BYTE) | Yes |
| 108 | NJA_OVERALL_TEST_RES | Overall Inspection Result | VARCHAR2(1 BYTE) | Yes |
| 109 | NJA_RPM_BYPASS | RPM Bypass | VARCHAR2(1 BYTE) | Yes |
| 110 | NJA_TEST_FEE | Test Fee | NUMBER(5,2) | Yes |
| 111 | NJA_STIK_NO | Sticker Number | VARCHAR2(8 BYTE) | Yes |
| 112 | NJA_CERT_NO | Certification Number | VARCHAR2(21 BYTE) | Yes |
| 114 | NJA_SAF_COND_1 | Safety Condition 1 | NUMBER(4,0) | Yes |
| 115 | NJA_SAF_COND_2 | Safety Condition 2 | NUMBER(4,0) | Yes |
| 116 | NJA_SAF_COND_3 | Safety Condition 3 | NUMBER(4,0) | Yes |
| 117 | NJA_SAF_COND_4 | Safety Condition 4 | NUMBER(4,0) | Yes |
| 118 | NJA_SAF_COND_5 | Safety Condition 5 | NUMBER(4,0) | Yes |
| 119 | NJA_SAF_COND_6 | Safety Condition 6 | NUMBER(4,0) | Yes |
| 120 | NJA_SAF_COND_7 | Safety Condition 7 | NUMBER(4,0) | Yes |
| 121 | NJA_SAF_COND_8 | Safety Condition 8 | NUMBER(4,0) | Yes |
| 122 | NJA_SAF_COND_9 | Safety Condition 9 | NUMBER(4,0) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 123 | NJA_SAF_COND_10 | Safety Condition 10 | NUMBER(4,0) | Yes |
| 124 | NJA_SAF_COND_11 | Safety Condition 11 | NUMBER(4,0) | Yes |
| 125 | NJA_SAF_COND_12 | Safety Condition 12 | NUMBER(4,0) | Yes |
| 126 | NJA_SAF_COND_13 | Safety Condition 13 | NUMBER(4,0) | Yes |
| 127 | NJA_SAF_COND_14 | Safety Condition 14 | NUMBER(4,0) | Yes |
| 128 | NJA_SAF_COND_15 | Safety Condition 15 | NUMBER(4,0) | Yes |
| 129 | NJA_SAF_COND_16 | Safety Condition 16 | NUMBER(4,0) | Yes |
| 130 | NJA_SAF_COND_17 | Safety Condition 17 | NUMBER(4,0) | Yes |
| 131 | NJA_SAF_COND_18 | Safety Condition 18 | NUMBER(4,0) | Yes |
| 132 | NJA_SAF_COND_19 | Safety Condition 19 | NUMBER(4,0) | Yes |
| 133 | NJA_SAF_COND_20 | Safety Condition 20 | NUMBER(4,0) | Yes |
| 134 | NJA_OLD_STIK_DT | Old Sticker Date | DATE | Yes |
| 136 | NJA_INSP_EXP_DT | Inspection Expiration Date | DATE | Yes |
| 137 | NJA_INSP_COMPL_DT | Inspection Compliance Date | DATE | Yes |
| 139 | NJA_CHANGE_ICD_FL | Change ICD Flag N = No Change to ICD, Y = Change ICD, T = Title Change, R = Restore Reg, or F = Foreign Title. (Based on the PIF spec, should never be 'T', 'R' or 'F') | VARCHAR2(1 BYTE) | Yes |
| 141 | NJA_STIK_DT_ACT | Sticker Date - Actual | DATE | Yes |
| 142 | NJA_CREATE_DT | Date and Time Stamp when record was created | DATE | Yes |
| 143 | NJA_CREATE_LOGIN | Oracle Login ID of user who created record | VARCHAR2(30 BYTE) | Yes |
| 144 | NJA_UPDATE_DT | Date and Time Stamp when record was updated | DATE | Yes |
| 145 | NJA_UPDATE_LOGIN | Oracle Login ID of user who updated record | VARCHAR2(30 BYTE) | Yes |
| 159 | NJA_OBD2_VIN | VIN from OBD | VARCHAR2(19 BYTE) | Yes |
| 161 | NJA_MISFIRE_MON_RES | Misfire Monitor Result (continuous) (R = Ready, X = Not Ready, or U = Unused) | VARCHAR2(1 BYTE) | Yes |
| 162 | NJA_FUEL_SYS_MON_RES | Fuel System Monitor Result (continuous) | VARCHAR2(1 BYTE) | Yes |
| 163 | NJA_COMP_CMPNT_RES | Comprehensive Component Result (continuous) | VARCHAR2(1 BYTE) | Yes |
| 164 | NJA_CATALYST_MON_RES | Catalyst Monitor Result | VARCHAR2(1 BYTE) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 165 | NJA_HEAT_CAT_MON_RES | Heated Catalyst Monitor Result | VARCHAR2(1 BYTE) | Yes |
| 166 | NJA_EVAP_SYS_MON_RES | Evaporative System Monitor Result | VARCHAR2(1 BYTE) | Yes |
| 167 | NJA_SEC_AIR_SYS_MON_RES | Secondary Air System Monitor Result | VARCHAR2(1 BYTE) | Yes |
| 168 | NJA_AIR_COND_MON_RES | Air Conditioning System Monitor Result | VARCHAR2(1 BYTE) | Yes |
| 169 | NJA_OXY_SENS_MON_RES | Oxygen Sensor Monitor Result | VARCHAR2(1 BYTE) | Yes |
| 170 | NJA_HTD_OXY_SNS_MON_RES | Oxygen Sensor Heater Monitor Result | VARCHAR2(1 BYTE) | Yes |
| 171 | NJA_EGR_SYS_MON_RES | ERG System Monitor Result | VARCHAR2(1 BYTE) | Yes |
| 172 | NJA_TOT_UNSET_RDY_MON | Total Number of Unset Readiness Monitors | NUMBER(2,0) | Yes |
| 173 | NJA_READINESS_STATUS | Readiness Status (R-Ready or X-Not Ready) | VARCHAR2(1 BYTE) | Yes |
| 175 | NJA_OBD2_CONNECT_RES | OBD DLC Connector Result: (P)ass, (D)amaged, (M)issing, or (O)bstructed | VARCHAR2(1 BYTE) | Yes |
| 177 | NJA_COMMUNIC_RES | Communications Result, P-Pass, F-Fail, E-Excluded | VARCHAR2(1 BYTE) | Yes |
| 178 | NJA_MIL_COMMAND_STAT | MIL Commanded On, P or F | VARCHAR2(1 BYTE) | Yes |
| 179 | NJA_DTC1 | DTC1 | VARCHAR2(5 BYTE) | Yes |
| 180 | NJA_DTC2 | DTC2 | VARCHAR2(5 BYTE) | Yes |
| 181 | NJA_DTC3 | DTC3 | VARCHAR2(5 BYTE) | Yes |
| 182 | NJA_DTC4 | DTC4 | VARCHAR2(5 BYTE) | Yes |
| 183 | NJA_DTC5 | DTC5 | VARCHAR2(5 BYTE) | Yes |
| 184 | NJA_TOT_STORED_DTCS | Total Number of DTCs Stored | NUMBER(2,0) | Yes |
| 185 | NJA_PID_COUNT | PID Count (Total PID count for all ECMs) | NUMBER(2,0) | Yes |
| 186 | NJA_PCM_MUDULE_ID | PCM Module ID | VARCHAR2(2 BYTE) | Yes |
| 190 | NJA_CATALYST_DTC_PRES | Catalyst DTC Present. Set by Rules Engine. Y: Cat monitor must be ready, N: Normal Cat monitor process, C: Clear Y condition. | VARCHAR2(1 BYTE) | Yes |
| 191 | NJA_COUNTY_CODE | For Roadside and Bus teams | VARCHAR2(2 BYTE) | Yes |
| 192 | NJA_COUNTY_NAME | For Roadside and Bus teams | VARCHAR2(30 BYTE) | Yes |
| 193 | NJA_LOCATION_DESCRIPTION | Roadside/Bus team location description | VARCHAR2(200 BYTE) | Yes |
| 194 | NJA_LOCATION_ID | Roadside/Bus Team location ID | VARCHAR2(30 BYTE) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 195 | NJA_MUNICIPALITY_CODE | For Roadside and Bus teams | VARCHAR2(4 BYTE) | Yes |
| 196 | NJA_MUNICIPALITY_NAME | For Roadside and Bus teams | VARCHAR2(200 BYTE) | Yes |
| 197 | NJA_BYPASS_OBD_ALLOWED | OBD Bypass allowed for this test | VARCHAR2(1 BYTE) | Yes |
| 198 | NJA_CAT_RETEST_EXCLUSION | Excluded from Cat retest restrictions | VARCHAR2(1 BYTE) | Yes |
| 199 | NJA_CVN_EXCLUSION | CVN From OBD not required | VARCHAR2(1 BYTE) | Yes |
| 201 | NJA_EXEMPT_FROM_OBD | Excluded from any OBD test | VARCHAR2(1 BYTE) | Yes |
| 202 | NJA_EXEMPT_FROM_OPACITY | Excluded from Diesel Opacity test | VARCHAR2(1 BYTE) | Yes |
| 203 | NJA_EXEMPT_FROM_READINESS | Excluded from Readiness | VARCHAR2(1 BYTE) | Yes |
| 204 | NJA_EXEMPT_FROM_TSI | Exempt from TSI | VARCHAR2(1 BYTE) | Yes |
| 205 | NJA_REQUIRES_ROLL_TEST | Diesel Roll test required | VARCHAR2(1 BYTE) | Yes |
| 206 | NJA_AFTERCOOLED_ASPIRATED |  | VARCHAR2(1 BYTE) | Yes |
| 207 | NJA_BLOWER_ASPIRATED |  | VARCHAR2(1 BYTE) | Yes |
| 208 | NJA_INTERCOOLED_ASPIRATED |  | VARCHAR2(1 BYTE) | Yes |
| 209 | NJA_NATURALLY_ASPIRATED |  | VARCHAR2(1 BYTE) | Yes |
| 210 | NJA_SUPERCHARGER_ASPIRATED |  | VARCHAR2(1 BYTE) | Yes |
| 211 | NJA_TURBO_CHARGED_ASPIRATED |  | VARCHAR2(1 BYTE) | Yes |
| 212 | NJA_UNKNOWN_ASPIRATION |  | VARCHAR2(1 BYTE) | Yes |
| 219 | NJA_OPACITY_BYPASS | Diesel Opacity test bypassed for this test | VARCHAR2(1 BYTE) | Yes |
| 220 | NJA_DIESEL_OPACITY_CUTPOINT |  | NUMBER(2,0) | Yes |
| 221 | NJA_DIESEL_OPACITY_READING |  | NUMBER(4,1) | Yes |
| 222 | NJA_DIESEL_STACKSIZE_MEASURED |  | NUMBER(1,0) | Yes |
| 223 | NJA_DIESEL_STACKSIZE_USED |  | NUMBER(1,0) | Yes |
| 224 | NJA_DTC6 |  | VARCHAR2(5 BYTE) | Yes |
| 225 | NJA_DTC7 |  | VARCHAR2(5 BYTE) | Yes |
| 226 | NJA_DTC8 |  | VARCHAR2(5 BYTE) | Yes |
| 227 | NJA_DTC9 |  | VARCHAR2(5 BYTE) | Yes |
| 228 | NJA_DTC10 |  | VARCHAR2(5 BYTE) | Yes |
| 229 | NJA_EMISS_CLARIFICATION | General emission comment | VARCHAR2(200 | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
|  |  |  | BYTE) |  |
| 230 | NJA_EMISS_COMPL_DT | Emission compliance date | DATE | Yes |
| 231 | NJA_EMISS_EXP_DT | Emission expiration date | DATE | Yes |
| 232 | NJA_ENG_SIZE_UNIT_CODE |  | VARCHAR2(2 BYTE) | Yes |
| 233 | NJA_ENGINE_HP |  | NUMBER(5,1) | Yes |
| 234 | NJA_ENGINE_MAKE |  | VARCHAR2(30 BYTE) | Yes |
| 235 | NJA_ENGINE_YEAR |  | NUMBER(4,0) | Yes |
| 239 | NJA_INSP_EXTENT_CODE | Extent of insp: F-Full, P-Partial, T-Terminated | VARCHAR2(1 BYTE) | Yes |
| 240 | NJA_INSP_MODE_CODE | Cycle mode: C-Cycle, O-Off-cycle | VARCHAR2(1 BYTE) | Yes |
| 241 | NJA_LIQUID_LEAK_RESULT |  | VARCHAR2(1 BYTE) | Yes |
| 242 | NJA_MISC_EMISS_REJECT_COMMENT |  | VARCHAR2(200 BYTE) | Yes |
| 243 | NJA_MISC_EMISS_REJECT_RESULT |  | VARCHAR2(1 BYTE) | Yes |
| 244 | NJA_MVC_REG_CODE |  | NUMBER(3,0) | Yes |
| 245 | NJA_MVC_VEHICLE_TYPE_CODE |  | NUMBER(3,0) | Yes |
| 246 | NJA_NJ1_INDICATOR | For special processing, Y or N | VARCHAR2(1 BYTE) | Yes |
| 251 | NJA_OBD_BYPASSED |  | VARCHAR2(1 BYTE) | Yes |
| 252 | NJA_OBD_CALID |  | VARCHAR2(30 BYTE) | Yes |
| 253 | NJA_OBD_CODE_CLEARED_DISTANCE |  | NUMBER(6,0) | Yes |
| 254 | NJA_OBD_CODE_CLEARED_TIME |  | NUMBER(6,0) | Yes |
| 255 | NJA_OBD_CVN |  | VARCHAR2(30 BYTE) | Yes |
| 256 | NJA_OBD_MIL_ON_DISTANCE |  | NUMBER(6,0) | Yes |
| 257 | NJA_OBD_MIL_ON_TIME |  | NUMBER(6,0) | Yes |
| 258 | NJA_OBD_MODULE_ID_2 |  | VARCHAR2(30 BYTE) | Yes |
| 259 | NJA_OBD_MODULE_ID_3 |  | VARCHAR2(30 BYTE) | Yes |
| 260 | NJA_OBD_PID00 |  | VARCHAR2(30 BYTE) | Yes |
| 261 | NJA_OBD_PID20 |  | VARCHAR2(30 BYTE) | Yes |
| 262 | NJA_OBD_PID40 |  | VARCHAR2(30 BYTE) | Yes |
| 263 | NJA_OBD_PROTOCOL_CODE | Number for OBD protocol | VARCHAR2(2 BYTE) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 264 | NJA_OBD_RPM |  | NUMBER(6,0) | Yes |
| 265 | NJA_OBD_TYPE | Type of OBD from vehicle OBD response | VARCHAR2(30 BYTE) | Yes |
| 266 | NJA_OBD_WARMUPS |  | NUMBER(6,0) | Yes |
| 267 | NJA_OBD2_VIN_SRC |  | VARCHAR2(1 BYTE) | Yes |
| 268 | NJA_OVERALL_TAMPER_RESULT |  | VARCHAR2(1 BYTE) | Yes |
| 269 | NJA_PRIMARY_EMISS_TEST_RESULT |  | VARCHAR2(1 BYTE) | Yes |
| 270 | NJA_REPAIR_DATE |  | DATE | Yes |
| 271 | NJA_REPAIR_FACILITY_LIC_ID |  | VARCHAR2(9 BYTE) | Yes |
| 272 | NJA_REPAIR_TECH_LIC_ID |  | VARCHAR2(9 BYTE) | Yes |
| 273 | NJA_RIT_CD | Insp Type (IM, SBC, etc.) - not populated (See INSPECTION_RESULTS table) | VARCHAR2(3 BYTE) | Yes |
| 274 | NJA_RPCC_CD | PIF Class Code | VARCHAR2(1 BYTE) | Yes |
| 275 | NJA_SAF_COND_21 |  | NUMBER(4,0) | Yes |
| 276 | NJA_SAF_COND_22 |  | NUMBER(4,0) | Yes |
| 277 | NJA_SAF_COND_23 |  | NUMBER(4,0) | Yes |
| 278 | NJA_SAF_COND_24 |  | NUMBER(4,0) | Yes |
| 279 | NJA_SAF_COND_25 |  | NUMBER(4,0) | Yes |
| 280 | NJA_SAF_COND_26 |  | NUMBER(4,0) | Yes |
| 281 | NJA_SAF_COND_27 |  | NUMBER(4,0) | Yes |
| 282 | NJA_SAF_COND_28 |  | NUMBER(4,0) | Yes |
| 283 | NJA_SAF_COND_29 |  | NUMBER(4,0) | Yes |
| 284 | NJA_SAF_COND_30 |  | NUMBER(4,0) | Yes |
| 285 | NJA_SAFETY_CLARIFICATION | General Safety comment | VARCHAR2(200 BYTE) | Yes |
| 286 | NJA_SAFETY_COMPL_DT | Safety compliance date | DATE | Yes |
| 288 | NJA_SAFETY_EXP_DT | Safety expiration date | DATE | Yes |
| 291 | NJA_TSI_BYPASS | TSI was bypassed for this test | VARCHAR2(1 BYTE) | Yes |
| 292 | NJA_VEHICLE_VIN | VIN from vehicle | VARCHAR2(19 BYTE) | Yes |
| 293 | NJA_VEHICLE_VIN_SRC |  | VARCHAR2(1 BYTE) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 298 | NJA_WORK_ORDER_NO | Repair order number | VARCHAR2(8 BYTE) | Yes |
| 299 | NJA_GAS_CAP_FLOW_RATE | Flow rate of gas cap test | VARCHAR2(3 BYTE) | Yes |
| 300 | NJA_ETNJ2_SW_VER | Software version (current) | VARCHAR2(30 BYTE) | Yes |
| 301 | NJA_VEHICLE_SEATING_CAPACITY | Bus only | NUMBER(3,0) | Yes |
| 303 | NJA_RETROFIT_PRESENT |  | VARCHAR2(1 BYTE) | Yes |
| 304 | NJA_ENGINE_SPEED | Engine speed of the vehicle: L (Low), M (medium), H (High). | VARCHAR2(1 BYTE) | Yes |
| 305 | NJA_OPACITY_TEST_TYPE | Diesel opacity test done: S (Snap Test), P (Power Brake Test), R (Rolling Test). | VARCHAR2(1 BYTE) | Yes |
| 306 | NJA_REPAIR_CONFIRMATION_ID | Repair Confirmation ID of the latest repair record for this vehicle | VARCHAR2(30 BYTE) | Yes |
| 307 | NJA_RPM_EXCLUSION | Excluded from RPM reading | VARCHAR2(1 BYTE) | Yes |
| 308 | NJA_CONTINUOUS_MONIT_XCLUSION | Excluded from continuous monitor rqmts | VARCHAR2(1 BYTE) | Yes |
| 309 | NJA_INDICATOR_LIGHT_RESULT | Overall result for Indicator Lights | VARCHAR2(1 BYTE) | Yes |
| 310 | NJA_PRIMARY_EMISS_TEST_CODE | Code for primary emission test: D-Diesel, I-Idle, O- OBD, T-TSI, N-None | VARCHAR2(1 BYTE) | Yes |
| 311 | NJA_CORRECTION_USER_ID | System user ID | VARCHAR2(30 BYTE) | Yes |
| 312 | NJA_PREV_RES_SYS_NO | ResSysNo (Primary Key) of previous insp record | NUMBER(11,0) | Yes |
| 313 | NJA_FULL_INSP_FL | Full Inspection Requested (Y or N) | VARCHAR2(1 BYTE) | Yes |
| 314 | NJA_OFFLINE_FL | Insp performed offline (Y or N) | VARCHAR2(1 BYTE) | Yes |
| 315 | NJA_COMMERCIAL_IND |  | VARCHAR2(1 BYTE) | Yes |
| 316 | NJA_OMNI_2_IND |  | VARCHAR2(1 BYTE) | Yes |
| 317 | NJA_TRNPORT_IND |  | VARCHAR2(1 BYTE) | Yes |
| 318 | NJA_CONFIG_VERSION | Software configuration version used | VARCHAR2(30 BYTE) | Yes |
| 319 | NJA_P5_VERSION | P5 Gas Bench version used | VARCHAR2(15 BYTE) | Yes |
| 320 | NJA_SAF_COND_31 |  | NUMBER(4,0) | Yes |
| 321 | NJA_SAF_COND_32 |  | NUMBER(4,0) | Yes |
| 322 | NJA_SAF_COND_33 |  | NUMBER(4,0) | Yes |
| 323 | NJA_SAF_COND_34 |  | NUMBER(4,0) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 324 | NJA_SAF_COND_35 |  | NUMBER(4,0) | Yes |
| 325 | NJA_SAF_COND_36 |  | NUMBER(4,0) | Yes |
| 326 | NJA_SAF_COND_37 |  | NUMBER(4,0) | Yes |
| 327 | NJA_SAF_COND_38 |  | NUMBER(4,0) | Yes |
| 328 | NJA_SAF_COND_39 |  | NUMBER(4,0) | Yes |
| 329 | NJA_SAF_COND_40 |  | NUMBER(4,0) | Yes |
| 335 | NJA_BUS_APPROVAL_NUMBER | Bus only | VARCHAR2(12 BYTE) | Yes |
| 336 | NJA_BUS_TERMINAL_ID | Bus only | VARCHAR2(10 BYTE) | Yes |
| 337 | NJA_BUS_NUMBER | Bus only - optional | VARCHAR2(10 BYTE) | Yes |

NJAS Deprecated/Discarded Data Elements

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 5 | NJA_SW_VER | Software Version - Legacy | NUMBER(3,2) | Yes |
| 16 | NJA_VHCL_TYPE | Vehicle Type(P/T/H) | VARCHAR2(1 BYTE) | Yes |
| 17 | NJA_BODY_STYLE | Body Style | NUMBER(1,0) | Yes |
| 18 | NJA_CERT_CLASS | EPA Certification Class. | VARCHAR2(5 BYTE) | Yes |
| 20 | NJA_ASM_ETW | ASM Equivalent Test Weight | NUMBER(4,0) | Yes |
| 21 | NJA_ETW_SRC | ETW Input Source(M = VRT V = VID, or D = DEFPARM.DAT) | VARCHAR2(1 BYTE) | Yes |
| 28 | NJA_VRT_REC_NO | VRT Record Number | NUMBER(5,0) | Yes |
| 29 | NJA_ARMSTDS_REC_NO | ASMSTDS Record Number | NUMBER(5,0) | Yes |
| 30 | NJA_RSN_F_N_TESTABLE | Reason for non-dyno testable (ASM) | NUMBER(1,0) | Yes |
| 31 | NJA_DYNO_TESTABLE | Dyno Testable | VARCHAR2(1 BYTE) | Yes |
| 34 | NJA_PREV_TEST_DT | Previous Cycle - Initial Test Date -OR- Previous Test Date? | DATE | Yes |
| 38 | NJA_EMISS_TEST_TYPE | Emission Test Type (performed) - A=ASM,I=Idle,2=2500 (or TSI),N=None | VARCHAR2(1 BYTE) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 39 | NJA_TEST_SEQ_NO | Test Sequence Number Performed (1 to 7) | NUMBER(1,0) | Yes |
| 40 | NJA_LOW_MILE_EXEMP | Low Mileage Exemption | VARCHAR2(1 BYTE) | Yes |
| 41 | NJA_TIRE_DRY | Tire Drying | VARCHAR2(1 BYTE) | Yes |
| 42 | NJA_TIN | TIN Number | VARCHAR2(8 BYTE) | Yes |
| 43 | NJA_REL_HUMID | Relative Humidity | NUMBER(3,1) | Yes |
| 44 | NJA_AMB_TEMP | Ambient Temperature | NUMBER(5,2) | Yes |
| 45 | NJA_BAR_PRESS | Barometric Pressure | NUMBER(4,2) | Yes |
| 46 | NJA_HCF | Humidity Correction Factor | NUMBER(3,2) | Yes |
| 47 | NJA_DCF_IDLE | Dilution Correction Factor - Idle - 1st Chance | NUMBER(3,2) | Yes |
| 48 | NJA_DCF_IDLE_2 | Dilution Correction Factor - Idle - 2nd Chance | NUMBER(3,2) | Yes |
| 49 | NJA_DCF_2500 | Dilution Correction Factor - 2500 - 1st Chance | NUMBER(3,2) | Yes |
| 50 | NJA_DCF_2500_2 | Dilution Correction Factor - 2500 - 2nd Chance | NUMBER(3,2) | Yes |
| 51 | NJA_DCF_ASM5015 | Dilution Correction Factor - ASM | NUMBER(3,2) | Yes |
| 52 | NJA_GAS_CAP_ADAP_AVL | Gas Cap - Adapter Available | VARCHAR2(1 BYTE) | Yes |
| 53 | NJA_GAS_CAP_REPL | Gas Cap Replaced | VARCHAR2(1 BYTE) | Yes |
| 54 | NJA_OVERALL_GAS_CAP_RES | Gas Cap Overall Result | VARCHAR2(1 BYTE) | Yes |
| 55 | NJA_HC_CTPT_IDLE | HC Cutpoint - Idle Test | NUMBER(4,0) | Yes |
| 56 | NJA_CO_CTPT_IDLE | CO Cutpoint - Idle Test | NUMBER(4,2) | Yes |
| 57 | NJA_HC_CTPT_2500 | HC Cutpoint - 2500 RPM Test | NUMBER(4,0) | Yes |
| 58 | NJA_CO_CTPT_2500 | CO Cutpoint - 2500 RPM Test | NUMBER(4,2) | Yes |
| 59 | NJA_HC_CTPT_ASM | HC Cutpoint - ASM | NUMBER(4,0) | Yes |
| 60 | NJA_CO_CTPT_ASM | CO Cutpoint - ASM | NUMBER(4,2) | Yes |
| 61 | NJA_NO_CTPT_ASM | NO Cutpoint - ASM | NUMBER(4,0) | Yes |
| 62 | NJA_ASM_RESTART_CNT | ASM Restart Counter (0 to 9) | NUMBER(1,0) | Yes |
| 63 | NJA_ASM_VIOLAT | Acceleration Violations - ASM (0 to 9) | NUMBER(1,0) | Yes |
| 64 | NJA_ASM_HP | HP ASM | NUMBER(3,1) | Yes |
| 65 | NJA_DC_HC_RES_ASM | Dilution Corrected HC Result - ASM | NUMBER(4,0) | Yes |
| 66 | NJA_DC_CO_RES_ASM | Dilution Corrected CO Result - ASM | NUMBER(4,2) | Yes |
| 67 | NJA_CO2_RES_ASM | CO2 Result - ASM | NUMBER(3,1) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 68 | NJA_DC_NO_RES_ASM | Dilution Corrected NO Result - ASM | NUMBER(4,0) | Yes |
| 69 | NJA_O2_RES_ASM | O2 Result - ASM | NUMBER(3,1) | Yes |
| 70 | NJA_ASM_RPM_RDNG | RPM Reading - ASM | NUMBER(4,0) | Yes |
| 71 | NJA_OVERALL_ASM_RES | ASM Overall Result | VARCHAR2(1 BYTE) | Yes |
| 72 | NJA_RPM_2500 | RPM - 2500 RPM - 1st Chance | NUMBER(4,0) | Yes |
| 73 | NJA_DC_HC_2500 | Dilution Corrected HC - 2500 RPM - 1st Chance | NUMBER(4,0) | Yes |
| 74 | NJA_DC_CO_2500 | Dilution Corrected CO - 2500 RPM - 1st Chance | NUMBER(4,2) | Yes |
| 75 | NJA_CO2_2500 | CO2 - 2500 RPM - 1st Chance | NUMBER(3,1) | Yes |
| 76 | NJA_OVERALL_2500_RES | 2500 RPM 1st Chance Overall Result | VARCHAR2(1 BYTE) | Yes |
| 77 | NJA_RPM_2500_2 | RPM - 2500 RPM - 2nd Chance | NUMBER(4,0) | Yes |
| 78 | NJA_DC_HC_2500_2 | Dilution Corrected HC - 2500 RPM - 2nd Chance | NUMBER(4,0) | Yes |
| 79 | NJA_DC_CO_2500_2 | Dilution Corrected CO - 2500 RPM - 2nd Chance | NUMBER(4,2) | Yes |
| 80 | NJA_CO2_2500_2 | CO2 - 2500 RPM - 2st Chance | NUMBER(3,1) | Yes |
| 81 | NJA_OVERALL_2500_RES_2 | 2500 RPM 2nd Chance Overall Result | VARCHAR2(1 BYTE) | Yes |
| 82 | NJA_IDLE_RPM | RPM - Idle - 1st Chance | NUMBER(4,0) | Yes |
| 83 | NJA_DC_HC_IDLE | Dilution Corrected HC - Idle - 1st Chance | NUMBER(4,0) | Yes |
| 84 | NJA_DC_CO_IDLE | Dilution Corrected CO - Idle - 1st Chance | NUMBER(4,2) | Yes |
| 85 | NJA_CO2_IDLE | CO2 - Idle - 1st Chance | NUMBER(3,1) | Yes |
| 86 | NJA_OVERALL_IDLE_RES | Idle Test 1st Chance Overall Result | VARCHAR2(1 BYTE) | Yes |
| 87 | NJA_IDLE_RPM_2 | RPM - Idle - 2nd Chance | NUMBER(4,0) | Yes |
| 88 | NJA_DC_HC_IDLE_2 | Dilution Corrected HC - Idle - 2nd Chance | NUMBER(4,0) | Yes |
| 89 | NJA_DC_CO_IDLE_2 | Dilution Corrected CO - Idle - 2nd Chance | NUMBER(4,2) | Yes |
| 90 | NJA_CO2_IDLE_2 | CO2 - Idle - 2nd Chance | NUMBER(3,1) | Yes |
| 91 | NJA_OVERALL_IDLE_RES_2 | Idle Test 2nd Chance Overall Result | VARCHAR2(1 BYTE) | Yes |
| 92 | NJA_EXH_EMISS_TEST_RES | Exhaust Emission Test Result | VARCHAR2(1 BYTE) | Yes |
| 95 | NJA_CAT_CONV_RES | Catalytic Converter Inspection Result | VARCHAR2(1 BYTE) | Yes |
| 96 | NJA_OBD2_FAULT_CD | Old Field, no longer used | VARCHAR2(20 BYTE) | Yes |
| 101 | NJA_LIGHT_RES | Lights Result (NOTE: has some bad data) | VARCHAR2(1 BYTE) | Yes |
| 104 | NJA_MISC_RES | Miscellaneous Items Result | VARCHAR2(1 BYTE) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 113 | NJA_STIK_DT_ORDN | Sticker Date Overridden | VARCHAR2(1 BYTE) | Yes |
| 135 | NJA_REG_ISSUE_DT | Registration Issuance Date | DATE | Yes |
| 138 | NJA_INSP_ALLOW_DT | Inspection Allowance Date | DATE | Yes |
| 140 | NJA_STIK_DT_CAL | Sticker Date - Calculated | DATE | Yes |
| 146 | NJA_LANE_NO | CIF Lane Number | VARCHAR2(8 BYTE) | Yes |
| 147 | NJA_POS1_TECH_ID | CIF Inspector ID Position 1 | VARCHAR2(9 BYTE) | Yes |
| 148 | NJA_POS1_TECH_SRC | CIF Inspector Position 1 ID Input Source (B/M) | VARCHAR2(1 BYTE) | Yes |
| 149 | NJA_GVWR_RANGE | CIF GVWR Range (1 = <= 6000 lbs, 2 = > 6000 lbs and <= 8500 lbs, 3 = > 8500 lbs) | NUMBER(1,0) | Yes |
| 150 | NJA_OPT_IDLE_TEST_SEL | CIF Optional Idle Tests Selected | VARCHAR2(1 BYTE) | Yes |
| 151 | NJA_POS3_TECH_ID | CIF Inspector ID Position 3 | VARCHAR2(9 BYTE) | Yes |
| 152 | NJA_POS3_TECH_SRC | CIF Inspector Position 3 ID Input Source (B = Scanned ID, M = Manual Entry, or C = Changed Safety Result) | VARCHAR2(1 BYTE) | Yes |
| 153 | NJA_WAIT_TIME | CIF Wait Time | VARCHAR2(8 BYTE) | Yes |
| 154 | NJA_MIL_CHK_ENG_STATUS | Unknown | VARCHAR2(1 BYTE) | Yes |
| 155 | NJA_VRT_VERSION_DT | VRT Table Version Date | DATE | Yes |
| 156 | NJA_ASMSTDS_VERSION_DT | ASMSTDS Table Version Date | DATE | Yes |
| 157 | NJA_OBD2_TEST_FL | Was OBD protocol code | NUMBER(1,0) | Yes |
| 158 | NJA_OBD2_CTLYST_RETEST | OBD Catalyst Retest (source SYS.DAT). Controlled cat failure retest process. | NUMBER(1,0) | Yes |
| 160 | NJA_OBD2_MOD_YR_RETEST | OBD Model Year Retest (source SYS.DAT). Set to 1996. | NUMBER(4,0) | Yes |
| 174 | NJA_CHK_ENG_LIGHT_ON | Malfunction Indicator Light On (Check Engine Light ON KOER). Still populated. | VARCHAR2(1 BYTE) | Yes |
| 176 | NJA_BULB_CHECK_RES | Bulb Check Result (MIL On KOEO) Still populated. | VARCHAR2(1 BYTE) | Yes |
| 187 | NJA_OVERALL_OBD2_RES | Overall OBD Result | VARCHAR2(1 BYTE) | Yes |
| 188 | NJA_ADVISORY_PASS | OBD Bypass (Advisory Pass is original fieldname) | VARCHAR2(1 BYTE) | Yes |
| 189 | NJA_OVR_EXH_OBD2_EM_RES | Overall OBD + Exhaust Emission Result | VARCHAR2(1 BYTE) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 200 | NJA_EXEMPT_FROM_LIFT | Excluded from lift | VARCHAR2(1 BYTE) | Yes |
| 213 | NJA_CO_CTPT_2500_2 |  | NUMBER(4,2) | Yes |
| 214 | NJA_CO_CTPT_IDLE_2 |  | NUMBER(4,2) | Yes |
| 215 | NJA_CUTPOINT_INDICATOR_2500 | Type of cutpoint | VARCHAR2(1 BYTE) | Yes |
| 216 | NJA_CUTPOINT_INDICATOR_2500_2 | Type of cutpoint | VARCHAR2(1 BYTE) | Yes |
| 217 | NJA_CUTPOINT_INDICATOR_IDLE | Type of cutpoint | VARCHAR2(1 BYTE) | Yes |
| 218 | NJA_CUTPOINT_INDICATOR_IDLE_2 | Type of cutpoint | VARCHAR2(1 BYTE) | Yes |
| 236 | NJA_EPA_MOBILE6_CODE |  | VARCHAR2(10 BYTE) | Yes |
| 237 | NJA_HC_CTPT_IDLE_2 |  | NUMBER(4,0) | Yes |
| 238 | NJA_HC_CTPT_2500_2 |  | NUMBER(4,0) | Yes |
| 247 | NJA_O2_READING_2500 |  | NUMBER(3,1) | Yes |
| 248 | NJA_O2_READING_2500_2 |  | NUMBER(3,1) | Yes |
| 249 | NJA_O2_READING_IDLE |  | NUMBER(3,1) | Yes |
| 250 | NJA_O2_READING_IDLE_2 |  | NUMBER(3,1) | Yes |
| 287 | NJA_SAFETY_CONDITION_CHANGED | A safety condition code was changed | VARCHAR2(1 BYTE) | Yes |
| 294 | NJA_WAIT_END_TIME |  | DATE | Yes |
| 295 | NJA_WAIT_START_TIME |  | DATE | Yes |
| 296 | NJA_WAIT_SYSTEM_CODE | Wait System active code (A-Active, I-Inactive, N-None | VARCHAR2(1 BYTE) | Yes |
| 297 | NJA_WAIT_TICKET_SCANNED_CODE | Scan wait ticket status (Y/N) | VARCHAR2(1 BYTE) | Yes |
| 302 | NJA_ETNJ2_FLAG | Flag to indicate if the record is from the ETNJ2 program Y or N. Now always Y. | VARCHAR2(1 BYTE) | Yes |
| 330 | NJA_OIL_TEMP | Oil Temperature | NUMBER(3,0) | Yes |
|  | Discard |  |  |  |
| 94 | NJA_FUEL_PRESS_RES | Fuel System Pressure Test Result | VARCHAR2(1 BYTE) | Yes |
| 289 | NJA_STACK_MEASURED | dup field | VARCHAR2(30 BYTE) | Yes |
| 290 | NJA_STACK_SIZE | dup field | NUMBER(1,0) | Yes |
| 331 | NJA_OPACITY_FAST_PASS_CODE | Wager data - never implemented | VARCHAR2(1 BYTE) | Yes |

| ID | COLUMN_NAME | Description | DATA_TYPE | NULLABLE |
| --- | --- | --- | --- | --- |
| 332 | NJA_IDLE_BASELINE_ACC_LEVEL | Wager data - never implemented | NUMBER(3,0) | Yes |
| 333 | NJA_SNAP_ACC_TRIGGER_LEVEL | Wager data - never implemented | NUMBER(4,0) | Yes |
| 334 | NJA_POSTTESTDRIFT | Wager data - never implemented | NUMBER(4,1) | Yes |

Inspection Results Table

| ID | COLUMN_NAME | DATA_TYPE | NULLABLE | DATA_DEFAULT | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 1 | RES_SYS_NO | NUMBER(11,0) | No |  | The number to uniquely Identify this Inspection_results record |
| 2 | RES_ETS_SYS_NO | NUMBER(6,0) | Yes |  | Number to uniquely Identify an ET System |
| 3 | RES_ETS_LANE_TABLE_DES | VARCHAR2(1 BYTE) | Yes | 'E' | Pointer to ET_SYSTEMS table or LANES Table. |
| 5 | RES_SESSION_ID | NUMBER(12,0) | Yes |  | The Session Id for this Record. |
| 7 | RES_ST_TEST | VARCHAR2(3 BYTE) | Yes |  | Type of the Station that Performed the Test. |
| 8 | RES_TEST_DT | DATE | Yes |  | Date of test or date of creation of this record |
| 9 | RES_SMR_DESCRIPTOR | VARCHAR2(4 BYTE) | Yes |  | Flag indicating which arced table the child record is in |
| 10 | RES_VHCL_DPL_SYS_NO | NUMBER(11,0) | Yes |  | The sys no of either vehicles or dpl_vehicles. |
| 11 | RES_VHCL_DPL_TABLE_DES | VARCHAR2(1 BYTE) | No |  | Flag saying out of the table’s vehicles and dpl_vehicles to which table does this belong to? |
| 12 | RES_ODOMETER | NUMBER(7,0) | Yes |  | Current Odometer |
| 14 | RES_STIK_NO | VARCHAR2(8 BYTE) | Yes |  | Sticker Number |
| 17 | RES_EMISS_RES | VARCHAR2(1 BYTE) | Yes |  | Emission Inspection Disposition |
| 18 | RES_OVERALL_RES | VARCHAR2(1 BYTE) | Yes |  | Record Inspection Disposition |
| 19 | RES_SAFETY_RES | VARCHAR2(1 BYTE) | Yes |  | Safety Inspection Disposition |
| 21 | RES_REP_DT | DATE | Yes |  | Repair Date |
| 24 | RES_DMV_INSP_EXP_DT | DATE | Yes |  | Inspection due date prior to this test. This is populated from dmv_vehicles table. |
| 25 | RES_TEST_TYPE | VARCHAR2(1 BYTE) | Yes |  | Test Type (I/R) |
| 26 | RES_CREATE_DT | DATE | Yes | SYSDATE | Date and Time Stamp of when This Record was Created |
| 27 | RES_CREATE_LOGIN | VARCHAR2(30 BYTE) | Yes | USER | Oracle Login ID of User Who Created This Record. |
| 28 | RES_UPDATE_DT | DATE | Yes |  | Date and Time Stamp of when This Record was Updated |
| 29 | RES_UPDATE_LOGIN | VARCHAR2(30 BYTE) | Yes |  | Oracle Login ID of User Who Updated This Record. |
| 30 | RES_PREV_RES_SYS_NO | NUMBER(11,0) | Yes |  |  |
| 33 | RES_CURR_INSP_CAT | VARCHAR2(2 BYTE) | Yes | ??? | Also I/R |

| ID | COLUMN_NAME | DATA_TYPE | NULLABLE | DATA_DEFAULT | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 38 | RES_COVERT_FL | VARCHAR2(1 BYTE) | Yes |  |  |
| 39 | RES_PLATE_MATCH_FL | VARCHAR2(1 BYTE) | Yes |  |  |
| 40 | RES_VIN_MATCH_FL | VARCHAR2(1 BYTE) | Yes |  |  |
| 42 | RES_INSP_TYPE_CODE | VARCHAR2(5 BYTE) | Yes |  | Identifies the type of inspection transaction. Though no FK is defined, this must be present in the R_INSPECTION_TYPES Table |
| 43 | RES_CHILD_REC_DESCRIPTOR | VARCHAR2(20 BYTE) | Yes |  | Identifies the child Table(s) for this Header Table record. NJA means Regular Inspection that went to the Good Table, NJVI means Regular Inspection that went to the Bad Table, SCV means Collector Vehicle Inspection, SDS-NJA means Sticker Replacement Transaction that went to the Good Table, SDS-NJVI means Sticker Replacement Transaction that went to the Bad Table and SDS-SCV means Sticker Replacement Transaction that went to the Collector Vehicle Inspections Table |
| 44 | RES_VOIDED_RECORD_FLAG | VARCHAR2(1 BYTE) | Yes |  | Identifies whether this record is an inspection conducted to test the software. Permitted values are Y or N. Y means, exclude this record while determining the previous inspection record for a vehicle. Populated from the attribute IsVoidTestRecord of the inspection data XML |
| 45 | RES_CORRECTION_USER_ID | VARCHAR2(30 BYTE) | Yes |  |  |

Inspection Results Deprecated Data Elements

| ID | COLUMN_NAME | DATA_TYPE | NULLABLE | DATA_DEFAULT | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4 | RES_SCR | VARCHAR2(1 BYTE) | Yes |  | The Source--For Future Use. Previously to distinguish the Protocol. |
| 6 | RES_BATCH_FL | VARCHAR2(1 BYTE) | Yes |  | Whether the test is done in batch mode. |
| 13 | RES_DISPLAY_FL | VARCHAR2(1 BYTE) | Yes |  | FFU |
| 15 | RES_CHANGED_FL | VARCHAR2(1 BYTE) | Yes |  | FFU |
| 16 | RES_MCI_PAID_FL | VARCHAR2(1 BYTE) | Yes |  | MCI Paid Flag. |
| 20 | RES_TAMPER_RES | VARCHAR2(1 BYTE) | Yes |  | Tampering Inspection Disposition |
| 22 | RES_VID_MATCH | VARCHAR2(1 BYTE) | Yes |  | Vehicle found in Vehicles/dpl_vehicles table."Y" -Found , "N" - Not found |
| 23 | RES_MIS_MATCH | VARCHAR2(1 BYTE) | Yes | 'Z' | Flag to indicate whether we found a match/ mis-match in the license plate numbers present on VID and CICS; Allowed values : 1 to 9 |
| 31 | RES_BILLING_IND | VARCHAR2(1 BYTE) | Yes |  |  |
| 32 | RES_PRIM_SEC_IND | VARCHAR2(1 BYTE) | Yes |  |  |
| 34 | RES_PRIM_EXP_DT | DATE | Yes |  |  |
| 35 | RES_PRIM_ODO_RDNG | NUMBER(7,0) | Yes |  |  |
| 36 | RES_INITIAL_EEI_IND | VARCHAR2(1 BYTE) | Yes |  |  |
| 37 | RES_PINK_CARD_IND | VARCHAR2(1 BYTE) | Yes |  | Pink Card Indicator |
| 41 | RES_ETNJ2_FLAG | VARCHAR2(1 BYTE) | Yes |  |  |

This is a list of all tables in the legacy VID.

| Table Name | # of Cols | # of Indexes |
| --- | --- | --- |
| ACCESSES | 14 | 3 |
| ADDRESSES | 19 | 4 |
| APPLICABLE_SAFETY_CHECKS | 10 | 1 |
| ASSIGNMENTS | 10 | 3 |
| ASSIGNMENTS_HISTORY | 11 | 0 |
| BAD_INSP_DETAILS_HIST | 316 | 0 |
| BEGIN_TESTS | 13 | 1 |
| BIOMETRIC_DATA | 8 | 1 |
| CALIBRATIONS | 66 | 4 |
| CALL_TRXS | 6 | 2 |
| CERT_CLASSES | 9 | 1 |
| CIF_AUDITS | 9 | 0 |
| CIF_CALIBRATIONS | 69 | 1 |
| CIF_CALIBRATIONS_LOG | 5 | 0 |
| CIF_COLL_INSPECTIONS | 74 | 1 |
| CIF_COLL_INSPECTIONS_HIST | 76 | 0 |
| CIF_COLL_VHCL_BEGIN_TESTS | 15 | 0 |
| CIF_DUP_PASSED_STICKERS | 10 | 0 |
| CIF_DUP_VIRS | 10 | 0 |
| CIF_ERROR_RECORDS | 8 | 1 |
| CIF_LANES | 9 | 3 |
| CIF_LANE_ACTIVATIONS | 9 | 0 |
| CIF_LANE_CALIBRATIONS | 9 | 0 |
| CIF_LANE_DEACTIVATIONS | 11 | 0 |
| CIF_LANE_LOGINS | 9 | 0 |
| CIF_LANE_LOGOUTS | 9 | 0 |
| CIF_REF_UPD_FILES | 7 | 1 |
| CIF_REPLACEMENT_STICKERS | 22 | 1 |
| CIF_SESSIONS | 5 | 1 |
| CIF_ST_WAIT_TIMES | 9 | 2 |
| CIF_SYS_REF_UPD_FILES | 7 | 1 |
| CIF_TECH_LOGINS | 10 | 0 |
| CIF_TECH_LOGOUTS | 10 | 0 |
| CIF_WAIVERS | 17 | 1 |
| CIF_WAIVER_AUTHORIZATIONS | 18 | 1 |
| CIF_WAIVER_INQUIRIES | 10 | 0 |
| COMMUNICATIONS | 12 | 1 |
| COMPONENTS_VERSION_HIST | 10 | 1 |

| Table Name | # of Cols | # of Indexes |
| --- | --- | --- |
| CORPORATIONS | 12 | 2 |
| COVERT_HISTORY | 14 | 1 |
| COVERT_VEHICLES | 12 | 2 |
| CURB_IDLE_EXCEPTIONS | 9 | 1 |
| DEBUGS | 6 | 1 |
| DIESEL_SNAP_RESULTS | 11 | 1 |
| DPL_VEHICLES | 31 | 2 |
| ENF_AUDITORS | 6 | 1 |
| ENF_AUDITS | 10 | 1 |
| ESC_ASMS | 15 | 1 |
| ESC_ASM_HISTS | 16 | 1 |
| ESC_ASM_VERSIONS | 6 | 1 |
| ESC_TSIS | 12 | 0 |
| ESC_TSI_HISTS | 14 | 0 |
| ETNJ2_CALIBRATIONS | 75 | 1 |
| ETNJ2_CONFIG_PARAMS | 8 | 2 |
| ETNJ2_ERROR_LOG | 7 | 1 |
| ETNJ2_IT_CORPORATIONS | 18 | 0 |
| ETNJ2_IT_STATIONS | 14 | 0 |
| ETNJ2_IT_STATION_LICENSES | 14 | 0 |
| ETNJ2_IT_TECHNICIANS | 17 | 0 |
| ETNJ2_IT_VEHICLES | 16 | 0 |
| ETNJ2_MAILS | 12 | 2 |
| ETNJ2_MAIL_GROUPS | 8 | 1 |
| ETNJ2_MAIL_RECIPIENTS | 13 | 1 |
| ET_SYSTEMS | 31 | 5 |
| ET_SYSTEMS_HISTORY | 10 | 0 |
| ET_SYSTEMS_PHONES | 9 | 1 |
| EVENTS | 8 | 1 |
| FACILITY_BUSINESS_HOURS | 19 | 1 |
| FAIL_TYPE_DAY_REPORT | 7 | 1 |
| FUEL_FILES | 9 | 1 |
| GLOBAL_ERRORS | 9 | 1 |
| GOOD_INSP_DETAILS_HIST | 313 | 0 |
| HP_UPDATES | 9 | 1 |
| INCOMPLETE_TESTS | 15 | 3 |
| INSPECTED_VEHICLES_HIST | 33 | 1 |
| INSPECTION_RESULTS | 45 | 3 |
| INSPECTION_RESULTS_HIST | 49 | 0 |
| INSPECTION_TAMPER_DETAILS | 7 | 1 |

| Table Name | # of Cols | # of Indexes |
| --- | --- | --- |
| INSPECTION_TASKS | 14 | 1 |
| INSP_CONDITIONS | 12 | 1 |
| INSP_INDICATOR_LIGHT_DETS | 8 | 1 |
| INSP_RES_ERROR | 2 | 0 |
| INSP_RETEST_UPDATES | 7 | 0 |
| INSP_UPDATE | 19 | 0 |
| INSP_UPD_ERROR | 2 | 0 |
| INVALID_CIF_REQUEST | 2 | 0 |
| IT_BEGIN_TESTS | 21 | 0 |
| IT_CALIBRATIONS | 60 | 0 |
| IT_CIF_CALIBRATIONS | 63 | 0 |
| IT_COMMUNICATIONS | 10 | 0 |
| IT_ENF_AUDITS | 4 | 0 |
| IT_ET_SYSTEMS | 10 | 0 |
| IT_INCOMPLETE_TESTS | 9 | 0 |
| IT_INSPECTION_RESULTS | 6 | 0 |
| IT_NJDMV_VEHICLES | 35 | 1 |
| IT_PREVIOUS_CALLS | 20 | 0 |
| IT_REPAIRS | 15 | 0 |
| IT_STATIONS | 18 | 0 |
| IT_UPD_CORPORATIONS | 19 | 0 |
| IT_UPD_LICENSES | 15 | 0 |
| IT_UPD_STATIONS | 16 | 0 |
| IT_UPD_TECHNICIANS | 20 | 0 |
| IT_VID_TO_CICS | 52 | 1 |
| IT_VID_TO_CICS_ERRORS | 52 | 1 |
| IT_VIN_LIC_CORRECTIONS | 10 | 1 |
| LICENSES | 22 | 6 |
| LICENSES_HISTORY | 26 | 0 |
| LOCKOUTS | 14 | 3 |
| MAIL_ROUTES | 10 | 3 |
| MESSAGES | 12 | 1 |
| NJAS | 337 | 7 |
| NJAS_HISTORY | 340 | 6 |
| NJDMV_VEHICLES | 31 | 3 |
| NJ_MILESTONES | 6 | 1 |
| NJ_VEHC_INSPS | 340 | 1 |
| NON_CONFORMING_MESSAGES | 11 | 2 |
| OBDVRT | 11 | 1 |
| OBD_EXCEPTIONS | 14 | 1 |

| Table Name | # of Cols | # of Indexes |
| --- | --- | --- |
| OWNERS | 16 | 1 |
| OWNERSHIPS | 8 | 3 |
| PB_SEC_APPLICATIONS | 7 | 1 |
| PB_SEC_OBJECTS | 8 | 2 |
| PB_SEC_OBJ_ASSIGNMENTS | 7 | 3 |
| PB_SEC_OBJ_GROUPS | 7 | 1 |
| PB_SEC_OBJ_USER_ACCESSES | 6 | 3 |
| PB_SEC_USERS | 7 | 1 |
| PB_SEC_USER_ASSIGNMENTS | 6 | 3 |
| PB_SEC_USER_GROUPS | 7 | 1 |
| PB_VER_VERIFICATIONS | 8 | 1 |
| PERSONS | 11 | 1 |
| PREVIOUS_CALLS | 26 | 1 |
| REPAIRS | 24 | 1 |
| REPAIR_APP_USERS | 7 | 1 |
| REPAIR_DETAILS_DIESEL | 6 | 1 |
| REPAIR_DETAILS_OTHER | 6 | 1 |
| RESPONSIBILITIES | 8 | 4 |
| RULES | 8 | 0 |
| R_ADR_TYPES | 6 | 1 |
| R_AGENCIES | 6 | 1 |
| R_BIOMETRIC_STATUSES | 6 | 1 |
| R_BUS_STATUSES | 6 | 1 |
| R_BUS_TERMINALS | 13 | 1 |
| R_COMPONENTS_LIST_MASTER | 12 | 1 |
| R_COMPONENT_TYPES | 6 | 1 |
| R_COST_LIMITS | 10 | 1 |
| R_COUNTIES | 6 | 1 |
| R_CURB_IDLE_DATA_REQS | 16 | 1 |
| R_CURB_IDLE_TEST_CONFIG | 7 | 1 |
| R_DIESEL_REPAIR_CODES | 6 | 1 |
| R_EPA_MOBILE_6_CODES | 6 | 1 |
| R_FUEL_TYPES | 6 | 1 |
| R_FUEL_TYPE_OBD_DATA_REQS | 7 | 1 |
| R_INSPECTION_TYPES | 6 | 1 |
| R_LICENSE_STATUSES | 6 | 1 |
| R_LIC_TYPES | 12 | 1 |
| R_LOCKOUTS | 10 | 2 |
| R_LOCKOUTS_LIST | 12 | 1 |
| R_LOCKOUT_GROUPS | 6 | 1 |

| Table Name | # of Cols | # of Indexes |
| --- | --- | --- |
| R_LOCKOUT_TRIGGERS | 7 | 1 |
| R_MAIL_GROUPS | 6 | 1 |
| R_MFRS | 6 | 1 |
| R_MILESTONES | 2 | 0 |
| R_MSG_CDS | 6 | 1 |
| R_MVC_REG_CODES | 6 | 1 |
| R_MVC_VEHICLE_TYPES | 7 | 1 |
| R_OBD_DATA_REQUIREMENTS | 7 | 1 |
| R_OBD_PROTOCOL_CODES | 6 | 1 |
| R_OBD_TEST_CONFIGURATION | 7 | 1 |
| R_PIF_CLASS_CODES | 6 | 1 |
| R_RANDOMS | 2 | 0 |
| R_READINESS_TYPES | 6 | 1 |
| R_REGION_CDS | 6 | 1 |
| R_REPAIR_CODES | 6 | 1 |
| R_RSP_TYPES | 6 | 1 |
| R_SAFETY_CHECKS | 9 | 1 |
| R_SAFETY_CHECK_CATEGORIES | 9 | 1 |
| R_SAFETY_CHECK_GROUPS | 6 | 1 |
| R_SAFETY_RES_CDS | 6 | 1 |
| R_SECURITY_QUESTIONS | 6 | 1 |
| R_STATES | 7 | 1 |
| R_TSI_DATA_REQUIREMENTS | 19 | 1 |
| R_TSI_TEST_CONFIGURATION | 7 | 1 |
| R_VEHCL_PICKLIST_VERSIONS | 7 | 1 |
| R_ZIPCDS | 7 | 1 |
| SECONDS | 21 | 1 |
| SESSIONS | 12 | 1 |
| STATIONS | 39 | 2 |
| STATIONS_HISTORY | 39 | 0 |
| STATION_STARTUPS | 16 | 2 |
| STICKER_YEAR_DAY_REPORT | 7 | 1 |
| SYSTEM_FILES | 25 | 2 |
| TAMPERS | 10 | 3 |
| TECHNICIANS | 27 | 2 |
| TECHNICIANS_HISTORY | 31 | 0 |
| TECH_RANDOMS | 6 | 0 |
| TEXTS | 7 | 2 |
| TSI_EXCEPTIONS | 10 | 1 |
| UPDATE_ESC_ASMS | 17 | 2 |

| Table Name | # of Cols | # of Indexes |
| --- | --- | --- |
| UPDATE_VRTS | 24 | 2 |
| VEHCL_PICKLIST_VER_HIST | 9 | 1 |
| VEHICLES | 24 | 2 |
| VEHICLE_MODELS | 9 | 2 |
| VEHICLE_PICK_LIST | 14 | 1 |
| VEHICLE_RECORD_HISTS | 9 | 1 |
| VEH_EXCEPTIONS | 4 | 0 |
| VERSIONS | 8 | 2 |
| VER_STATUSES | 10 | 3 |
| VID_TO_OIT | 40 | 0 |
| VID_TO_OIT_ERRORS | 40 | 0 |
| VIN_LIC_CORRECTIONS | 10 | 1 |
| VRTS | 22 | 1 |
| VRT_VERSIONS | 6 | 1 |
| WEEK_GLOBAL_ERRORS | 9 | 0 |
| WORKSTATIONS | 14 | 1 |
| WORKSTATIONS_HISTORY | 14 | 0 |
| WORKSTATION_DAY_REPORT | 12 | 1 |
| WORKSTATION_LOCKOUTS | 13 | 1 |

# Appendix 3.5B: Legacy DEXML and XML Schema

DEXML Security Using WSS4J

What is DEXML

DEXML	is	a		source	independent		data exchange	service.	It		acts		as	a	proxy, providing	data	to	client	applications from	various	legacy	sources.

What is WSS4J

WSS4J	is	the	Apache	implementation	of the	OASIS	WS-Security	specification.

WSS4J	can	be	used	to	sign	and	verify

SOAP	Messages	in	a	web	service.

DEXML Security Using WSS4J

DEXML Features

### Provides	CRUD	access		to	 data	from disparate		sources	while	concealing the	data	source	and	interface requirements

- Provides	metadata	describing	data

as	DEXML	message-blocks

- Provides	a	mechanism	to	discover

available	data

DEXML Security Using WSS4J

DEXML Features, continued

- Uses	an	XML	schema	to	communicate

requests	and	responses

- Supports	casual	client	access	via RMI-IIOP	or	formal	access	via secure	web	service

- Uses Service Level Agreements to describe permissions, limits and fees for each client

DEXML Security Using WSS4J

DEXML Features, continued

- Exposes	interfaces	for	internal MyMVC	clients,		external	GSN	clients and	external	Internet	clients

- Can	interface	with	an	accounting application	to		record	and	track	fee for	service	client	transactions

DEXML Security Using WSS4J

DEXML Security

RMI-IIOP CLIENTS

- An	RMI_IIOP	interface		offering better	performance	than	the	Web Service	is	available	to	internal clients

- Only MyMVC applications running on the MVC server stack can use this interface

DEXML Security Using WSS4J

DEXML Security

WEB SERVICE CLIENTS

- The	service	endpoint	is	not

discoverable

- Clients	must	be	pre-authorized

- WS-Security is  used  to  validate clients and encrypt elements of the SOAP message

DEXML Security Using WSS4J

DEXML Security

WEB SERVICE CLIENTS, continued

- Clients	must	pass	a	username

token	within	the	SOAP	request

- X.509	certificates	are	used	to authenticate	requests	and	secure messaging

DEXML Security Using WSS4J

web container brick21/njapps3

AXIS Gateway Servlet

If DEXML_WS returns a Java object, it is serialized, and converted to XML. The XML response is then packaged in a SOAP message.

WSS4J Security Handler

This handler encrypts the body of the SOAP message using the public key provided by the client during setup. A message digest is then generated and signed using the DEXML_WS X.509 certificate before the SOAP message is forwarded on to the requestor.

web container brick21/njapps3

AXIS Gateway Servlet

If DEXML_WS returns a Java object, it is serialized, and converted to XML. The XML response is then packaged in a SOAP message.

WSS4J Security Handler

This handler encrypts the body of the SOAP message using the public key provided by the client during setup. A message digest is then generated and signed using the DEXML_WS X.509 certificate before the SOAP message is forwarded on to the requestor.

EJB container brick21/njapps3 DEXML application processes web service request and formats response message:

EJB container brick21/njapps3 DEXML application processes web service request and formats response message:

Client Request (within GSN)

HTTP Request from Client contains SOAP message encrypted using DEXML_WS public key

SOAP Header: Contains encrypted UserName token SOAP Body: contains encrypted DEXML request

Client Request (within GSN)

HTTP Request from Client contains SOAP message encrypted using DEXML_WS public key

SOAP Header: Contains encrypted UserName token SOAP Body: contains encrypted DEXML request

IN	Request Processing Flow

OUT

EJB container brick21/njapps3

DEXML application processes web service request and formats response message

EJB container brick21/njapps3

DEXML application processes web service request and formats response message

Client Request (within GSN)

HTTP Response from DEXML_WS

SOAP Header containing client encryption key info SOAP Body containing encrypted DEXML Request

Client Request (within GSN)

HTTP Response from DEXML_WS

SOAP Header containing client encryption key info SOAP Body containing encrypted DEXML Request

brick01 Web Server

Response is forwarded to client

brick01 Web Server

Response is forwarded to client

web container brick21/njapps3

WSS4J Security Handler

This handler validates the UserName and password in the token and uses the private key from the Trust Key- store to decrypt the SOAP request.

AXIS Gateway Servlet

The SOAP request is deserialized and a Java object is passed to the target application (MVC_DEXML_WS).

web container brick21/njapps3

WSS4J Security Handler

This handler validates the UserName and password in the token and uses the private key from the Trust Key- store to decrypt the SOAP request.

AXIS Gateway Servlet

The SOAP request is deserialized and a Java object is passed to the target application (MVC_DEXML_WS).

brick01 Web Server

Request is forwarded to brick21/njapps3

brick01 Web Server

Request is forwarded to brick21/njapps3

Request Flow

Request Flow

## DEXML Security Using WSS4J

### Setup for GSN clients

#### A	self-signed	X.509	certificate	with public	and		private	keys	is	generated and	added	to	the	DEXML	key-store		on brick21/njapps3

- The	DEXML	public	key	is	sent	to	all

pre-authorized	GSN	clients.

- A Username and password is created for each DEXML client and added to a configuration file

DEXML Security Using WSS4J

Setup for GSN clients, continued

#### Each	DEXML		client	must	generate their		own	X.509	certificate.	The client		public	key	is	then	imported into	the	DEXML		key-store

- Client certificates can be self- signed or signed by a Certificate Authority

DEXML Security Using WSS4J

Setup for Internet clients

- An X.509 certificate signed by a CA is generated and added to the DEXML key-store on brick21/njapps1

- The DEXML public key is sent to all pre-authorized Internet clients.

- A Username and password is created for each DEXML client and added to a configuration file

DEXML Security Using WSS4J

Setup for Internet clients, continued

#### Each	DEXML		client	must	generate their		own	X.509	certificate.	The client		public	key	is	then	imported into	the	DEXML		key-store

- Client	certificates	must	be	signed

by	a	valid	Certificate	Authority

DEXML Web Service

URL for WSDL Using RPC/Encoded Style

https://emvc.state.nj.us/MVC_DEXML_WS/services/DEXMLBusinessSB?wsdl

URL for WSDL Using Document/Literal Style

https://emvc.state.nj.us/MVC_DEXML_WS/services/DEXMLBusinessSBDocLiteral?wsdl

EndPoint URLs

https://emvc.state.nj.us/MVC_DEXML_WS/services/DEXMLBusinessSB

https://emvc.state.nj.us/MVC_DEXML_WS/services/DEXMLBusinessSBDocLiteral

URL for WSDL Using Document/Literal

https://emvc.state.nj.us/MVC_DEXML_WS/services/DEXMLBusinessSBDocLiteral?wsdl

Method 1 : public DexmlDocumentVO doDexmlProcess(DexmlDocumentVO dexmlDocumentVO) Method 2 : public String doDexmlDocumentProcess(String xmlRequestDocument)

The second method is same for both RPC/Encoded & Document/Literal.

- The Object DexmlDocumentVO represents both Request and Response.

- Fields for DexmlDocumentVO are as below

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| dexmlVersion | 1.0 | Yes for Request | For request only |
| returnStatusCode | 1000 for success -1 for failure | Yes for Response | For Response only (Other error codes are to be defined meanwhile) |
| Document Type | REQUEST OR RESPONSE OR UPDATE_REQUEST | Yes | For both Request and Response |
| RequestDocumentHeaderVO | Object | Yes for Request |  |
| RequestDocumentVO | Object | Yes for Request | Read Request |
| UpdateRequestDocumentVO | Object | Yes for Update Request | Update Request |
| ResponseDocumentVO | Object | No | Available only for successful requests |

- Fields for RequestDocumentHeaderVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| slaId | 12345 | yes | For future use(Service Level Agreement ID) Presently it can be any value other than a whitespace. |
| endUserId | Any end user id | yes | DesktopUser Id. There is no way to check the validity/authentication by DEXML so it can be any value sent by client |
| endUserIpAdd | End User Machine IP address | yes | MainFrame DB requires this value |
| clientIpAdd | Client Applcn Host IP Address | yes | Http Remote (Client)Host/proxy Address and this value must be same |
| clientUserId | Used for Basic Authentication | yes | Http Remote User and this value must be same |

- Fields for RequestDocumentVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| refId |  | yes | Name of the Control Block |
| NoOfMsgBlocks |  | yes | Message Block count comprising the Control block(It looks like this value is redundant, But for some reason it is required on MainFrame side ) |
| MsgBlockVO | Object Array | yes | One or more objects |

- Fields for MsgBlockVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| msgBlockName |  | yes | Name of the Message Block Asking for |
| KeyInfoVO | Array | yes | One or more objects |

- Fields for KeyInfoVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| keyStrucName |  | yes | Name of the Message Block Asking for |
| KeyValuePairsVO | Object | yes |  |

- Fields for KeyValuePairsVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| FieldDataVO | Object Array | yes | At least one Object |
| RelativeKeyVO | Object Array | no | For future use |

- Fields for FieldDataVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| fieldName |  | yes |  |
| fieldType |  | no |  |
| fieldFormat |  | no | Format for date fields |
| fieldValue |  | yes |  |

- Fields for RelativeKeyVO (Not Used Now)

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |

- Fields for UpdateRequestDocumentVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| RefId |  | yes | Name of the Control Block |
| UMBlockVO | Array | yes | Array contains Atleast one object |

- Fields for UMBlockVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| MBlockName |  | yes | Message block name |
| KeyInfoVO | Array |  | Zero or more |
| UMBlockUpdateDataVO | Array |  | Atleast one |
| UMBlockVO |  |  | Zero or more(for child relationship) |

- Fields for UMBlockUpdateDataVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| FieldDataVO | Array | yes | Atleast one |

- Fields for ResponseDocumentVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| ResultInfoVO |  | yes |  |
| MBlockSetVO | Array |  | Zero or more |

- Fields for ResultInfoVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| StatusCode | 1000 for success | yes | 1000 for success(other error codes are to be defined meanwhile) |
| StatusMessage |  | yes |  |

- Fields for MBlockSetVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| MBlockName |  | yes |  |
| MBlockVO | Array |  | Zero or more |
| MBlockSetVO | Array |  | Zero or more( for child relationship) |

- Fields for MBlockVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| FieldNameAndValueVO | Array |  | Zero or more |

- Fields for FieldNameAndValueVO

| Field/Object | Valid Values | Mandatory | Notes |
| --- | --- | --- | --- |
| FieldName |  | yes |  |
| FieldType |  | no |  |
| FieldFormat |  | no |  |
| FieldValue | String Array | yes | Atleast one value |

URL for WSDL Using Document/Literal

https://emvc.state.nj.us/MVC_DEXML_WS/services/DEXMLBusinessSBDocLiteral?wsdl

Method: public String doDexmlDocumentProcess(String xmlRequestDocument)

It takes an XML Document and sends an XML Document . The Request/Response XML Document is based on the following single XSD (DexmlDocument.xsd).

<?xml version="1.0" encoding="UTF-8"?>

<xsd:schema xmlns:xsd="http://www.w3.org/2001/XMLSchema" xmlns:DexmlDoc="http://dexmlws/vo" targetNamespace="http://dexmlws/vo" >

<xsd:element name="DexmlDocument">

<xsd:complexType>

<xsd:sequence>

<!-- The value for DexmlVersion is 1.0-->

<xsd:element name="DexmlVersion" type="xsd:string" />

<!-- A value of 1000 for ReturnStatusCode is success in getting response-->

<xsd:element name="ReturnStatusCode" type="xsd:string" minOccurs="0" maxOccurs="1" />

<!-- A value for DocumentType is REQUEST OR RESPONSE -->

<xsd:element name="DocumentType" type="xsd:string" />

<xsd:element name="RequestDocumentHeader" type="DexmlDoc:RequestDocumentHeader" minOccurs="0" maxOccurs="1" />

<xsd:element name="RequestDocument" type="DexmlDoc:RequestDocument" minOccurs="0" maxOccurs="1" />

<xsd:element name="UpdateRequestDocument" type="DexmlDoc:UpdateRequestDocument" minOccurs="0" maxOccurs="1" />

<xsd:element name="ResponseDocument" type="DexmlDoc:ResponseDocument" minOccurs="0" maxOccurs="1" />

</xsd:sequence>

</xsd:complexType>

</xsd:element>

<xsd:complexType name="RequestDocumentHeader">

<xsd:sequence>

<xsd:element name="SlaId" type="xsd:string" />

<!-- SlaId is for future use(Service Level Agreement ID), value not required -->

<xsd:element name="EndUserId" type="xsd:string" />

<!-- EndUserId is Desktop User Id, value required -->

<xsd:element name="EndUserIpAdd" type="xsd:string" />

<!-- EndUserIpAdd is End User Machine IP address, value required -->

<xsd:element name="ClientUserId" type="xsd:string" />

<!-- ClientUserId is Used for Basic Authentication, value required, Http Remote User and this value must be same -->

<xsd:element name="ClientIpAdd" type="xsd:string" />

<!-- ClientIpAdd is Client Application Host IP Address, value required, Http Remote Host Address and this value must be same -->

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="RequestDocument">

<xsd:sequence>

<xsd:element name="RefId" type="xsd:string" />

<!-- RefId is the Control Block name, value required -->

<xsd:element name="NoOfMsgBlocks" type="xsd:string" />

<!-- NoOfMsgBlocks is Message block count in the Control Block, value required -->

<xsd:element name="MsgBlock" type="DexmlDoc:MsgBlock" minOccurs="1" maxOccurs="unbounded" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="UpdateRequestDocument" >

<xsd:sequence>

<xsd:element name="RefId" type="xsd:string" />

<!-- RefId is the Control Block name, value required -->

<xsd:element name="UMBlock" type="DexmlDoc:UMBlock" minOccurs="1" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="ResponseDocument">

<xsd:sequence>

<xsd:element name="ResultInfo" type="DexmlDoc:ResultInfo" />

<xsd:element name="MBlockSet" type="DexmlDoc:MBlockSet" minOccurs="0" maxOccurs="unbounded"/>

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="ResultInfo" >

<xsd:sequence>

<xsd:element name="StatusCode" type="xsd:string" />

<!-- A value of 1000 for StatusCode is success in getting response -->

<xsd:element name="StatusMessage" type="xsd:string" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="MBlockSet" >

<xsd:sequence>

<xsd:element name="MBlockName" type="xsd:string" />

<!-- MBlockName is Name of the Message Block -->

<xsd:element name="MBlock" type="DexmlDoc:MBlock" minOccurs="0" maxOccurs="unbounded" />

<xsd:element name="MBlockSet" type="DexmlDoc:MBlockSet" minOccurs="0" maxOccurs="unbounded" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="MBlock" >

<xsd:sequence>

<xsd:element name="FieldNameAndValue" type="DexmlDoc:FieldNameAndValue" minOccurs="0" maxOccurs="unbounded" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="UMBlock" >

<xsd:sequence>

<xsd:element name="MBlockName" type="xsd:string" />

<!-- MBlockName is Name of the Message Block -->

<xsd:element name="KeyInfo" type="DexmlDoc:KeyInfo" minOccurs="0" maxOccurs="unbounded" />

<xsd:element name="UMBlockUpdateData" type="DexmlDoc:UMBlockUpdateData" minOccurs="1" maxOccurs="unbounded" />

<xsd:element name="UMBlock" type="DexmlDoc:UMBlock" minOccurs="0" maxOccurs="unbounded" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="UMBlockUpdateData" >

<xsd:sequence>

<xsd:element name="FieldData" type="DexmlDoc:FieldData" minOccurs="1" maxOccurs="unbounded" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="MsgBlock" >

<xsd:sequence>

<xsd:element name="MsgBlockName" type="xsd:string" />

<xsd:element name="KeyInfo" type="DexmlDoc:KeyInfo" minOccurs="1" maxOccurs="unbounded" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="KeyInfo" >

<xsd:sequence>

<xsd:element name="KeyStrucName" type="xsd:string" />

<xsd:element name="KeyValuePairs" type="DexmlDoc:KeyValuePairs" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="KeyValuePairs" >

<xsd:sequence>

<xsd:element name="FieldData" type="DexmlDoc:FieldData" minOccurs="1" maxOccurs="unbounded" />

<xsd:element name="RelativeKey" type="DexmlDoc:RelativeKey" minOccurs="0" maxOccurs="unbounded" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="RelativeKey" >

<xsd:sequence>

<xsd:element name="RelativeKeyName" type="xsd:string" />

<xsd:element name="RelativeMsgBlockName" type="xsd:string" />

<xsd:element name="RelativeMsgBlockIndex" type="xsd:string" />

<xsd:element name="RelativeFieldName" type="xsd:string" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="FieldData" >

<xsd:sequence>

<xsd:element name="FieldName" type="xsd:string" />

<xsd:element name="FieldType" type="xsd:string" minOccurs="0" maxOccurs="1" />

<xsd:element name="FieldFormat" type="xsd:string" minOccurs="0" maxOccurs="1" />

<xsd:element name="FieldValue" type="xsd:string" />

</xsd:sequence>

</xsd:complexType>

<xsd:complexType name="FieldNameAndValue" >

<xsd:sequence>

<xsd:element name="FieldName" type="xsd:string" />

<xsd:element name="FieldType" type="xsd:string" minOccurs="0" maxOccurs="1"/>

<xsd:element name="FieldFormat" type="xsd:string" minOccurs="0" maxOccurs="1"/>

<xsd:element name="FieldValue" type="xsd:string" minOccurs="1" maxOccurs="unbounded" />

</xsd:sequence>

</xsd:complexType>

</xsd:schema>

CLVLDR

This Control Block consists of Message Block MLVLD. The corresponding Key Block is KLVLD.

Field Names of Message Block MLVLD of CLVLDR are as below

  - MLVLD_MATCH_DLN

  - MLVLD_MATCH_SSN

  - MLVLD_DOB_MATCH

  - MLVLD_DVR_DESEASE_STATUS

  - MLVLD_DVR_LIC_TY

  - MLVLD_DVR_LIC_EXP_DATE

Below is an Example using method doDexmlDocumentProcess(String xmlRequestDocument) for getting the data for ContalBlock CLVLDR.

REQUEST XML Document:

<?xml version="1.0" encoding="UTF-8" ?>

<ns1:DexmlDocument xmlns:ns1="http://dexmlws/vo">

<DexmlVersion>1.0</DexmlVersion>

<DocumentType>REQUEST</DocumentType>

<RequestDocumentHeader>

<SlaId>SlaId</SlaId>

<EndUserId> A00011240011552</EndUserId>

<EndUserIpAdd>10.37.70.232</EndUserIpAdd>

<ClientUserId>dexml001</ClientUserId>

<ClientIpAdd>10.37.70.232</ClientIpAdd>

</RequestDocumentHeader>

<RequestDocument>

<RefId>CLVLDR</RefId>

<NoOfMsgBlocks>1</NoOfMsgBlocks>

<MsgBlock>

<MsgBlockName>MRPRT</MsgBlockName>

<KeyInfo>

<KeyStrucName>KLVLD</KeyStrucName>

<KeyValuePairs>

<FieldData>

<FieldName>DLN</FieldName>

<FieldValue>A00011240011552</FieldValue>

</FieldData>

<FieldData>

<FieldName>SSN</FieldName>

<FieldValue>145067830</FieldValue>

</FieldData>

<FieldData>

<FieldName>DOB</FieldName>

<FieldFormat>MMDDYYYY</FieldFormat>

<FieldValue>11091955</FieldValue>

</FieldData>

</KeyValuePairs>

</KeyInfo>

</MsgBlock>

</RequestDocument>

</ns1:DexmlDocument>

RESPONSE XML Document:

<?xml version="1.0" encoding="UTF-8" ?>

<ns1:DexmlDocument xmlns:ns1="http://dexmlws/vo">

<DexmlVersion>1.0</DexmlVersion>

<ReturnStatusCode>1000</ReturnStatusCode>

<DocumentType>RESPONSE</DocumentType>

<ResponseDocument>

<ResultInfo>

<StatusCode>1000</StatusCode>

<StatusMessage>REQUEST/RESPONSE IS OK.</StatusMessage>

</ResultInfo>

<MBlockSet>

<MBlockName>MLVLD</MBlockName>

<MBlock>

<FieldNameAndValue>

<FieldName>MLVLD_MATCH_DLN</FieldName>

<FieldValue>Y</FieldValue>

</FieldNameAndValue>

<FieldNameAndValue>

<FieldName>MLVLD_MATCH_SSN</FieldName>

<FieldValue>Y</FieldValue>

</FieldNameAndValue>

<FieldNameAndValue>

<FieldName>MLVLD_DOB_MATCH</FieldName>

<FieldValue>Y</FieldValue>

</FieldNameAndValue>

<FieldNameAndValue>

<FieldName>MLVLD_DVR_LIC_TY</FieldName>

<FieldValue>D</FieldValue>

</FieldNameAndValue>

<FieldNameAndValue>

<FieldName>MLVLD_DVR_DESEASE_STATUS</FieldName>

<FieldValue>A</FieldValue>

</FieldNameAndValue>

<FieldNameAndValue>

<FieldName>MLVLD_DVR_LIC_EXP_DATE</FieldName>

<FieldFormat>MMDDYYYY</FieldFormat>

<FieldValue>02292008</FieldValue>

</FieldNameAndValue>

</MBlock>

</MBlockSet>

</ResponseDocument>

</ns1:DexmlDocument>

Vehicle Inspection Maintenance Data (AIM)

  - Vehicle Registration Information

    - No Changes to the requirements

      - VIN#

      - Registration Plate

      - Received Date

      - Received Time

      - Registration issue date

      - Plate Jurisdiction

      - ICD Flag

      - Registration Expiration Date

      - Doc Type

      - Vehicle Type

  - Vehicle Inspection Information

(This section is preliminary. Final data elements will be defined during detail design.)

    - No Changes to the current requirements, with the exception of:

    - Inspection Registration Suspension (will not be needed)

    - Vin Source (will not be needed)

    - Plate Source (will not be needed)

    - EEI Test Indicator (will not be needed)

      - Inspection Cert #

      - Inspection Date

      - Inspection Expiration Date

      - Inspection Compliance Date

      - Inspection Allow Date

      - Inspection Tran Sequence #

      - Inspection Category

      - Inspection Registration State Code

      - Primary / Sec Indicator

      - Facility ID

      - Inspector ID

      - Inspection Time

      - Status Indicator

      - Billing Date

      - Billing Code

      - Inspection match indicator

      - Cert Class

      - Date error flag

      - Test gas cap

      - Analyzer ID

      - Odometer

      - Overall Result

      - Credential Result

      - Emission Result

      - Safety Result

      - Test Performed

      - Collector Vehicle

      - NJ Sticker Number

Here is the information and layout for the VID data:

At the end of	each day, after inspection processing is completed, program	AIMSD105 processes the daily transaction	records	and creates a VID file ( lpmxaimp.vid.updt.dtl(+1) ).

The above file, created in job IOCAIM01, step AIMSD105 , triggers a connect direct	job named IOCVEHCD.

IOCVEHCD transmits the VID file to MCI via connect direct process named MCIVEH.

This job runs Tuesday to Friday and on Sundays.

******************************************************************

**	AIMWSVID	10/17/1999	**

**	MOTOR VEHICLE SERVICES	**

**	VEHICLE INSPECTION MAINTENANCE	**

**	* *	A I M	* *	**

**	**

**	RECORD LENGTH: 150	**

******************************************************************  01	VID-REC.

05	VID-PROCESS-CD	PIC X(01).

88	PROCESS-ADD	VALUE 'A'.

88	PROCESS-CHANGE	VALUE 'C'.

88	PROCESS-VIN	VALUE 'V'.

88	PROCESS-EOF		VALUE 'X'. 05	VID-VIN	PIC X(19).

05	VID-CORRECTED-VIN	PIC X(19).

05	VID-REG-PL-NO	PIC X(10).

05	VID-REG-EXP-DT.

10	VID-REG-EXP-CCYY.

15	VID-REG-EXP-CC	PIC X(02).

15	VID-REG-EXP-YY	PIC X(02).

10	VID-REG-EXP-MM	PIC X(02).

05	VID-REG-SUS-STA-CD	PIC X(01).

88	REG-IS-SUSPENDED	VALUE 'Y'.

88	REG-NOT-SUSPENDED		VALUE 'N'. 05	VID-VEH-MK	PIC X(03).

05	VID-VEH-MDL	PIC X(03).

05	VID-VEH-TY	PIC X(02).

88	STATION-WAGON	VALUE '01'.

88	TWO-DOOR	VALUE '02'.

88	CONVERTIBLE	VALUE '03'.

88	FOUR-DOOR	VALUE '04'.

88	LIMOUSINE	VALUE '06'.

88	TRUCK	VALUE '07'.

88	VAN	VALUE '08'.

88	PICK-UP-TRUCK	VALUE '09'.

88	OMNIBUS	VALUE '14'.

88	MOTORCYCLE	VALUE '20'.

88	PASSENGER-VAN	VALUE '22'. 05	VID-VEH-YR.

10	VID-VEH-CC	PIC X(02).

10	VID-VEH-YY	PIC X(02). 05	VID-VEH-CLR	PIC X(05).

05	VID-VEH-FUEL	PIC X(01).

05	VID-INSP-EXP-DT.

10	VID-INSP-EXP-CCYY.

15	VID-INSP-EXP-CC	PIC X(02).

15	VID-INSP-EXP-YY	PIC X(02).

10	VID-INSP-EXP-MM	PIC X(02).

|  | 10 VID-INSP-EXP-DD | PIC X(02). |
| --- | --- | --- |
| 05 | VID-INSP-COMP-DT. |  |
|  | 10 VID-INSP-COMP-CCYY. |  |
|  | 15 VID-INSP-COMP-CC | PIC X(02). |
|  | 15 VID-INSP-COMP-YY | PIC X(02). |
|  | 10 VID-INSP-COMP-MM | PIC X(02). |
|  | 10 VID-INSP-COMP-DD | PIC X(02). |
| 05 | VID-REG-PL-STA-CD | PIC X(01). |
|  | 88 ACTIVE-PLATE | VALUE 'A'. |
|  | 88 INCATIVE-PLATE | VALUE 'I'. |
| 05 | VID-SPEC-VEH-IND | PIC X(01). |
|  | 88 SPECIAL-VEH | VALUE 'Y'. |
|  | 88 NOT-SPECIAL-VEH | VALUE 'N'. |
| 05 | VID-COLLECTOR-VEH-IND | PIC X(01). |
|  | 88 COLLECTOR-VEH | VALUE 'Y'. |
|  | 88 NOT-COLLECTOR-VEH | VALUE 'N'. |
| 05 | VID-SAFETY-INSP-REQ | PIC X(01). |
|  | 88 SAFETY-INSP-REQ | VALUE 'Y'. |
|  | 88 SAFETY-INSP-NOT-REQ | VALUE 'N'. |
| 05 | VID-EMISS-INSP-REQ | PIC X(01). |
|  | 88 EMISS-INSP-REQ | VALUE 'Y'. |
|  | 88 EMISS-INSP-NOT-REQ | VALUE 'N'. |
| 05 | VID-REG-ISSUE-DT. |  |
|  | 10 VID-REG-ISSUE-CCYY. |  |
|  | 15 VID-REG-ISSUE-CC | PIC X(02). |
|  | 15 VID-REG-ISSUE-YY | PIC X(02). |
|  | 10 VID-REG-ISSUE-MM | PIC X(02). |
|  | 10 VID-REG-ISSUE-DD | PIC X(02). |
| 05 | VID-INSP-ALLOW-DT. |  |
|  | 10 VID-INSP-ALLOW-CCYY. |  |
|  | 15 VID-INSP-ALLOW-CC | PIC X(02). |
|  | 15 VID-INSP-ALLOW-YY | PIC X(02). |
|  | 10 VID-INSP-ALLOW-MM | PIC X(02). |
|  | 10 VID-INSP-ALLOW-DD | PIC X(02). |
| 05 | VID-DT-CHANGE-IND | PIC X(01). |
| 05 | VID-VEH-CERT | PIC X(05). |
| 05 | VID-RECORD-COUNT | PIC 9(07). |
| 05 | FILLER | PIC X(26). |

➔

Daily file sent to Verizon BLS Inspector License data:

01	MCI-INSP-REC.

| 05 |  | M-I-PROCESS-CD | PIC | X(01). |
| --- | --- | --- | --- | --- |
| 05 |  | M-I-REC-KEY. |  |  |
|  | 10 | M-I-DOC-TY | PIC | X(03). |
|  | 10 | M-I-DOC-NUM | PIC | X(06). |
| 05 |  | M-I-INSP-L-NAME | PIC | X(17). |
| 05 |  | M-I-INSP-F-NAME | PIC | X(09). |
| 05 |  | M-I-INSP-MIDI | PIC | X(01). |
| 05 |  | M-I-INSP-STA-CD | PIC | X(01). |
| 05 |  | M-I-INSP-ISS-DT | PIC | X(08). |
| 05 |  | M-I-INSP-SUSP-DT | PIC | X(08). |
| 05 |  | M-I-INSP-RSTR-DT | PIC | X(08). |
| 05 |  | M-I-INSP-EXP-DT | PIC | X(08). |
| 05 |  | M-I-PIN-REF-NUM | PIC | X(06). |
| 05 |  | M-I-PIN-PASSWORD | PIC | X(17). |
| 05 |  | M-I-INSP-DOB | PIC | X(08). |
| 05 |  | M-I-INSP-AUTOPIC | PIC | X(15). |

BLS Business License data: 01	MCI-LICN-REC.

05	M-L-PROCESS-CD	PIC X(01).

05	M-L-CORP-CD	PIC X(15).

05	M-L-SITE-CD	PIC X(15).

05	M-L-REC-KEY.

10	M-L-DOC-TY	PIC X(03).

10    M-L-DOC-NUM      PIC X(06). 05    M-L-LICN-STA-CD     PIC X(01). 05    M-L-LICN-ISS-DT     PIC X(08). 05    M-L-LICN-SUSP-DT    PIC X(08). 05    M-L-LICN-RSTR-DT    PIC X(08). 05    M-L-LICN-EXP-DT       PIC X(08).

BLS Corporation Data:

|  | 01 | MCI-CORP-REC. 05 M-C-PROCESS-CD |  | PIC | X(01). |
| --- | --- | --- | --- | --- | --- |
|  |  | 05 | M-C-CORP-CD | PIC | X(15). |
|  |  | 05 | M-C-NAME | PIC | X(27). |
|  |  | 05 | M-C-STREET | PIC | X(27). |
|  |  | 05 | M-C-CITY | PIC | X(14). |
|  |  | 05 | M-C-STATE | PIC | X(02). |
|  |  | 05 | M-C-ZIP-CD | PIC | X(09). |
|  |  | 05 | M-C-PHONE-NUMBR | PIC | X(10). |
|  |  | 05 | M-C-FAX-NUMBR | PIC | X(10). |
|  |  | 05 | M-C-PRINC-L-NAME | PIC | X(17). |
|  |  | 05 | M-C-PRINC-F-NAME | PIC | X(09). |
|  |  | 05 | M-C-PRINC-M-INT | PIC | X(01). |
|  |  | 05 | M-C-PRINC-TITLE | PIC | X(03). |
|  |  | 05 | M-C-PRINC-PHONE-NUM | PIC | X(10). |
| BLS | Site Data: |  |  |  |  |
|  | 01 | MCI-CORP-REC. |  |  |  |
| 05 |  |  | M-C-PROCESS-CD | PIC | X(01). |
| 05 |  |  | M-C-CORP-CD | PIC | X(15). |
| 05 |  |  | M-C-NAME | PIC | X(27). |
| 05 |  |  | M-C-STREET | PIC | X(27). |
| 05 |  |  | M-C-CITY | PIC | X(14). |
| 05 |  |  | M-C-STATE | PIC | X(02). |
| 05 |  |  | M-C-ZIP-CD | PIC | X(09). |
| 05 |  |  | M-C-PHONE-NUMBR | PIC | X(10). |
| 05 |  |  | M-C-FAX-NUMBR | PIC | X(10). |
| 05 |  |  | M-C-PRINC-L-NAME | PIC | X(17). |
| 05 |  |  | M-C-PRINC-F-NAME | PIC | X(09). |
| 05 |  |  | M-C-PRINC-M-INT | PIC | X(01). |
| 05 |  |  | M-C-PRINC-TITLE | PIC | X(03). |
| 05 |  |  | M-C-PRINC-PHONE-NUM | PIC | X(10). |

When a vehicle completes inspection (successful or not) an ‘AIMU’ transaction is generated by the vendor system and passed to the OIT mainframe.

These transactions invoke program AIMSL020, which formats the information into an inspection record and writes it out to a transaction

database.

The transactions are formatted using copybook ‘AIMCA10’ .

The control is then passed to the program AIMSL025 which reads the data in the inspection transaction database and generates SQL updates to the vehicle and inspection databases.

Attached is the layout for the data received via copybook AIMCA10.

******************************************************************

**	AIMCA10	05/05/2000	**

**	**

**	MOTOR VEHICLE SERVICES	**

**	VEHICLE INSPECTION MAINTENANCE	**

**	* *	A I M	* *	**

**	**

**	LANE RECORD (V.000505A)	**

**	**

**	RECORD LENGTH: 500	**

**--------------------------------------------------------------**

** VER/MOD	DATE	USER-ID REASON	**

** --- --- ---------- ------- ----------------------------------**

| ** ** | 01 | 01 | 07/29/2002 | SZI | 1.Added new CommArea CA-NJ-STCKR- ** NUM to Ca-Inspection-Block and ** |
| --- | --- | --- | --- | --- | --- |
| ** |  |  |  |  | reduced CA-TEST-AREA BY 7 bytes.** |
| ** |  |  |  |  | ** |
| ** | 01 | 02 | 03/19/2003 | SZI | 1.Moved CommArea CA-NJ-STCKR-NUM ** |
| ** |  |  |  |  | to the of end commarea. ** |
| ** | 01 | 03 | 04/25/2003 | SZI | 1.Added new field On Boad Diagnos-** |
| ** |  |  |  |  | tic OBD_OVERALL_RESULT_IND with ** |
| ** |  |  |  |  | 1 bytes length and value Pass or ** |
| ** |  |  |  |  | fail, then reduce lengh of Com- ** |
| ** |  |  |  |  | munication Test Area from 218 ** |
| ** |  |  |  |  | to 217. ** |
| ** | 01 | 03 | 08/11/2003 | SZI | 1.Extended CA-NJ-STCKR-NUM from 7 ** |
| ** |  |  |  |  | to 8 bytes, then reduce length ** |
| ** |  |  |  |  | of Communication Test Area from ** |
| ** |  |  |  |  | 217 to 216. ** |

******************************************************************  02 CA-LANE-REC.

05 CA-ACTION-FLAG	PIC X.

88 CA-PRODUCTION	VALUE '0'.

88 CA-XCTL-NEXT-PROG	VALUE '1'.

88 CA-RETURN-WITH-TRAN	VALUE '2'.

88 CA-RETURN-WITHOUT-TRAN	VALUE '3'.

88 CA-RETURN-FROM-AIMSL025	VALUE '4'. 05 CA-HEADER-BLOCK.

10 CA-HEADER-ID	PIC X(4).

88 CA-INQUIRY-TRANSACTION	VALUE 'AIMI'.

88 CA-UPDATE-TRANSACTION	VALUE 'AIMU'.

10 CA-TRAN-SEQ-NUM	PIC 9(11).

10 CA-FACILITY-ID.

15 CA-FACILITY-TY	PIC X(3).

88 CA-CENTRAL-FACILITY	VALUE 'CIF'.

88 CA-EMISSION-REPAIR	VALUE 'ERF'.

88 CA-FLEET-FACILITY	VALUE 'PFF'.

88 CA-HYBRID-FACILITY	VALUE 'HIF'.

88 CA-MOBILE-INSPECTIONS	VALUE 'MIT'.

88 CA-MOTORCYCLE-FACILITY	VALUE 'PIM'.

88 CA-PRIVATE-FACILITY	VALUE 'PIF'.

88 CA-SPECIAL-REFEREE-FACILITY VALUE 'SIF'.

15 CA-FACILITY-NUM	PIC X(6).

10 CA-ANALIZER-ID	PIC X(8).

10 CA-MISMATCH-SWITCH	PIC X.

88 CA-MISMATCH-NO

VALUES ' ', 'B', 'N', '1', '2', '6'.

88 CA-MISMATCH-YES

VALUES 'A', 'C', 'D', 'Y', '3',

'4', '5', '7', '8', '9'.

10 CA-CRITICAL-DATEERROR-FLAG	PIC X.

88 CA-DATEERROR-NO	VALUES ' ', 'N'.

88 CA-DATEERROR-YES	VALUE 'Y'.

10 CA-TEST-TME-STMP	PIC X(26).

10 CA-ERR-IND	PIC X(8).

88 CA-OTHER-ERROR-CODES	VALUE '???'.

88 CA-NO-ERRORS	VALUE SPACES.

88 CA-DUPLICATE-KEY	VALUE 'DUPEKEY'.

88 CA-UNKNOWN-VEHICLE	VALUE 'UNKNOWN'.

10 CA-ACK-STA	PIC X(3).

88 CA-INQUIRY-RESPONSE	VALUE 'INQ'.

88 CA-PROBLEM-RESPONSE	VALUE 'BAD'.

88 CA-UPDATE-RESPONSE	VALUE 'ACK'. 05 CA-VEH-BLOCK.

10 CA-VIN	PIC X(19).

10 CA-INSP-EXPIR-DT-CHAR	PIC X(8).

10 CA-INSP-EXPIR-DT

REDEFINES CA-INSP-EXPIR-DT-CHAR.

15 CA-INSP-EXP-CCYY	PIC 9(4).

15 CA-INSP-EXPIR-CCYY

REDEFINES CA-INSP-EXP-CCYY.

20 CA-INSP-EXPIR-CC	PIC 9(2).

20 CA-INSP-EXPIR-YY	PIC 9(2).

15 CA-INSP-EXPIR-MM	PIC 9(2).

15 CA-INSP-EXPIR-DD	PIC 9(2).

10 CA-INSP-COMPL-DT-CHAR	PIC X(8).

10 CA-INSP-COMPL-DT

REDEFINES CA-INSP-COMPL-DT-CHAR.

15 CA-INSP-COMP-CCYY	PIC 9(4).

15 CA-INSP-COMPL-CCYY

REDEFINES CA-INSP-COMP-CCYY.

20 CA-INSP-COMPL-CC	PIC 9(2).

20 CA-INSP-COMPL-YY	PIC 9(2).

15 CA-INSP-COMPL-MM	PIC 9(2).

15 CA-INSP-COMPL-DD	PIC 9(2).

10 CA-INSP-ALLOWANCE-DT-CHAR	PIC X(8).

10 CA-INSP-ALLOWANCE-DT

REDEFINES CA-INSP-ALLOWANCE-DT-CHAR.

15 CA-INSP-ALLOWNCE-CCYY	PIC 9(4).

15 CA-INSP-ALLOWANCE-CCYY

REDEFINES CA-INSP-ALLOWNCE-CCYY.

|  | 20 CA-INSP-ALLOWANCE-CC | PIC 9(2). |
| --- | --- | --- |
|  | 20 CA-INSP-ALLOWANCE-YY | PIC 9(2). |
|  | 15 CA-INSP-ALLOWANCE-MM | PIC 9(2). |
|  | 15 CA-INSP-ALLOWANCE-DD | PIC 9(2). |
| 10 | CA-REG-ISSUE-DT-CHAR | PIC X(8). |
| 10 | CA-REG-ISSUE-DT |  |

REDEFINES CA-REG-ISSUE-DT-CHAR.

15 CA-REGISSUE-CCYY	PIC 9(4).

15 CA-REG-ISSUE-CCYY

REDEFINES CA-REGISSUE-CCYY.

20 CA-REG-ISSUE-CC	PIC 9(2).

20 CA-REG-ISSUE-YY	PIC 9(2).

15 CA-REG-ISSUE-MM	PIC 9(2).

15 CA-REG-ISSUE-DD	PIC 9(2).

10 CA-ICD-FLAG	PIC X.

88 CA-ICD-CHANGE-MSO	VALUE 'Y'.

88 CA-ICD-FOREIGN-TITLE	VALUE 'F'.

88 CA-ICD-TITLE-CHG-CASUAL-SALE	VALUE 'T'.

10 CA-LOW-MILEAGE-IND	PIC X.

88 CA-LOW-MILEAGE-NO	VALUE 'N'.

88 CA-LOW-MILEAGE-YES	VALUE 'Y'.

10 CA-ODOMETER-READING	PIC 9(7).

10 CA-VEHICLE-TYPE	PIC X(2).

10 CA-VEHICLE-MAKE.

15 CA-VEH-MK	PIC X(3).

15 FILLER	PIC X(14) VALUE SPACE.

10 CA-VEHICLE-MODEL.

15 CA-VEH-MDL	PIC X(3).

15 FILLER	PIC X(20) VALUE SPACE.

10 CA-VEH-YR	PIC 9(4).

10 CA-VEHICLE-FUEL-TYPE.

15 CA-VEH-FUEL	PIC X.

15 FILLER	PIC X(3) VALUE SPACE. 05	CA-REG-BLOCK.

10 CA-PL-NUM	PIC X(10).

10 CA-PL-JURIS	PIC X(2).

88 CA-NJ-VEHICLE	VALUE 'NJ'.

10 CA-DOCU-TY	PIC X(2).

88 CA-COLL-VEH-INITIAL	VALUE 'CI'.

88 CA-COLL-VEH-RENEWAL	VALUE 'CR'.

88 CA-REG-DOCUMENT	VALUE 'RG'.

10 CA-REG-STA-CD	PIC X.

88 CA-REG-ACTIVE	VALUE 'A'.

88 CA-REG-INACTIVE	VALUE 'I'.

10 CA-REG-SUSPENDED-STA-CD	PIC X.

88 CA-REG-NOT-SUSPENDED	VALUE 'N'.

88 CA-REG-SUSPENDED	VALUE 'Y'.

10 CA-REG-EXPIR-DT-CHAR	PIC X(6).

10 CA-REG-EXPIR-DT

REDEFINES CA-REG-EXPIR-DT-CHAR.

15 CA-REG-EXPIR-CCYY.

20 CA-REG-EXPIR-CC	PIC 9(2).

20 CA-REG-EXPIR-YY	PIC 9(2).

15 CA-REG-EXPIR-MM	PIC 9(2).

10 CA-NJ1-IND	PIC X.

88 CA-NJ1-NO	VALUES 'N', ' '.

88 CA-NJ1-YES	VALUE	'Y'. 05 CA-INSPECTION-BLOCK.

10 CA-INSP-SEQ-NUM.

15 CA-INSP-FACILITY-ID	PIC X(9).

15 CA-INSP-JULIAN-DATE-CCYYDDD	PIC X(7).

15 CA-INSP-SECS-SINCE-MIDNIGHT	PIC X(5).

10 CA-MCI-TIN-NUM	PIC X(8).

10 CA-CERTIFICATION-CLASS	PIC X(5).

10 CA-BILLING-INDICATOR	PIC X.

88 CA-NON-BILLABLE	VALUE '0'.

88 CA-BILLED-AS-NOT-ENHANCED	VALUE '1'.

88 CA-BILLED-AS-ENHANCED	VALUE '2'.

88 CA-BILLED-AS-4WD-ENHANCED	VALUE '3'.

88 CA-BILLED-AS-COLLECTOR	VALUE '4'.

88 CA-PIF-NON-BILLABLE	VALUE '9'.

10 CA-MCI-TRANSACTION-DATE	PIC X(10).

10 CA-MCI-TRANS-DATE

REDEFINES CA-MCI-TRANSACTION-DATE.

|  | 15 CA-MCI-MM | PIC X(2). |  |
| --- | --- | --- | --- |
|  | 15 FILLER | PIC X. |  |
|  | 15 CA-MCI-DD | PIC X(2). |  |
|  | 15 FILLER | PIC X. |  |
|  | 15 CA-MCI-CCYY | PIC X(4). |  |
| 10 | CA-CUR-INSP-CAT | PIC X(2). |  |
|  | 88 CA-COURTESY-INSPECTION | VALUE 'CO'. |  |
|  | 88 CA-INITIAL-INSPECTION | VALUE 'IN'. |  |
|  | 88 CA-METT-INSPECTION | VALUE 'MT'. |  |
|  | 88 CA-NON-APPLICABLE | VALUE 'NA'. |  |
|  | 88 CA-RETEST-INSPECTION | VALUE 'RE'. |  |
| 10 | CA-EMISSION-TEST-PERFORMED | PIC X(2). |  |
|  | 88 CA-ETEST-NOT-PERFORMED | VALUE 'N '. |  |
|  | 88 CA-ASM5015-TEST | VALUE 'A '. |  |
|  | 88 CA-EMISSION-TEST-NA | VALUE 'N '. |  |
|  | 88 CA-IDLE-TEST | VALUE 'I '. |  |
|  | 88 CA-2500-TEST | VALUE '2 '. |  |
| 10 | CA-TEST-OVERALL | PIC X. |  |
|  | 88 CA-OVERALL-ADMIN-WAIVER | VALUE 'A'. |  |
|  | 88 CA-OVERALL-FAIL | VALUE 'F'. |  |
|  | 88 CA-OVERALL-PASS | VALUE 'P'. |  |
|  | 88 CA-OVERALL-PATTERN-FAILURE | VALUE 'T'. |  |
|  | 88 CA-OVERALL-WAIVER | VALUE 'W'. |  |
|  | 88 CA-OVERALL-48HRS | VALUE '4'. |  |
| 10 | CA-TEST-CREDENTIALS | PIC X. |  |
|  | 88 CA-CREDENTIALS-ADMIN-WAIVER | VALUE 'A'. |  |
|  | 88 CA-CREDENTIALS-FAIL | VALUE 'F'. |  |
|  | 88 CA-CREDENTIALS-PASS | VALUE 'P'. |  |
|  | 88 CA-CREDENTIALS-WAIVER | VALUE 'W'. |  |
| 10 | CA-TEST-EMISSION | PIC X(2). |  |
|  | 88 CA-EMISSION-ADMIN-WAIVER | VALUE 'A'. |  |
|  | 88 CA-EMISSION-FAIL | VALUE 'F'. |  |
|  | 88 CA-EMISSION-NOT-APPLIC | VALUE 'N'. |  |
|  | 88 CA-EMISSION-PASS | VALUE 'PP'. |  |
|  | 88 CA-EMISSION-PATTERN-FAILURE | VALUE 'T'. |  |
|  | 88 CA-EMISSION-WAIVER | VALUE 'PW'. |  |
|  | 88 CA-EMISSION-48HRS | VALUE '4'. |  |
| 10 | CA-TEST-SAFETY | PIC X. |  |
|  | 88 CA-SAFETY-ADMIN-WAIVER | VALUE 'A'. |  |
|  | 88 CA-SAFETY-FAIL | VALUE 'F'. |  |
|  | 88 CA-SAFETY-NOT-APPLIC | VALUE 'N'. |  |
|  | 88 CA-SAFETY-PASS | VALUE 'P'. |  |
|  | 88 CA-SAFETY-WAIVER | VALUE 'W'. |  |
| 10 | CA-PRIMARY-TEST-IND | PIC X. |  |
|  | 88 CA-PRIMARY-TEST | VALUE 'P'. |  |
|  | 88 CA-SECONDARY-TEST | VALUE 'S'. |  |
| 10 | CA-COLLECTOR-IND | PIC X. |  |
|  | 88 CA-COLL-ACTIVE | VALUE 'A'. |  |
|  | 88 CA-COLL-EXPIRED | VALUE 'E'. | PIC |
|  | 88 CA-COLL-PENDING | VALUE 'P'. |  |
|  | 88 CA-COLL-REVOKED | VALUE 'R'. |  |
|  | 88 CA-COLL-SUSPENDED | VALUE 'S'. |  |
|  | 88 CA-NOT-COLL-VEH | VALUE ' '. |  |
| 10 | CA-INSPECTOR-ID | PIC X(9). |  |
| 10 | CA-VIN-INPUT-SOURCE | PIC X. |  |
|  | 88 CA-VIN-BARCODE | VALUE 'B'. |  |
|  | 88 CA-VIN-MANUAL | VALUE 'M'. |  |
|  | 88 CA-VIN-VEHICLE | VALUE 'V'. |  |

10	CA-PLATE-INPUT-SOURCE	PIC X.

88	CA-PLATE-BARCODE	VALUE 'B'.

88	CA-PLATE-MANUAL	VALUE 'M'.

10	CA-EEI-TEST-INDICATOR	PIC X.

88	CA-EEI-TEST-PERFORMED	VALUE 'Y'.

88	CA-EEI-TEST-NOT-PERFORMED	VALUE 'N'.

10	CA-PINK-CARD-INDICATOR	PIC X.

88	CA-PINK-CARD-NO	VALUE 'N'.

88	CA-PINK-CARD-YES	VALUE 'Y'.

10	CA-TEST-GAS-CAP	PIC X.

88	CA-GAS-CAP-FAIL	VALUE 'F'.

88	CA-GAS-CAP-NOT-APPLICABLE	VALUE 'N'.

88	CA-GAS-CAP-PASS	VALUE 'P'.

88	CA-GAS-CAP-PREV-PASS	VALUE 'Z'.

10 CA-OBD-OVERALL-RESULT-IND	PIC X(01).

88	CA-OBD-OVERALL-RESULT-PAS	VALUE 'P'.

88	CA-OBD-OVERALL-RESULT-FAIL	VALUE 'F'.

88	CA-OBD-OVERALL-RESULT-PRV-PAS VALUE 'Z'.

88	CA-OBD-OVERALL-RESULT-NO-USED VALUE ' '.

10 CA-NJ-STCKR-NUM	PIC X(8). 05	CA-TEST-AREA	PIC X(216).

➔

### XML Schema

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:complexType name="AuditInformationType">

<xs:annotation>

<xs:documentation>

<summary>Basic identifying information about an audit</summary>

<remarks>This class holds basic information about an audit such as IDs and comments</remarks>

</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="AuditDateTime" type="xs:dateTime">

<xs:annotation>

<xs:documentation>

<summary>Timestamp of the audit</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="WorkstationID" type="xs:string">

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Workstation ID</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="FacilityID" type="xs:string"> |
|  |  |  | <xs:annotation> |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>Facility ID</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="UserID" type="xs:string"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>User ID</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="AuditComments" type="xs:string">

<xs:annotation>

<xs:documentation>

<summary>Auditor's comments about the audit</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PrinterAuditResultsType">

<xs:annotation>

<xs:documentation>

<summary>Results of a printer audit</summary>

<remarks>This class holds information about a printer audit including results for different ways of priting</remarks>

</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="PlainPaperResults" type="PassFailCodeType">

| plain paper</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

| plain paper</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Result for printing on

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ManualFeedResults" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>Result for printing when manually feeding paper</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="OBDAuditResultsType">

<xs:annotation>

<xs:documentation>

<summary>Results of an OBD audit</summary>

<remarks>This class holds information about the results of an OBD audit including overall and individual results.</remarks>

</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="OverallResult" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>Overall result of the OBD audit</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="EquipmentVisualCheckResult" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>Result of equipment visual check</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="CommCheckResult" type="PassFailCodeType"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>Result of comm check</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="AccuracyCheckResult" type="PassFailCodeType">

<xs:annotation>

| check</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

| check</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>Result of accuracy

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="RPMCheckResult" type="PassFailCodeType"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>Result of RPM check</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="CommProtocol" type="ObdProtocolCodeType"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>Comm protocol used</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="PCMModuleID1" type="xs:string"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>PCM module ID 1</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="PIDCount1" type="xs:int"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>PID count 1</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="PID1CResponse" type="Integer1DigitType"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>PID 1C response</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="MILCommandedOn" type="PassFailCodeType"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>MIL commanded on</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ReadinessResultMisfire" type="ReadinessResultType">

<xs:annotation>

<xs:documentation>

<summary>Misfire readiness result</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="ReadinessResultFuelControl" |

<summary>Fuel control readiness result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ReadinessResultComprehensive" type="ReadinessResultType">

<xs:annotation>

<xs:documentation>

<summary>Comprehensive readiness result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ReadinessResultCatalyst" type="ReadinessResultType">

<xs:annotation>

<xs:documentation>

<summary>Catalyst readiness result</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="ReadinessResultHeatedCatalyst" |
| type="ReadinessResultType"> <xs:annotation> |  |  |  |

<xs:documentation>

<summary>Heated catalyst readiness result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ReadinessResultEvap" type="ReadinessResultType">

<xs:annotation>

<xs:documentation>

<summary>Evap readiness result</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="ReadinessResultSecondaryAir" |
| type="ReadinessResultType"> <xs:annotation> |  |  |  |

<xs:documentation>

<summary>Secondary air readiness result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ReadinessResultAC" type="ReadinessResultType">

<xs:annotation>

<xs:documentation>

<summary>AC readiness result</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="ReadinessResultO2Sensor" |

<summary>O2 sensor readiness result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ReadinessResultO2SensorHeater" type="ReadinessResultType">

<xs:annotation>

<xs:documentation>

<summary>O2 sensor heater readiness result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ReadinessResultEGR" type="ReadinessResultType">

<xs:annotation>

<xs:documentation>

<summary>EGR readiness result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="DTC1" type="xs:string">

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>DTC 1</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="DTC2" type="xs:string"> |
|  |  |  | <xs:annotation> |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>DTC 2</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="DTC3" type="xs:string"> |
|  |  |  | <xs:annotation> |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>DTC 3</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="DTC4" type="xs:string"> |
|  |  |  | <xs:annotation> |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>DTC 4</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="DTC5" type="xs:string"> |
|  |  |  | <xs:annotation> |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>DTC 5</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="DTC6" type="xs:string"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>DTC 6</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="OBDVIN" type="xs:string">

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>OBD VIN</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="CALID" type="xs:string"> |
|  |  |  | <xs:annotation> |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>CAL ID</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="CVN" type="xs:string"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>CVN</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="DieselAuditResultsType">

<xs:annotation>

<xs:documentation>

<summary>Results of a diesel test equipment audit</summary>

<remarks>This class holds the results of an audit for all equipment involved in diesel testing.</remarks>

</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="OverallDieselResult" type="PassFailCodeType">

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Overall result</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="StationVisualResult" |
| type="PassFailCodeType"> <xs:annotation> |  |  |  |

| result</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

| result</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>Visual inspection

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="OpacityReading1" type="xs:string">

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Opacity reading 1</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="OpacityStandard1" type="xs:string"> |

<xs:annotation>

<xs:documentation>

<summary>Opacity standard 1</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="OpacityCheckResult1" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>Opacity check result 1</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="OpacityReading2" type="xs:string">

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Opacity reading 2</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="OpacityStandard2" type="xs:string"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>Opacity standard 2</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="OpacityCheckResult2" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>Opacity check result 2</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="OpacityReading3" type="xs:string">

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Opacity reading 3</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="OpacityStandard3" type="xs:string"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>Opacity standard 3</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="OpacityCheckResult3" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>Opacity check result 3</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="WeatherStationTemp" type="xs:int"> |

<xs:annotation>

<xs:documentation>

<summary>Weather station temp</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="WeatherStationTempStandard" type="xs:int"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>Weather station temp standard</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="WeatherStationTempCheckResult" |
| type="PassFailCodeType"> <xs:annotation> |  |  |  |

<xs:documentation>

<summary>Weather station temp check result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="WeatherStationRH" type="xs:string">

| humidity</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

| humidity</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Weather station relative

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="WeatherStationRHStandard" type="xs:string">

| humidity standard</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

| humidity standard</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Weather station relative

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="WeatherStationRHCheckResult" type="PassFailCodeType">

| humidty check result</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

| humidty check result</summary> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Weather station relative

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="WeatherStationBaro" type="xs:string"> |
|  |  |  | <xs:annotation> |

<xs:documentation>

<summary>Weather station barometer</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="WeatherStationBaroStandard" type="xs:string">

<xs:annotation>

<xs:documentation>

<summary>Weather station barometer

standard</summary>

</xs:documentation>

</xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="WeatherStationBaroCheckResult" |
| type="PassFailCodeType"> <xs:annotation> |  |  |  |

<xs:documentation>

<summary>Weather station barometer check result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="ExhaustTempCheckResult" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>Exhaust temp check result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RPMDevice1CheckResult" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>RPM device 1 check result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RPMDevice2CheckResult" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>RPM device 2 check result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RPMDevice3CheckResult" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>RPM device 3 check result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RPMDevice4CheckResult" type="PassFailCodeType">

<xs:annotation>

<xs:documentation>

<summary>RPM device 4 check result</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="AuditRecordType">

<xs:annotation>

<xs:documentation>

<summary>Record for the audit log file</summary>

<remarks>This class represents the information recorded in each entry to the audit log file.</remarks>

</xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="AuditInformation" type="AuditInformationType">

<xs:annotation>

<xs:documentation>

<summary>Basic information about the audit</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="PrinterAuditResults" type="PrinterAuditResultsType">

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>

<summary>Printer results</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="OBDAuditResults" |
| type="OBDAuditResultsType"> <xs:annotation> |  |  |  |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:annotation> |
| --- | --- | --- | --- | --- |

<xs:documentation>

<summary>OBD results</summary>

</xs:documentation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="DieselAuditResults" |
| type="DieselAuditResultsType"> <xs:annotation> |  |  |  |

<xs:documentation>

<summary>Diesel results</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:element name="AuditRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="AuditRecord" type="AuditRecordType" maxOccurs="unbounded">

<xs:annotation>

<xs:documentation>

<summary>List of audit records in the file.</summary>

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="AuditResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BasePersistResponse" />

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"/>

<xs:element name="Authenticate">

<xs:complexType>

| minOccurs="1" maxOccurs="1"> |  |  |  |  |
| --- | --- | --- | --- | --- |

| minOccurs="1" maxOccurs="1"> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="Facility"

| name="Client" minOccurs="1" maxOccurs="1"> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| name="Client" minOccurs="1" maxOccurs="1"> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

<xs:complexType>

<xs:sequence>

<xs:element

<xs:complexType>

<xs:sequence>

<xs:element name="Password" type="xs:string" minOccurs="1" maxOccurs="1">

| in the document |  |  |
| --- | --- | --- |
| available upon |  |  |

| in the document |  |  |
| --- | --- | --- |
| available upon |  |  |

<xs:annotation><xs:documentation>

The password as generated following the algorithm outlined

"ETNJ2_Interface_Specification_EIS_PasswordAlgorithm.doc"

request from VERIZON.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="Token"	type="TokenType" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

A token that was randomly generated by the client.

</xs:documentation></xs:annotation>

</xs:element>

</xs:sequence>

<xs:attribute name="clientId" type="xs:string" use="required">

<xs:annotation><xs:documentation>

The client Id.

</xs:documentation></xs:annotation>

</xs:attribute>

</xs:complexType>

</xs:element>

</xs:sequence>

<xs:attribute name="facilityLicenseId" use="required" type="xs:string">

<xs:annotation><xs:documentation>

The facility license id.

</xs:documentation></xs:annotation>

</xs:attribute>

</xs:complexType>

</xs:element>

<xs:element name="Lockout" type="LockoutType" minOccurs="0" maxOccurs="unbounded">

<xs:annotation><xs:documentation>

|  |  |  |  |  | this client has |
| --- | --- | --- | --- | --- | --- |

|  |  |  |  |  | this client has |
| --- | --- | --- | --- | --- | --- |

This is a list of all the lockout that have either been set or cleared since the last time

communicated with EIS.

</xs:documentation></xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="AuthenticateResponse">

<xs:complexType>

<xs:complexContent>

| minOccurs="0" maxOccurs="1"> |  |  |  |  |
| --- | --- | --- | --- | --- |

| minOccurs="0" maxOccurs="1"> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="Facility"

| name="Client" minOccurs="0" maxOccurs="1"> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| name="Client" minOccurs="0" maxOccurs="1"> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

<xs:complexType>

<xs:sequence>

<xs:element

<xs:complexType>

<xs:attribute name="clientId" use="required" type="xs:string">

<xs:annotation><xs:documentation>

The client id (either CIF station controller or PIF workstation id).

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="biometricUsage" use="required" type="YesNoType">

<xs:annotation><xs:documentation>

Is biometric scanning required at this client?

</xs:documentation></xs:annotation>

</xs:attribute>

|  |  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- | --- |

</xs:complexType>

</xs:element>

name="LicenseStatusCode" type="LicenseStatusCodeType" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

What is the license status of this facility?

|  |  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- | --- |

</xs:documentation></xs:annotation>

</xs:element>

name="FleetIndicator" type="xs:boolean" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

Is this a fleet facility?

|  |  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- | --- |

</xs:documentation></xs:annotation>

</xs:element>

name="MobileIndicator" type="xs:boolean" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

Is this a mobile facility?

|  |  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- | --- |

</xs:documentation></xs:annotation>

</xs:element>

name="FacilityProgramCode" type="FacilityProgramCodeType" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

Is this facility a centralized or decentralized facility?

|  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  |  |  | <xs:element |
| --- | --- | --- | --- | --- | --- | --- |

</xs:documentation></xs:annotation>

</xs:element>

name="FacilityOperationCode" type="FacilityOperationCodeType" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

Is this a State or contractor run facility?

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="PIFClassCode" type="PIFClassCodeType" minOccurs="0" maxOccurs="1">

<xs:annotation><xs:documentation>

The PIF class code.		This element will be excluded for non-PIF facilities.

</xs:documentation></xs:annotation>

</xs:element>

</xs:sequence>

<xs:attribute name="facilityLicenseId" use="required" type="xs:string">

<xs:annotation><xs:documentation>

The facility license id.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="licenseIssueDate" use="optional" type="xs:date">

<xs:annotation><xs:documentation>

The date the facility license was issued.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="licenseExpirationDate" use="optional" type="xs:date">

<xs:annotation><xs:documentation>

The expiration date of the facility license.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="licenseRestoreDate" use="optional" type="xs:date">

<xs:annotation><xs:documentation>

The restoration date of the facility license.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="licenseSuspensionDate" use="optional" type="xs:date">

<xs:annotation><xs:documentation>

The suspension date of the facility license.

</xs:documentation></xs:annotation>

</xs:attribute>

</xs:complexType>

</xs:element>

<xs:element name="UpdateAuditor" type="YesNoType"			minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

'Y' if there is an update to the auditors table for this client else 'N'.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="UpdateInspector" type="YesNoType"			minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

'Y' if there is an update to the inspectors table for this client else 'N'.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="NewMail" type="YesNoType"			minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

'Y' if there a new mail message for this facility.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="ConfigVersion" type="ConfigVersionType"		minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The overall configuration version of the reference data and rules engine.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="SoftwareVersion" type="SoftwareVersionType"		minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The expected software version number for this client.	This value will be based on the station license

type and the client id.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="RefFileVersion" type="RefFileVersionType"	minOccurs="0" maxOccurs="unbounded">

<xs:annotation><xs:documentation>

A list of version number for each individual piece for reference data and the the current rules

engine version.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="VPLVersion" type="VPLVersionType"			minOccurs="0" maxOccurs="unbounded">

<xs:annotation><xs:documentation>

A list of version number for each model year in the Vehicle pick list.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="Lockout" type="LockoutType"			minOccurs="0" maxOccurs="unbounded">

<xs:annotation><xs:documentation>

A complete list of lockouts current set on the VID.

</xs:documentation></xs:annotation>

|  |  |  |  |  |  | </xs:element> |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | <xs:element |

name="NetworkConfiguration" type="NetworkConfigurationType" minOccurs="1" maxOccurs="1"/>

<xs:element name="Time" type="xs:dateTime"			minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The current time on the EIS server.

</xs:documentation></xs:annotation>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="RefFileVersionType">

<xs:simpleContent>

| type="ReferenceDataFilenameType"> |  |  |  |
| --- | --- | --- | --- |

| type="ReferenceDataFilenameType"> |  |  |  |
| --- | --- | --- | --- |

<xs:extension base="xs:string">

<xs:annotation><xs:documentation>

The current version of this reference file.

</xs:documentation></xs:annotation>

<xs:attribute name="filename"

| reference data type. |  |  |  |  |
| --- | --- | --- | --- | --- |

| reference data type. |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:annotation><xs:documentation>

This is the file name of this

</xs:documentation></xs:annotation>

</xs:attribute>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="VPLVersionType">

<xs:complexContent>

<xs:extension base="VersionType">

<xs:attribute name="year" type="xs:gYear" use="required"/>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="VersionType">

<xs:attribute name="version" type="xs:integer"/>

</xs:complexType>

<xs:simpleType name="TokenType">

<xs:restriction base="xs:string">

<xs:length value="20"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="LockoutType">

<xs:attribute name="lockoutCode"	use="required" type="LockoutCodeType">

<xs:annotation><xs:documentation>

A coded value for this lockout.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="setDate"	use="required" type="xs:dateTime">

<xs:annotation><xs:documentation>

The date that this lockout was set.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="setBy"	use="required" type="LockoutActionByType">

<xs:annotation><xs:documentation>

This should be the id of the user or system that initiated that set this lockout.

On a client machine this could be either an auditor id or client id.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="setComment"	use="required" type="LockoutActionCommentType">

<xs:annotation><xs:documentation>

This should contain a useful comment about why this lockout was set.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="clearDate"	use="optional" type="xs:dateTime">

<xs:annotation><xs:documentation>

This is the date that this lockout was cleared.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="clearBy"	use="optional" type="LockoutActionByType">

<xs:annotation><xs:documentation>

This should be the id of the user or system that initiated that set this lockout.

On a client machine this could be either an auditor id or client id.

The attribute is required if the clearDate has been populated.

</xs:documentation></xs:annotation>

</xs:attribute>

<xs:attribute name="clearComment" use="optional" type="LockoutActionCommentType">

<xs:annotation><xs:documentation>

This should contain a useful comment about why this lockout was cleared.

The clear comment by should be populated if the clear date has been set.

</xs:documentation></xs:annotation>

</xs:attribute>

</xs:complexType>

<xs:simpleType name="LockoutActionByType">

<xs:restriction base="xs:string">

<xs:maxLength value="30"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LockoutActionCommentType">

<xs:restriction base="xs:string">

<xs:maxLength value="80"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LockoutCodeType">

<xs:restriction base="xs:string">

<xs:enumeration value="1">

<xs:annotation><xs:documentation>

STATE LOCKOUT

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="8"> |

<xs:annotation><xs:documentation>

FAIL TO PAY FOR COMMUNICATIONS

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="13"> |

<xs:annotation><xs:documentation>

Facility License Inactive

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="14"> |

<xs:annotation><xs:documentation>

Non-Payment Maintenance

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="15"> |

<xs:annotation><xs:documentation>

System Clock Failure

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="16"> |

<xs:annotation><xs:documentation>

Audit Failure

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="17"> |

<xs:annotation><xs:documentation>

Analyzer Warmup in Progress

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="18"> |

<xs:annotation><xs:documentation>

Analyzer Warmup	Failure

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="19"> |

<xs:annotation><xs:documentation>

Leak Check Required (# of hours)

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="20"> |

<xs:annotation><xs:documentation>

Leak Check Failure

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="21"> |

| hours) |  |  |  |
| --- | --- | --- | --- |

| hours) |  |  |  |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>

Analyzer Check/Calibration Required (#

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="22"> |

<xs:annotation><xs:documentation>

Analyzer Calibration Failure

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="23"> |

<xs:annotation><xs:documentation>

O2 Sensor Not Installed

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="24"> |

<xs:annotation><xs:documentation>

O2 Sensor Failure

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="25"> |

<xs:annotation><xs:documentation>

Calibration Cylinder Expired

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="26"> |

<xs:annotation><xs:documentation>

Calibration Cylinder Out of Tolerance

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="27"> |

<xs:annotation><xs:documentation>

Leak Check Required (# days)

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="28"> |

<xs:annotation><xs:documentation>

Analyzer Check/Calibration Required (# days)

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="29"> |

<xs:annotation><xs:documentation>

(#) RPM Bypasses in (# days)

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="30"> |

<xs:annotation><xs:documentation>

Gas Cap Tester Calibration Required (# days)

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="31"> |

<xs:annotation><xs:documentation>

Gas Cap Tester Failure

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="32"> |

<xs:annotation><xs:documentation>

Smokemeter Calibration Failure

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="33"> |

<xs:annotation><xs:documentation>

General Purpose Print Error

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="34"> |

<xs:annotation><xs:documentation>

Biometrics Required and No Device Functioning

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="35"> |

<xs:annotation><xs:documentation>

No Documents Scanned in (#) Inspections

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="36"> |

<xs:annotation><xs:documentation>

Reference Table Corrupt

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="37"> |

<xs:annotation><xs:documentation>

Software Configuration Not Current

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="38"> |

<xs:annotation><xs:documentation>

(#) Invalid VID Transactions

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="39"> |

<xs:annotation><xs:documentation>

(#) Offline Tests in (#) Days

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="40"> |

<xs:annotation><xs:documentation>

OBDII Interface not responding

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="41"> |

<xs:annotation><xs:documentation>

Smokemeter Calibration Required (# days)

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="42"> |

<xs:annotation><xs:documentation>

Gas Cap Lockout

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="43"> |

<xs:annotation><xs:documentation>

OBD Lockout

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="44"> |

<xs:annotation><xs:documentation>

Smoke Meter Lockout

</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="45"> |

<xs:annotation><xs:documentation>

Gas Bench Lockout

</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="CorporationCodeType">

<xs:restriction base="xs:string">

<xs:maxLength value="15"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="NetworkConfigurationType">

<xs:sequence>

<xs:element name="NumOffLineTest" type="xs:integer"

minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The maximum number of consecutive test that can be performed off-line within

"NumOffLineDays" before the workstation locks itself out.

</xs:documentation> </xs:annotation>

</xs:element>

<xs:element name="NumOffLineDays" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The maximum number of consecutive day that a work bench can remain off-line before

the workstation locks itself out.

</xs:documentation></xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="NumHoursRequiredLeakCheck" |
| type="xs:integer" minOccurs="1" maxOccurs="1"> |  |  |  |

|  |  |  |  | <xs:annotation><xs:documentation> |
| --- | --- | --- | --- | --- |
|  |  |  |  | The maximum number of hou |

|  |  |  |  | <xs:annotation><xs:documentation> |
| --- | --- | --- | --- | --- |
|  |  |  |  | The maximum number of hou |

rs that a gas

analyzer must perform a leak check before being used in an

inspection.	When exceeded, the gas analyzer is locked out, though other components of the

workstation and aspects of the inspection may continue.

</xs:documentation></xs:annotation>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="NumHoursBetweenGasBenchCal" |
| type="xs:integer" minOccurs="1" maxOccurs="1"> |  |  |  |

|  |  |  | <xs:annotation><xs:documentation> |
| --- | --- | --- | --- |
|  |  |  | The maximum number of hours |
| bench calibration before the workstation locks itself out. </xs:documentation></xs:annotation> |  |  |  |

|  |  |  | <xs:annotation><xs:documentation> |
| --- | --- | --- | --- |
|  |  |  | The maximum number of hours |
| bench calibration before the workstation locks itself out. </xs:documentation></xs:annotation> |  |  |  |

between a gas

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="NumDaysBetweenLeakTestCal" |
| type="xs:integer" minOccurs="1" maxOccurs="1"> |  |  |  |

|  |  |  |  | <xs:annotation><xs:documentation> |
| --- | --- | --- | --- | --- |
|  |  |  |  | The maximum number of days |
| test | before the workstation locks itself out. </xs:documentation></xs:annotation> |  |  |  |

|  |  |  |  | <xs:annotation><xs:documentation> |
| --- | --- | --- | --- | --- |
|  |  |  |  | The maximum number of days |
| test | before the workstation locks itself out. </xs:documentation></xs:annotation> |  |  |  |

between a leak

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="NumDaysBetweenGasBenchCal" |
| type="xs:integer" minOccurs="1" maxOccurs="1"> |  |  |  |

|  |  |  |  | <xs:annotation><xs:documentation> |
| --- | --- | --- | --- | --- |
|  |  |  |  | The maximum number of days between a |
| bench calibration before before the workstation locks itself out. </xs:documentation></xs:annotation> |  |  |  |  |
|  |  |  | </xs:element> |  |
|  |  |  | <xs:element name="RPMBypassesLimit" type="xs:integer" |  |
| minOccurs="1" maxOccurs="1"> |  |  |  |  |

|  |  |  |  | <xs:annotation><xs:documentation> |
| --- | --- | --- | --- | --- |
|  |  |  |  | The maximum number of days between a |
| bench calibration before before the workstation locks itself out. </xs:documentation></xs:annotation> |  |  |  |  |
|  |  |  | </xs:element> |  |
|  |  |  | <xs:element name="RPMBypassesLimit" type="xs:integer" |  |
| minOccurs="1" maxOccurs="1"> |  |  |  |  |

gas

<xs:annotation><xs:documentation>

The maximum number of RPM bypasses allowed within the NumInspPerRPMBypassLimit

before the workstation locks itself out.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="NumInspPerRPMBypassLimit" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

This is the number of inspection that the RPMLimit is applied to.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="NumDaysBetweenGasCapCal" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The maximum number of days between gas cap calibrations before the workstation locks itself out.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="NumDaysBetweenSmokemeterCal" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The maximum number of days between smoke-meter calibration before the workstation locks itself out.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="MaxNumInspWithoutScanningDoc" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The maximum number contiguous inspections where the vehicle document is not scanned for

VIN and/or license plate before the workstation locks itself out.

</xs:documentation></xs:annotation>

</xs:element>

<xs:element name="NumInvalidTransactions" type="xs:integer" minOccurs="1" maxOccurs="1">

<xs:annotation><xs:documentation>

The maximum number of invalid EIS transactions that may occur before the workstation

locks itself out.

</xs:documentation></xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:schema>

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"/>

<xs:element name="AuditorsRequest"	type="AuthenticatedUserRequest"/>

<xs:element name="AuditorsResponse"	type="AuditorResponse"/>

<xs:element name="InspectorsRequest" type="AuthenticatedUserRequest"/>

<xs:element name="InspectorsResponse" type="InspectorResponse"/>

<xs:complexType name="AuthenticatedUserRequest">

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AuditorResponse">

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="Auditor" type="AuthenticatedUserType" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="InspectorResponse">

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="Inspector" type="AuthenticatedUserType" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="AuthenticatedUserType">

<xs:sequence>

<xs:element name="LastName">

|  |  |  | <xs:simpleType> |
| --- | --- | --- | --- |
|  |  |  | <xs:restriction base="xs:string"> |

<xs:minLength value="0"/>

<xs:maxLength value="20"/>

</xs:restriction>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="FirstName"> |

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="FirstName"> |

</xs:simpleType>

|  |  |  | <xs:simpleType> |
| --- | --- | --- | --- |
|  |  |  | <xs:restriction base="xs:string"> |

<xs:minLength value="0"/>

<xs:maxLength value="30"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="MiddleInitial" minOccurs="0" maxOccurs="1">

|  |  |  | <xs:simpleType> <xs:restriction base="xs:string"> |
| --- | --- | --- | --- |
|  |  |  | <xs:minLength value="0"/> |
|  |  |  | <xs:maxLength value="1"/> |
|  |  |  | </xs:restriction> |
|  |  |  | </xs:simpleType> |
|  |  |  | </xs:element> |
|  |  |  | <xs:element name="BiometricCollectionStatus"> |
|  |  |  | <xs:simpleType> |
|  |  |  | <xs:restriction base="xs:string"> |
|  |  |  | <xs:enumeration value="C"> |

<xs:annotation><xs:documentation>Collected</xs:documentation></xs:annotation>

|  |  |  |  |  |  | </xs:enumeration> |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | <xs:enumeration value="N"> |

<xs:annotation><xs:documentation>Not Collected</xs:documentation></xs:annotation>

</xs:enumeration>

<xs:enumeration value="U">

<xs:annotation><xs:documentation>Unable to Collect</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="BiometricUsage">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="N">

<xs:annotation><xs:documentation>Do not use Biometrics</xs:documentation></xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation><xs:documentation>Use Biometrics Always</xs:documentation></xs:annotation>

</xs:enumeration>

<xs:enumeration value="O">

<xs:annotation><xs:documentation>May override Biometrics</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="BiometricLastUpdated" type="xs:dateTime" minOccurs="0" maxOccurs="1"/>

<xs:element name="AccessCode" type="AccessCodeType" />

<xs:element name="Expiry" type="xs:date" />

<xs:element name="Status" type="LicenseStatusType" />

</xs:sequence>

<xs:attribute name="licenseId" type="AuthenticatedUserIdType"/>

</xs:complexType>

<xs:complexType name="AccessCodeType">

<xs:simpleContent>

<xs:extension base="xs:base64Binary">

<xs:attribute name="salt" type="SaltType"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:simpleType name="SaltType">

<xs:restriction base="xs:string">

<xs:length value="20"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LicenseStatusType">

<xs:restriction base="xs:string">

<xs:enumeration value="R">

<xs:annotation><xs:documentation>Revoked</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="A"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Active</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="S"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Suspended</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="P"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Pending</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

</xs:schema>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:include schemaLocation="inspection.xsd"	/>

<xs:element name="beginInspectionRequest" type="beginInspectionRequestType"/>

<xs:element name="beginInspectionResponse" type="beginInspectionResponseType"/>

<xs:complexType name="beginInspectionRequestType" >

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="VinDocument" type="VinDocumentType" minOccurs="1" maxOccurs="1"/>

<xs:element name="PlateNumber" type="PlateNumberType"		minOccurs="1" maxOccurs="1"/>

<xs:element name="NJ1Indicator" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="beginInspectionResponseType" >

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="PrevInspection" type="InspectionType"		minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:element name="beginCollectorVehicleInspectionRequest" type="beginTestCollectorVehicleRequest"/>

<xs:element name="beginCollectorVehicleInspectionResponse" type="beginTestCollectorVehicleResponse"/>

<xs:complexType name="beginTestCollectorVehicleRequest" >

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="VINDocument" type="VinDocumentType" minOccurs="1" maxOccurs="1"/>

<xs:element name="PlateNumber" type="PlateNumberType" minOccurs="1" maxOccurs="1"/>

<xs:element name="NJ1Indicator" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="Voucher" type="VoucherType"	minOccurs="1" maxOccurs="1"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="beginTestCollectorVehicleResponse" >

|  |  | <xs:complexContent> <xs:extension base="BaseResponseType"> |
| --- | --- | --- |
|  |  | <xs:sequence> |
|  |  | <xs:element name="CollectorVehicleIndicator" |

type="YesNoType" minOccurs="1" maxOccurs="1"/>

<xs:element name="PrevOdometerReading" type="PrevOdometerReadingType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="VoucherType">

<xs:attribute name="voucherExpirationDate" type="xs:date"/>

<xs:attribute name="voucherSource"	type="VoucherSourceType"/>

</xs:complexType>

</xs:schema>

bus.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:element name="BusTerminalRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="BusTerminalResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="CommercialBusVersion"	type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="BusTerminal" type="BusTerminalType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="BusTerminalType">

<xs:sequence>

<xs:element name="TerminalId" type="String10" minOccurs="0" maxOccurs="1"></xs:element>

<xs:element name="TerminalAddress" type="AddressType" minOccurs="0" maxOccurs="1"></xs:element>

<xs:element name="CompanyName" type="String50" minOccurs="0" maxOccurs="1"></xs:element>

<xs:element name="BusNumber" type="String10" minOccurs="0" maxOccurs="1"></xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="AddressType">

<xs:sequence>

<xs:element name="StreeAddress" type="String50" minOccurs="0" maxOccurs="1"></xs:element>

<xs:element name="City" type="String35" minOccurs="0" maxOccurs="1"></xs:element>

<xs:element name="County" type="String50" minOccurs="0" maxOccurs="1"></xs:element>

<xs:element name="State" type="String2" minOccurs="0" maxOccurs="1"></xs:element>

<xs:element name="Zip" type="String10" minOccurs="0" maxOccurs="1"></xs:element>

</xs:sequence>

</xs:complexType>

</xs:schema>

commercialBus.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:include schemaLocation="inspectionConditions.xsd"	/>

<xs:element name="CommercialBusRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="CommercialBusResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="CommercialBusVersion"	type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="Condition" type="InspectionConditionType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:simpleType name="CredentialsResultCodeType">

<xs:annotation>

<xs:documentation>An enumeration for CredentialsResultCodeType

</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| fail</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| fail</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="3">

<xs:documentation>30-day

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="A"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Advisory</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="F"> |

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="P">

<xs:annotation>

<xs:documentation>Pass</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="S">

<xs:annotation>

<xs:documentation>out-of-service</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="GasCapLeakRateType">

<xs:union memberTypes="FlowRateType FlowRateStringType"/>

</xs:simpleType>

<xs:simpleType name="FlowRateType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="3"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="FlowRateStringType">

<xs:restriction base="xs:string">

<xs:enumeration value="QP"/>

<xs:enumeration value="QF"/>

<xs:enumeration value="MF"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="VinMaskType">

<xs:restriction base="xs:string">

<xs:maxLength value="19"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="RPMLimitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="6"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="COLimitType">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="5"/>

<xs:fractionDigits value="2"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ModeTimerLimitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="5"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="HCLimitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="5"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="LicenseStatusCodeType">

<xs:restriction base="xs:string">

<xs:enumeration value="A">

<xs:annotation><xs:documentation>Active</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="R"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Revoked</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="S"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Suspended</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="P"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Pending</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="C"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Cancelled</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="InspectorDataType">

<xs:sequence>

<xs:element name="InspectorId" type="InspectorIdAndSourceType"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="InspectorIdAndSourceType">

<xs:simpleContent>

<xs:extension base="InspectorIdType">

<xs:attribute name="source" type="InspectorIdSourceType"/>

<xs:attribute name="authSource" type="InspectorAuthSourceType"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:simpleType name="ConfigVersionType">

<xs:restriction base="xs:string">

<xs:maxLength value="30"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SoftwareVersionType">

<xs:restriction base="xs:string">

<xs:maxLength value="30"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="BaseRequestType">

<xs:attribute name="protocolVersion" type="xs:string" fixed="1.00"/>

<xs:attribute name="source"	type="SourceType"/>

<xs:attribute name="id"	type="xs:string"/>

</xs:complexType>

<xs:complexType name="BaseResponseType">

<xs:attribute name="protocolVersion" type="xs:string" fixed="1.00"/>

<xs:attribute name="source"	type="SourceType"/>

<xs:attribute name="processingTime"	type="xs:duration"/>

<xs:attribute name="id"	type="xs:string"/>

</xs:complexType>

<xs:complexType name="BasePersistResponse">

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="Result" type="SimpleResultType"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="BasePersistInspectionResponse">

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="Result" type="PersistInspectionResultType"/>

<xs:element name="SystemUID" type="xs:integer"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:simpleType name="AuditorIdType">

<xs:restriction base="xs:string">

<xs:pattern value="AUD[0-9]{6}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="CollectorVehicleStatusType">

<xs:restriction base="xs:string">

<xs:enumeration value="I">

<xs:annotation>

<xs:documentation>Initial</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation>Renewal</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="DTCCodeType">

<xs:restriction base="xs:string">

<xs:pattern value="[0-9A-Z]{5}"></xs:pattern>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="FacilityIdType">

<xs:restriction base="xs:string">

<xs:pattern value="(BIT|CIF|DEC|DEF|ERF|HIF|MIT|PFF|PIF|PIM|SIF)[0-9]{6}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="InspectorIdType">

<xs:restriction base="xs:string">

<xs:pattern value="INL[0-9]{6}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="MakeType">

<xs:restriction base="xs:string">

<xs:maxLength value="17" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ModelType">

<xs:restriction base="xs:string">

<xs:maxLength value="23" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="OdometerReadingType">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="9999999"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PassFailAdvisoryCodeType">

<xs:annotation>

<xs:documentation>An enumeration for PassFailAdvisoryCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="A">

<xs:annotation>

<xs:documentation>Advisory</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="F">

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="P">

<xs:annotation>

<xs:documentation>Pass</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PassFailCodeType">

<xs:annotation>

<xs:documentation>An enumeration for PassFailCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="F">

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="P">

<xs:annotation>

<xs:documentation>Pass</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PlateSourceType">

<xs:annotation>

<xs:documentation>An enumeration for PlateSourceType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="B">

<xs:annotation>

<xs:documentation>Scanned Document Barcode (Reg or VIR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>Manual Entry</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="PlateNumberType">

<xs:simpleContent>

<xs:extension base="SimplePlateType">

<xs:attribute name="plateSource" type="PlateSourceType" use="optional" >

<xs:annotation>

<xs:documentation>

The plate source needs to be made optional to

support

legacy collector vehicle inspection where the plate

source what not stored.

</xs:documentation>

</xs:annotation>

</xs:attribute>

<xs:attribute name="jurisCode" type="String2" use="optional">

<xs:annotation>

<xs:documentation>

support

The jurisdiction code needs to be made optional to

legacy collector vehicle inspection where the

jurisdiction

code what not stored.

</xs:documentation>

</xs:annotation>

</xs:attribute>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="PrevOdometerReadingType">

<xs:simpleContent>

<xs:extension base="OdometerReadingType">

<xs:attribute name="prevInspDate" type="xs:date"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:simpleType name="ReadinessResultType">

<xs:annotation>

<xs:documentation>An enumeration for ReadinessResultType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="E">

<xs:annotation>

<xs:documentation>Excluded</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation>Ready</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="X">

<xs:annotation>

<xs:documentation>Not ready</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="RepairFacilityIdType">

<xs:restriction base="xs:string">

<xs:pattern value="([ERF|PIF|PFF|DEC|DEF][0-9]{6})|SELF"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="RepairTechIdType">

<xs:restriction base="xs:string">

<xs:pattern value="(ERT|INL)[0-9]{6}"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SimplePlateType">

<xs:restriction base="xs:string">

<xs:minLength value="1"/>

<xs:maxLength value="10" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SimpleResultType">

<xs:restriction base="xs:string">

<xs:enumeration value="Committed" />

<xs:enumeration value="Error" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PersistInspectionResultType">

<xs:restriction base="xs:string">

<xs:enumeration value="Committed" />

<xs:enumeration value="CommittedWithErrors" />

<xs:enumeration value="Error" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SimpleVINType">

<xs:restriction base="xs:string">

<xs:maxLength value="19" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SourceType">

|  |  | <xs:restriction base="xs:string"> |
| --- | --- | --- |
|  |  | <xs:enumeration value="PAR"> |

<xs:annotation><xs:documentation>Parsons</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="SGS"> |

<xs:annotation><xs:documentation>SGS</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="VehicleGVWRType">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0" />

<xs:maxInclusive value="100000" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="VehicleIdType">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:totalDigits value="11"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="VehicleYearType">

<xs:restriction base="xs:integer">

<xs:minInclusive value="1900" />

<xs:maxInclusive value="2049" />

</xs:restriction>

</xs:simpleType>

<xs:complexType name="VinDocumentType">

<xs:simpleContent>

<xs:extension base="SimpleVINType">

<xs:attribute name="vinSource" type="VinSourceType" />

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="VinObdType">

<xs:simpleContent>

<xs:extension base="SimpleVINType"/>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="VinVehicleType">

<xs:simpleContent>

<xs:extension base="SimpleVINType">

<xs:attribute name="vinSource" type="VinSourceType" />

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:simpleType name="VoucherSourceType">

<xs:restriction base="xs:string">

<xs:enumeration value="B">

<xs:annotation>

<xs:documentation>Bar Code</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>Manual Entry</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="YesNoType">

<xs:annotation>

<xs:documentation>An enumeration for YesNoType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="N">

<xs:annotation>

<xs:documentation>No/False</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Y">

<xs:annotation>

<xs:documentation>Yes/True</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PlateMatchFlagType">

<xs:restriction base="xs:string">

<xs:enumeration value="1">

<xs:annotation><xs:documentation>VIN/Plate matched both in OIT and previous test</xs:documentation></xs:annotation>

</xs:enumeration>

<xs:enumeration value="2">

<xs:annotation><xs:documentation>VIN/Plate matched in OIT, VIN/Plate did not match in previous test</xs:documentation></xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation><xs:documentation>VIN/Plate did not match in OIT, VIN/Plate matched in previous test</xs:documentation></xs:annotation>

</xs:enumeration>

<xs:enumeration value="4">

<xs:annotation><xs:documentation>VIN/Plate did not match in OIT, VIN/Plate did not match in previous

test</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="9"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Offline</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="VinMatchFlagType">

<xs:restriction base="xs:string">

<xs:enumeration value="1">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
|  |  |  | <xs:documentation> |

OIT Up?	Yes

OIT/MVC Match?	Yes

Prev. Test Match? Yes

</xs:documentation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="2"> |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="2"> |

</xs:annotation>

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
|  |  |  | <xs:documentation> |

OIT up?	Yes

OIT/MVC Match?	Yes

Prev. Test Match? No

</xs:documentation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="3"> |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="3"> |

</xs:annotation>

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
|  |  |  | <xs:documentation> |

OIT up?	Yes

OIT/MVC Match?	No

Prev. Test Match? Yes

</xs:documentation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="4"> |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="4"> |

</xs:annotation>

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
|  |  |  | <xs:documentation> |

OIT up?	Yes

OIT/MVC Match?	No

Prev. Test Match? No

</xs:documentation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="5"> |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="5"> |

</xs:annotation>

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
|  |  |  | <xs:documentation> |

OIT up?	No

OIT/MVC Match?	Yes

Prev. Test Match? Yes

</xs:documentation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="6"> |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="6"> |

</xs:annotation>

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
|  |  |  | <xs:documentation> |

OIT up?	No

OIT/MVC Match?	Yes

Prev. Test Match? No

</xs:documentation>

|  |  |  | </xs:annotation> |
| --- | --- | --- | --- |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="7"> |
|  |  |  | <xs:annotation> |
|  |  |  | <xs:documentation> |
|  |  |  | OIT up? No |
|  |  |  | OIT/MVC Match? No |
|  |  |  | Prev. Test Match? Yes |
|  |  |  | </xs:documentation> |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="8"> |
|  |  |  | <xs:annotation> |
|  |  |  | <xs:documentation> |
|  |  |  | OIT up? No |
|  |  |  | OIT/MVC Match? No |
|  |  |  | Prev. Test Match? No |
|  |  |  | </xs:documentation> |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="9"> |

<xs:annotation><xs:documentation>Offline</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="CurrencyType">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="5"/>

<xs:fractionDigits value="2"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer1DigitType">

<xs:restriction base="xs:int">

<xs:minInclusive value="0" />

<xs:maxInclusive value="9" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer2DigitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="2"/>

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer3DigitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="3"/>

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer9DigitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="9"/>

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer4DigitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="4"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer5DigitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="5"/>

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer6DigitType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="6"/>

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer1To24Type">

<xs:restriction base="xs:integer">

<xs:minInclusive value="1"/>

<xs:maxInclusive value="24"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer2To12Type">

<xs:restriction base="xs:integer">

<xs:minInclusive value="2"/>

<xs:maxInclusive value="12"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer0To16Type">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="16"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Integer3To6Type">

<xs:restriction base="xs:integer">

<xs:minInclusive value="3"/>

<xs:maxInclusive value="6"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Decimal4Dot1Type">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="4"/>

<xs:fractionDigits value="1"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Decimal4Dot3Type">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="4"/>

<xs:fractionDigits value="3"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Decimal5Dot1Type">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="5"/>

<xs:fractionDigits value="1"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Decimal4Dot2Type">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="4"/>

<xs:fractionDigits value="2"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Decimal3Dot2Type">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="3"/>

<xs:fractionDigits value="2"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="Decimal3Dot1Type">

<xs:restriction base="xs:decimal">

<xs:totalDigits value="3"/>

<xs:fractionDigits value="1"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String1">

<xs:restriction base="xs:string">

<xs:maxLength value="1"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String2">

<xs:restriction base="xs:string">

<xs:maxLength value="2"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String3">

<xs:restriction base="xs:string">

<xs:maxLength value="3"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String4">

<xs:restriction base="xs:string">

<xs:maxLength value="4"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String8">

<xs:restriction base="xs:string">

<xs:maxLength value="8"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String10">

<xs:restriction base="xs:string">

<xs:maxLength value="10"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String12">

<xs:restriction base="xs:string">

<xs:maxLength value="12"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String15">

<xs:restriction base="xs:string">

<xs:maxLength value="15"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String35">

<xs:restriction base="xs:string">

<xs:maxLength value="35" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String21">

<xs:restriction base="xs:string">

<xs:maxLength value="21" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String30">

<xs:restriction base="xs:string">

<xs:maxLength value="30" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String40">

<xs:restriction base="xs:string">

<xs:maxLength value="40"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String50">

<xs:restriction base="xs:string">

<xs:maxLength value="40"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="String200">

<xs:restriction base="xs:string">

<xs:maxLength value="200" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="WorkstationIdType">

<xs:restriction base="xs:string">

<xs:length value="8"/>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="FacilityProgramCodeType">

<xs:annotation>

<xs:documentation>An enumeration for facility program codes

</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="C">

<xs:annotation>

<xs:documentation>Central</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="D">

<xs:annotation>

<xs:documentation>Decentral</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="FacilityOperationCodeType">

<xs:annotation>

<xs:documentation>An enumeration of facility operation codes

</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="C">

<xs:annotation>

<xs:documentation>Contractor</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="S">

<xs:annotation>

<xs:documentation>State</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="WaitSystemCodeType">

<xs:annotation>

<xs:documentation>An enumeration for WaitSystemCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="A">

<xs:annotation>

<xs:documentation>Active Wait System</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="I">

<xs:annotation>

<xs:documentation>Inactive Wait System</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="N">

<xs:annotation>

<xs:documentation>No Wait System</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="InspectorAuthSourceType">

<xs:annotation>

<xs:documentation>An enumeration for InspectorAuthSourceType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="A">

<xs:annotation>

<xs:documentation>Biometric Authentication</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>Manual PIN Entry</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="InspectorIdSourceType">

<xs:annotation>

<xs:documentation>An enumeration for InspectorIDSourceType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="B">

<xs:annotation>

<xs:documentation>Barcode from ID</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>Manual Entry</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="DLCResultCodeType">

<xs:restriction base="xs:string">

<xs:enumeration value="D">

<xs:annotation>

<xs:documentation>Damaged</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>Missing</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="O">

<xs:annotation>

<xs:documentation>Obstructed</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="P">

<xs:annotation>

<xs:documentation>Pass</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="CatDTCCodeType">

<xs:restriction base="xs:string">

<xs:enumeration value="C">

<xs:annotation>

<xs:documentation>Cat DTC Code Clear/Reset (Clear/Reset)</xs:documentation>

|  |  |  | </xs:annotation> |
| --- | --- | --- | --- |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="N"> |
|  |  |  | <xs:annotation> |
|  |  |  | <xs:documentation>Initialized Cat DTC |
| (None/Neutral)</xs:documentation> |  |  |  |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="Y"> |
|  |  |  | <xs:annotation> |
|  |  |  | <xs:documentation>Cat DTC Code Set |

(Yes/Set)</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="DisplUnitCodeType">

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Inches</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Inches</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="CI">

<xs:documentation>Cubic

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="LI"> |

<xs:annotation>

<xs:documentation>Liters</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="CC">

<xs:annotation>

<xs:documentation>Cubic Centimeter</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="OtherEngineCodeType">

<xs:restriction base="xs:string">

<xs:enumeration value="E">

<xs:annotation>

<xs:documentation>Electric</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation>Rotary</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="TransmissionCodeType">

<xs:restriction base="xs:string">

<xs:enumeration value="A">

<xs:annotation>

<xs:documentation>Automatic</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>Manual</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="AspirationCodeType">

<xs:restriction base="xs:string">

|  |  |  | <xs:enumeration value="A"> <xs:annotation> |
| --- | --- | --- | --- |
| <xs:documentation>Aftercooled</xs:documentation> |  |  |  |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="B"> |
|  |  |  | <xs:annotation> |
|  |  |  | <xs:documentation>Blower</xs:documentation> |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="I"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Intercooled</xs:documentation> |  |  |  |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="N"> |
|  |  |  | <xs:annotation> |
|  |  |  | <xs:documentation>Naturally |
| aspirated</xs:documentation> |  |  |  |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="S"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Supercharger</xs:documentation> |  |  |  |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="T"> |
|  |  |  | <xs:annotation> |
|  |  |  | <xs:documentation>Turbo |
| charged</xs:documentation> |  |  |  |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="U"> |
|  |  |  | <xs:annotation> |
|  |  |  | <xs:documentation>Unknown</xs:documentation> |
|  |  |  | </xs:annotation> |
|  |  |  | </xs:enumeration> |

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="FuelType">

<xs:annotation>

<xs:documentation>An enumeration of fuel types</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="G">

<xs:annotation>

<xs:documentation>Gasoline</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="D">

<xs:annotation>

<xs:documentation>Diesel</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="N">

<xs:annotation>

<xs:documentation>None</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="O">

<xs:annotation>

<xs:documentation>Other</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="OverallSafetyResultType">

<xs:annotation>

<xs:documentation>An enumeration for OverallSafetyResultTypee</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Fail</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Fail</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="3">

<xs:documentation>30-day

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="4"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Fail</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Fail</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>48-hour

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="F"> |

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="N"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| required</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| required</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>not

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="P"> |

<xs:annotation>

<xs:documentation>Pass</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="S"> |
|  |  |  | <xs:annotation> |

<xs:documentation>Out-of-Service</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="X"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| performed</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| performed</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Fail, not

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="Z"> |

Already</xs:documentation>

<xs:annotation>

<xs:documentation>Passed

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="OverallEmResultCodeType">

<xs:annotation>

<xs:documentation>An enumeration for EmResultCodeType

</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="F">

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="N"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| required</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| required</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>not

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="P"> |

<xs:annotation>

<xs:documentation>Pass</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="S"> |
|  |  |  | <xs:annotation> |

<xs:documentation>out-of-service</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="X"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| performed</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| performed</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Fail, not

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="Z"> |

Already</xs:documentation>

<xs:annotation>

<xs:documentation>Passed

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<!-- Generated Enumerations -->

<xs:simpleType name="EmResultCodeType">

<xs:annotation>

<xs:documentation>An enumeration for EmResultCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="F">

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="N"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| required</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| required</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>not

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="P"> |

<xs:annotation>

<xs:documentation>Pass</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="R"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Pass</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Pass</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Repaired to

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="S"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Pass</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Pass</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Repaired to

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="X"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| performed</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| performed</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Fail, not

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="Z"> |

Already</xs:documentation>

<xs:annotation>

<xs:documentation>Passed

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="EPAMobile6CodeType">

<xs:annotation>

<xs:documentation>An enumeration for EPAMobile6CodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (Passenger Cars)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (Passenger Cars)</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="LDGV">

<xs:documentation>Light-Duty Gasoline

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="LDGT1"> |

<xs:annotation>

<xs:documentation>Light-Duty Gasoline Trucks

- (0-6,000 lbs. GVWR, 0-3,750 lbs. LVW)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="LDGT2">

<xs:annotation>

<xs:documentation>Light-Duty Gasoline Trucks

- (0-6,000 lbs. GVWR, 3,751-5,750 lbs. LVW)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="LDGT3">

<xs:annotation>

<xs:documentation>Light-Duty Gasoline Trucks

- (6,001-8,500 lbs. GVWR, 0-5,750 lbs. ALVW)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="LDGT4">

<xs:annotation>

<xs:documentation>Light-Duty Gasoline Trucks

- (6,001-8,500 lbs. GVWR, greater than 5,751 lbs. ALVW)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="HDGV2b">

<xs:annotation>

<xs:documentation>Class 2b Heavy-Duty Gasoline Vehicles (8,501-10,000 lbs. GVWR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="HDGV3">

<xs:annotation>

<xs:documentation>Class 3 Heavy-Duty Gasoline Vehicles (10,001-14,000 lbs. GVWR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="HDGV4">

<xs:annotation>

<xs:documentation>Class 4 Heavy-Duty Gasoline Vehicles (14,001-16,000 lbs. GVWR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="HDGV5">

<xs:annotation>

<xs:documentation>Class 5 Heavy-Duty Gasoline Vehicles (16,001-19,500 lbs. GVWR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="HDGV6">

<xs:annotation>

<xs:documentation>Class 6 Heavy-Duty Gasoline Vehicles (19,501-26,000 lbs. GVWR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="HDGV7">

<xs:annotation>

<xs:documentation>Class 7 Heavy-Duty Gasoline Vehicles (26,001-33,000 lbs. GVWR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="HDGV8a">

<xs:annotation>

<xs:documentation>Class 8a Heavy-Duty Gasoline Vehicles (33,001-60,000 lbs. GVWR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="HDGV8b">

<xs:annotation>

<xs:documentation>Class 8b Heavy-Duty Gasoline Vehicles (>60,000 lbs. GVWR)</xs:documentation>

</xs:annotation>

</xs:enumeration>

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| (Passenger Cars)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| (Passenger Cars)</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="LDDV">

<xs:documentation>Light-Duty Diesel Vehicles

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="LDDT12"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 1and | 2 (0-6,000 lbs. |  | GVWR)</xs:documentation> </xs:annotation> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 1and | 2 (0-6,000 lbs. |  | GVWR)</xs:documentation> </xs:annotation> |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV2b"> |
|  |  |  | <xs:annotation> |
| Vehicles (8,501-10,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV2b"> |
|  |  |  | <xs:annotation> |
| Vehicles (8,501-10,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Light-Duty Diesel Trucks

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV3"> |
|  |  |  | <xs:annotation> |
| Vehicles (10,001-14,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV3"> |
|  |  |  | <xs:annotation> |
| Vehicles (10,001-14,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Class 2b Heavy-Duty Diesel

<xs:documentation>Class 3 Heavy-Duty Diesel

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV4"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (14,001-16,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (14,001-16,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Class 4 Heavy-Duty Diesel

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV5"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (16,001-19,500 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (16,001-19,500 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV6"> |
|  |  |  | <xs:annotation> |
| Vehicles (19,501-26,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV6"> |
|  |  |  | <xs:annotation> |
| Vehicles (19,501-26,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Class 5 Heavy-Duty Diesel

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV7"> |
|  |  |  | <xs:annotation> |
| Vehicles (26,001-33,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV7"> |
|  |  |  | <xs:annotation> |
| Vehicles (26,001-33,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Class 6 Heavy-Duty Diesel

<xs:documentation>Class 7 Heavy-Duty Diesel

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV8a"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (33,001-60,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (33,001-60,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Class 8a Heavy-Duty Diesel

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDV8b"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (>60,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles (>60,000 lbs. GVWR)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Class 8b Heavy-Duty Diesel

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="MC"> |

<xs:annotation>

<xs:documentation>Motorcycles

(Gasoline)</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDGB"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Transit and Urban)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Transit and Urban)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Gasoline Buses (School,

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDBT"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Buses</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Buses</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Diesel Transit and Urban

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HDDBS"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Buses</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Buses</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="LDDT34"> |
|  |  |  | <xs:annotation> |
| and 4 (6,001-8,500 lbs. GVWR)</xs:documentation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="LDDT34"> |
|  |  |  | <xs:annotation> |
| and 4 (6,001-8,500 lbs. GVWR)</xs:documentation> |  |  |  |

<xs:documentation>Diesel School

<xs:documentation>Light-Duty Diesel Trucks 3

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="FuelCodeType">

<xs:annotation>

<xs:documentation>An enumeration for FuelCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Gas</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Gas</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="CNGA">

<xs:documentation>Compressed Natural

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="DIES"> |

<xs:annotation>

<xs:documentation>Diesel</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="ELEC"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Electric</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="ELED"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Diesel</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Diesel</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Electric /

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="ELEG"> |

Gasoline</xs:documentation>

<xs:annotation>

<xs:documentation>Electric /

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="GASO"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Gasoline</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="GCNG"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Natural Gas</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Natural Gas</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Gasoline / Compressed

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="GHYD"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Hydrogen</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Hydrogen</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Gasoline /

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="GLNG"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Natural Gas</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Natural Gas</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Gasoline / Liquified

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="GLPG"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Petroleum Gas</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Petroleum Gas</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Gasoline / Liquified

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="GMET"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| (M85)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| (M85)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Gasoline / Methane

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="GPRO"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Propane</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Propane</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Gasoline /

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="HYDR"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Hydrogen</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="METH"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Methanol</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="PROP"> |

<xs:annotation>

<xs:documentation>Propane</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="InspTypeCodeType">

<xs:annotation>

<xs:documentation>An enumeration for InspTypeCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Replacement</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Replacement</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="RPL">

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CVQ"> |
|  |  |  | <xs:annotation> |
| Qualification</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CVQ"> |
|  |  |  | <xs:annotation> |
| Qualification</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Sticker

<xs:documentation>Collector Vehicle

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBA"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial Bus Audit

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBC"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial Bus

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBD"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| - Dual Use</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| - Dual Use</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBG"> |
|  |  |  | <xs:annotation> |
| Garage/Complaint</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBG"> |
|  |  |  | <xs:annotation> |
| Garage/Complaint</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial Bus Compliance

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBR"> |
|  |  |  | <xs:annotation> |
| Roadside/Illegal Compliance</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBR"> |
|  |  |  | <xs:annotation> |
| Roadside/Illegal Compliance</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial Bus

<xs:documentation>Commercial Bus

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBS"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Self-inspect</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Self-inspect</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial Bus

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="CBX"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Specification</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Specification</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial Bus

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="DC"> |

Compliance</xs:documentation>

<xs:annotation>

<xs:documentation>Diesel

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="IMC"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>IM

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="MC"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Motorcycle

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="SBC"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Compliance</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="SBX"> |
|  |  |  | <xs:annotation> |
| Specification</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="SBX"> |
|  |  |  | <xs:annotation> |
| Specification</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>School Bus

<xs:documentation>School Bus

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="SBA"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Audit</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Audit</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>School Bus

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="SBS"> |

| Camp</xs:documentation> |  |  |  |
| --- | --- | --- | --- |

| Camp</xs:documentation> |  |  |  |
| --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>School Bus Summer

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PIFClassCodeType">

<xs:annotation>

<xs:documentation>An enumeration for PIFClassCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="1">

<xs:annotation>

<xs:documentation>OBD-Only</xs:documentation>

</xs:annotation>

</xs:enumeration>

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| TSI</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| TSI</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="2">

<xs:documentation>OBD and

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="3"> |

<xs:annotation>

<xs:documentation>Opacity Only</xs:documentation>

<xs:documentation>Opacity and OBD</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="5">

<xs:annotation>

<xs:documentation>OBD, TSI, and Opacity</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="MonitorResultCodeType">

<xs:annotation>

<xs:documentation>An enumeration for MonitorResultCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation>Ready</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="U">

<xs:annotation>

<xs:documentation>Unsupported</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="X">

<xs:annotation>

<xs:documentation>Not ready</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="MVCRegCodeType">

<xs:annotation>

<xs:documentation>An enumeration for MVCRegCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:integer">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 1</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 1</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="1">

<xs:documentation>Passenger WC

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="2"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 2</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 2</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Passenger WC

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="3"> |

<xs:annotation>

<xs:documentation>Passenger WC 3</xs:documentation>

<xs:documentation>Passenger WC 4</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="5"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 5</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 5</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Passenger WC

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="6"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 6</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 6</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Passenger WC

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="7"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 7</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 7</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Passenger WC

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="8"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 8</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 8</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Passenger WC

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="9"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Van</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Van</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commuter

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="11"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="12"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicles</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>No Fee

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="13"> |

<xs:annotation>

<xs:documentation>Taxi</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="14">

<xs:annotation>

<xs:documentation>Limousine</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="15">

<xs:annotation>

<xs:documentation>Pass at Comm

Fee</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="16"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Veh-Aggregate</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Veh-Aggregate</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Comm

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="17"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 1</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 1</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>School Vehicle

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="18"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 2</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 2</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>School Vehicle

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="19"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Trailer</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Trailer</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>4 Year Comm

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="20"> |

<xs:annotation>

<xs:documentation>Bogey</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="21"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| trailer</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| trailer</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="22"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Trailer</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Trailer</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Private Utility

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="23"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Trailer</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Trailer</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Special

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="24"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Motorcycle</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Motorcycle</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Commercial

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="31"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Truck</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Truck</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Farm

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="32"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Truck</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Truck</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Light Duty Tow

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="33"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Truck</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Truck</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Heavy Duty Tow

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="39"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Disposal</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Disposal</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Solid Waste

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="41"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicle</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicle</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Constructor

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="51"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Use</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Use</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Farm

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="52"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Tractor</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Tractor</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Agricultural

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="53"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Motorcycle</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="54"> |

<xs:annotation>

<xs:documentation>Omnibus</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="56"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Equipment</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Equipment</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Contr

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="58"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Worker</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Worker</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Migrant Farm

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="61"> |

<xs:annotation>

<xs:documentation>Moped</xs:documentation>

</xs:annotation>

</xs:enumeration>

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicle</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicle</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="64">

<xs:documentation>Historic

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="73"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 73</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 73</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Chapter

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="81"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Pleasure</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Pleasure</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Boat

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="82"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Livery</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Livery</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Boat

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="84"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Manufacturer</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Manufacturer</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Boat

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="85"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Commercial</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Commercial</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Boat

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="87"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Res</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Res</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Snowmobile

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="88"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Res</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Res</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Snowmobile Non

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="89"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Resident</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Resident</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>ATV

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="90"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Non-Resident</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Non-Resident</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>ATV

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="99"> |

<xs:annotation>

<xs:documentation>Farm Labor

Transport</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="MVCVehicleTypeCodeType">

<xs:annotation>

<xs:documentation>An enumeration for MVCVehicleTypeCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:integer">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Wagon</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Wagon</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="1">

<xs:documentation>Station

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="2"> |

<xs:annotation>

<xs:documentation>2 Door</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>Convertible</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="4">

<xs:annotation>

<xs:documentation>4 Door</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="5"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Home</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Home</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Motor

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="6"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Limousine</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="7"> |

<xs:annotation>

<xs:documentation>Truck</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="8">

<xs:annotation>

<xs:documentation>Van</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="9"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Truck</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Truck</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Pickup

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="10"> |

<xs:annotation>

<xs:documentation>Semi-Tractor</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="11"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Tractor</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Tractor</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Farm

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="12"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Tractor</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Tractor</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Titled Farm

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="13"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicle</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Vehicle</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Non-Conventional

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="14"> |

<xs:annotation>

<xs:documentation>Omnibus</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="15">

<xs:annotation>

<xs:documentation>Bogey</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="16">

<xs:annotation>

<xs:documentation>Trailer</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="17"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Trailer</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Trailer</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Non-Titled

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="18"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Home</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Home</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Mobile

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="19"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Motorcycle</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="20"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Motorcycle</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Motorcycle</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Non-Titled

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="21"> |

<xs:annotation>

<xs:documentation>Moped</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="22"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Van</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Van</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Pass

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="23"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Snowmobile</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="24"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Equip</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Equip</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Farm Use

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="25"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Driller</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Driller</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Well

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="26"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>All-Terrain</xs:documentation> </xs:annotation> |  |  |  |
|  |  |  | </xs:enumeration> |
|  |  |  | <xs:enumeration value="31"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Boat</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Boat</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Open

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="32"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Boat</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Boat</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Cabin

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="33"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Boat</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Boat</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>House

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="34"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Boat</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Boat</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Sail

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="35"> |

| Boat</xs:documentation> |  |  |  |
| --- | --- | --- | --- |

| Boat</xs:documentation> |  |  |  |
| --- | --- | --- | --- |

<xs:annotation>

<xs:documentation>Auxiliary

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="36">

<xs:annotation>

<xs:documentation>Canoe</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="37">

<xs:annotation>

<xs:documentation>Other</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="38">

<xs:annotation>

<xs:documentation>Other</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="98">

<xs:annotation>

<xs:documentation>Vehicle</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="99">

<xs:annotation>

<xs:documentation>Vehicle</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ObdProtocolCodeType">

<xs:annotation>

<xs:documentation>An enumeration for ObdProtocolCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="00">

<xs:annotation>

<xs:documentation>None</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="01"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| requested</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| requested</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>init

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="02"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| VPW</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| VPW</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>SAE J1850 10.4K

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="03"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| PWM</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| PWM</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>SAE J1850 41.6K

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="04"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 9141-2</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 9141-2</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>ISO

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="05"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 2000)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| 2000)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>ISO 14230-4 (Keyword

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="06"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| bit header)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| bit header)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>ISO 15765-4 CAN 500K (11

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="07"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| bit header)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| bit header)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>ISO 15765-4 CAN 500K (29

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="08"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| bit header)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| bit header)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>ISO 15765-4 CAN 250K (11

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="09"> |

<xs:annotation>

<xs:documentation>ISO 15765-4 CAN 250K (29 bit header)</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="PrimaryEmTestCodeType">

<xs:annotation>

<xs:documentation>An enumeration for PrimaryEmTestCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Test</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Test</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="D">

<xs:documentation>Diesel Opacity

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="I"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Test</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Test</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Idle

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="N"> |

<xs:annotation>

<xs:documentation>No test</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="O"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Test</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Test</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>OBD

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="T"> |

<xs:annotation>

<xs:documentation>TSI

Test</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="ReferenceDataFilenameType">

<xs:annotation>

<xs:documentation>An enumeration for ReferenceDataFilenameType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SafetyResultCodeType">

<xs:annotation>

<xs:documentation>An enumeration for SafetyResultCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Fail</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Fail</xs:documentation> </xs:annotation> |  |  |  |

<xs:enumeration value="3">

<xs:documentation>30-day

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="4"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Fail</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Fail</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>48-hour

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="F"> |

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="N"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| required</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| required</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>not

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="P"> |

<xs:annotation>

<xs:documentation>Pass</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="R"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Pass</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Pass</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Repaired to

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="S"> |
|  |  |  | <xs:annotation> |

<xs:documentation>Out-of-Service</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="X">

<xs:annotation>

<xs:documentation>Fail, not performed</xs:documentation>

</xs:annotation>

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="Z"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Already</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Already</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Passed

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="A"> |
|  |  |  | <xs:annotation> |
| <xs:documentation>Advisory</xs:documentation> |  |  |  |
|  |  | </xs:annotation> |  |
|  |  | </xs:enumeration> |  |

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="VinSourceType">

<xs:annotation>

<xs:documentation>An enumeration for VinSourceType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| (Reg | or VIR)</xs:documentation> </xs:annotation> |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| (Reg | or VIR)</xs:documentation> </xs:annotation> |  |  |

<xs:enumeration value="B">

<xs:documentation>Scanned Document Barcode

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="M"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Entry</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| Entry</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Manual

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="S"> |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| (deprecated)</xs:documentation> </xs:annotation> |  |  |  |

|  |  |  | <xs:annotation> |
| --- | --- | --- | --- |
| (deprecated)</xs:documentation> </xs:annotation> |  |  |  |

<xs:documentation>Scanned Windshield Sticker

|  |  |  | </xs:enumeration> |
| --- | --- | --- | --- |
|  |  |  | <xs:enumeration value="V"> |

Vehicle</xs:documentation>

<xs:annotation>

<xs:documentation>Scanned

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="AuthenticatedUserIdType">

<xs:restriction base="xs:string">

<xs:pattern value="INL[0-9]{6}|AUD[0-9]{6}"/>

</xs:restriction>

</xs:simpleType>

|  |  | <xs:simpleType name="AuthenticatedUserTypeType"> |
| --- | --- | --- |
|  |  | <xs:restriction base="xs:string"> |
|  |  | <xs:enumeration value="I"> |

<xs:annotation><xs:documentation>Inspector</xs:documentation></xs:annotation>

|  |  |  | </xs:enumeration> <xs:enumeration value="A"> |
| --- | --- | --- | --- |

<xs:annotation><xs:documentation>Auditor</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="SnapTriggerCodeType">

<xs:annotation>

<xs:documentation>An enumeration for SnapTriggerCodeType</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="A">

<xs:annotation>

<xs:documentation> Acoustic trigger </xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="O">

<xs:annotation>

<xs:documentation> Opacity trigger </xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="T">

<xs:annotation>

<xs:documentation> Time trigger </xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

</xs:schema>

diesel.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:include schemaLocation="inspectionConditions.xsd"	/>

<xs:element name="DieselRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="DieselResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="DieselVersion" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="Condition" type="InspectionConditionType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

endOfDayReport.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:element name="persistEndOfDayReportRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="Report" type="EODReportType" minOccurs="1" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="persistEndOfDayReportResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BasePersistResponse" />

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="EODReportType">

<xs:sequence>

<xs:element name="ReportingDate" type="xs:date" minOccurs="1" maxOccurs="1"/>

<xs:element name="WorkstationID" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="Total"	type="xs:integer" minOccurs="1" maxOccurs="1"/>

<xs:element name="TotalPass"	type="xs:integer" minOccurs="1" maxOccurs="1"/>

<xs:element name="TotalFail"	type="xs:integer" minOccurs="1" maxOccurs="1"/>

<xs:element name="TotalPassByStickerYear" type="PassInspectionByStickerYearType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="TotalFailByFailureType" type="TotalFailByFailureType"	minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="PassInspectionByStickerYearType">

<xs:attribute name="year"	type="xs:gYear"	use="required"/>

<xs:attribute name="totalPass" type="xs:integer" use="required"/>

</xs:complexType>

<xs:complexType name="TotalFailByFailureType">

<xs:attribute name="type"	type="SafetyResultCodeType" use="required"/>

<xs:attribute name="totalFail" type="xs:integer" use="required"/>

</xs:complexType>

</xs:schema>

file.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:element name="FileRequest" >

<xs:complexType>

<xs:complexContent>

| minOccurs="0" maxOccurs="unbounded"> |  |  |  |  |
| --- | --- | --- | --- | --- |

| minOccurs="0" maxOccurs="unbounded"> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="File"

| name="filename" type="xs:string"/> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| name="filename" type="xs:string"/> |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

<xs:complexType>

<xs:attribute

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="FileResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="File" minOccurs="0" maxOccurs="unbounded">

<xs:complexType>

<xs:simpleContent >

<xs:extension base="xs:base64Binary">

<xs:attribute name="filename" type="xs:string" />

<xs:attribute name="version" type="xs:string"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

fuelCodes.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"/>

<xs:element name="FuelCodesRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="FuelCodesResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="FuelCodesVersion" type="xs:string"		minOccurs="1" maxOccurs="1"/>

<xs:element name="CodeToType" type="FuelCodeToFuelCodeTypeType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="FuelCodeToFuelCodeTypeType">

<xs:sequence>

<xs:element name="FuelCodeDescription" type="xs:string"/>

<xs:element name="FuelType" type="FuelType"/>

<xs:element name="FuelCode" type="FuelCodeType"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd" xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd" />

<xs:element name="persistInspectionResponse">

<xs:complexType>

<xs:complexContent>

<xs:extension base="BasePersistInspectionResponse" />

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="persistInspectionRequest">

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="Inspection" type="InspectionType" minOccurs="1" maxOccurs="1" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="InspectionType">

<xs:sequence>

<xs:element name="TestRecordNumber"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="FacilityData"	type="FacilityDataType" minOccurs="0" maxOccurs="1"/>

<xs:element name="LocationData"	type="LocationDataType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VehicleData"	type="VehicleDataType" minOccurs="0" maxOccurs="1"/>

<xs:element name="RegData"	type="RegDataType" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspectionParms"	type="InspectionParmsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspectionResults" type="InspectionResultsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="WaitTime"	type="WaitTimeType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="RepairData"	type="RepairDataType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="BusData"	type="BusDataType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

<xs:attribute name="isVoidTestRecord" type="xs:boolean" use="optional" default="false"/>

</xs:complexType>

<xs:complexType name="BusDataType">

<xs:sequence>

<xs:element name="TerminalId"	type="String10" minOccurs="0" maxOccurs="1"/>

<xs:element name="TerminalAddress"	type="AddressType" minOccurs="0" maxOccurs="1"/>

<xs:element name="CompanyName"	type="String50" minOccurs="0" maxOccurs="1"/>

<xs:element name="BusNumber"	type="String10" minOccurs="0" maxOccurs="1"/>

<xs:element name="ApprovalNumber"	type="String12" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="AddressType">

<xs:sequence>

<xs:element name="StreetAddress"	type="String50" minOccurs="0" maxOccurs="1"/>

<xs:element name="City"	type="String35" minOccurs="0" maxOccurs="1"/>

<xs:element name="State"	type="String2" minOccurs="0" maxOccurs="1"/>

<xs:element name="Zip"	type="String10" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="RegDataType">

<xs:sequence>

<xs:element name="PlateNumber"	type="PlateNumberType" minOccurs="0" maxOccurs="1"/>

<xs:element name="RegIssueDate"	type="xs:date" minOccurs="0" maxOccurs="1"/>

<xs:element name="NJ1Indicator"	type="YesNoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="MVCRegCode"	type="MVCRegCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="MVCVehicleTypeCode"	type="MVCVehicleTypeCodeType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="RepairDataType">

<xs:sequence>

<xs:element name="RepairFacilityLicenseId" type="RepairFacilityIdType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The emission repair facility license id that the repair was performed at or 'SELF'</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RepairTechId" type="RepairTechIdType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>Emission Repair Technician (ERT) ID</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RepairConfirmationID" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>

A unique repair confirmation number (generated by system at repair entry time) printed on the	Repair result form given

to motorist at the ERF.	This is not applicable for self-repairs.

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RepairDate" type="xs:date" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>

The date the repair technician performed the repair. Not applicable for self-repairs.

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="RepairOrderId" type="xs:string" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>

The work order number provided by the ERF. Not applicable for self-repairs.

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="WaitTimeType">

<xs:choice>

<xs:element name="WaitTimeValues" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="WaitDuration"	type="xs:duration" minOccurs="0" maxOccurs="1"/>

<xs:element name="WaitStartTime" type="xs:dateTime"	minOccurs="0" maxOccurs="1"/>

<xs:element name="WaitEndTime"	type="xs:dateTime"	minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="WaitTimeExceptions" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="WaitTicketScannedCode" type="YesNoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="WaitSystemCode"	type="WaitSystemCodeType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:choice>

</xs:complexType>

<xs:complexType name="LocationDataType">

<xs:sequence>

<xs:element name="LocationId"	type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="LocationDescription" type="String200" minOccurs="0" maxOccurs="1"/>

<xs:element name="CountyCode"	type="String2"	minOccurs="0" maxOccurs="1"/>

<xs:element name="CountyName"	type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="MunicipalityCode"	type="String4"	minOccurs="0" maxOccurs="1"/>

<xs:element name="MunicipalityName"	type="String200" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="FacilityDataType">

<xs:sequence>

<xs:element name="FacilityLicenseId"	type="FacilityIdType"

<xs:element name="PIFClassCode"	type="PIFClassCodeType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="VehicleDataType">

<xs:sequence>

<xs:element name="VehicleID"	type="VehicleIdType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VehicleSource"	type="String1" minOccurs="0" maxOccurs="1"/>

<xs:element name="VinData"	type="VinDataType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VehiclePhysicalParms" type="VehiclePhysicalParmsType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="VinDataType">

<xs:sequence>

<xs:element name="VinDocument" type="VinDocumentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VinObd"	type="VinObdType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="VinVehicle"	type="VinVehicleType"	minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="VehiclePhysicalParmsType">

<xs:sequence>

<xs:element name="VehicleYear"	type="VehicleYearType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VehicleMake"	type="MakeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VehicleModel"	type="ModelType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VehicleGVWR"	type="VehicleGVWRType" minOccurs="0" maxOccurs="1"/>

<xs:element name="FuelCode"	type="FuelCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="EPAMobile6Code"	type="EPAMobile6CodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="Displacement"	type="Decimal5Dot1Type" minOccurs="0" maxOccurs="1"/>

<xs:element name="DisplUnitCode"	type="DisplUnitCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="CylindersCode"	type="CylindersCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="TransmissionCode"	type="TransmissionCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="DualExhaust"	type="YesNoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="EngineYear"	type="VehicleYearType" minOccurs="0" maxOccurs="1"/>

<xs:element name="EngineMake"	type="String30" minOccurs="0" maxOccurs="1"/>

<xs:element name="EngineHP"	type="Decimal5Dot1Type" minOccurs="0" maxOccurs="1"/>

<xs:element name="StackSize"	type="StackSizeIntegerType" minOccurs="0" maxOccurs="1"/>

<xs:element name="StackMeasured"	type="xs:string"

<xs:element name="AspirationCode"	type="AspirationCodeType" minOccurs="0" maxOccurs="7"/>

<xs:element name="NumberOfPassengers"	type="Integer4DigitType" minOccurs="0" maxOccurs="1"/>

<xs:element name="RetrofitPresent"	type="YesNoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="EngineSpeed"	type="EngineSpeedType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="InspectionParmsType">

<xs:sequence>

<xs:element name="CatDTCCode"	type="CatDTCCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="CertificateNumber"	type="String21" minOccurs="0" maxOccurs="1"/>

<xs:element name="SystemUID"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="PreviousSystemUID"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspDate"	type="xs:date" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspStartTime"	type="xs:dateTime" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspEndTime"	type="xs:dateTime" minOccurs="0" maxOccurs="1"/>

<xs:element name="Task"	type="TaskType" minOccurs="0" maxOccurs="4"/>

<xs:element name="InspTypeCode"	type="InspTypeCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspCycleTypeCode"	type="InspCycleTypeCode" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspModeCode"	type="InspModeCode" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspExtentCode"	type="InspExtentCode" minOccurs="0" maxOccurs="1"/>

<xs:element name="OldStickerDate"	type="xs:date" minOccurs="0" maxOccurs="1"/>

<xs:element name="OdometerReading"	type="OdometerReadingType" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspComplianceDate"	type="xs:date" minOccurs="0" maxOccurs="1"/>

<xs:element name="InspExpirationDate"	type="xs:date" minOccurs="0" maxOccurs="1"/>

<xs:element name="EmComplianceDate"	type="xs:date" minOccurs="0" maxOccurs="1"/>

<xs:element name="EmExpirationDate"	type="xs:date"

<xs:element name="CommercialIndicator"	type="String1" minOccurs="0" maxOccurs="1"/>

<xs:element name="Omni2Indicator"	type="String1" minOccurs="0" maxOccurs="1"/>

<xs:element name="TransportIndicator"	type="String1" minOccurs="0" maxOccurs="1"/>

<xs:element name="VehicleConditionOK"	type="String1" minOccurs="0" maxOccurs="1"/>

<xs:element name="ExceptionParms"	type="ExceptionParmsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="ConfigVersion"	type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="P5VersionNumber"	type="String15" minOccurs="0" maxOccurs="1"/>

<xs:element name="VinMatchFlag"	type="VinMatchFlagType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>

This should mirror the value received in the Begin test message.

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="PlateMatchFlag"	type="PlateMatchFlagType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>

This should mirror the value received in the Begin test message.

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="ExceptionParmsType">

<xs:sequence>

<xs:element name="ExemptFromOBD"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="ExemptFromTSI"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="ExemptFromOpacity"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="ExemptFromLift"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="ExemptFromReadiness"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="RequiresRollTest"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="CVNExclusion"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="CATRetestExclusion"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="BypassOBDAllowed"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="RPMExclusion"	type="YesNoType" minOccurs="0" maxOccurs="1" />

<xs:element name="ContinuousMonitorExclusion" type="YesNoType" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="TaskType">

<xs:sequence>

<xs:element name="InspectorData" type="InspectorDataType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>The id of the inspector that performed this task</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="TaskStartTime"	type="xs:dateTime"	minOccurs="0" maxOccurs="1"/>

<xs:element name="TaskEndTime"	type="xs:dateTime"	minOccurs="0" maxOccurs="1"/>

</xs:sequence>

<xs:attribute name="workstationId"	type="WorkstationIdType"	use="optional"/>

<xs:attribute name="softwareVersion" type="SoftwareVersionType" use="optional"/>

<xs:attribute name="configVersion"	type="ConfigVersionType"	use="optional"/>

</xs:complexType>

<xs:simpleType name="ChangeICDCodeType">

<xs:annotation>

<xs:documentation>

An enumeration for ChangeICDCodeType

</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="E">

<xs:annotation>

<xs:documentation>use Insp Exp Date</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="F">

<xs:annotation>

<xs:documentation>

Foreign vehicle (OIT only)

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>

Mobile (Roadside) test occurred

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="N">

<xs:annotation>

<xs:documentation>

No (already changed this cycle)

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation> Registration (OIT only)

</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="T">

<xs:annotation>

<xs:documentation>Title Change (OIT only)</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="Y">

<xs:annotation>

<xs:documentation>Yes</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="InspCycleTypeCode">

<xs:restriction base="xs:string">

<xs:enumeration value="I">

<xs:annotation>

<xs:documentation>Initial Inspection</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation>Inspection Retest</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="C">

<xs:annotation>

<xs:documentation>Courtesy Inspection</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="InspModeCode">

<xs:restriction base="xs:string">

<xs:enumeration value="C">

<xs:annotation>

<xs:documentation>Cyclical</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="O">

<xs:annotation>

<xs:documentation>Off-cycle</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="InspExtentCode">

<xs:restriction base="xs:string">

<xs:enumeration value="F">

<xs:annotation>

<xs:documentation>Full Inspection</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="P">

<xs:annotation>

<xs:documentation>Partial Inspection</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="InspectionResultsType">

<xs:sequence>

<xs:element name="OverallResult"	type="PassFailCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SafetyDetails" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:choice>

<xs:element name="IMSafetyDetails"	type="IMSafetyDetailsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="CollectorVehicleDetails" type="CollectorVehicleDetailsType" minOccurs="0" maxOccurs="1"/>

</xs:choice>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="CredentialsDetails"	type="CredentialsDetailsType" minOccurs="0" maxOccurs="1" />

<xs:element name="EmissionDetails"	type="EmissionDetailsType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="CredentialsDetailsType">

<xs:sequence>

<xs:element name="OverallCredentialsResult" type="CredentialsResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SafetyCondition"	type="xs:integer" minOccurs="0" maxOccurs="10" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="CollectorVehicleDetailsType">

<xs:sequence>

<xs:element name="CollectorVehicleStatus"	type="CollectorVehicleStatusType" minOccurs="0" maxOccurs="1"/>

<xs:element name="CollectorVehicleVoucherDate"	type="xs:date" minOccurs="0" maxOccurs="1"/>

<xs:element name="OdometerReading"	type="OdometerReadingType" minOccurs="0" maxOccurs="1"/>

<xs:element name="PrevOdometerReading"	type="PrevOdometerReadingType" minOccurs="0" maxOccurs="1"/>

<xs:element name="PrevInspDate"	type="xs:date" minOccurs="0" maxOccurs="1"/>

<xs:element name="Qualified"	type="YesNoType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="IMSafetyDetailsType">

<xs:sequence>

<xs:element name="OverallSafetyResult"	type="OverallSafetyResultType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SafetyConditionChanged"	type="YesNoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="BrakeResult"	type="SafetyResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="ExhaustSystemResult"	type="SafetyResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SafetyEquipmentResult"	type="SafetyResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SteeringSuspensionResult"	type="SafetyResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="MiscSafetyRejectionComment" type="String200" minOccurs="0" maxOccurs="1"/>

<xs:element name="MiscSafetyRejectionResult"	type="SafetyResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SafetyClarification"	type="String200" minOccurs="0" maxOccurs="1"/>

<xs:element name="SafetyCondition"	type="xs:integer" minOccurs="0" maxOccurs="30" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="EmissionDetailsType">

<xs:sequence>

<xs:element name="OverallEmResult"	type="OverallEmResultCodeType" minOccurs="0" maxOccurs="1" />

<xs:element name="PrimaryEmTestCode"	type="PrimaryEmTestCodeType" minOccurs="0" maxOccurs="1" />

<xs:element name="PrimaryEmDetails"	type="PrimaryEmDetailsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="GasCapDetails"	type="GasCapDetailsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="VisibleSmokeResult"	type="EmResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="LiquidLeakResult"	type="EmResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="TamperDetails"	type="TamperDetailsType" minOccurs="0"	maxOccurs="1" />

<xs:element name="IndicatorLightDetails"	type="IndicatorLightDetailsType" minOccurs="0" maxOccurs="1" />

<xs:element name="MiscEmRejectionComment" type="String200" minOccurs="0" maxOccurs="1"/>

<xs:element name="MiscEmRejectionResult"	type="EmResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="EmClarification"	type="String200" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

<xs:attribute name="tsiBypass"	type="xs:boolean" use="optional"/>

<xs:attribute name="obdBypass"	type="xs:boolean" use="optional"/>

<xs:attribute name="opacityBypass" type="xs:boolean" use="optional"/>

</xs:complexType>

<!--

=======================================================

|

| Primary Emission Test Types

|

+=======================================================

-->

<xs:complexType name="PrimaryEmDetailsType">

<xs:sequence>

<xs:element name="PrimaryEmResult"	type="EmResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="PrimaryEm" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:choice>

<xs:element name="DieselEmDetails" type="DieselEmDetailsType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="IdleEmDetails"	type="IdleEmDetailsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="OBDEmDetails"	type="OBDEmDetailsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="TSIEmDetails"	type="TSIEmDetailsType" minOccurs="0" maxOccurs="1"/>

</xs:choice>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="SnapOpacityDetailsType">

<xs:sequence>

<xs:element name="SnapOpacityReading"	type="Decimal4Dot1Type" minOccurs="0" maxOccurs="1"/>

<xs:element name="SnapTriggerCode"	type="SnapTriggerCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SnapTriggerLevel"	type="Integer3DigitType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="DieselEmDetailsType">

<xs:sequence>

<xs:element name="OpacityTestType"	type="OpacityTestType" minOccurs="0" maxOccurs="1"/>

<xs:element name="OpacityCutpoint"	type="Integer2DigitType" minOccurs="0" maxOccurs="1"/>

<xs:element name="OpacityReading"	type="Decimal4Dot1Type" minOccurs="0" maxOccurs="1"/>

<xs:element name="StackSizeMeasured" type="StackSizeMeasuredType" minOccurs="0" maxOccurs="1"/>

<xs:element name="StackSizeUsed"	type="StackSizeIntegerType" minOccurs="0" maxOccurs="1"/>

<xs:element name="OilTemp"	type="Integer3DigitType" minOccurs="0" maxOccurs="1"/>

<xs:element name="OpacityFastPassCode"	type="YesNoType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SnapOpacityDetails"	type="SnapOpacityDetailsType" minOccurs="0" maxOccurs="9"/>

<xs:element name="IdleBaseLineAcousticLevel"	type="Integer3DigitType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SnapAcousticTriggerLevel"	type="Integer3DigitType" minOccurs="0" maxOccurs="1"/>

<xs:element name="PostTestDrift"	type="Decimal4Dot1Type" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="IdleEmDetailsType">

<xs:sequence>

<xs:element name="IdleTestFinal"	type="TSITestReadingType"	minOccurs="0" maxOccurs="1" />

<xs:element name="IdleTestFirst"	type="TSITestReadingType"	minOccurs="0" maxOccurs="1" />

</xs:sequence>

<xs:attribute name="rpmBypassCode" type="YesNoType"	use="optional"/>

</xs:complexType>

<xs:complexType name="OBDEmDetailsType">

<xs:sequence>

<xs:element name="ReadinessMonitorDetails" type="ReadinessMonitorDetailsType" minOccurs="0" maxOccurs="1"/>

<xs:element name="UnsetMonitorCount"	type="Integer2DigitType" minOccurs="0" maxOccurs="1"/>

<xs:element name="ReadinessResult"	type="ReadinessResultType" minOccurs="0" maxOccurs="1"/>

<xs:element name="DLCResultCode"	type="DLCResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdCommResultCode"	type="PassFailCodeType"

minOccurs="0" maxOccurs="1"/>

<xs:element name="MilCommandOnResultCode"	type="PassFailCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="DTCCode"	type="DTCCodeType" minOccurs="0" maxOccurs="10" />

<xs:element name="DTCStoredCount"	type="Integer2DigitType" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdProtocolCode"	type="ObdProtocolCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdModuleID1"	type="TwoDigitHex" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdModuleID2"	type="TwoDigitHex" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdModuleID3"	type="TwoDigitHex" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdPID00"	type="String30" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdPID20"	type="String30" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdPID40"	type="String30" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdPIDCount"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdRPM"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdCalID"	type="String30" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdCVN"	type="String30" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdType"	type="TwoDigitHex" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdMILOnDistance"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdMILOnTime"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdWarmUps"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdCodeClearedDistance"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

<xs:element name="ObdCodeClearedTime"	type="xs:integer" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="TwoDigitHex">

<xs:restriction base="xs:string">

<xs:pattern value="[0-9A-F]{0,2}"></xs:pattern>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="TSIEmDetailsType">

<xs:sequence>

<xs:element name="IdleTestFinal"	type="TSITestReadingType" minOccurs="0" maxOccurs="1" />

<xs:element name="IdleTestFirst"	type="TSITestReadingType" minOccurs="0" maxOccurs="1" />

<xs:element name="HighIdleTestFinal" type="TSITestReadingType" minOccurs="0" maxOccurs="1" />

<xs:element name="HighIdleTestFirst" type="TSITestReadingType" minOccurs="0" maxOccurs="1" />

</xs:sequence>

<xs:attribute name="rpmBypassCode" type="YesNoType" use="optional"/>

</xs:complexType>

<xs:complexType name="TSITestReadingType">

<xs:sequence>

<xs:element name="TailpipeResult"	type="EmResultCodeType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="Cutpoints">

<xs:complexType>

<xs:sequence>

<xs:element name="CutpointHC" type="Integer4DigitType" minOccurs="0" maxOccurs="1"/>

<xs:element name="CutpointCO" type="Decimal4Dot2Type"	minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="CutpointIndicator" type="CutpointIndicatorType" minOccurs="0" maxOccurs="1"/>

<xs:element name="DilutionCorrectionFactor" type="Decimal3Dot2Type" minOccurs="0" maxOccurs="1"/>

<xs:element name="GasReadings">

<xs:complexType>

<xs:sequence>

<xs:element name="CO2Rdg" type="Decimal3Dot1Type"	minOccurs="0" maxOccurs="1"/>

<xs:element name="CORdg"	type="Decimal4Dot2Type"	minOccurs="0" maxOccurs="1"/>

<xs:element name="HCRdg"	type="Integer4DigitType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="O2Rdg"	type="Decimal3Dot1Type"	minOccurs="0" maxOccurs="1"/>

<xs:element name="RPMRdg" type="Integer4DigitType"	minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

<!--

=======================================================

|

| Secondary Emission Test Types

|

+=======================================================

-->

<xs:complexType name="GasCapDetailsType">

<xs:sequence>

<xs:element name="GasCapResult"	type="EmResultCodeType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="FlowRate"	type="GasCapLeakRateType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

<xs:attribute name="adapterAvailable" type="YesNoType" use="optional" default="Y" />

<xs:attribute name="replacedCode"	type="YesNoType" use="optional" default="N" />

</xs:complexType>

<xs:complexType name="TamperDetailsType">

<xs:sequence>

<xs:element name="TamperResult"	type="EmResultCodeType" minOccurs="0" maxOccurs="1" />

<xs:element name="TamperItemDetails" type="TamperItemDetailsType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="TamperItemDetailsType">

<xs:sequence>

<xs:element name="TamperItemCode"	type="TamperItemCodeType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="TamperItemResult" type="EmResultCodeType"	minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="TamperItemCodeType">

<xs:restriction base="xs:string">

<xs:enumeration value="C">

<xs:annotation>

<xs:documentation>Catalytic Converter</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation>Diesel Retrofit</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="D">

<xs:annotation>

<xs:documentation>Diesel DPF System</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="N">

<xs:annotation>

<xs:documentation>Diesel NOx Control System</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="IndicatorLightDetailsType">

<xs:sequence>

<xs:element name="IndicatorLightResult" type="EmResultCodeType" minOccurs="0" maxOccurs="1" />

<xs:element name="IndicatorLightItemDetails" type="IndicatorLightItemDetailsType"	minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="IndicatorLightItemDetailsType">

<xs:sequence>

<xs:element name="IndicatorLightItemCode" type="IndicatorLightItemCodeType" minOccurs="1" maxOccurs="1" />

<xs:element name="LightCheckResult"	type="PassFailCodeType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>

Pass: Indicator light is On when Key On Engine Off.

</xs:documentation>

</xs:annotation>

</xs:element>

<xs:element name="LightOnResult"	type="PassFailCodeType" minOccurs="0" maxOccurs="1">

<xs:annotation>

<xs:documentation>

Pass: Indicator light is Off when Engine On.

</xs:documentation>

</xs:annotation>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="IndicatorLightItemCodeType">

<xs:restriction base="xs:string">

<xs:enumeration value="D">

<xs:annotation>

<xs:documentation>Diesel DPF</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>OBD MIL</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="N">

<xs:annotation>

<xs:documentation>NOx Indicator</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="CutpointIndicatorType">

<xs:restriction base="xs:string">

<xs:enumeration value="F">

<xs:annotation>

<xs:documentation>Fast Fail</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="P">

<xs:annotation>

<xs:documentation>Fast Pass</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation>Regular</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="StackSizeMeasuredType">

<xs:restriction base="xs:decimal">

<xs:minInclusive value="3" />

<xs:maxInclusive value="7" />

</xs:restriction>

</xs:simpleType>

<xs:complexType name="ReadinessMonitorDetailsType">

<xs:sequence>

<xs:element name="AirConditioningResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="CatalystResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="CompComponentResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="EgrSystemResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="FuelSystemResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="HeatedCatalystResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="MisfireResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="OxygenSensorHeaterResult" type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="OxygenSensorResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="SecondaryAirSystemResult" type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

<xs:element name="EvapSystemResult"	type="MonitorResultCodeType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="CylindersCodeType">

<xs:choice>

<xs:element name="NumOfCylinders" type="Integer0To16Type" minOccurs="0" maxOccurs="1"/>

<xs:element name="OtherEngineCode"	type="OtherEngineCodeType" minOccurs="0" maxOccurs="1"/>

</xs:choice>

</xs:complexType>

<xs:simpleType name="EngineSpeedType">

<xs:annotation>

<xs:documentation> Diesel engine speed.

</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="L">

<xs:annotation>

<xs:documentation>Low speed diesel engine</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="M">

<xs:annotation>

<xs:documentation>Medium	speed diesel engine</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="H">

<xs:annotation>

<xs:documentation>High speed diesel engine</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="OpacityTestType">

<xs:annotation>

<xs:documentation> Opacity test method.

</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:string">

<xs:enumeration value="S">

<xs:annotation>

<xs:documentation>Snap Acceleration Test</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="P">

<xs:annotation>

<xs:documentation>Power Brake Test</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="R">

<xs:annotation>

<xs:documentation>Rolling Acceleration Test</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:simpleType name="StackSizeIntegerType">

<xs:restriction	base="xs:int">

<xs:minInclusive value="3" />

<xs:maxInclusive value="7" />

</xs:restriction>

</xs:simpleType>

</xs:schema>

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:element name="InspectionConditionsRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="InspectionConditionsResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="InspectionConditionsVersion" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="Condition" type="InspectionConditionType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="InspectionConditionType">

<xs:sequence>

<xs:element name="DisplaySequence">

|  |  |  | <xs:simpleType> |
| --- | --- | --- | --- |
|  |  |  | <xs:restriction base="xs:int"> |

<xs:maxInclusive value="999"/>

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="SafetyCheckCategoryCode">

|  |  |  |  | </xs:simpleType> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:simpleType> |
| --- | --- | --- | --- | --- |

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:length value="3"></xs:length>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="ConditionNumber"> |

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="ConditionNumber"> |

</xs:restriction>

|  |  |  | <xs:simpleType> |
| --- | --- | --- | --- |
|  |  |  | <xs:restriction base="xs:int"> |

<xs:maxInclusive value="9999"/>

<xs:minInclusive value="0"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="ConditionDescription">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:maxLength value="50"/>

</xs:restriction>

|  |  |  | </xs:simpleType> |
| --- | --- | --- | --- |
|  |  |  | </xs:element> |
|  |  |  | <xs:element name="ConditionResult" |

type="ConditionResultType"/>

</xs:sequence>

<xs:attribute name="id">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:maxLength value="9"/>

</xs:restriction>

</xs:simpleType>

</xs:attribute>

</xs:complexType>

<xs:simpleType name="ConditionResultType">

<xs:restriction base="xs:string">

<xs:enumeration value="F">

<xs:annotation>

<xs:documentation>Fail</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="A">

<xs:annotation>

<xs:documentation>Advisory</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="4">

<xs:annotation>

<xs:documentation>48-Hour</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="S">

<xs:annotation>

<xs:documentation>Out-of-Service</xs:documentation>

</xs:annotation>

</xs:enumeration>

<xs:enumeration value="3">

<xs:annotation>

<xs:documentation>30 day</xs:documentation>

</xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

</xs:schema>

loopback.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"/>

<xs:element name="LoopbackRequest">

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="message" nillable="false" type="xs:string" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="LoopbackResponse">

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="message" nillable="false" type="xs:string" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"/>

<xs:element name="MailRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="MailResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="Message" type="MessageType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="MessageType">

<xs:simpleContent>

<xs:extension base="xs:string">

|  |  |  |  | <xs:attribute name="messageId" type="MessageIdType" |
| --- | --- | --- | --- | --- |
| /> |  |  |  |  |
|  |  |  |  | <xs:attribute name="subject"> |
|  |  |  |  | <xs:simpleType> |

<xs:restriction base="xs:string">

<xs:maxLength value="76"/>

</xs:restriction>

</xs:simpleType>

</xs:attribute>

<xs:attribute name="createDate" type="xs:dateTime"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:element name="ConfirmMailRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="MessageId" type="MessageIdType" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="ConfirmMailResponse">

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:simpleType name="MessageIdType">

<xs:restriction base="xs:integer">

<xs:totalDigits value="10"/>

</xs:restriction>

</xs:simpleType>

</xs:schema>

nonConforming.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:element name="persistNonConformingRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="messsage" type="xs:base64Binary" minOccurs="1" maxOccurs="unbounded"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="persistNonConformingResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BasePersistResponse" />

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

obdDataRequirements.xsd

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"/>

<xs:element name="OBDDataRequirementsRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType" />

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="OBDDataRequirementsResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="OBDDataRequirementVersion" type="xs:string"	minOccurs="1" maxOccurs="1"/>

<xs:element name="OBDDataRequirements"		type="OBDDataRequirementsType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="OBDDataRequirementsType" >

<xs:sequence>

<xs:element name="ExtendedPidsStartModelYear" type="xs:integer"/>

<xs:element name="ExtendedPidsEndModelYear" type="xs:integer"/>

<xs:element name="ContinuousMonitorSupportRequired" type="YesNoType"/>

<xs:element name="VinRequiredStartModelYear" type="xs:integer"/>

<xs:element name="VinRequiredEndModelYear" type="xs:integer"/>

<xs:element name="CalidRequiredStartModelYear" type="xs:integer"/>

<xs:element name="CalidRequiredEndModelYear" type="xs:integer"/>

<xs:element name="CvnRequiredStartModelYear" type="xs:integer"/>

<xs:element name="CvnRequiredEndModelYear" type="xs:integer"/>

<xs:element name="ExtendedDataStartModelYear" type="xs:integer"/>

<xs:element name="ExtendedDataEndModelYear" type="xs:integer"/>

</xs:sequence>

<xs:attribute name="fuelType" type="FuelType"/>

</xs:complexType>

</xs:schema>

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:element name="OBDExceptionRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="OBDExceptionResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="ObdExceptionVersion" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="ObdException" type="OBDExceptionType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="OBDExceptionType">

<xs:sequence>

<xs:element name="VehicleMake" type="MakeType"		minOccurs="0" maxOccurs="1"/>

<xs:element name="VehicleModel" type="ModelType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="VehicleYear" type="xs:gYear"		minOccurs="0" maxOccurs="1"/>

<xs:element name="VINMask" type="VinMaskType" minOccurs="0" maxOccurs="1"/>

<xs:element name="ExcludedFromReadiness" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="ExcludedFromOBDIITesting" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="RPMExclusion" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="ContinuousMonitorsExclusion" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="CVNExclusion" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="CATRetestExclusion" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="BypassOBDAllowed" type="YesNoType"	minOccurs="1" maxOccurs="1"/>

</xs:sequence>

</xs:complexType>

<xs:simpleType name="VinMaskType">

<xs:restriction base="xs:string">

<xs:maxLength value="19"/>

</xs:restriction>

</xs:simpleType>

</xs:schema>

obdTestConfiguration.xsd

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"/>

<xs:element name="OBDTestConfigurationRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType" />

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="OBDTestConfigurationResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="OBDTestConfigurationVersion" type="xs:string"	minOccurs="1" maxOccurs="1"/>

<xs:element name="OBDTestConfiguration"		type="OBDTestConfigurationType" minOccurs="1" maxOccurs="1" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="OBDTestConfigurationType" >

<xs:sequence>

<xs:element name="BulbCheckRetryCount" type="xs:integer"/>

<xs:element name="BulbCheckRetryCountdownTime" type="xs:integer"/>

<xs:element name="InterfaceCommunicationsRetryCount" type="xs:integer"/>

<xs:element name="VehicleCommunicationsRetryCount" type="xs:integer"/>

<xs:element name="VehicleCommunicationsCountdownTime" type="xs:integer"/>

<xs:element name="RpmRetryCount" type="xs:integer"/>

<xs:element name="ContinuousMonitorsReadinessRetryCount" type="xs:integer"/>

<xs:element name="MILOnNoDtcsRetryCount" type="xs:integer"/>

<xs:element name="MaximumVehicleAgeForBypass" type="xs:integer"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

previousInspection.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:include schemaLocation="inspection.xsd"	/>

<xs:element name="PreviousInspectionRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:attribute name="vinDocument" type="SimpleVINType" />

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="PreviousInspectionResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="inspection" type="InspectionType" minOccurs="0" maxOccurs="1" />

</xs:sequence>

<xs:attribute name="vinDocument" type="SimpleVINType" />

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

schoolBus.xsd

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:include schemaLocation="inspectionConditions.xsd"	/>

<xs:element name="SchoolBusRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType"/>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="SchoolBusResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="SchoolBusVersion" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="Condition" type="InspectionConditionType" minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"/>

<xs:element name="VehiclePickListRequest" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="Year" type="xs:gYear" minOccurs="1" maxOccurs="1"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="VehiclePickListResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BaseResponseType">

<xs:sequence>

<xs:element name="VplVersion" type="xs:string" minOccurs="1" maxOccurs="1"/>

<xs:element name="VplYear"	type="xs:gYear" minOccurs="1" maxOccurs="1"/>

<xs:element name="VplValue" type="VehiclePickListDataType"	minOccurs="0" maxOccurs="unbounded" />

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:complexType name="VehiclePickListDataType">

<xs:sequence>

<xs:element name="VehicleMake" minOccurs="1" maxOccurs="1">

|  |  |  |  | </xs:simpleType> |
| --- | --- | --- | --- | --- |

|  |  |  |  | </xs:simpleType> |
| --- | --- | --- | --- | --- |

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:maxLength value="17"/>

</xs:restriction>

|  |  |  | </xs:element> |
| --- | --- | --- | --- |
|  |  |  | <xs:element name="VehicleModel" minOccurs="1" |
| maxOccurs="1"> <xs:simpleType> |  |  |  |

<xs:restriction base="xs:string">

<xs:maxLength value="23"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="VehicleYear"	type="xs:gYear" minOccurs="1" maxOccurs="1"/>

<xs:element name="BodyStyle"	type="xs:string" minOccurs="0" maxOccurs="1" />

<xs:element name="EngineSize"	type="xs:string" minOccurs="0"		maxOccurs="1" />

<xs:element name="EngineDisplacement" type="VplEngineDisplacementType" minOccurs="0" maxOccurs="1" />

<xs:element name="TransmissionType" type="TransmissionCodeType"	minOccurs="0" maxOccurs="1" />

<xs:element name="VplFuelType"	type="xs:string" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

<xs:simpleType name="VplEngineDisplacementType">

<xs:annotation><xs:documentation>Engine size in CC</xs:documentation></xs:annotation>

<xs:restriction base="xs:integer">

<xs:minExclusive value="0"/>

<xs:maxExclusive value="9999"/>

</xs:restriction>

</xs:simpleType>

</xs:schema>

<?xml version="1.0"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://etnj.appsolgrp.com/vid/xsd"

xmlns="http://etnj.appsolgrp.com/vid/xsd"

elementFormDefault="qualified">

<xs:include schemaLocation="common.xsd"	/>

<xs:element name="WorkstationConfigurationResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BasePersistResponse" />

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="WorkstationConfigurationRequest" >

<xs:complexType>

<xs:complexContent>

| minOccurs="0" maxOccurs="1"> |  |  |  |  |
| --- | --- | --- | --- | --- |

| minOccurs="0" maxOccurs="1"> |  |  |  |  |
| --- | --- | --- | --- | --- |

<xs:extension base="BaseRequestType">

<xs:sequence>

<xs:element name="Biometrics"

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="On">

<xs:annotation><xs:documentation>On</xs:documentation></xs:annotation>

</xs:enumeration>

<xs:enumeration value="Off">

<xs:annotation><xs:documentation>Off</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="GasCapId" type="String50"		minOccurs="0" maxOccurs="1"/>

<xs:element name="OBDTestEquipmentId"		type="String50"	minOccurs="0" maxOccurs="1"/>

<xs:element name="SmokeMeterId" type="String50"		minOccurs="0" maxOccurs="1"/>

<xs:element name="TSIWorkbenchId" type="String50"		minOccurs="0" maxOccurs="1"/>

<xs:element name="SoftwareUpdateDate"		type="xs:dateTime"	minOccurs="0" maxOccurs="1"/>

<xs:element name="SoftwareVersion" type="SoftwareVersionType"	minOccurs="0" maxOccurs="1"/>

<xs:element name="WorkstationID" type="WorkstationIdType"	minOccurs="1" maxOccurs="1"/>

<xs:element name="WorkstationStatus" type="WorkstationStatusType" minOccurs="0" maxOccurs="1"/>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:simpleType name="WorkstationStatusType">

<xs:restriction base="xs:string">

<xs:enumeration value="I">

<xs:annotation><xs:documentation>Inactive</xs:documentation></xs:annotation>

</xs:enumeration>

<xs:enumeration value="A">

<xs:annotation><xs:documentation>Active</xs:documentation></xs:annotation>

</xs:enumeration>

</xs:restriction>

</xs:simpleType>

<xs:element name="persistWorkstationConfigurationResponse" >

<xs:complexType>

<xs:complexContent>

<xs:extension base="BasePersistResponse" />

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

# Appendix 3.6A: BIT/MIT Vehicle Specifications and Diagrams

# 2014-15 HINO 195

# 2019 Ford E450 Inspection Truck

# 2025 Ford F550 Replacement Inspection Truck

# Other Vehicles To Be Upfitted With Inspection Equipment

# 2023 Chevrolet Tahoe(s)

# 2024 Jeep Grand Cherokee Laredo(s)

# 2012-13 Chevrolet Express Van(s)

# 2016 Dodge Caravan(s)

# 2012 Chevrolet Tahoe(s)

# 2020 Dodge Durango(s)